KENWOOD TS940S INTERFACE CONTROLLER          Vers. 1.3

This program is being distributed as a freeware program for

private use only and is Copyright Protected.  You are encouraged to

distribute these files to other interested amateurs under the

following conditions;

1) All files will be transferred intact without change.

(TS940CTL.EXE, TS940CTL.DOC and READ-ME.

May contain files TS940-1.EXE and TS940-2.EXE.)

2) No financial gain may be obtained through the use, sale

or distribution of this software.

* * * * * * * * * * * *

The commands are fairly clear in the menu portion of the screen.

The following is a more explicit description;

F1 - MEMORY   -  "F1" toggles the MEMORY function on and off.  In this

mode, you are capable of writing the memory status to

a selected VFO by using "V".

F2 - SL.TUNE  -  "F2" allows you to set either the low or high side of the

IF Slope Tuning.  This function will LOCK UP the

associated control on the transceiver.  If the high side

has been engaged using this program, the low side control

will remain functional and vice versa.  Of course if both

sides have been engaged using this program, then both

controls will be locked.   You may return the controls to

normal function by using a control C (^C).  The IF tuning

will be reset to its normal settings as well.

>  F3 - M. CHNL+ -  "F3" increases the MEMORY CHANNEL by one (1).

F4 - VBT      -  "F4" engages the Variable Band Tuning in the same fashion

as F2 for the Slope Tuning.  The same control LOCK UP

applies in this function as well.

>  F5 - M. BANK+ -  "F5" increases the MEMORY BANK by one (1).  The left

digit above the frequency is the Bank selected.

F6 - PG. SCAN -  "F6" engages the Program Scan function.  The transceiver

will scan between the frequencies in Memory Channels

9 and 0 in the current Memory Bank.  Thus you have the

capabilities of 4 scan frequencies by changing the Memory

Bank.  Keys are limited to F6 and F8 in this mode.

F7 - M. SCAN  -  "F7" toggles the MEMORY SCAN off and on.  Only

F7 and F8 are functional during this function.

F8 - S. HOLD  -  "F8" will hold either the Memory Scan or Program Scan

functions.  Hitting the key again will disable the function.

Disabling the scan function will also disable it.

F9 - M. WRITE -  "F9" allows  you to write  directly to  the transceivers

memory  channels.  You  may select  either  the  current

operating status or enter a new frequency and mode without

disturbing the current settings.

F10- SEND     -  "F10"; refer to "*".

">" denotes commands where repetative commands occur with continued key press.

Cont...)              KENWOOD TS940S INTERFACE CONTROLLER      Vers. 1.3

V - VFO-B    -  "V" will toggle between VFO A and VFO B.

F - FREQUNCY -  "F" will allow you to enter a frequency directly into

either VFO A or B.  If the frequency is being entered

into the current VFO, you will be requested for the

MODE as well.  "ESC" will leave the mode unchanged.

M - MODE     -  "M" allows you to change the mode (LSB, USB, etc) in the

current VFO ONLY.

A - ANT.TUNE -  "A" turns on the Auto Antenna Tuner.  Hitting "A"

again before the timer has ended the function will

terminate this operation.

SPECIAL NOTE: The Graphics Display does not represent a true indication

of the transceiver antenna tuning condition.

* - SEND     -  "*" will key up the transmitter (PTT).  Using this key a

second time will return you to receive mode.  F10 serves

the same function.  This was to allow for right or left

handed operators to use this function while holding a

pencil in either hand. (At least on my keyboard)

R - RIT      -  "R" toggles the RIT on and off.

X - XIT      -  "X" toggles the XIT on and off.

L - F.LOCK   -  "L" toggles the F.LOCK on and off.

S - SPLIT    -  "S" toggles the SPLIT frequency operation on and off.

Q - QUIT     -  "Q" obviously terminates the program returning control

back to the transceiver if any controls have been locked

due to this programs operation.

If you have entered a command which requires information from the

operator, you may abort that operation with either the "ESC" or "ENTER"

key.  If the input expected is one key stroke, the "ESC" may be used.

The system will reset after a 10 second wait with no key press. If more

than one key stroke is expected (eg. frequency, etc), the "ENTER" key with

no input will abort the operation.  This input is not controlled by the

auto-reset.

ARROW KEYS

>  RT  - INC.  -  The right arrow key increases the VFO frequency by 10 cycles.

>  LFT - DEC.  -  The left arrow key decreases the VFO frequency by 10 cycles.

SHIFT-ARROW KEY will increase or decrease the frequency by 100 cycles.

CONTROL-ARROW KEY will increase or decrease the frequency by 1 kHz.

>  + and - keys will increase or decrease the frequency by 10 kHz.

OFF SET TUNING

>  UP  - INC.    -  The up arrow key increases the off set frequency.

>  DWN - DEC.    -  The down arrow key decreases the off set frequency.

C   - CLR.    -  Resets the off set frequency to "0.000".

During the frequency change operation, I have sacrificed screen

updating for speed.  If an arrow key is held down, the screen will not

be updated until the key is released.

Cont...)            KENWOOD TS940S INTERFACE CONTROLLER      Vers. 1.3

Requirements

This program was written on and for the IBM XT and clones.  It

requires the installation of the Kenwood Interface Kit IF-10B and a

TTL-RS232 converter such as Kenwood's IF-232C.

I might mention that I did experience considerable difficulty getting

the computer to recognize that the interface was connected.  I received an

ERROR #24 DEVICE ERROR.  I eventually shorted pins 6,8 and 20 on the

RS-232 cable at the computer end and that solved the problem.

******   WARNING  ******

I have tested this program using DoubleDos using 128K of upper

memory.  I feel you should be made aware of the following experience;

1)  By design, this program locks out the keyboard during times when

keyboard input is undesirable.  In Dbl-Dos the keyboard buffer

remains active and at points where I have stripped the buffer,

this operation does not appear to be effective in Dbl-Dos.

Using the arrow keys in repetative mode will store the keyboard

activity in the buffer. (i.e. the frequency shift will continue

long after the key has been released).

2)  Also, when you terminate the program using the "Q" command, it

appears Dbl-Dos is leaving the system in a locked up state.  I

feel the keyboard lockout, as described above, is the root of

the problem.  With a little effort (hitting different keys), I

was able to regain system control.

I have tested this program using Turbo mode (8Mhz) and normal mode

(4.77 Mhz).  It appears to be working correctly in both although the speed

is conciderably slower in the normal mode.  If anyone has any problems or

suggestions for improvement, please let me know so that I may use the

information in any future updates.  I would appreciate any feed back.  I

can be reached;

via Packet Radio      VE3COO @ VE3NUU

or by mail          Richard (Dick) Tunks  VE3COO

1220 Falgarwood Dr.

Oakville, Ontario

L 6 H   2 L 3

(C) 1989-VE3COO                                               March 19, 1989