AMTOR/PACKET Link (APLINK) Version 3.81, June 10, 1989

Copyright (c) 1989 - Victor D. Poor, W5SMM

[AUTOFORWARDING SUPPLEMENT TO APLINK.DOC]

AMTOR SEMI-AUTOMATIC FORWARDING

Beginning with Version 3.7, APLINK now includes an Amtor Semi-

Automatic message forwarding feature.  This feature permits a sysop

to forward one or more messages to another APLINK system without

passing the traffic thru the packet network.

It is not expected that sysops who have v.h.f. access to the

National Packet Network in the U.S. would use this feature for

domestic traffic.  This feature is more suited to passing traffic

over long h.f. hauls where packet has not proven successful or is

not available.  In other words: do not abuse this feature - use

packet if you have it.

This feature is called semi-automatic because, unlike packet, an

operator must initiate the transfer.  It is simply not socially

acceptable to begin transmission of an amtor signal without

listening first to make sure the frequency is not already in use.

Full-automatic would require blind transmissions - a sure way to

alienate your fellow operators.

HOW TO USE IT

First, set up a routing file named AMTFWD.A$ (more on this below).

Then, while running APLINK, link with a another APLINK station that

you wish to move traffic to or receive traffic from.  Press F3 F8.

It will take it from there - move all the available traffic both

ways and wrap up the link.  (Traffic may be sent to any APLINK

station in this way but stations running Versions 3.5X or lower

will not automatically "reverse forward" traffic.)

That's all there is to it.  But first...

SETTING UP THE ROUTING FILE

Using your Norton Editor, create a file named AMTFWD.A$.  In that

file there will be one or more lines of text, each line will have

either two or three call letters (or packet-like routing

designators).  On each line the first call is the 'target' call and

will match either the TO field or the AT field in the message

headers.  The next call on that line is the APLINK station the

message should be forwarded to.  A third optional call is an

alternate APLINK station that the message may also be forwarded to.

The 'target' call may have a trailing 'wild card', the * character.

It will match charaters up to the * symbol and if it makes it that

far will consider it a good match, BUT, only when checking the AT

field.  A wild card entry will not match the TO field of a message.

End the file with the word END.  Don't leave any blank lines or

extraneous text in the file other than extra white space between

calls to make the file look orderly.  You may create a file with

no calls in it but you still need one line with the word END on it.

Here is what a file at WA8DRZ might look like:

W5JSO     AH6D

96*       AH6D

K3NIO     HL9TH     AH6D

HL9*      HL9TG     AH6D

VK*       VK2AGE    AH6D

ZL*       VK2AGE    AH6D

TI*       TI2ALG

END

Here is how this file would work.  Any messages at WA8DRZ for W5JSO

will be sent out to AH6D in Hawaii.  Any messages with a zip code

in the AT field beginning with 96 will go to AH6D, also.  Any

messages for K3NIO will go to HL9TG in Camp Humphreys, Korea or to

AH6D, whichever station links up and initiates a transfer first.

All HL9 prefix traffic will go to either HL9TG or AH6D, etc, etc.

How about traffic that does not match any of the above?  It goes

out on packet just like it always did (assuming there is something

in the AT field) or waits to be picked up by the addressee.

If the call of the station to which you are linked appears in

either the TO or AT field it will be unconditionally forwarded to

that station.  (No sense fooling around forwarding through a third

party if you have the bird in hand.)

SOME THINGS TO WORRY ABOUT

IMPORTANT!  CAUTION!  Be careful to co-ordinate with your fellow

APLINK sysops.  There is no protection yet in the system to stop

an endless round robin message where station A forwards to station

B who forwards to station C who forwards to station A.

As long as no AMTFWD.A$ file exists, APLINK is the way it always

was.  Once you set up an AMTFWD.A$ file make sure that you or the

station you are forwarding to moves the traffic in a timely manner

(unless the file only contains an END).  If a message in your

system is set for amtor routing it will not move until the link is

made and the autoforwarding has started.  IT WILL NOT GO OUT ON THE

PACKET PORT NO MATTER HOW LONG IT SITS THERE.

It is possible to duplicate a message in the process of a transfer.

If a message was received but the acknowledgment was mutilated it

will be sent again and appear as a new message to the receiving

system.  For the sake of simplicity I have not added protocol to

prevent this from happening but then it is not very serious fault

either.

If the link fails or the system retrys the same message three times

without a successful completion, APLINK will close the link and

return to standby MBO mode.  Further forwarding will not occur

until manually restarted.

PROGRAMMING NOTE

Hierarchical routing is not yet implemented in APLINK but will be

eventually.