PK232COM VERSION 1.45  PAGE 1

PK232COM Version 1.45 (c) 1987 by G3ZCZ

(C) Copyright 1987 - All Rights Reserved

By Joe Kasser G3ZCZ

POB 3419,

Silver Spring,

Md., 20901, USA.

A Personal Packet Terminal Program for the TNC1, TNC2 and most of

all for the PK232.

PK232COM is written and supported by Joe Kasser, G3ZCZ.

The program is distributed as a Shareware product. You may freely

copy  and  share the product for non commercial  use,  with  your

friends,  associates  and other radio hams.  If you decide to use

the product, you are asked to become a registered user by sending

a QSL card and a contribution  to the author (suggested amount of

$35.00 or equivalent in foreign currency.

The  contribution covers the cost of distribution of the  updated

disk,  and  supports  the continual development of "Software  for

Amateur  Radio".   Upon receipt of your  contribution,  you  will

receive  one  free update disk CUSTOMISED with YOUR CALLSIGN  (if

you write the type of TNC on the QSL card.), mail (electronic and

regular)  support,  and notice of further releases.   The  update

disk  will contain any new versions of the software on this  disk

plus  any  new  software,  accompanying the  series  of  articles

currently  being written on the applications of  data  processing

concepts of microcomputers to amateur radio.

This  product may not be sold or distributed with another product

without the express written permission of Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ  will only support unmodified copies of  this

software.   Your comments and suggestions for changes are however

welcome.   If  you  are  the first to suggest a  change  that  is

implemented,  you  will be sent a complimentary copy of the  disk

with the change incorporated.

Potential Commercial Users please contact Joe Kasser directly for

details of Site licensing.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 2

CONTENTS

-------------

1.0  INTRODUCTION

1.1  PROGRAM HIGHLIGHTS

1.1.1     MORSE/BAUDOT/AMTOR/ASCII

1.1.2     PACKET RADIO

2.0  CRT DISPLAY

2.1  Status Display

2.2  Incoming Text Display

2.3  Outgoing Text Display

2.4  Command/Status Display.

2.5  Alarm Tones.

3.0  Function keys.

3.1  Function Keys

3.1.1    Capture to Disk Toggle

3.1.2    Transmit a file

3.1.3    CAllsign Sequence.

3.1.4    Brag tape

3.1.5    Call CQ

3.1.6    Change Communications Modes

3.1.7    Call/Connect

3.1.8    Disconnect or Receive

3.1.9    Command Mode

3.1.10   Transmit Mode

3.2  Alt Function keys

3.2.1    Exit or return to DOS.

3.2.2    Initialize the PK-232.

3.2.3    Connect Status

3.2.4    Set Terminal Mode

3.2.4.1   SOLO

3.2.4.2   TRAFFIC

3.2.4.3   CQ/BEACON

3.2.4.4   READ THE MAIL

3.2.4.5   Multiple Connect Modes.

3.2.4.5.1 The Individual  Multi

Connect Mode.

3.2.4.5.2  The Conference Multi

Connect Mode.

3.2.4.5.3  Entering      the

conference mode

3.2.4.5.4  Connecting  in  the

multi user mode

3.2.4.6   FEC

3.2.4.7   ALISTEN

3.2.4.8   ACHG

3.2.4.9   ARQ

3.2.4.10  Force LETTERS Case

3.2.4.11  Override Mailbox/Beacon.

3.2.4.12  Turn on Mailbox/Beacon.

3.2.5    Monitor Channel Activity

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 3

3.2.6    Change Packet Connect / Contest / CQ text

Message

3.2.6.1 Change Packet Connect Message

3.2.6.2 Change Contest Message

3.2.6.3 Change CQ text Message

3.2.7    Enter Call Sign

3.2.8    Not used.

3.2.9    Activate Transmit Buffer

3.2.10   Help

3.3  Shift Function Keys

3.3.1     Dir

3.3.2     Rename

3.3.3     List of Outgoing Messages to be sent.

3.3.4     Delete

3.3.5     List of messages that have been sent.

3.3.6     Mail Beacon Activate

3.3.7     Retry a call/connect request

3.3.8     Not used.

3.3.9     Connected Indicator Reset.

3.3.10    OVER function.

3.4  Control Function Keys

3.4.1     Day Flag

3.4.2     CWID

3.4.3     Message Uploading

3.4.3.1   Leaving a Note in your system.

3.4.3.2   Leaving  a message on an  other

PK232COM system [QSP].

3.4.3.3   Uploading  messages to  another

PK232COM system.

3.4.4     Show last few log entries.

3.4.5     Automatic CQ Sender

3.4.6     SELCAL

3.4.7     Beacon  Activate  Toggle/CQ  Wait  Delay

Adjust.

3.4.7.1   Beacon Mode Toggle/On-Off

3.4.7.2   CQ Delay Time Adjust

3.4.8     Not used.

3.4.9     Transmit Buffer Clear

3.4.10    Log QSO

3.5  Shift/Baudrate Control keys

3.5.1     The RIGHT ARROW Key (Increase Baudrate).

3.5.2     The LEFT ARROW Key (Decrease Baudrate).

3.5.3     The UP ARROW Key (Change shift).

3.5.4     The DELETE Key (toggle USOS).

3.6  Additional keys

3.6.1     The HOME Key.

3.6.2     The DOWN ARROW key.

3.6.3     The Alt A key (Alert Call Set/Toggle).

3.6.4     The Alt B key (BAUDOT/ASCII  baud  Rate

Set Key).

3.6.5     The Alt C key (Connect/Call).

3.6.6     The Alt D key (Display Inhibit).

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 4

3.6.7     The Alt F key (Digipeat Monitor Set/Toggle).

3.6.8     The Alt P key (Printer Toggle).

3.6.9     The Alt R key (Message Relay toggle).

3.6.10    The Alt S key (Sound Set/Toggle).

3.6.11    The Alt T key (Target Call Set/Toggle).

3.6.12    The Alt X key (Exit or return to DOS).

3.6.13    The PgUp key  (Scrolls back Incoming).

3.6.14    The Control Z key (AMTOR +?).

4.0  PK232COM disk files

4.1  PK232COM.COM

4.2  PK232COM.RUN

4.3  PK232COM.SYS

4.3.1     The callsign.

4.3.2     The AMTOR SELCAL.

4.3.3     The CQ Text.

4.3.4     The fourth and fifth lines (Band and Power).

4.3.5     The CQ Delay time.

4.3.6     The Message Relay Capability (QSP).

4.3.7     The Communications Port.

4.3.8     The TNC type.

4.3.9     The Terminal Baud Rate.

4.3.10    The QTC_Snatch flag

4.3.11    The remaining lines.

4.4  PK232COM.LOG

4.5  PK232COM.DIR

4.6  CONFIG.SYS

4.7  *.MBX

5.0  Initialization

6.0  Running the Program

7.0  Mailbox features (AMTOR/Packet Modes Only)

7.1  Answering Machine.

7.2  Mail Beacon (Annunciator).

7.3  How to leave a message

7.4  Remote File Downloading (Packet Mode Only).

7.5  Remote Beacon shutdown.

APPENDIX 0. UPDATES/REVISION HISTORY.

APPENDIX 1. BUGS and anomalies.

A1.1 Command/Converse mode anomalies.

A1.2 Buffer messages in AMTOR Beacon CQ mode.

A1.3 FEC +? responses in AMTOR Beacon CQ mode.

A1.4 PK232COM versions 1.00 to 1.39.

A1.5 Using a CGA card with a monochrome monitor.

A1.6 The Packet Conference Mode.

A1.7 Packet Conference Mode disconnects.

A1.8 Slow Screen Updating.

A1.9 Strange Events.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 5

APPENDIX 2. MESSAGES IN A LOCAL AREA NETWORK

A2.1 LAN Protocol (G3ZCZ Version).

A2.2 Message Format.

A2.3 File downloading.

A2.4 Path Determination to a Dx station.

APPENDIX 3  BASIC NC/L DICTIONARY

A3.1  :QBM: To download a file.

A3.2  :QMH: To request a call monitored list ('MH').

A3.3  :QSM: To request a repeat of a message.

A3.4  :QSP: To leave a message for another station.

A3.5  :QRT: To  shut  down  a  packet  mode  beacon

station.

A3.6  :QRU: To upload messages.

A3.7  :QNO: Error or function not present/active.

A3.8  :QJG: No more messages pending.

A3.9  :QRV: Ready for message.

A3.10 :QSL: Confirm receipt of message.

A3.11 :QTC: Messsage list.

A3.12 :QYU: YAPP format file upload.

A3.13 :QYD: YAPP format file download.

APPENDIX 4. USING OTHER TNC'S.

4.1 TNC2 (MFJ 1270)

A4.1.1     Default Commands.

A4.1.2     Program differences.

4.2 TNC1 (HD-4040)

A4.2.1     Default Commands.

A4.2.2     Program differences.

APPENDIX 5. NOTES.

APPENDIX 6. REGISTRATION FORM.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 6

1.0  INTRODUCTION

PK232COM  is a Personal Digital Communications  Terminal  Program

for  a Packet Radio TNC.   while it takes advantage of the  extra

features  of the PK-232 which is made by  Advanced  Electronics

Applications Inc.  MANY OF THE PACKET RADIO FEATURES WILL WORK ON

A TNC1 AND MOST OF THESE FEATURES SHOULD WORK ON ANY TNC2.  It is

designed  to allow anyone to use and get the most out of the  PK-

232  for  Morse,  ASCII,  BAUDOT,  AMTOR as well as Packet  Radio

communications  without having to keep the PK-232 or  other  TNC2

manual handy.

PK232COM  is  a sophisticated program.   In its  basic  state  it

allows  you to use the TNC in an optimal manner.   It  configures

the  TNC for you to maximise the communications efficiency in the

communications  mode of your choice.   That means,  for  example,

when  working  Packet on HF you need to program the  PK232  para-

meters  to  different values than you would use on  VHF  to  make

maximum  use  of  the mode.   One significant difference  is  the

length of the packet itself,  for the longer it is,  the  greater

the  probability of QRM destroying it.  This program will  adjust

the packet parameters for you.

Since  the  computer is now involved,  other features  have  been

added  to  simplify operation,  and several  features  have  been

automated.   All  these  operations are performed using  function

keys as documented below.   It will take a while to learn how  to

use  this program in a manner that suits you,  read this document

and  have  fun,  after all,  isn't that one of  the  purposes  of

Amateur Radio.

This  documentation  file is not a manual.  It is  a  preliminary

document.   It  is written to remind you of what the program  can

do.   If  there is enough interest (registrations) I will  expand

it.   It  is  not  designed  to explain  the  theory  behind  the

different communications modes.   Read the PK232 Manual for  more

information.    This   program   is  designed  to  optimize   the

configuration  of  the PK232 in each communications mode  and  to

provide some smart terminal features.

1.1  PROGRAM HIGHLIGHTS

1.1.1     MORSE/BAUDOT/AMTOR/ASCII

Automatic  CQ caller.   Will call CQ repetetively and  signal

you when a reply is received.

SELCAL.

Automatic capture to disk of all ingoing and outgoing text of

any QSO.

AMTOR/PACKET  Selective  answering machine and MAILBOX  using

NC/L command dialog.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 7

1.1.2     PACKET RADIO

Automatic disk recording of all packet radio connects.  Auto-

matic indication of Packet connects.

AMTOR/Packet  Local  Area  Network (LAN)  message  store  and

forward.

Capable of automatic connect attempts to download a QTC  from

an other station in the LAN.

Digipeat monitoring and capture.

Alert signal to let you know when a predetermined call  shows

up in a packet header on frequency.

Conference mode in multi- connect situations.

Path determination to Dx station via :QMH:.

Indicator  that a specific station designated as the 'target'

call   connected   in  Packet  mode,   or  linked  to   AMTOR

Beacon/Mailbox while you were away.

Automatic  NET/ROM  path set up from  PK232COM.DIR  call/path

directory file.

AMTOR/PACKET  Automatic  Answering  Machine  capability  with

display of message queue.

Automatic  Beacon Mode CQ caller in Packet and AMTOR communi-

cations modes.

Will  call  CQ repetitively and either work the  connect  and

keep  going  after disconnect or signal you when a  reply  is

received.

Automatic logbook entries for Packet and Mailbox/Beacon  mode

AMTOR  Connects,  semi  automatic logbook entries  for  other

modes.

Log  file is compatible to that of the G3ZCZ CONTEST  program

can  be processed by the DBASE  Logbook Package of PC-HAM  by

G3ZCZ  for  indexed  listings,  tracking of  DXCC  and  other

AWARDS, etc.

Contest  operation,  sends standard mesage and  automatically

increments QSO count.

Automatic  optimized  configuration of the TNC for each  com-

munications mode.

FUNCTION KEY 'OVER' FEATURE

Morse,  ASCII,  BAUDOT communication modes are HALF DUPLEX.   One

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 8

station  is transmitting at any time.   You thus have to use  the

Transmit/Converse  and Receive/Disconnect function keys  to  turn

the transmitter on and off respectively.

AMTOR  is also HALF DUPLEX but has some of the attributes of full

duplex.   In  this  mode  you  control  who  is  talking  through

software,  only one station can send data at a time,  but you can

type  characters in  to the buffer and they will  be  transmitted

automatically  when the other station turns the QSO over to  you.

Once   you  have  linked  with  someone,   you  do  not  use  the

Receive/Disconnect function UNTIL THE QSO IS OVER.  Both stations

are communicating automatically thanks to the TNC.   If you  want

the  other operator to respond type the +?  key sequence (or  use

shift function key 10).

Packet  Radio communications  on the other hand is a full  duplex

mode  of communications.   Once you have connected with  someone,

you  do not use the Receive/Disconnect function UNTIL THE QSO  IS

OVER.   Both  stations are communicating automatically thanks  to

the  TNC.   If you want the other operator to respond type the  >

key followed by the 'Enter' key,  better yet,  use shift function

key 10.

Note  that  Morse,  AMTOR,  ASCII and BAUDOT are  character  mode

communications  in  that each character you type is sent  as  you

type them.   In Packet Communications, the TNC stores them up and

sends out a burst or packet when you type the 'Enter' key or  you

type enough characters to fill up a packet.   Studies have  shown

that  the human attention span when sitting at the keyboard is of

the  order  of 2 seconds.  If you are in a keyboard  to  keyboard

packet radio connect,  remember this and don't forget to use  the

'Enter'  frequently,  especially  before you stop to think  about

what to send next.

Any  Terminal  Node  Controller  (TNC)  is  a  complex  piece  of

equipment.   It can operate in a Command Mode,  in which you tell

it to do something,  or in a Converse Mode in which you are using

it  to talk to other stations.   Many people confuse the two when

first  getting on Packet.   If you monitor the channels you  will

recognize command mode TNC instructions on the air,  and when you

use  the  TNC you will receive the "?error" reply when  you  type

something  thinking  that you are in the Converse  Mode  but  are

really in the Command Mode.   PK232COM is designed to make Packet

operation simple using high level (Function key) commands.

This Version has the following functions built in to it in a menu

driven format.

Morse Code  20 wpm (CW).

BAUDOT  with default of 45 WPM narrow shift (LSB).   You may  set

other  baud rates directly,  or use the left and right arrow keys

to change the baud rate (see below).

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 9

ASCII  with default 110 BAUDS narrow shift (LSB).   You  may  set

other  baud rates directly,  or use the left and right arrow keys

to change the baud rate (see below).

Packet VHF or HF baud rates and shifts (LSB).

AMTOR 100 BAUDS narrow shift (LSB).

All  communications between the computer and the TNC normally use

COMM1  and operate in the 8 BIT NO PARITY and 1 stop  bit  RS-232

Serial Port configuration.  You CAN SEND FOREIGN LANGUAGE CHARAC-

TERS  FROM  THE  KEYBOARD OR FILES.   If you want  to  limit  the

character  set  to the 7 bit ASCII set,  set the  TNC  parameters

accordingly.   The baud rate defaults to 1200, but you may set it

to 2400, 4800 or even 9600 if your CPU clock is fast enough.

Access  to  the  TNC Command mode is provided in  case  the  user

wishes  to  override  any  of these defaults.

IF YOU USE THIS PROGRAM, YOU STILL MUST READ THE TNC MANUAL.

Note   that   AMTOR   and  PACKET  are  full  duplex   modes   of

communications.   When you connect with someone, there is no need

to use the transmit and receive function keys.   In fact you will

have  problems  if you do.   Do not disconnect until the  QSO  is

over.

WARNING  ....   THIS PROGRAM OPERATES IN AN OPEN LOOP  MODE.   IT

REMEMBERS WHAT INSTRUCTIONS OR COMMANDS IT GAVE THE TNC.   IF FOR

SOME REASON,  THE TNC IS NOT SYNCHRONIZED TO THE PROGRAM YOU WILL

NOT GET CORRECT DISPLAYS ON THE SCREEN.

2.0  CRT DISPLAY

The CRT display contains four areas as follows.

-------------------------------------------------

|             STATUS DISPLAY                    |

-------------------------------------------------

|                                               |

|                                               |

|             INCOMING TEXT DISPLAY             |

|                                               |

|                                               |

|                                               |

-------------------------------------------------

|             OUTGOING TEXT DISPLAY             |

|        ____________________________           |

|       |____________________________|          |

|                                               |

-------------------------------------------------

2.1  Status Display

A  one  line display at the top of the  screen  showing  the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 10

status of the communications system.

The  left side of the screen normally displays the  "capture

to  disk"  or  logging status,  the  center  of  the  screen

displays  status  information  depending  on  which  of  the

operational modes is active.

2.2  Incoming Text Display

The  center section of the screen displays data coming  back

from the TNC.   This consists of data received via the radio

communications link,  TNC status information and transmitted

data echoed back by the TNC.

2.3  Outgoing Text Display

The  lower area of the screen shows data being sent  to  the

TNC.   This consists of command data from the program to the

TNC, text typed by the user and files being transmitted.

2.4  Command/Status Display.

This  screen  located above the Outgoing Text Display  shows

the  command dialog between the program (PK232COM)  and  the

TNC.  If  you  want to see the dialog,  set the  TNC  'ECHO'

parameter to 'ON'.  The default should be 'OFF'.

2.5  Alarm Tones.

PK232COM  sounds a different number of alarm tones or chimes

according to the following events;

1    Digipeat Detected.

File Alarm Error.

2    PK232COM :QTC:  message detected,  automatic QTC_Snatch

connect request in progress.

Tones toggled back on.

3    Alert Call heard in packet header.

5    Somebody connected.

The  alarm  tones  may be toggled ON/OFF  by  means  of  the

Alternate S key.

3.0  Function keys.

PK232COM  can be used as a simple Terminal Interface Program with

the  10  main function keys.   The control,  alternate and  shift

function keys provide advanced features.  You should first become

familiar with the function keys described in Section 3.1 and then

expand  your  use  of the program to  incorporate  the  remaining

expanded function keys.  Don't try to learn them all at once.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 11

The operation of each of the function keys are explained below.

3.1  Function Keys

3.1.1    Capture to Disk Toggle

This  key toggles the "capture to disk"  function.

When  the  function is active,  all incoming  text

(including 'Echo as sent' outgoing text) is stored

on  a  disk  (floppy or hard)  in  a  file  called

'PK232COM.RUN'.   If  this file does not exist the

first time that PK232COM is loaded, it is created.

From that time on,  successive activations  append

new data to the contents of the file.

Each  time the function is activated  a  datestamp

(or timetag) will be written to the file.

When  a  Packet station connects the  'capture  to

disk'  function  will be  activated  automatically

even  if  you  have  not  set  it  active.  Packet

connects  will thus be recorded in their  entirety

automatically  unless you turn off the 'Capture to

disk' during the course of the QSO.  The  function

will  be deactivated at disconnect time unless you

have toggled it on.

You  may  later edit the file at your  convenience

with your favorite word processor.

PK232COM  will automatically close the log capture

file when you terminate the program and return  to

DOS.

3.1.2    Transmit a file

This function allows you to transmit an ASCII file

via  the PK-232.   You may abort the  transmission

sequence  at  any  time by depressing any  of  the

function keys,  the keys on or associated with the

numerical pad, or the ESC key.

During the file transfer,  the status window  will

show the different states of the transmission.  It

begins  with  a signal that the message  is  being

sent.   If  the  text  is long and fills  the  TNC

buffer,  you  will see a message 'QRX  TNC  BUFFER

FULL'  possibly followed by  'TNC NOT RESPONDING'.

This  latter message will seem to blink  periodic-

ally.   When  the TNC signals that it is ready  to

accept  more  text,   the  message  sequence  will

resume.

In the Packet Communications mode,  when the  file

has been loaded into the TNC the computer will add

the ^Z character to the outgoing text to signal to

the  recipient that the file transmission is over.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 12

The  computer  will  signal you  (using  the  bell

character,  when the file has been loaded into the

TNC,   Note  that is not the same as the time that

the TNC has transmitted the file,  because the TNC

incorporates a character buffer.

3.1.3   Call Sign Sequence.

This  function  lets  you transmit  the  following

sequence <other call) DE <your call>.  For example

if  you are 4X6AA and you are working  G3ZCZ  then

this function would transmit the sequence G3ZCZ de

4X6AA.    You   use   this  at  the  start  of   a

transmission in any mode except Packet.

3.1.4    Brag tape

This  function  allows you to transmit a  standard

text file historically known as a "Brag Tape"  via

the  TNC.   The  brag  tape file  must  be  called

PK232COM.TXT,  and  should be created with a  word

processor in the non-document mode.   Transmission

of  the Brag Tape may aborted in a similar  manner

to that of a regular file (see above).

3.1.5    Call CQ

You  use  this function key to call  CQ.   In  the

Packet Communications mode,  the TNC will transmit

a line of text along with the CQ packet.  The line

of  text  is set up in the PK232COM.SYS file  des-

cribed in section 4.

In the AMTOR communications mode, your SELCAL will

be  transmitted along with the CQ call in the  FEC

mode.

3.1.6    Change Communications Modes

This   function   allows   you   to   change   the

communications  mode  of  the  PK-232.   Allowable

modes are Morse code, BAUDOT, AMTOR standby, AMTOR

Alisten,  ASCII, Packet and Signal.   In the Packet

mode  you  can  choose VHF or HF  operation  in  a

single step by entering the letters V or H.

When  you  select the AMTOR standby mode  you  are

placed  into the 'standby receive' mode.   If  you

want to read the mail on an existing AMTOR QSO  in

progress,  select  the AMTOR Alisten or  chirpcopy

mode.   If you are in AMTOR standby mode, you will

not be able to see signals of the chirpcopy mode.

Use  the 'G' option to select the  'SIGNAL'  mode.

Read   the   PK232  documentation   carefully   to

understand  how this mode works.   Once the  PK232

has identified the digital signal it is tuned  to,

use the same function key a second time to  select

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 13

the relevant mode and set the baud rates and shift

to display the data.

3.1.7    Call/Connect

You  use this function to call a station heard  on

the  band,  or to attempt a connect  with  another

packet station.  You may also use it to call 'QRZ'

or even 'TEST'.

In  AMTOR the call will be sent in the ARQ mode to

the SELCAL of the desired station.

In the Packet Communications mode you can create a

directory  file  and  keep  a list  of  names  and

callsigns.   The computer will look up a name  and

call the station by its callsign.   For example if

you  put

Joe G3ZCZ

as a line in the file,  when you tell the computer

to  connect  to  Joe,  it will try to  connect  to

G3ZCZ.   If you enter

4X4HF  4X4HF V 4X1AA 4Z4ZB 4X4IL

you  only  need  to type '4X4HF'  to  set  up  the

correct  connect  path.   See section  4.5   which

describes    the  PK232COM.DIR  file  for  further

information.

You  can also use this function key to  automate a

NET/ROM  path connection.   In the manual  NET/ROM

path  setting up mode,  you would first connect to

your local node,  then when you are connected with

it,  connect  to the next node,  and so on through

any  intermediate  nodes  within  the  whole  path

before  you finally connect with your  destination

station.   You may have digipeaters in any of  the

paths.

For example if you wish to connect to K1HTV who is

in  range  of  the  SSCT  node,  and  you  are  in

Washington DC,  you would first have to connect to

your   local   NET/ROM  node   (ELK),   then   the

intermediate  nodes,  SNJ3,  NNJ,  WMA before  you

could try to connect with K1HTV.

If  you put the following line in the PK232COM.DIR

file

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

the NET/ROM sequence will be automatic.

PK232COM will issue a 'disconnect' command to  the

TNC  in  the  event the  NET/ROM  connect  attempt

fails.   This function is designed for known paths

not for general path determination.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 14

In the Packet communications mode,  if you want to

loop  back  through  someone else  enter  the  '/'

character  followed by the call of the station you

want  to  loop back through.   For  example  if  I

(G3ZCZ)  want to loop back through 4X6AA,  I  just

need to enter /4X6AA as the call to connect to and

the  computer will tell the TNC to try to  connect

to G3ZCZ via 4X6AA.

In  a  multi  connect situation the  program  will

automatically select a free IO channel to try  the

connect on.

3.1.8    Disconnect or Receive

This  function  key  returns the  station  to  the

receive communications (PK232 command)  mode.   It

may  also  be  used  to  abort  the  automatic  CQ

sequence.   If  you use it to abort a CQ  sequence

while the transmission is in progress,  flush  the

PK232  transmit buffer using Control function  key

9.

In  a  multi  connect situation the  program  will

prompt   you  to  tell  it  which  IO  channel  to

disconnect.

3.1.9    Command Mode

This  function puts the TNC into the command mode.

At this time,  the program is transparent and acts

as  a dumb terminal (except for  placing  incoming

and  outgoing text in the relevant windows).   you

use  this  mode  for overriding  any  of  the  TNC

default conditions.   (SEE I TOLD YOU THAT YOU HAD

TO READ THE TNC MANUAL).

3.1.10   Transmit Mode

This  function places the TNC into the transmit or

converse mode.   The transmitter is turned on  and

the  contents of the transmit buffer is  transmit-

ted.   In the non packet communications modes, the

outgoing  characters will be echoed back from  the

TNC  and  displayed in the INCOMING window on  the

CRT as they are transmitted on the air.

3.2  Alt Function keys

3.2.1    Exit or return to DOS.

Exits  the program and returns you to  DOS.   When

you exit the program,  the TNC is put into the VHF

default  mode in which it rejects calls  from  any

other  stations.   (Read about the 'CFROM' command

in  the  TNC manual).   you can thus if  you  wish

leave  it powered,  and if you also leave the  VHF

radio  powered  up,   you  will  be  active  as  a

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 15

digipeater,  but at the same time anyone trying to

connect with you will receive a busy signal.   The

MBX function (Reading the Mail) is also cleared at

this time.

3.2.2    Initialize the PK-232.

Sets up the defaults for the PK-232 from the  file

PK232COM.SYS.   If  you have put batteries in  the

PK232,  you should normally only have to run  this

function  when  you  first power up the  PK232  or

change the batteries.

It will also set the date and time within the  TNC

from the computer clock,  and,  when it has loaded

the configuration file, will propmt you to set the

PK232  Terminal  mode.   If you are using a  TNC2,

select 'V'.

3.2.3    Connect Status

When  you  are in the Packet communications  mode,

this  function  lets  you see  the  state  of  the

connection  in the Command Window.   It will  show

you  if  there are any unacknowledged  packets  en

route  to  the other station.   If you  have  been

sending text, it is advisable to use this function

before you disconnect, just to ensure that all the

data  has  been received and acknowledged  by  the

other party.   You may use this function in either

the TNC Command or the TNC Converse mode.   If you

use  it when in the Converse mode,  it will  leave

you  in  the  command mode,  so  remember  to  use

function key 10 to return to the converse mode.

In a multi connect situation, it will show you the

status  on  all the IO channels (It uses  the  'C'

TNC  command in the single user mode and the  'CS'

TNC command in a multi user situation).

You may also use this function to reset the status

window  display after a relink or any other  event

which was not reflected in the status window.

3.2.4    Set Terminal Mode

This   key   allows  you  to  set   the   terminal

configuration mode.  In Packet communications many

stations  can  use the channel at the  same  time.

Refer  to the "MONITOR' and 'MBX' commands in  the

PK232 manual.  These modes give you some degree of

control  as  to  how the  packets  from  different

stations  are  displayed on ypur screen.

In  the PACKET Communications mode,  the  function

works in the following manner.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 16

You can set four Packet terminal modes as follows.

3.2.4.1   SOLO

In  this  mode,  you will only  see  messages

addressed to you.  You will only get messages

from  people  who  connect  to   you.   (This

corresponds to 'MONITOR 0').

3.2.4.2   TRAFFIC

In this mode you will see most of the traffic

on  channel.   you can use this mode to check

that the TNC is working. (This corresponds to

'MONITOR  4' or 'MONITOR ON').

3.2.4.3   CQ/BEACON

In  This  mode,  you will see CQ  and  BEACON

packets on the channel.  (This corresponds to

'MONITOR  1').  This  mode only works on  the

PK232.

3.2.4.4   READ THE MAIL

You  can  read  the mail on a  BBS  or  other

station and get bulletins without  connecting

to that station yourself.   You use this mode

to do so.   you will be prompted for the call

whose packets you wish to monitor.   Remember

to  enter the call of the station sending the

packets,  not  the call of the one  receiving

them.   (This corresponds to 'MONITOR 0'  and

'MBX' callsign).  This mode only works on the

PK232.

You  can  also use this function  to  monitor

both sides (read the mail) on a QSO on a busy

packet channel.

You  may  want  to  turn  the  Digipeat  Data

Display mode off when reading the mail.

The  Packet  Terminal  mode of operation  will  be

shown  in  the  Status window at the  top  of  the

screen.

If  you  are reading the mail on  a  station,  the

callsign of that station will be shown next to the

word "MAIL".

If  you  are connected to someone,  the call  (and

path) will be shown prefixed by the "*>" sequence.

3.2.4.5 Multiple Connect Modes.

You  also use this function to go into and out  of

the multiple connect mode.   The default condition

is one connect at a time.   Should you opt for the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 17

multiple connect mode, make sure you know what you

are doing

When  you  enter  a  multiple  connect  mode,  the

program will change the stream change character or

the  CHSWITCH character (in the case of the  PK232

from  a default $00) to the  '\'  (backslash).  At

this  time  don't transmit raw directory files  in

or you will see an error message.   You don't have

to  be  in the command mode to  use  the  CHSWITCH

character.   Read  all  about CHSWITCH in the  TNC

manual.   When you return to the single user  mode

the program will reset the stream character to the

default $00.

When  you  are  in a  multiple  connect  mode,  an

asterisk  will be shown in the status window  next

to the word 'PACKET'.

PK232COM  supports  two  multiple  connect  modes,

individual and conference.

An extract of an actual 'off the air' CRT  display

(showing  received text only) in a multi user mode

is  shown  below.   Note  each  line  of  text  as

received  by the PK232 is prefixed (by the  PK232)

with  the  IO channel number and callsign  of  the

station its connected to .

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP....>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO  CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

0:4X4IC:

2: *** DISCONNECTED: 4Z4ZB

3.2.4.5.1  The Individual Multi  Connect

Mode.

This is the normal Multi Connect Mode as

described in the TNC manual.   Here  you

are  connected to up to 10 stations  and

will  send different traffic to each  of

them.    Each  time  you  wish  to  send

something  to a particular station,  you

must  select the IO channel the  station

is connected on before typeing the  text

or sending the file.

You select the channel by typing the '\'

character followed by (in the PK232) the

channel number.   For example if you are

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 18

connected  to G8BTB on IO channel 0  and

to  G3ZCZ on IO channel 1,  then to talk

to G8BTB you would type '\0' before  any

text, and similarly to send something to

G3ZCZ  you would type a '\1' before  the

text  you're going to send me.   If  you

are  not  using a PK232,  read  the  TNC

manual  to  determine if the IO  streams

are identified by numbers or by letters.

The  TNC2  standard  configuration  uses

letters.

3.2.4.5.2  The Conference Multi  Connect

Mode.

In  the  conference mode  on  the  other

hand,  everything  that you type at  the

keyboard  is transmitted to each station

that  you are connected with.   Thus  if

you are linked to two stations each line

will be packeted twice by the TNC.   You

don't  have to worry about  sending  the

wrong thing to the wrong person, as they

will all get the stuff.

In the conference mode, as the same text

is transmitted on all IO  channels,  the

throughput  will thus be  reduced,  slow

down  your  typing  speed or  use  short

packets.   Should you use function  keys

alt  3 or 5 to check the link status  or

call monitored list, you may not be able

to  get  back into the converse mode  if

you  have too many packets  outstanding.

If that happens,  wait a while and  then

try   to  set  the  converse  mode  with

function key 10.  You may have to try it

a few times,  depending on how good your

links are.

DON'T   TRY  TO  SEND  A  FILE  IN   THE

CONFERENCE  MODE,  ITS NOT DESIGNED  FOR

IT. KEEP THE PACKETS SHORT.

3.2.4.5.3 Entering the conference mode.

You enter this mode by selecting the '0'

option  in the terminal mode select menu

(Function key Alt 4).  You terminate the

mode by selecting the '1' option.

3.2.4.5.4  Connecting in the multi  user

mode.

You  connect in the usual  manner  using

function  key  7.   You MUST WAIT FOR  A

CONNECTION TO BE ESTABLISHED OR TIME OUT

before attempting to connect to the next

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 19

station.  If you do not, you will get an

error message back from the TNC.

The conference mode will only work in the PK232 if

DCDCONN  is  set to ON.   So if you  are  using  a

PK232,  PK232COM will do it for you when you enter

the  conference  terminal mode in case you  forgot

or/and  haven't  set  the  PK232COM.SYS  file   to

include  the line 'DCD ON',  At the same time  set

PK232COM.SYS  default states to show the calls  of

other  stations on each incoming line in the  muti

connect modes.  (Read the TNC manual for how to do

this).

The conference mode will only work in the TNC2  if

DCDCONN  is jumpered so that Pin 8 of  the  RS-232

Interface   changes  state  when  you  connect  to

someone else.   If you get error messages and  see

the  characters  '\J'  on your  screen,  then  the

jumper probably isn't set.

The conference mode does not work on the TNC1.

In the conference mode, if any one of the stations

disconnects,  the  TNC will return to the  command

mode,  and  you must use function key 10 to return

the  program to the converse mode before you  type

another character of text.

After setting the terminal configuration mode,  it

returns you to the TNC command mode.

In  the  AMTOR  communications  mode  you  can  do

several things as follows;

3.2.4.6   FEC

Places  the  terminal into the  FEC  transmit

mode.  This puts you on the air, anything you

type will be transmitted.

3.2.4.7   ALISTEN

Read the Mail on a QSO.  Unfortunately,  when

you  tune in a chirpy AMTOR QSO in the ARQ or

standby  mode you won't get any copy  on  the

QSO  in progress.   You can however read  the

mail on one by entering the ALISTEN mode.

3.2.4.8   ACHG

Force a changeover or 'break in'.  If for any

reason   you  want  to  interrupt  the  other

station you send a 'BREAK' signal in software

this way.

3.2.4.9   ARQ

Return to the ARQ standby mode.  You must  be

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 20

in  this mode to receive signals from another

station  who is calling you.  You may  do  it

this  way or via the main Mode   Menu.   When

you  tune for CQ calls,  they sound  somewhat

like  fast RTTY with a ringing sound  similar

to the connect alarm (but at a faster ringing

rate) used by PK232COM.

3.2.4.10   Force LETTERS Case

This  function lets you force the receiver to

the  letters  case  if for  some  reason  you

appear  to be in the numbers case.   It  will

return you to the converse mode.

3.2.4.11  Override ROBOT Mailbox/Beacon

This  function  lets you override  the  ROBOT

BEACON  and  chat  to  a  station  that   has

linked  to  the  ROBOT.   Use  it  after  the

message  in  PK232COM.TXT has been  sent,  ie

when  the other station is sending a  message

to you.

3.2.4.12  Turn on Mailbox/Beacon.

This  function  lets you turn on the  Mailbox

during  a regular AMTOR QSO so that the other

station  can  leave a message or  download  a

file remotely.

3.2.5    Monitor Channel Activity

This  function  lets you see what calls the  PK232

has  heard recently on the Packet  channels.   You

can see up to 18, with the data and time they were

last  logged  by the PK232.   If they  were  heard

directly  an asterisk (*) will be listed  next  to

the   call  sign.    If  they  were  heard   being

re  transmitted via another station,  the asterisk

(*) will not be shown.   Refer to the 'MH' command

in  the  PK232 manual.   If you are using  a  TNC2

refer to the MH command in the TNC manual for  the

particulars of the display.

The  MH  list  shows up in a  special  window  and

freezes until you depress any key.  While the list

is  frozen,  PK232COM  is not monitoring  received

data, so don't freeze it too long.

This function does not work on the TNC1.

3.2.6    Change Packet Connect/Contest Message

3.2.6.1   Change Packet Connect Message

This   function  allows  you  to  change  the

message  that  will be  received  by  another

packet station when it connects to you. Refer

to the 'CTEXT' command in the TNC manual.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 21

3.2.6.2 Change Contest Message

This function first toggles the contest mode.

If  the  contest mode  is  active,  the  word

'TEST' and the number of the next QSO will be

displayed  in  the status  window.  When  the

contest  mode  is toggled on,  this  function

then  allows  you  to set  the  starting  QSO

number  and  write  in the  non  changing  or

standard message that will be transmitted  as

part  of  an  exchange of messages  during  a

contest.

The contest function is toggled off the  next

time  you select this function,  or when  you

change  the PK-232 Communications  mode  (ie.

switch from Morse to BAUDOT).

Note  that while this does give you some semi

automatic contest operation by using function

keys,  there is no checking of callsigns  for

duplicates  built  into the  program.   In  a

contest,  you  enter the callsigns  and  call

the other station in the normal manner,  then

use   this  function  key  to  transmit   the

exchange.  In Morse, and RTTY don't forget to

use function key 10 to turn on the transmiter

first.

3.2.6.3 Change CQ text Message

This  function  allows  you  to  change   the

contents of the CQ text message string.  This

is  the string that is transmitted along with

the  CQ call.   The default value  is  loaded

from the PK232COM.SYS file.

3.2.7    Enter Call Sign

This  key lets you enter the call of  any  station

calling  you  into the program.  If you  type  the

enter key in response to the prompt, you can later

use  the  F3  (function key  3)  to  transmit  the

sequence  "de  <yourcall>".  You may also use  the

Shift F7 key to call that station in the non AMTOR

modes.

In AMTOR,  you use it to enter the callsign of the

station you want to or have called by means of the

SELCAL.

You  should also use this to enter a callsign of a

station  that called you,  if you were not  in  an

autoCQ mode.

3.2.8    Not used.

You  don't want to disconnect accidentally if  you

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 22

just depress function key 8 without the 'Alt' key.

3.2.9    Activate Transmit Buffer

The  use  of this key sets PK232COM to  the  "Type

Ahead"  mode.   Any  characters then typed at  the

keyboard  will  be placed into  the  TNC  transmit

buffer.  The contents of the buffer will be trans-

mitted  over the air when the transmit function is

invoked [FK10].

This feature allows you to 'type-ahead' as you can

enter text into the transmit buffer while  viewing

incoming  words in the INPUT window in the screen.

Any  characters  typed from the keyboard  will  be

echoed in the OUTGOING window on the screen.

THIS FUNCTION IS NOT ACTIVE IN THE AMTOR or PACKET

modes.   Packet is full duplex, while in AMTOR the

PK-232 buffers the data until you receive the next

+? sequence.

3.2.10   Help

The use of this key will activate the HELP menu in

a  differnet  window on the screen.   This  window

will remain on display until a character is  typed

at the keyboard, or a character is received by the

computer from the PK-232.

3.3  Shift Function Keys

3.3.1     Dir

This  function  shows  you the  directory  of  the

logged in (active) disk drive.

3.3.2     Rename

This  function allows you to rename a  file.   You

could  for  example rename a document  file  to  a

"callsign.OUT"  file so that someone will  receive

it automatically next time they connect.

3.3.3     List of Outgoing Messages

This function shows you the outgoing messages that

have not been sent. In other words, all files with

the  file  extension  of '.OUT'.See  below  for  a

description  of outgoing messages in your  "Packet

Radio Answering Machine".

3.3.4     Delete

This function allows you to delete a file

3.3.5     List of messages that have been sent.

This  function  shows you the messages  that  have

been  sent.   In other words,  all files with  the

file extension of '.OLD'.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 23

3.3.6     Mail Beacon Activate

Using this function will also load a 'Mail :' list

into  your  Packet communications mode Beacon  and

initiate  a  beacon transmission every 30  minutes

(Refer  to the BTEXT command in the  TNC  manual).

If  no  mail is pending then beacon  transmissions

are  inhibited.   This conforms to good  operating

practice on crowded Packet channels.

3.3.7     Retry

This  function allows you to retry a connect  that

for some reason did not go through the first time.

It save wear and tear on your fingers particularly

if  the  desired connect path is via a  number  of

digipeat  relays  or through a number  of  NET/ROM

NODES.

In  the  non  packet  modes,   it  allows  you  to

retransmit  a  call  to  another  station  without

having to re-enter the call (FK7).

3.3.8     Not used.

You  don't want to disconnect accidentally if  you

just  depress  function key 8 without the  'Shift'

key.

3.3.9 Connected Indicator Reset.

You  use this function key to reset the PACKET and

AMTOR   beacon  mode  "connect  has  taken   place

indicator".  This is a flashing [C] display in the

status window.   Whenever someone else connects or

links  up to you this display will  be  activated.

It  is  reset by this function key.   If  you  are

monitoring  Packet traffic on a channel,  you  may

not  notice  that a connect has  previously  taken

place.   This  indicator  alerts you to check  the

contents  of  the PK232COM.RUN disk file  for  any

incoming messages.   The number shown with the 'C'

is  the number of connects that have  taken  place

since  the Indicator was last reset.   If a 'happy

face'  is flashing along with the  connect  count,

your    "target   station"   was   amongst   those

connecting.

3.3.10  OVER function.

In  the Packet communications mode  this  function

sends  your  callsign followed by the >  and  Line

feed  characters.   You  may  use this  to  signal

'OVER' in a packet contact.

In  the  AMTOR communications mode  this  function

sends  a  callsign sequence followed by  the  '+?'

sequence  of  characters  which are  the  software

'OVER' signal.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 24

In   the   MORSE  and  RTTY  (Baudot  and   ASCII)

communications  modes,   this  function  lets  you

transmit  the following sequence <other  call)  DE

<your  call> and returns you to the PK232  Command

Mode.   For  example if you are 4X6AA and you  are

working  G3ZCZ then this function  would  transmit

the  sequence 'G3ZCZ de 4X6AA'.   You use this  at

the  END  of  a transmission in  any  mode  except

Packet.

If  you have typed something into the 'type  ahead

buffer' then touch this function,  the contents of

the  buffer  will will be transmitted followed  by

the callsign sequence.

3.4  Control Function Keys

3.4.1     Day Flag

This  function key puts the TNC into  the  command

mode and loads the date into it from the computer.

You  use  this to reset the TNC clock if it  isn't

that stable,  or to flag a date and time into  the

log file when in the non Packet modes. For example

if you want to time tag a qso.

This function does not work on the TNC1.

3.4.2     CWID

This   function   key  is  used  to  send   a   CW

identification  when  operating BAUDOT,  ASCII  or

AMTOR.   You use this if your license requires you

to  identify  in  morse code as  well  as  in  the

operating mode.  In AMTOR, the CWID is sent in the

FEC  mode.

The  identification consists of the  sequence  'DE

<your  callsign>'  where  your call sign  is  that

entered into the PK232 as  'MYCALL'.

3.4.3     Message Upload

PK232COM  contains store and forward routines  for

messages   in  a  Local  Area  Network  (LAN)   as

described  in Appendix 2.   Use this  function  to

upload a message to another system compatable with

PK232COM.

When you invoke this function,  the computer  will

ask  you  to tell it if you want to leave a  note,

QSP or QRU.

3.4.3.1   Leaving a Note in your system.

In all modes you can leave a brief  note

(message) for another station.  When you

invoke this function the header is writ-

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 25

ten  to  the message file,  then as  you

enter the text it is stored on the  disk

in the usual format.

WHILE WRITING YOUR NOTE,  ALL OTHER FUN-

CTIONS  OF  PK232COM  ARE  INHIBITED  OR

LOCKED OUT.  THUS IF SOMEONE CONNECTS TO

YOU,  THE TNC WILL KNOW THT BUT PK232COM

WILL NOT.  BE WARNED !!!

After   you  have  left  the  note   and

received    the   'QSL'   message   from

PK232COM,  if  you  are  in  the  Packet

communications mode, push function key 8

or  function  key  shift 6 to  load  the

callsign  of  the  recipient  into   the

beacon.

When you leave a note,  leave it for the

root  call,  for example to leave a note

for   KL7LF/KH3  leave  it  for   KL7LF,

similarly to leave a note for   WB8WKA-1

leave it for WB8WKA.   PK232COM is smart

enough to extract the root call from any

callsign it links/connects to.

3.4.3.2    Leaving a message on an other  PK232COM

system [QSP].

The  QSP  function lets  you  enter  the

callsign  of  the  station you  wish  to

leave a message for.   It will then send

the text sequence :QSP: <callsign>.  The

other PK232COM HOST station will  either

reply  :QNO:  followed  by a  number  or

:QRV:  <callsign>.   The  'QNO'  message

means  that  the other  computer  cannot

accept  the message,  the :QRV:  message

means that it is waiting for you to send

it.

If you get the :QRV: reply, go ahead and

type the message,  or send the text file

containing the message.  If you type the

message,  use a control Z (^Z) character

and  carriage  return to  terminate  the

message.    (In  AMTOR,   PK232COM  will

translate  the  ^Z  to   :EOF:   +?   to

terminate  the message).   You will then

receive  either a :QSL:  frm  the  other

station  meaning  that  the  message  is

accepted,  or  a  :QNO:  followed  by  a

number  which means that something  went

wrong and the message was not stored.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 26

3.4.3.3    Uploading messages to another  PK232COM

system.

The QRU function allows you to upload  a

number of messages to the other station.

You   use  it  when  you  have  messages

pending on your system and want to  pass

them  to a third computer for storage in

the  network,  while you take your  com-

puter off-line.

You  can only use this function  with  a

station  designated as a store and  for-

ward mail box, for which you have set up

a  <callsign>.MBX file (see section  7).

This  function  may be triggered by  you

using   this  control  key  or  by   the

connecting  station sending you a  :QRU:

command.

When the sequence is over,  the computer

sending  the messages will transmit  the

NC/L  command word :QJG:  to signal that

the  sequence is complete.   Should  you

receive it before you get any  messages,

then there weren't any pending.

If  you  want to download messages  from

another  PK232COM system,  connect to it

and type the sequence :QRU:  followed by

the 'Enter' character.  That system will

then  respond to you as if the  operator

there had initiated a QRU function.

Appendix  2 contains more information on  the  LAN

capabilities of PK232COM.

3.4.4     Show last few log entries.

This function displays the log entries made during

the  active  session.  If no QSO's have been  made

since  the program was loaded the  display  window

will be blank.   Up to 5 log entries will be disp-

layed in the window.   You use this function for a

quick look at which stations have connected to you

in your absence.

3.4.5     Automatic CQ Sender

This function will send a CQ sequence,  wait about

a  minute  or so  and then repeat the CQ  sequence

every  minute  or so until it receives  your  call

back from someone else.   When it does so, it will

alert you with a Connect Alarm signal and reset to

the normal or listening mode.

When the Automatic CQ sequence is in operation, an

indication  will  be shown in the  status  window.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 27

The display will be constant while the transmitter

is  sending the CQ test,  and will blink while the

receiver is listening for a reply.

The  time  delay  between calls is set  using  The

Control Function key 7.

The  automatic CQ function will only work  in  the

PK-232  non Packet modes if the 'EAS' mode is  set

to  'ON'  per  the PK232COM.SYS  file.   In  other

words,  you must be able to see the CQ text pop up

in the outgoing window and then slowly echo in the

incoming  window for the function  to  work.   The

reason  for  this is that the "K K K" sequence  is

used  to arm the software to look for  your  call-

sign.   If it wasn't,  the program would recognize

your  call  being echoed to the screen  and  think

that you had called yourself.

In AMTOR you will get the connect alarm only  when

the  replying station transmits the '+?' character

combination.

The  check for the detection of your callsign  and

the connect alarm sequence only occur after a line

feed character have been received.

Under most conditions in AMTOR,  many in RTTY, and

some  in  MORSE,   PK232COM  will  recognize   the

callsign  of  the  station answering your  CQ  and

display  it in the status window.   In  the  event

that  it fails completely,  the  sequence  '--?--'

will  show  up in the status  window.   Since  the

recognition  algorithm use just look for the first

word following the 'DE ' sequence and assumes that

it has the callsign,  be careful to check it  when

operating under QRM conditions.   You can override

the  automatic call selection with the Enter Call-

sign function key.

The  automatic  CQ sequence is cancelled  when  an

incoming call is detected, you push the disconnect

function  key,  or  if you  change  communications

modes.

3.4.6     SELCAL

This  function  key toggles the SELCAL feature  on

and off.   When the SELCAL is activated,  as shown

in the status window,  the incoming text from  the

PK232 is scanned.   When either the letters "ZCZC"

or  'your  callsign' (as entered in line 1 of  the

PK232COM.SYS file),  the capture-to-disk is turned

on.   All  subsequent text will be stored  in  the

PK232COM.RUN  disk  file until the letters  "NNNN"

are received.  At that time the capture-to-disk is

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 28

turned off.

You   may   use  this  feature   when   monitoring

commercial  RTTY  broadcasts  to record  news  and

other  information,  or when calling CQ  to  start

recording  all  incoming text and keep the QSO  on

disk.

You may use the loggletoggle function to  override

the automatic turnoff if you wish.

3.4.7     Beacon Toggle On/Off/CQ delay time set.

This  function toggles the beacon option  for  the

automatic  CQ  caller  in  the  AMTOR  and  Packet

communications modes.  When active and CQ is being

called,  a  [B]  will be displayed in  the  status

window.  In  this mode,  the Automatic CQ sequence

will  resume after the AMTOR link is dropped or  a

Packet Communications mode disconnect.

In the beacon mode,  the last few logbook  entries

will  be  displayed  in the logpage  window  which

overlays the outgoing text window at the bottom of

the screen.

In the Packet mode,  the CTEXT message is the only

transmission from you that the other station  will

receive, so make it good.

In the AMTOR Mailbox/Beacon mode,  the contents of

the  PK232COM.TXT file will be transmitted to  the

station who called you.   The capture-to-disk will

then  be turned on to record one transmission from

that station.   The QSO will then be logged to the

logbook file.   Don't forget to check the  entries

in case the computer made a mistake in recognizing

the call sign of the other station.

THE BEACON MODE IS DESIGNED TO ENCOURAGE PEOPLE TO

QSY OFF THE CROWDED PACKET HF CHANNELS. DO NOT USE

IT  ON  A CROWDED HF CHANNEL OR ON AN  ACTIVE  VHF

CHANNEL.  IF YOU DO, YOU WILL SOON FIND THAT OTHER

STATIONS  HAVE  SET  THEIR TNC'S  TO  REJECT  YOUR

PACKETS,  YOU WILL NOT WORK ANYBODY, YOU WILL JUST

BA A SOURCE OF QRM AND WILL BE KNOWN AS SUCH.

THE  MODE IS ALSO DESIGNED FOR ROBOT  BEACONS  FOR

THE HIGHER FREQUENCIES.   FOR EXAMPLE IF YOU LEAVE

IT  RUNNING  ON  10 METERS YOU NEVER KNOW  WHEN  A

SHORT OPENING WILL ALLOW A QSO TO SOME PART OF THE

WORLD.   A DX CONTACT YOU WOULD HAVE MISSED OTHER-

WISE.    DO NOT LEAVE IT RUNNING ON 20 METERS WHEN

THE BAND IS OPEN.   YOU WILL JUST BE CAUSING  QRM.

FURTHER  APPLICATIONS  ARE  IN  METEORSCATTER  AND

OTHER "EXOTIC" MODES OF COMMUNICATIONS.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 29

If  you  are using this mode in  a  special  event

station  or  DX-pedition you may  type  characters

into the transmit buffer,  while the other station

is sending to you.   When he, she or even it turns

it back to you, you will transmit those characters

before the sign off sequence.

In both the Packet and AMTOR communications modes,

the  robot may be shut down by stations connecting

to you and telling you to QRT.

3.4.7.1   Beacon Mode Toggle/On-Off

This  function allows you to toggle  the

Beacon mode on and off.

3.4.7.2   CQ Delay Time Adjust

This  function allows you to change  the

time   that  the  program  waits  for  a

response  after  a  CQ  call  until  the

following  one  is   transmitted.    The

default   value   is  loaded  from   the

PK232COM.SYS  file.   You  may  want  to

experiment  since the actual  time  will

depend  on  the clock frequency  of  the

computer.   Make  it a small number such

as 4 or 6 when operating AMTOR or a  DX-

pedition,  and  make it a larger  number

such  as  30 when operating  on  crowded

packet  channels.   The maximum  allowed

value is 255.

3.4.8     Not used.

You  don't want to disconnect accidentally if  you

just  depress  function key 8 without the  'Shift'

key.

3.4.9     Transmit Buffer Clear

This  function is used if the PK232  continues  to

transmit  the  characters you put into  the  'type

ahead  buffer'  after  you have tried to  shut  it

down.

3.4.10    Logbook.

PK232COM  performs automatic logbook  entries  for

Packet  and AMTOR Beacon mode Connects,  and  semi

automatic  logbook entries for other  modes.   The

Log  file  is  compatible to that of  the  CONTEST

program  in  the G3ZCZ PC-HAM package and  can  be

processed by the DBASE  Logbook Package of  PC-HAM

by  G3ZCZ for indexed listings,  tracking of  DXCC

and other AWARDS,  etc.  This is ideal for special

event stations and DX-peditions, for the whole QSL

process  and further statistical analyses of their

operation has been automated.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 30

For this function to work,  the file  PK232COM.LOG

must be present on the disk.  If it isn't, it will

be  created for you.  After one logging  operation

has occurred, edit out the initial text.  You must

perform  this  editing operation before  appending

the  contents of the logbook file to  the  logbook

database of the PC-HAM LOGBOOK Package.   Edit the

log  before appending it to the DBASE package,  as

there  may be some null characters at the  end  of

the  file which may be recognised as a blank entry

by DBASE

It  is  also advisable not to let the each of  the

files get too big.

When  an  AMTOR  beacon  mode  or  packet  connect

occurs,  the connect is automatically logged.  The

connect  path is placed in the comments section of

the logbook entry.   In the non packet modes,  the

log data entry is set up for you with default  599

values for the reports exchanged.  You invoke this

function to enter the data into the log.

When you invoke the function you will be presented

with  the  following  typical  menu  in  the  menu

window.

LOG ENTRY

DATE     TIME   CALL     BND  TX  RX  MODE  POWER S R   COMMENTS

87/04/27 17:28             20  599     RTTY   10   - -

Date,tIme,Call,Bnd,Tx,Rx,Mde,Pwr,qSl snt,cOmmnts or Logit (*) ?

You  should  reply with the capitalized  character

associated with each item.   For example,  if  you

want to change the Band,  enter the letter 'b', to

change the cOments,  enter the letter 'o'.   Touch

the  'ENTER'  key after you have made your  choice

and enter the text when prompted.   You will  then

be presented with an updated entry.

To  enter the data into the logbook,  use the  'l'

key, to save the data but not make a log entry use

the '*' character.   You would do that if you want

to change the band or power information.

You  may use the 'qSl sent' entry to tag the  fact

that you have written out a QSL card.

The band and power information are set up from the

default  values  stored in the configuration  file

PK232COM.SYS.   They  are displayed in the  status

window.   If  you are using the logging  function,

you  should change them when changing bands/powers

and  quit  the  logging  function  with  the   '*'

character so that you don't put a blank entry into

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 31

the logbook.

Note  that you can also use this function to enter

the odd SSB contact into the logbook file.  If you

do,  use  the  characters 'SSB' or 'FM' for  those

voice modes to remain compatible with the rest  of

the DBASE logbook package.

NOTE   Do  not use the comma character ',' in  the

comments field.   If you examine the log entry you

will see that all items are separated by  comma's.

If  you use a comma character in the comments  you

will  screw  up  the  transfer of  data  into  the

logbook  package.

The comments field in the logbook is restricted to

a maximum of 10 characters.   If you try and enter

more than 10, the surplus will be ignored.

When  you  examine the DBASE logbook file,  if  it

looks funny or the data are in the  wrong,  place,

then  you probably have the wrong number of  comma

characters in some line.  Delete the .DBF file and

re-edit the .LOG file, to make sure that there are

the correct number of comma characters.   You will

recognise the pattern readily enough.

3.5  Shift/Baudrate Control keys

The  following  keys are used to control the Shift (Wide  or

narrow) and the Radio link baudrate in the BAUDOT, ASCII and

PACKET communications modes.

3.5.1     RIGHT ARROW speeds up the baud rate.

3.5.2     LEFT ARROW slows down the baud rate.

3.5.3     UP  ARROW toggles the shift from Wide to

Narrow and back.

3.5.4     DELETE toggles the USOS function.

In  the PACKET mode the baudrate may be changed between 300,

600, 1200 and 2400 bauds.  In the other modes all baud rates

supported by the PK232 may be selected.

The  state  of each of the above functions are displayed  in

the  status  window at the top of the screen.   If  USOS  is

active,  'USOS' will be shown.  The shift is shown as 'W' or

'N'  for  wide or narrow respectively.

3.6  Additional keys

3.6.1     The HOME Key

The home key will clear the window screen that the

cursor  happens to be in at the time that the  key

was depressed,  and will then position the  cursor

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 32

at the top left hand corner of that window.

3.6.2    The DOWN ARROW key

The  DOWN  ARROW  key toggles the  Normal  Reverse

Sideband  function  in  all  communications  modes

except  MORSE.   The sideband is depicted  in  the

status  window by the up and down arrows.   If the

down  arrow  is showing,  you are  set  for  Lower

Sideband (LSB) which is the normally used one.

If  you have reversed the sidebands a flashing  up

arrow will be displayed.

If you set your transceiver to LSB you will be set

correctly for all modes.

For  CW operation you may set your rig to  the  CW

position and ignore the sideband setting.   If you

use FSK just set it so that it works for you.

3.6.3     The Alt A key (Alert toggle).

This  key toggles the Alert call function  on  and

off.   If it is off, you are prompted for the call

to  be monitored,  if it is on,  it is turned off.

The toggle is set when you enter a  callsign,  and

cleared  when you depress the 'Enter' key  instead

of a callsign.

The  Alert  call  will be shown  flashing  in  the

status window when in the disconnected state.

The   Alert  function  is  used  in   the   Packet

communications  mode  when  disconnected  and  the

terminal set for 'TRFC' or 'CQ/BEACON'.   PK232COM

will  scan  the packet headers received  from  the

TNC,  and,  when  it  sees a packet originated  or

digipeated if the MRPT parameter in the TNC is set

to 'ON', (the default state in PK232COM.SYS by the

way) by the station whose call you have entered as

the  'Alert' call,  it will sound an alarm at  the

console.

NOTE:  If the terminal is set for 'CQ/BEACON'  the

Alert  will  only sound if the  specified  station

transmits CQ or BEACON packets.

3.6.4     The Alt B key (Baudot/ASCII Baud rate set).

This   key   allows   you   to   set   a   desired

communications  baud rate directly.   You will  be

prompted  to  enter the desired  rate.   Use  this

function to avoid having to shift through too many

intermediate states,  if for example,  you want to

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 33

make a direct change from 45 to 100 bauds.

3.6.5    The Alt C key (connect request).

This key performs the same task as Function Key 7.

The  Alt C key has been provided for those  people

whose  fingers have learnt YAPP and don't wish  to

be re-educated.

3.6.6    The Alt D key (display inhibit).

This key toggles the 'Display Inhibit' mode.   Use

it  to  inhibit direct writing to the screen  when

running  under  a multi-tasking  operating  system

such as Doubledos.   It has been tested and  works

under Doubledos Version 4.0.

3.6.7     The Alt F key (Digipeat monitor toggle)

This  key toggles the digipeat display function on

and  off.   If  the Digipeat Display  Function  is

active, any signals digipeated through you will be

stored to the PK232COM.RUN file and a 'beep'  will

be sounded for each packet being digipeated.   The

status display window will also show a [D] at that

time.   The  MDIGI  function is set 'ON' when  the

Digipeat display function is 'ON', this means that

digipeats  will show up while you are connected to

someone.   The default state for the Digipeat Dis-

play Function is OFF.

3.6.8     The Alt P key (printer toggle).

This key toggles the printer on and off.  When the

printer  is turned on,  any incoming data will  be

printed  in  real time.   If you intend  to  print

files  received  at  1200  bauds  in  the   packet

communications   mode,  use a print spooler or you

may  loose  data.   The printer  is  automatically

turned  off in the packet mode when  a  disconnect

occurs.

3.6.9     The Alt R key (Message Relay toggle)

This  key  toggles the Store and  forward  Message

Relay  Capability of your system on and  off.   If

the  Message Relay Capability is  active  (toggled

on),  any  one who connects can store a message in

your  system  using the  ':QSP:  <callsign>'  NC/L

function.  If  the Message Relay Capability is not

active  (IE.  toggled off) anyone who attempts  to

':QSP: a message' will receive a ':QNO: 1' "error"

message.   The  status display window will show  a

[R]  when the Message Relay Capability is  active.

The default state for the Message Relay Capability

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 34

is loaded from the PK232COM.SYS file.

When  the Store and forward capability is  active,

the suffix 'R' will be added to the

'*** PK232COM 1.42'

message  received  by  anyone connecting  to  your

system   to  notify  them  that  they  can   store

messages.   In this manner they don't have to  try

to  leave a message and then get ':QNO:  1'  error

message to find out if the feature is toggled off.

3.6.10   The Alt S key (Sound Set/Toggle).

This  key  toggles the sound on and off.   If  the

sound  is  off,   a  flashing  indicator  will  be

displayed in the Status Window.  When the sound is

toggled on again, a chime will sound to verify the

audio.

3.6.11     The Alt C key (Target Call set/toggle).

This  key toggles the Target call function on  and

off.   If it is off, you are prompted for the call

to  be monitored,  if it is on,  it is turned off.

The toggle is set when you enter a  callsign,  and

cleared  when you depress the 'Enter' key  instead

of a callsign.

The  Target  call  will be shown flashing  in  the

status window when in the disconnected state.

The  Target  call function is used in  the  Packet

communications  mode when  disconnected.  PK232COM

will monitor the stations connecting to you.  When

it  sees  a connect by the station which you  have

entered  as  the 'Target' call,  it will  set  the

flashing  Connect Count display to show  a  'happy

face'.

3.6.12    The Alt X key (Exit or return to DOS).

This  function is the same as the Alternate  Func-

tion 1 key.  It exits the program and returns  you

to DOS.  When you exit the program, the TNC is put

into  the  VHF  default mode in which  it  rejects

calls  from any other stations.   (Read about  the

'CFROM' command in the TNC manual).   you can thus

if  you  wish leave it powered,  and if  you  also

leave the VHF radio powered up, you will be active

as  a  digipeater,  but at the  same  time  anyone

trying  to  connect with you will receive  a  busy

signal.   The  MBX function (Reading the Mail)  is

also cleared at this time.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 35

3.6.13    The PgUp key (Scrolls back incoming text).

This  key lets you repeat the display of  incoming

text  which  has  scrolled  out  of  the  incoming

window.   you  may print it or capture it to  disk

during the repeat.

3.6.14    The Control Z key.

This key is used in the Packet communications mode

to  signal  the  end of a file  or  message,  when

loading  messages into another PK232COM system  or

into a BBS.   In the AMTOR communications mode, it

is translated into the sequence ' :EOF: +? ' which

can  be used as a fast break-in changeover  or  to

terminate   a  message  being  stored  in  another

PK232COM AMTOR mailbox. .cp7

4.0  PK232COM disk files

The following disk files are associated with the program.

4.1  PK232COM.COM

This is the program.  You activate it from DOS by typing the

word  PK232COM  followed by ENTER in the  normal  manner  of

invoking DOS commands.

4.2  PK232COM.RUN

This  is  the  file containing the captured  incoming  text.

This file contains incoming data from the TNC recorded while

the 'capture to disk' function was active.

4.3  PK232COM.SYS

This  file  contains the configuration information for  both

the program and the PK-232.  YOU MUST EDIT IT before you run

the  program  on  the air the first  time.   Use  your  word

processor in the NON-Document mode.

The contents of the file are as follows.

4.3.1     The first line.

The first line  contains your callsign.

Please  do  not use G8BTB.   Replace  G8BTB   with

yours.   You  may  use / characters if  you  wish.

This  is  the call sign that will be used  in  the

non- Packet  modes.   You  may thus use  "portable

designators"   such  as   G3ZCZ/A,   G3ZCZ/W3   or

G3ZCZ/4X,  which you cannot do in Packet.   IF YOU

DON'T CHANGE THE CALL SIGN,  YOU WILL GET AN ERROR

MESSAGE AND THE PROGRAM WILL NOT RUN.

This is the call that will be monitored for in the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 36

Digipeat monitoring mode.   In this mode which  is

toggled  on and off by the Alt F key,  if  someone

uses  you  as  a  digipeater  while  you  are  not

connected to anyone else, the event will be logged

to  disk.   This  mode  will  only  work  if  your

callsign  as  stored  in  this  line  is  a  valid

(uppercase only) packet callsign.

For  example,  in the PK-232,  if the callsign  is

G3ZCZ then the sequence >G3ZCZ> will be recognised

as  a  digipeat and the event  logged.   Thus  for

example  4X6AA*>G3ZCZ>4Z4ZB  would be logged as  a

digipeat event.

Now  if the callsign in this line is G3ZCZ/4X then

the  event  will not be recognised as  a  digireat

because  the  computer  would be  checking  for  a

G3ZCZ/4X  in the packet header and it would  never

see  G3ZCZ/4X.    This  option  is  inhibited  for

reciprocal  callsign  operators due to  the  AX2.5

protocol itself.

If  you have a / character in your  callsign,  and

use  the  base  call for  packet  identifying  the

location  in the CQ text,  then the function  will

work  if  you place the "portable location"  as  a

suffix.   For example, if the call is G3ZCZ/4X and

the  packet  identification  is  G3ZCZ,  then  the

digipeat  monitor  will  work.  If  the  call  was

entered  as  4X/G3ZCZ the digipeat  monitor  would

check for 4X and would not appear to work.

Store  your callsign using UPPERCASE characters if

you want this option to work (IE.  G3ZCZ).  If you

use lower case (ie. g3zcz), the option will not be

enabled as recognition will never take place.  The

PK232 automatically converts case to upper case in

the non packet communications modes.

If  you  are using an SSID (eg.  G3ZCZ-8)  in  the

packet  communications mode,  you should enter the

whole call INCLUDING the SSID.

4.3.2     The second line.

The   second   line  contains  the  AMTOR   SELCAL

configured  for your callsign.   You use it  again

later to tell the PK232 what the SELCAL is.   This

line just tells it to PK232COM.

4.3.3     The third line.

The   third  line  contains  the  line   of   text

transmitted  by  the PK-232 in a CQ call  (in  any

mode except for MORSE). Think of this message as a

CQTEXT line.  Edit this line as well.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 37

4.3.4     The fourth and fifth lines

The band and power information are set up from the

default  values  stored in the configuration  file

PK232COM.SYS.   They  are displayed in the  status

window.   If  you operate on another band or  your

transmitter  uses  a different power level  change

the  default  values in the file to  reflect  your

custom default state.

The  fourth  line contains the  band  information.

The  default  value is 2 for 2  Meters.   Use  the

Meter  value  for  the band to  keep  the  display

correct.

The   fifth  line  contains  the  value  for   the

transmitter power.  The default value is 10 for 10

watts.

4.3.5     The sixth line

The  sixth line contains a number which tells  the

program how long to wait before transmitting a  CQ

call when in the beacon (automatic) mode.  A value

of  4 corresponds to about 2 minutes.   This value

of time will vary depending on the clock speed  of

your  computer.   You may have to experiment  with

different  values.   The default value is 0  which

inhibits the beacon mode.

4.3.6     The seventh Line

The  seventh  line configures the  LAN  store  and

forward  capacity of the system also known as  the

Message Relay Capability.  If it is a '0' (default

state)  your computer will not accept messages for

forwarding.   If it is a '1', it will.  See Appen-

dix 2 for details about LANs.

Remember,  if you configure your computer to store

and forward messages,  you have a moral obligation

to deliver them.

The  status of this function shows up as an [R] in

your  status  display and in the  connect  message

which  is transmitted to the other station at con-

nect  time,  as an 'R' suffix to the message  when

active.

4.3.7     The eighth line

This  line  contains  the  default  Communications

port.   The default value is 1.  You must leave it

set  to 1.   THIS VERSION OF PK232COM CAN ONLY  BE

USED WITH COMM1.

4.3.8     The ninth line (TNC type)

This  line  tells  the program which TNC  you  are

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 38

using.   The  PK232 command set is  slightly  dif-

ferent to those of TNC2 in the packet mode.   Thus

the  command dialog has to be changed somewhat  to

make the 'SOLO,  TRAFFIC, CQ/BEACON and MAIL' ter-

minal  modes  work correctly for different  TNC's.

NOTE  that program does not check the TNC  in  the

Non Packet modes, so for example if you try to set

a  TNC2  to AMTOR,  PK232COM will send  the  PK232

commands  to the TNC2 which should then return  an

error message.

Valid TNC's are PK232

TNC1

TNC2          (don't type the '.').

The  TNC2  used to test PK232COM 1.41 was  an  MFJ

1270.

4.3.9     The Tenth line (Terminal Baud Rate).

This  line  tells PK232COM what baud rate the  TNC

is set to. This is the baudrate on the RS-232 line

between  the  TNC and the  computer.   Valid  baud

rates for PK232COM are 1200,  2400, 4800 and 9600.

If  you  specify  something  else  PK232COM   will

default to 1200.

If the TNC does not use an interrupt driven serial

port  you may experience problems when using  baud

rates greater than 1200.  The PK-232 does not like

baud  rates faster than 2400,  and  sometimes  not

even that.  Use 1200 until you have tested and are

sure that a faster one will work.

This  program  has been tested on a TNC2  using  a

terminal  baud  rate  of  9600  bauds  and  worked

correctly.

4.3.10  The QTC _Snatch or Automatic Connect  Attempt

Flag.

If the value entered on this line is a 1, then the

QTC_Snatch  Flag is enabled.   if the value on the

line is a 0, then the flag is disabled.

If the QTC_Snatch flag is enabled,  PK232COM scans

the contents of packets appearing in the  Inwindow

on  the screen while the terminal is disconnected.

Should   it  recognise  the  three  text   strings

'PK232COM',   ':QTC:',  and 'your callsign' in the

packet,  it  will then issue a connect request  to

the callsign of the PK232COM station that  origin-

ated  the beacon transmission.   If the connect is

made,  the other station will send you the message

automatically which PK232COM will capture to disc.

The  connect  will time out and  terminate  a  few

minutes later.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 39

Hopefully  it will not try to connect to you if it

hears  your beacon with your call in  it  (someone

left you a message) via a digipeater.

The  normal connect request will be made DIRECT to

the station.   A digipeat path will not be invoked

as this feature is designed to trigger a  normally

silent  station  standing  by  on  the  LAN  (good

operating practice).   If you expect messages from

stations  you  can only connect with via  a  digi-

peater,   place  the  path  in  your  PK232COM.DIR

directory file.   It should be there anyhow if you

connect to the station regulaly.   For example, if

you  expect messages from 4Z4ZB whom you can  only

work  if digipeated via 4X1AA then place an  entry

like

4Z4ZB   4Z4ZB V 4X1AA

in the PK232COM.DIR  file.

4.3.11    The remaining lines

The  remaining lines contain commands to  the  PK-

232.  For  further information about them refer to

the TNC manual.   The example in the  PK232COM.SYS

file is as follows.

CONSTAMP ON

CHCALL ON

CHDOUBLE ON

CMSG ON

CTEXT SHALOM you have connected with G3ZCZ/4X

daystamp on

DCD ON

eas on

echo off

flow off

mycall G8BTB

MYALIAS JOE

MYSEL GBTB

newmode on

NUCR ON

nulf on

users 1

usos on

If  you use a different TNC (other than the PK232) you  will

have to edit this command list.

4.4  PK232COM.LOG

This  file  contains the log entries.   It will  be  created

automatically to intialize the logging function.  The format

of the logbook file is the same as that created by the G3ZCZ

CONTEST.BAS  program  and  both files  are  designed  to  be

appended  into the G3ZCZ LOGBOOK DATABASE package  presented

as part of the PC-HAM software package.  It should be edited

with a word processor in the non document mode to delete the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 40

first  line  and clean it up before the file is appended  to

the DBASE logging package data files.

4.5  PK232COM.DIR

This  is  a  directory file.  You use it  to  store  connect

NET/ROM   or   digipeat  paths  as  well  as   name/callsign

relationships.

Examples are

Joe  G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR   4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

You  create  this  file with your wordprocessor in  its  non

document mode.   You must leave AT LEAST one space character

between the first (key) word and the connect path.  When you

type in the key word,  the program does not care about upper

or  lower case.   If the program does not find the key word,

it will try to connect with whatever you typed in.

If  you  already  have  a YAPP.DIR file just  rename  it  to

PK232COM.DIR.

An example of a PK232COM.DIR entry for a NET/ROM path is

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

where the '!' separates each stage of the link.

Consider the manual sequence to make the connection.   K1HTV

is  the station the contact is being attempted with,  and he

is  located  within  range of the SCCT  NET/ROM  node.   The

station  attempting the connet is located in the  Washington

DC area one digipeat away from the ELK NET/ROM node.

The  connection to the local node is performed  first.   the

operator has to type ' C ELK v WB4APR-5' to make that  link.

When  the  connect with the node is achieved,  the  operator

must type 'C SNJ3' to connect to the next node.   Each  time

the  connection  is  made,  the operator must  type  in  the

command to make the next one, until finally the last node is

reached and a connect request can be made to the destination

station  (either  direct or via  a  digipeater).   Sometimes

there  may be an intermediate non NET/ROM connection between

two stations somewhere in the path.

This procedure is tedious and time consuming, because it may

take  several  seconds to complete each stage of  the  link.

PK232COM automates this sequence by scanning the line in the

PK232COM.DIR  file  associated  with the  key  callsign  and

transmitting each section of the line when it recognises the

'Connected  to ' reply from each NET/ROM node in  the  path.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 41

When it finally gets to the destination it sounds a chime to

alert you that the connect has been achieved.

If  for  any  reason the  connection  fails,  PK232COM  will

disconnect the link.

The  format  of  the entry in the PK232COM.DIR  file  is  as

follows.

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

The key word is K1HTV.   PK232COM then scans the line to see

if  there is a '!' in it.   If there is it knows that it  is

dealing  with  a  NET/ROM path.   It then tells the  TNC  to

connect  to the characters following the key word up to  the

'!'  sign to the TNC in the command mode.   In this case  it

tells the TNC to connect to ELK via WB4APR-5.  When the  TNC

sends back the connected message,  PK232COM tells the TNC to

try to connect to the next node in the path,  namely in this

example SNJ3.  The sequence continues each time a connection

is made until the link is complete.

To  set  up  a directory entry just copy the  format  shown.

Place  a '!' sign after the uplink path and  different  node

segments. Do not place one after the downlink path.

NOTE:  All  path entries in PK232COM.DIR must be on a single

line.

4.6  CONFIG.SYS

You  must have a CONFIG.SYS file on the boot disk if you are

going  to  use the disk logging features.   This  file  must

contain the line

FILES = 20

See   the  DOS  manual  for  more  information  about   the

CONFIG.SYS file.

4.7  *.MBX

These  are the files associated with the stations  you  have

designated  as  store and forward mailboxes.   Thus  if  for

example  you  want  to  use 4Z4ZB as  a  store  and  forward

mailbox,  you  will have to create a file  called  4Z4ZB.MBX

using the non-document mode of your word processor.  In this

file  you will put a list of the call signs of the  stations

4Z4ZB  can  connect to reliably.   If for example  you  want

4Z4ZB to be able to store messages from you to 4X6AA, 4X6LR,

G3ZCZ, ON8IK, F0WN,and G8BTB, you would put those call signs

in the file, one call on a line as follows.

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 42

When the QRU function is invoked either by you using Control

Function key 3 or by 4Z4ZB connecting to you and sending you

the command :QRU:  ,  any messages addressed to any of those

stations  (EG.  G3ZCZ.OUT and ON8IK.OUT) will be transferred

from  you  to  4Z4ZB just as if you had  tried  to  QSP  the

messages  manually  or as if G3ZCZ or ON8IK   had  connected

directly to you.  The file names will be changed to *.OLD.

You  must  create  one file for each  designated  store  and

forward station, ie. 4Z4ZB.MBX, G3ZCZ.MBX etc.

5.0  Initialization

5.1  Copy the files to a working disk.

5.2  Edit PK232COM.SYS. If you are upgrading from a previous

version  of PK232COM,  note that the new .SYS file  may

not compatible with the old one.

5.3  Copy the new PK232COM.SYS onto the old one.

5.4  Put  the  working disk into the active drive  (the  one

with the > prompt).

5.5  Make sure that the file CONFIG.SYS exists and  contains

the statement FILES=20.

6.0 Running the Program

6.1  Make sure that the PK-232 is connected to the computer.

6.2  Make  sure that the DOS calendar date and time are  set

to UTC.

6.3  Run the program by typing the word PK232COM followed by

the ENTER key.

6.4  When  the program begins,  you will default  to  PACKET

mode on VHF (Just like the PK-232 at switch on) in  the

TRAFFIC terminal mode.  If the PK-232 is not set to the

correct  baud rate and parity type the * characters  as

explained in the PK-232 manual.

6.5  Initialize  the TNC using Alternate Function key 2.

If  the PK232 does not have any batteries in it,  make  sure

that  the  RS-232 link between the TNC and the  computer  is

connected   then   load   and  run   PK232COM.    When   the

initialisation is over,  power up the TNC and type a few '*'

characters  as requested in the manual.   Once you  see  the

sign on message,  configure the TNC using Alternate function

key number 2.  To save you going through this rigmarole each

time  you power up the unit,  put some batteries in the TNC.

Use  good  quality  ones so that they don't  leak  over  the

circuit board.

7.0 Mailbox features

7.1 Answering Machine.

The  program includes an AMTOR (Robot beacon CQ  mode  only)

and  PACKET mode   SMART "answering machine" facility.   You

can  leave messages on your disk (in the same  directory  as

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 43

the  PK232COM.* files,  if you are using a hard  disk),  for

different  stations.  When someone connects to you,  if  you

left  a message for him,  he (or she or even it as the  case

may be) and only that station will receive it automatically.

No one else will normally be able to download that message.

7.2  Mail Beacon (Annunciator).

To ensure that people know that you have left a message  for

them a 'MAIL for' list is loaded into your Packet Beacon and

transmitted  every 30 minutes (Refer to the BTEXT command in

the TNC manual) as  ':QTC:' followed by a list of calls.  If

no mail is pending then beacon transmissions are  inhibited.

This conforms to good operating practice on crowded channels

(at least inhibiting the beacon does).

The  mail beacon in the AMTOR mode is transmitted as part of

the beacon autocq message.  It is automatically updated when

a message is transmitted.   If you don't set the beacon (see

below)  you  will not transmit a mail beacon  text  in  your

AMTOR CQ message.

The  Mail  beacon text is set up either when  you  tell  the

computer to set it up using Shift function key 6 or when you

disconnect a packet QSO from another station.

In  this manner you may update the list while the program is

running,  for example by using Sidekick to write a  message,

or  preferably the NOTE function,  and then with a stroke of

Shift function key 6,  you will update the mail announcement

on your beacon.

Any  station using PK232COM Version 1.42+ can be  configured

(default   state)   to  automatically  attempt   a   connect

(QTC_Snatch)  when  seeing their callsign in someone  else's

PK232COM :QTC: beacon mail message list.

If  for some reason the message got lost  (eg.  because  the

link  was marginal),  :QSM:  in version 1.42+ will get you a

repeat of that message.

7.3  How to leave a message

You  can use the note function (Control function key  3)  or

type the message as a non-document file on Wordstar,  or  if

you  use another word processor,  you must leave the message

as an ASCII text file.

You  may also use the NOTE function built into the  program.

If you do,  the message will be given a header as if it came

in from someone else.   if you use a word processor to write

the  message,  you  will have to write in a header  or  show

somehow that the message is not 'live'.

You  just  name  the message file as  the  callsign  of  the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 44

station  to whom you wish the message to be sent at  connect

time.  You must also give it the filetype of 'OUT'.

For  example a message for G3ZCZ would be stored on the disk

as  a  file named 'G3ZCZ.OUT',  and similarly a  message  to

4X6AA would be stored on disk as '4X6AA.OUT'.

Once the message has been sent, an entry to that effect will

be made in the PK232COM.RUN  data capture file.  The message

file  will  then  be renamed and given  the  file  extension

'.OLD'.  At that time any previously existing mail file with

the  file extension '.OLD' will be deleted.   This stops the

addressee  receiving  the message on successive connects  if

you  can't delete it for some reason (you forget)  while  at

the same time, the message is still available to you in case

you need it.

You  should use a utility program if you want the  calls  in

the  mail beacon sorted.   You may for example wish to  sort

them  alphanumerically or in the order of the date that they

were left in your answering machine.

7.4  Remote File Downloading (Packet/AMTOR Modes Only).

There  comes  a time when you want to leave a file  on  your

system  for someone to download later.   You can do this  by

using NC/L to control the transfer.  You must leave the file

in  a subdirectory called \PK232COM\FILES.   You must  first

create  it if it does not exist.   The station who wants the

message just asks for it using :QBM:  <filename>,  note they

do  not need to use the full subdirectory path.   They  must

however leave a single space character between :QBM: and the

filename.  In AMTOR the Mailbox must be turned on for remote

control functions.

For example,  you have the latest ARRL DX bulletin,  and you

want to pass it on.  You could pass it to selected people by

copying  the file to individual messages with  the  filetype

'.OUT',  eg.  4X6AA.OUT,  4Z4ZB.OUT or even G3ZCZ.OUT if you

want me to get it, which wastes a lot of disk space.

On  the  other hand you could tell people that the file  was

available  for  downloading,  either in  the  CTEXT  connect

message line which everyone gets when connecting to you,  or

in individual ('.OUT') messages.

If  you  include   the  following  in  your  CTEXT   message

"ARRLDX.015 now available"

people will know that to download it, they just have to type

the  line ':QBM:  ARRLDX.015' (one space  character  between

:QBM:  and the file name) to get the file.   PK232COM is not

designed as a BBS,  however, if you copy the contents of the

\PK232COM\FILES subdirectory into a file called DIR (USE DIR

> DIR in DOS),  anyone can get a list of files the files you

have  stored  in it for downloading by  typeing  :QBM:  DIR.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 45

Since they do not get the real directory,  just what was  in

it  at the time you copied it into the file 'DIR',  you  may

keep  "hidden"  files in this subdirectory.   You  may  thus

leave  a file for someone to download,  tell them by leaving

them a message (which they will get automatically when  they

connect)  and  no one else connecting will know that  it  is

there.

When the QRU function is invoked either by you using Control

Function key 3 or by 4Z4ZB connecting to you and sending you

the  command :QRU:,  any messages addresses to any of  those

stations  (EG.  G3ZCZ.OUT and ON8IK.OUT) will be transferred

from  you  to  4Z4ZB just as if you had  tried  to  QSP  the

messages  manually or G3ZCZ or ON8IK  had connected to  you.

The file names will be changed from *.OUT to *.OLD.

You  may only use the QRU function with stations  designated

as Store and Forward mailboxes.  Put a list of stations that

you can connect to reliably in your <callsign>.MBX file  and

send  it  to other stations in your LAN also using  PK232COM

Rev  1.36 or higher.   They will then be able to dump  their

mail  messages on you and you on them.  If they do the  same

and  send  you  their file,  then you will be able  to  send

messages via them for stations that they can work.

While  QRU gives you the capability to bulk upload  messages

to  another station in your local area,  when you take  your

machine off line,  it may also be used to transfer  messages

between  two  LANs (such as the Baltimore and Washington  DC

Areas) via well sighted gateway digipeaters.

7.5 Remote Beacon shutdown.

I recognize that some stations may abuse the beacon facility

of  this  program.  I  have thus built into  the  program  a

'remote  shutdown' capability.   If someone running PK232COM

has their beacon timer set too often, connect to them in the

AMTOR   or  Packet Communications modes,  and tell  them  to

:QRT:  which will clear their beacon mode and take them  off

the air for a while at least.

In  the  Packet communications mode,  the CQ beacon is  shut

down but the 30 minute mail beacon is not touched.   In  the

AMTOR communications mode,  the CQ delay time is set to 255.

The  Mailbox  will thus still be active in both  modes,  but

will not beacon on the air at the frequency it was set at.

APPENDIX 0. UPDATES/REVISION HISTORY

1.30 PACKET  MODE  Automatic Logging of  Connects.  Automatic  CQ

beacon added.  AMTOR Time shown in 'OVER' sequence. Callsign

sequence text added in ARQ call.  Automatic CQ beacon  added

with  logging  in  beacon  mode.    NON  PACKET  MODES  Semi

automatic logging added.   Call sequence changed to one line

except for CQ calls. Retry sequence works in all modes.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 46

1.31 Contest  Mode.  Function key 3 operation slightly  modified.

AMTOR  answering machine added.  Overwrite of status  window

fixed.

1.32 Packet Mode Store and forward LAN :QSP: capability added.

1.33 :QBM:   message/file   downloading  added  in  Packet  Mode.

Digipeat traffic mode logging added. File handle bug.

1.34 File  handle  bug fixed,  logging/capture file  opens/closes

with  each  QSO.    Digipeat  detect  works  for  reciprocal

operators, ie >G3ZCZ>  for G3ZCZ/4X. Logbook file created if

non  existant.  Some changes in AMTOR beacon  signal  recog-

nition algorithm. PK232COM.DIR directory added.

1.35 Digipeat   detect  now  stores  data  as  well  as   address

information on disk.

1.36 LAN :QRU:  store and forward added, code condensed.  Display

of  Multiple  connect  mode added.  Changes  in  NC/L  :QBM:

changed  to :QRV:  in message mode only.   Digipeat  logging

does not indicate times of logging on/off.

1.37 AMTOR  now doesn't send Brag Tape in non beacon automatic CQ

mode.   Timeout added in PACKET Beacon mode to stop  station

connecting and holding system.

1.38 V and H (direct Packet mode VHF/HF) added to function key 6.

Code  condensed  again.  AMTOR  ROBOT override  manual  mode

added. Packet loop back call added.

1.39 CQ text change added to Alt F6.   AMTOR non beacon mode auto

CQ sends a callsign sequence and +?  when someone calls  in.

Note  feature added to Cont F3.  QRU echo's QSP  to  sending

terminal.   Printer toggle added.  RTTY 'RY' changed back to

single line in CQ mode.   AMTOR CQ works with EPROM  version

4/87.  Digipeat Display function now toggled on/off by Alt F

key (default is OFF).  CQ delay time now adjustable on-line.

Minor code changes within the program.

1.40 Thirdpary  callsign  correct in 'QNO 1' response  situation.

Alt R key (Message Relay Capability toggle) added.  TNC type

line  added to PK232COM.SYS.   TNC2 code  added.  Additional

Baud rates added.   Internal code condensed. An active Store

and  forward  Relay status is indicated in  connect  message

'*** PK232COM 1.40' by the 'R' suffix. SIGNAL mode [G] added

to  function 6.  Digipeat display function also now controls

'MDIGI'.

1.41 Packet  mode Baudrate,  shift reverses (change in 1.40)  now

done in command mode.  Delay added in Alternate function key

2 (TNC Initialise routine).  Commands transmitted in command

window, replies now show up in INwindow.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 47

1.42 Path  determination  capability  (get  other  stations  'MH'

list), :QMH: added, multi user modes including conferencing,

code  compaction.  Packet  mode Alert  function  added.  '/'

character  added  in message header between date  and  time,

Terminal baud rate now 1200/2400/4800/9600,  Receive  buffer

expanded  from  128 bytes to  1K  bytes.  QTC_Snatch  added.

Packet mode Target call function added.  Outgoing text color

changed  to  light green (shows up better on mono  CRT  used

with  CGA).  Timetag  of logged events now  includes  dates.

:QSM:  for repeat of messages,  AMTOR Robot/Mail Beacon/Mail

headers  now without ?  marks.  Digipeat display and logging

function  (Alternate F key) now works for TNC2.  Alt B  key,

Direct Baudot/ASCII baud rate settup added.  Alt F5 or  'MH'

now has own window.  QTC_Snatch bug in TNC2.

1.43 QTC_Snatch  bug fixed.  Target call function now also  works

properly  via  digipeater.  'DCD on' not sent to  non  PK232

TNC's.

1.44 'MH'  window  module  code changed.  NET/ROM path  added  in

PK232COM.DIR   file.   TNC1 capability added.   Remote  TNC2

disconnect now returns you to command mode.

1.45 TNC specific code modified.   PgUp key function added. AMTOR

function  key  8 now turns off transmitter and puts  you  in

ALIST   mode.   Mailbox   added  to   AMTOR   mode.    AMTOR

Robot/Mailbox  call recognition algorithm improved.   Target

call  changed  to Alternate T key.  Alt C key  now  same  as

Function key 7.   MH,  Help and DIR windows displays, freeze

until  key depressed.   Input buffer size increased  to  8K.

Check  added  on setting of CQ Delay.   Dialog condensed  to

save memory space.

APPENDIX 1. ANOMALIES (and BUGS)

A1.1 Since  the  Program  right  now has  no  way  to  check  the

command/converse mode status of the TNC, you may see various

cmd: and error messages from the TNC when using the function

key  commands.   Ignore them for now.   If you get an  error

message, try using the function key again.

A1.2 In the AMTOR Beacon CQ mode,  if the other station drops the

link while you are recording the text, the program will time

out  before logging the QSO and restarting the CQ  sequence.

If  someone else calls in before the timeout  has  occurred,

the text from both stations should be captured to disk,  but

the  log  entry may not show both stations,  thus the  'LOG'

must be compared with the 'RUN' file for QSL purposes.

A1.3 In the AMTOR Beacon CQ mode,  if another stations sends a +?

in  the  FEC  mode,  the  beacon will attempt  to  send  the

PK232COM.TXT file.  The PK232 however is not in the converse

mode at that time and will interpret the text of the file as

commands.   If  by some chance,  the text corresponds  to  a

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 48

valid  command  the  PK232 will execute it and you  may  not

notice  that it did until you begin to wonder why nobody  is

replying to the beacon.   As such I recommend that the first

character  of  each line in the file PK232COM.TXT be  a  non

valid command character such as a number or a period (.)  or

another punctuation sign.

A1.4 PK232COM versions 1.00 to 1.39 were written for a PK232 with

an Oct 1986 ROM.   There may be differences with later ROMs.

Version 1.39 works with the April 1987 ROM.

A1.5 If you are using a CGA card with a monochrome  monitor,  the

text  you  type into the OUTWINDOW will be fainter than  the

incomming text.   This is because it is set to light  green.

If  you  are using a Hercules compatable card you  will  not

notice any difference.

A1.6 The  conference mode has been tested on a PK232 and a  SANYO

MBC-675  which  has a clock speed of 4.7MHz.   If  you  have

problems  with  intermittent selection of  IO  channels  try

slowing down your CPU clock.  There is a software delay loop

in  the program that is used to allow the TNC time to switch

IO streams after receipt of the stream change command.  This

delay is there because PK232COM tests for a connect on  each

channel before outputting any text, so that it does not send

unaddressed packets on non connected IO channels.

A1.7 A  Packet conference Mode disconnect returns the TNC to  the

command  mode,  even if you remain connected to one or  more

other stations.

A1.8 When using the non packet modes the 'long' time the software

takes  to rewrite the screens for each character means  that

there is a  delay in fiollowing the typing.  If this bothers

you,  set  EAS  OFF for manual operation (from  the  command

mode),  but don't forget to turn it on when using the autocq

mode.

A1.9 Strange Events.

If  the TNC does not use an interrupt driven serial port you

may  experience problems when using baud rates greater  than

1200.   The PK-232 does not like PK232COM to use RS-232 baud

rates faster than 2400,  and sometimes not even  that.   Use

1200  until  you have tested and are sure that a faster  one

will work.

IF  YOU  HAVE ANY STRANGE PROBLEMs AND ARE USING  an  RS-232

TERMINAL  BAUD RATE to the TNC WHICH IS GREATER  THAN  1200,

set  the  Terminal Baud Rate to 1200 and see if the  problem

goes away.

As there are many different configurations of IBM PC  Compatables

and TNC's, I can't test all of them before releasing a version of

the program.   If a function does not work on your configuration,

after retrying it using 1200 bauds on the RS-232 line between the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 49

computer  and the TNC (see Section A1.9),  inhibit it and let  me

know  about  it.   It  will be made to  work  in  the  subsequent

release.

APPENDIX 2. Local Area Network Capablity

VHF  Packet  radio systems can be considered as part of  a  Local

Area  Network (LAN) in which messages can be left by one  station

in the computer belonging to a second station.  At HF the same is

true,  but  the area becomes greater.   The basic problem here is

that  people  can only send and receive messages to or  from  you

when you are on-line.   To compensate for this, BBS stations were

developed  which allowed both messages and bulletins to be stored

by anyone for later retrieval.

PK232COM  allows you to store messages in your computer so as  to

use  it as an automatic answering machine.   Thus by storing  the

messages in a file called by the name of callsign,  with the file

extension  '.OUT',  anyone  connecting to the computer  will  get

their  message without having to request it.    PK232COM  Version

1.32 extends that concept to cover the eventuality of you wanting

to take your computer off-line for some reason.  You may now load

the  message  into any other computer you can  connect  to  using

elements  of  the Q code adapted to the High Level  Network  Com-

munications  Language  (NC/L)  proposed in my book  Software  for

Amateur Radio,  (Book number 1560) published by TAB  Books,  Blue

Ridge Summit, Pa. 17214 USA.

A2.1 LAN Protocol (G3ZCZ Version)

The  protocol is as follows.  When connected to someone  who

has  their  computer configured as a host,  if you  want  to

store  a  message you send the following instruction to  the

other station :QSP: <callsign> where  <callsign> is the call

of the station that the message is for,  not the callsign of

the  host  station  in whose computer you  are  storing  the

message.  [Note  use  only  one space  character  after  the

:QSP:].   PK232COM  sends the  :QSP:  message  automatically

using Control Function Key 3.

For  example  if  you want to store a message for  4Z4ZB  in

4X6AA's computer which is configured as a Store and  Forward

system,  you  would first connect to 4X6AA and then send the

message as

:QSP: 4Z4ZB .

Better still use function key 3 to automate the sequence.

The  computer at 4X6AA will respond either with a  statement

saying  that  it is ready for you to go  ahead,  or  send  a

message saying that it can't comply.  If it is ready you get

a positive reply which will take the form :QRV:   <callsign>

which if you know the Q code, means " I am ready to accept a

message for  <callsign>".

At this time you may go ahead and send the message.   If you

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 50

type  the  message at the keyboard,  you may  use  either  a

control  Z  (^Z) character or the character  sequence  :EOF:

followed  by a carriage return (the ENTER key) to  terminate

the  message.   If you have first written the message into a

text file you may then send it using function key 2  and the

termination sequence is automatic.

Once  you  have  completed the  message,  the  other  (host)

computer  will  either  reply  that  the  message  has  been

successfully stored or give you an error message.

If the message is stored and ready to be sent next time  the

addressee  connects  to  that computer,  you  will  see  the

message :QSL:  on your screen.  If something went wrong, you

will  get  back  a negative message taking  the  form  :QNO:

followed  by  a  number.   The  number  tells  you  why  the

operation failed.

A2.2 Message Format

The  message  is stored just as if you had left it  in  your

system  (except that a header is added identifying the  time

of reception and the call of the sending station).   It will

contain  a header showing the date and time it was  received

and  your  callsign.   Should  a message  for  that  station

already be in the system,  yours will be appended to it.  In

the  event  the your upload is aborted,  the amount of  text

received  before  the abort occurred will be stored  as  the

message.

When you disconnect from the host station,  its mail  beacon

will be updated.

Once  the  message  is loaded in the host,  it can  only  be

deleted  by  the operator of the  host  station.   When  the

addressee connects to the host and receives the message, the

file type will change from '.OUT' to '.OLD'.

A2.3 File downloading

See section 7.4 for the use of :QBM: for downloading text or

ASCII files from another PK232COM system.

A2.4 Path Determination to a Dx station

If  you  want  to establish a digipeat  path  to  a  station

somewhat out of your direct range, you need to know which of

the  stations that you can connect to can hear that  desired

DX  station.   If  you could get a call monitored (MH  list)

from the stations that you connect to,  you would be able to

see  if  the  station you are connected to  has  heard  your

desired DX station.

PK232COM  uses  the :QMH:  command to request such  a  list.

When   PK232COM  (Version 1.42+) receives a  :QMH:  command,

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 51

it  drops  back  to the TNC command mode and issues  a  'MH'

command  to the TNC (except to a TNC1 which does not support

it).  It stores the list received in a temporary buffer, and

then  returns  to the converse mode and transmits  the  list

back to the station it is linked to.

By  judicious use of :QMH:  you can determine paths to other

stations.   Note however,  that just because one station can

hear another station,  it does not mean that it can work it.

For  example,  the  station  you are connected  to  (running

PK232COM Version 1.42 or greater) may be using a power level

of  1 watt or so,  while the station 200 miles away that  it

heard was using 100 watts.   Test the path yourself,  or/and

leave a message asking how reliable the connect path between

those two stations is.

If  you and a small group use a non standard packet  channel

for  inter-group communications,  you may also use the :QMH:

function to find out who the station you are connected with,

has heard recently, when you first sign on the LAN channel.

APPENDIX 3     BASIC NC/L DICTIONARY

Messages can be transferred into any other PK232COM computer  you

can connect with using elements of the Q code adapted to the High

Level  Network Communications Language (NC/L) proposed in my book

Software for Amateur Radio,  (Book number 1560) published by  TAB

Books, Blue Ridge Summit, Pa. 17214 USA.

To  receive  a message,  do nothing,  you will receive your  mail

automatically  when  connecting/linking with a PK232COM  station.

You cannot normally read messages adressed to another person.

A3.1  :QBM: To download a file, send

:QBM: filename.type

The filename.type is the file you want. For example

:QBM: dir

:QBM: help

:QBM: pk232com.doc

A3.2  :QMH: To  request  a  call monitored list ('MH')  from  the

station  that you are connected with  using  PK232COM

(1.42+), send

:QMH:

A3.3  :QSM: To request a repeat of a message from a station using

PK232COM (1.42+), send

:QSM:.

This  command  will  be valid as  long  as  the  host

station  has  not deleted the <yourcall>.OLD file  on

his  disk.   If  the file does not  exist,  you  will

receive a :QNO: 2 'error' response.

A3.4  :QSP: To leave a message, send

:QSP: callsign

the callsign is that of the station you wish to leave

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 52

a message for.

A3.5  :QRT: To  shut  down an  AMTOR/Packet  mode  Mailbox/beacon

station which is causing QRM,  link or connect to the

station and send

:QRT:

A3.6  :QRU: To  upload messages from one PK232COM (1.38+)  system

to your computer, send

:QRU:

Computer responses

A3.7  :QNO: 'NO' or error

QNO Error Values.

The  following  error numbers are  associated  with  message

store and forward operations.

?    What you typed was not an NC/L command.

1    Computer not configured as Store and Forward system.

2    Requested ASCII file/ message (:QBM:) does not exist.

3    You  made an error in the name of the callsign for whom

the  message  is  intended  (It  must  be  at  least  3

characters long).

4    File creation error in host system.

5    Error occurred during reception and storage of message.

Could  be  that the computer ran out of  space  on  the

disk,  or  something  else  went wrong in  storing  the

message.

6    :QRU:  You  are not authorised as a store  and  forward

mailbox.

7    :QRU: Error in opening <callsign>.MBX file.

8    :QRU: Error in closing <callsign>.MBX file.

9    :QRU:  Sequence   Error  in callsign of message to  go.

The bad callsign will be shown after the error number.

90   NC/L defined function not implemented in this release.

99   PK232COM compatable program, but requested function has

not been implemeted.

A3.8  :QJG: The  QRU  sequence is complete.   There are  no  more

messages pending.

A3.9  :QRV: callsign

The  computer is ready you to send the  message.  End

the message with a control Z (^Z) character,  or  the

sequence :EOF: .

A3.10 :QSL: callsign

Confirms receipt of message to that callsign.   It is

also used to acknowledge receipt of a ':QRT:' comand.

A3.11 :QTC: messsage list.

This  precedes a list of callsigns for whom  messages

are  stored up on a computer.   It is used in  Packet

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 53

Beacon  transmissions or AMTOR Beacon mode CQ  calls.

If PK232COM receives :QTC:  it will respond with  the

beacon text showing the mail queue.

You can configure your version of PK232COM to issue a

QTC_Snatch connect request to any other station using

the  PK232COM beacon transmission to signal that  you

have a message waiting.

PROPOSED EXTENSIONS

A3.12 :QYU:     YAPP format file upload.

A3.13 :QYD:     YAPP format file download.

APPENDIX 4. USING OTHER TNC'S.

PK232COM  is  designed for the PK232.   However the  PACKET  mode

capabilities  can  be  used  on other  TNC's.   There  are  minor

differences in the command dialog between different  TNC's.   The

dialog  differences affecting the operation of PK232COM are built

into  the  program  and set when the TNC type is  read  from  the

PK232COM.SYS file at the program load time.  The non packet modes

will obviously result in error messages if attempted on a  packet

mode TNC.

The   following  notes  apply  to  different  TNC's  or   digital

communications controllers.

A4.1 TNC2 (MFJ 1270)

A4.1.1     Default Commands.

Edit  the  PK232COM.SYS file taking out the  PK232  specific

commands,  namely  anything  that is not mentioned  in  your

manual.   Add  in the default state for the TNC commands not

included  in the file and make sure that you set  AUTOLF  ON

and MCON ON.   If you are going to use the conference  mode,

set the multi user defaults to show the callsign in the text

header that the TNC sends to the computer.

A4.1.2     Program differences.

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

MAIL Packet terminal mode (Alternate Function key 4).

CQ/BEACON Packet terminal mode (Alternate Function key 4).

TNC Transmit Buffer flush (Control Function key 9).

4.2 TNC1 (HD-4040)

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 54

A4.2.1     Default Commands.

Delete or rename the PK232COM.SYS file.  Rename the TNC1.SYS

file to PK232COM.SYS and edit it using your wordprocessor in

its non document mode to customize it to your station.

A4.2.2     Program differences.

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Day/Time functions (Control function key 1).

'MH' and remote :QMH: functions (Alternate Function key 5).

MAIL Packet terminal mode (Alternate Function key 4).

CQ/BEACON Packet terminal mode (Alternate Function key 4).

TNC Transmit Buffer flush (Control Function key 9).

Conference and multi-connect modes.

APPENDIX 5. NOTES

I'm   always  interested  in  your  bug  reports,   comments  and

suggestions for changes.  In particular, what if any, binary file

transfer mode(s) should be supported.

For  the  latest  information on  Updates,  read  the

UPDATE.* files on the distribution disk.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.45  PAGE 55

APPENDIX 6 REGISTRATION FORM.

To: Joe Kasser G3ZCZ

P O BOX 3419

SILVER SPRING

MD 20904.

CALL ________________   TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY    ___________________________________________

STATE   _________  POSTCODE  ______________________

TNC TYPE _____________________

I enclose a check for $35.00.  Please send me the latest  version

of PK232COM customised to my call and TNC,  and resister me as  a

user  of PK232COM.   I am currently using Shareware Version _____

which I obtained from  _________________________________________.

The  first  10 lines of my current PK232COM.SYS file  are  listed

in the blank space below.

Signature

Comments, likes, dislikes, wish list, etc.

PK232COM.DOC (c) G3ZCZ 1987

##################################################################################