Source is  TS Kelso

SYSOP, DataTech Node 019

BBS # ===> (512) 892-4180

Austin, TX

- NASA Prediction Bulletins -          May, 1986

Effective January 1986, this system began posting the most recent element sets

received from NASA Goddard for the following satellites or satellite series:

SeaSat 1, Salyut 7, Radio, GOES, NOAA, LandSat, UOSAT, OSCAR, and Meteor.

These elements will be maintained in ASCII format in the file NASA-PB.xxx,

where xxx indicates the current file number (a number from 000 to 999) and

will serve to indicate whether the file has been updated recently.

Data for each satellite will consist of three lines in the following format:

AAAAAAAAAAA

1 NNNNNU NNNNNAAA NNNNN.NNNNNNNN +.NNNNNNNN +NNNNN-N +NNNNN-N N NNNNN

2 NNNNN NNN.NNNN NNN.NNNN NNNNNNN NNN.NNNN NNN.NNNN NN.NNNNNNNNNNNNNN

Line 1 is a eleven-character name.

Lines 2 and 3 are the standard Two-Line Orbital Element Set Format identical

to that used by NASA and NORAD.  The format description is:

Line 2

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

10-11     International Designator (Last two digits of launch year)

12-14     International Designator (Launch number of the year)

15-17     International Designator (Piece of launch)

19-20     Epoch Year (Last two digits of year)

21-32     Epoch (Julian Day and fractional portion of the day)

34-43     First Time Derivative of the Mean Motion

or Ballistic Coefficient (Depending of ephemeris type)

45-52     Second Time Derivative of Mean Motion (Blank if N/A)

54-61     BSTAR drag term if GP4 general perturbation theory was used.

Otherwise, radiation pressure coefficient.

63-63     Ephemeris type

65-68     Element number

69-69     Check Sum (Modulo 10)

(Letters, blanks, periods = 0; minus sign = 1)

Line 3

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

09-16     Inclination [Degrees]

18-25     Right Ascension of the Ascending Node [Degrees]

27-33     Eccentricity (decimal point assumed)

35-42     Argument of Perigee [Degrees]

44-51     Mean Anomaly [Degrees]

53-63     Mean Motion [Revs per day]

64-68     Revolution number at epoch [Revs]

69-69     Check Sum (Modulo 10)

All other columns are blank or fixed.

Example:

NOAA 6

1 11416U          86 50.28438588 0.00000140           67960-4 0  5293

2 11416  98.5105  69.3305 0012788  63.2828 296.9658 14.24899292346978

Note that the International Designator fields are usually blank, as issued in

the NASA Prediction Bulletins.

Satellites will be ordered by their NASA Catalog Number.  The data file will

be updated as soon as possible after receipt of new element sets or whenever

element sets are received for the Space Shuttle.

Hopefully, this service will provide a central location for users to obtain

this data in a timely manner and help eliminate the tedious process of

entering data directly from the NASA Prediction Bulletins.

If any users wish to see other satellites included or have suggestions for

improvements in these data dissemination procedures, please leave a

message/comment for the SYSOP and I will get back to you.

- TS Kelso

#se leave a

message/comme