BEGIN_DOC Psychro.doc version 1.1, Aug. 4, 1993 J. Weisbin.

Psychro for the HP48 S and G series calculators, (c) 1993

James Weisbin, is a program for doing psychrometric calc-

ulations. Both SI and English units are supported, in a

manner consistent with the HP48 Equation Library. The

program is written entirely in System-RPL, for speed.

Psychro 1.1 is distributed as a library with ID number

1766, approximate size 8 Kbytes. Psychro is "shareware",

$15.00 U.S. to register. Send check to James Weisbin,

115 West Broadway, New York City, New York, 10013 U.S.A.)

e-mail: jsw@well.ca.sf.us or 73457.3462@compuserve.com

Registered users will get full documentation including

worked examples. Future versions may include a plot mode.

Changes from version 1.0:

1.  Added a fast repeatable key action to the scrolling

selection bar (hold down any of the arrow keys and

scroll).

WARNING AND DISCLAIMER: No claims or guarantees of accuracy

or completeness are in any way made or implied by this author.

USE AT YOUR OWN RISK.

This program can be copied and distributed provided that

this document accompanies it, and no modifications are

done without the author's permission.

PLEASE BACK UP YOUR CALCULATOR'S MEMORY BEFORE USING THIS

SOFTWARE. I am in no way responsible for any loss of data

through the use or misuse of this program. (Etc..!)

Psychro v1.1 is distributed as an auto-attaching library,

ID number 1766, size 8144.5 Bytes, checksum 70DB.

To install the library, first transfer the binary file

Psych11.lib to your machine using Kermit in Binary transfer

mode on both the HP48 and the host machine. Then put the

library on the stack, by pressing the menu key that shows

up labeled Psych11.lib (only PSYCH will appear in the menu).

You should then see this on the stack:

Library 1766: PSY...

Decide which port you want to store the library in. Note

that port 0 doesn't get backed up when you use the Archive

command, so you might want to use port 1 or 2 if you have

a battery backed card in one of those. Enter the number of

the port (e.g.: 1) and then press STO. Then turn your

machine off and on again. The screen will flash off and

back on again when the '48 reconfigures itself. Now press

the LIBRARY key, (right shift 2 on the GX) and keep pressing

NXT until you see PSYCH on the menu. If you don't see it,

it hasn't been installed properly - try again. To uninstall

the program, go to the MEMORY menu on the SX, or left shift

LIBRARY on the GX. Enter 1766 on the stack and press DETACH,

then PURGE the library by doing :port#:1766 PURGE.

To run the program, make sure that you have switched to

the directory where you wish to keep your data. Note:

Psychro "auto-attaches" at start-up, and is therefore

available in any directory, including the home directory.

If you've managed to install Psychro correctly, then go to

the directory that you wish to run Psychro from. Now press

PSYCH from the library menu to start the program.

You will now see a display with the title PSYCHRO, and

the following items:

Dry Bulb & Wet Bulb

Dry Bulb & Rel Humid

Dry Bulb & Dew Point

Dry Bulb & Hum Ratio

Dry Bulb & Vap Press.

Mixture Calculation

The top item will be highlighted (not the title). Press the arrow

keys to select the set of knowns you want to use. If you keep

pressing the arrow keys, you will get to the next "page", which

has only the one item Change Pressure.

The menu keys allow you to select either SI units (flag 60 clear)

or ENGlish units (flag 60 set). HELP brings up 2 pages of help.

VARS allows you to look at the results of your calculations (with

units attached) and copy them to the stack if desired (see VARS

instructions below). GO is the same as ENTER and means "do this

calculation now". EXIT quits the program. More details follow:

GO     Carry out the selected calculation. You will be prompted

for the data in either SI or ENG units. PSYCHRO does

all of its computations internally with real numbers

and then appends either SI or ENGlish units when in

VARS mode. You can append units at the prompt, but they

will be ignored - only the real part will be used. This

is also true even if you use the interactive stack to echo

a previous result into the command line. So, for example,

when in SI mode, entering '75_psi' at the temperature

prompt will be interpreted as 75 degrees C.

When the calculation is done, you will automatically be placed

in the VARS menu/display, and you can scroll around with the arrow

keys to select the data. Press ENTER or ->STK to copy the data to

the stack, and answer when prompted "copy what data" (one or all).

Press VIEW to see the selected data if not all of it is visible.

Press MAIN to exit VARS mode.

NOTE: If you change the pressure, and then use VARS without

first doing any calculations, all the results will be your old

data with the new pressure and will therefore not be correct.

REMEMBER TO RECALCULATE.

Mixture calcs: These are the same as above, but you will be

prompted for two sets of data (State 1 and State 2) and the

volume at each state.

NOTE: Volumes at each state are coverted to a mass AT THAT

CONDITION (ie using the density at that state) and then added

together for the total mass, which is then converted back to

volume at the final mixture condition. That's why the final

volume will usually differ from the sum of the 2 volumes given.

EXIT   quit the program. EXIT can also be initiated

by pressing the backspace key or ON (ATTN).

The following key presses are supported:

right-arrow,

down-arrow                   next item (with repeat)

up-arrow,

left arrow                   previous item (with repeat)

left-shift right-arrow,

left-shift down-arrow,

next                         next 6th item.

left-shift left-arrow,

left-shift up-arrow,

prev                         prev 6th item.

right-shift right-arrow,

right shift down-arrow       go to last item.

right-shift left-arrow

right-shift up-arrow         go to first (top) item.

right-shift OFF              pauses program and turns the machine

off. The 48 will beep when restarted,

and resume the operation of Psychro.

Psychro creates a global variable (library object) named

psyPAR, to which it saves the last results of your calcs.

The variable is written and read using utilities provided

as part of Raymond Hellstern's LIB library generator/decom-

piler (thanks to Raymond and Detlef Mueller for RPL48).

NOTE: The library ID (1766) is encoded in psyPAR, and

therefore can't be changed by simply decompiling the

library and editing $ROMID.  If you need to have a different

number, I'll send you a recompiled lib with the number of

your choice (registered users only).

Use this program to start Psychro if you always want your data

saved to the same directory (eg mydir):

%%HP: T(3)A(D)F(.);

\<< PATH \-> p

\<< HOME mydir

PSYCHRO p EVAL

\>>

\>>

The program checks for data within the following ranges:

dry bulb, wet bulb, and dewpoint temp: -60 to +70 degrees C

Pressure: 25 kpa (>+10,000 feet) to 200 kpa (< approx -2000 feet?).

Relative Humidity: 0 to 100%  (see note 1 below).

Humidity Ratio: 0 to .28 kg/kgda (see note 1 below).

Vapor Pressure: 0 to 32 kpa

Volume rate: 0 litres/second to MAXREAL.

The program may be innacurate or produce errors at extremes

approaching the values given above.

NOTE 1: Most psychrometric programs will not allow 0 to be input

for vapor pressure, humidity ratio, or relative humidity, since,

for perfectly dry air, "dew point temperature" is not defined.

Instead, I have allowed this input, and set the dew point temp.

to absolute zero. DO NOT INTERPRET A DEW POINT TEMPERATURE AT

ABSOLUTE ZERO TO BE VALID DATA. Use your common sense, and throw

out data which does not make any sense, such as negative values

for vapor pressure, humidity ratio, etc.

NOTE 2: The psychrometric "chart" is inherently inaccurate at low

temperatures, and will not agree with this program at extremely low

or high temperatures or pressures.

NOTE 3: Psychro converts all input to SI and does its calculations

internally in SI, then converts back to english if necessary.

Therefore, if you input a temperature, for example, of 70 deg F,

you might find that its changed to 69.999999 degrees on output.

Close enough for engineering work.

References:

ASHRAE "Book of Fundamentals" 1981

Psychro has been tested on rev "B" (SX) and rev "M" (GX)

calculators and should run on all models. Only supported

entry points are used, but please, PLEASE, please remember

to back up your data BEFORE running Psychro, just in case.

James Weisbin 115 West Broadway, New York City, New York,

10013, U.S.A.  email:jsw@well.sf.ca.us (James Weisbin) or

73457.3462@compuserve.com

END_DOC