#

File: akai.info,  Node: Top,  Next: (canon.info),  Up: (remotes.info)

******************************************************************************

AKAI

******************************************************************************

Version:		3.2

Date:			10 feb 94

* Menu:

* Technical info::

#

File: akai.info,  Node: Technical info,  Prev: Top,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Coding type:		Space

Carrier: 		39.2kHz

Space between trans:	11000us

+--------------------------------+                +--+

Header:      |                                |                |  |

..+                                +----------------+  +..

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+        +--+

Stop code:      |                                |        |  |

..+                                +--------+  +..

16T                  4T     T

8.8ms                2.2ms

T = 550us

The code is only transmitted once.

After that a stop code is transmitted.