#

File: oldpana.info,  Node: Top,  Prev: (nec.info),  Next: (oldtech.info),  Up: (remotes.info)

******************************************************************************

OLDER PANASONIC

******************************************************************************

Version:		3.2

Date:			20 Nov 93

* Menu:

* Technical info::

* Video::

* TV::

* Portable::

#

File: oldpana.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:	22 bits

Carrier:	32kHz

+--------+      +..

Header:   |        |      |

..+        +------+

8T     6T

+--+    +--+

1 is coded:  |  |    |  |

..+  +----+  +..

2T  6T  2T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

2T 2T 2T

T=420 us to approx 424 us (US of A and Canada)

T=454 us to approx 460 us (Europe and others)

Remote control transmitter data code (MN6030)

Head pulse  Custom code  Data code  Inverse custom code  Inverse data code

(8T high,   (5 bits)     (6 bits)   (1<==>0)             (1<==>0)

8T low

Followed by 90T low (for sync) = 41130us (Europe) = 37980us (USA)

#

File: oldpana.info,  Node: Video,  Prev: Technical info, Next: TV, Up: Top

******************************************************************************

Video

******************************************************************************

Code	Function

----	--------

#402	vtr

#6C2	timer rec

#202	1

#222	2

#242	3

#262	4

#282	5

#2A2	6

#2C2	7

#2E2	8

#302	9

#322	10/0

#482	-/--

#442	Ch +

#462	Ch -

#142	play

#002	stop

#042	rew

#062	ff

#0C2	pause

#102	rec

#182	still adv

#1E2	slow

#583	prog

#743	next

#0C3	vps

#5A2	clock/counter

#642	memory/repeat/search

#662	counter reset

#6A2	remaining time

#7A2	audio out

#

File: oldpana.info,  Node: TV,  Prev: Video, Next: Portable, Up: Top

******************************************************************************

TV

******************************************************************************

Code	Function

----	--------

#400	On/Off

#200	1

#220	2

#240	3

#260	4

#280	5

#2A0	6

#2C0	7

#2E0	8

#300	9

#320	0

#480	-/--

#420	C

#440	Ch +

#460	Ch -

#160	Mute

#540	TV/AV

#0E0	N

#580	<-|-> -

#560	<-|-> +

#1C0	Color -

#180	Color +

#5C0	Contrast -

#500	Contrast +

#1E0	Brightness -

#0C0	Brightness +

#380	Treble -

#4E0	Treble +

#3A0	Bass -

#5E0	Bass +

#740	TV

#760	TEXT

#3E0	Alarm

#780	Mix

#380	Reveal

#4E0	List Store

#5E0	Index

#680	Hold

#6E0	Time text

#6A0	Display cancel

#720	F/T/B

#3A0	List/F. Text

#340	Red

#360	Green

#6C0	Yellow

#7C0	Blue

#

File: oldpana.info,  Node: Portable,  Prev: TV, Up: Top

******************************************************************************

PORTABLE

******************************************************************************

Code for: Panasonic portable stereo component Model RX-DS660

ON    = 2093EDh

TAPE  = 36912Dh

TUNR  = 37910Dh

CD    = 35914Dh

AUX   = 3890EDh

VOL-  = 25934Dh

VOL+  = 24936Dh

1     = 16952Dh

2     = 17950Dh

3     = 1894EDh

4     = 1994CDh

5     = 1A94ADh

6     = 1B948Dh

7     = 1C946Dh

8     = 1D944Dh

9     = 1E942Dh

10    = 1095EDh  <- "0" KEY

10+   = 15954Dh  <- "." KEY

_TAPE_

<<    = 297ADh

>>    = 3978Dh

REC   = 896EDh

<     = 996CDh

>     = A96ADh

STOP  = 97EDh

_CD_

2D924D	REP

2B928D	<<_<  (SKIP/SEARCH)

2A92AD	>>_>  (SKIP/SEARCH)

||    = 2992CDh  ||

PLAY  = 2C926Dh  PLAY

PROG  = 2F920Dh  PROG

CANCL = 3091EDh  CANCL

With best regards,

Enrique Chavez                     echavez@as.arizona.edu