#

File: canon.info,  Node: Top, Prev: (akai.info), Next: (denon.info),  Up: (remotes.info)

******************************************************************************

CANON

******************************************************************************

Version:	3.2

Date:		15 Nov 93

* Menu:

* Technical info::

* Videocamera::

#

File: canon.info,  Node: Technical info,  Prev: Top,  Next: Videocamera,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Carrier:		37.5kHz

+--------------------+

Header:      |                    |

..+                    +--------

16T            8T     (8.8ms + 4.4ms)

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +....

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +....

T  T  T

T = 550us

Space between transmissions = 40ms

#

File: canon.info,  Node: Videocamera,  Prev: Technical info,  Up: Top

******************************************************************************

VIDEOCAMERA

******************************************************************************

GROUP	#A16E

Code	Function

----	--------

#C03F	Start/Stop

#B847	Zoom wide

#387C	Zoom tele

#807F	Play

#E817	Stop

#40BF	Rew

#00FF	FF

#20DB	Pause/Still

#E01F	REC

#708F	Char title

#F00F	Date

#B04F	Count reset

#609F	Tape return

#D827	On Screen