#

File: nec.info,  Node: Top,  Prev: (mitsubi.info),  Next: (oldpana.info),  Up: (remotes.info)

******************************************************************************

NEC

******************************************************************************

Version:		3.2

Date:			20 Nov 93

* Menu:

* Technical info::

* Video::

#

File: nec.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Carrier: 		39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+                +..

Header:      |                                |                |

..+                                +----------------+

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+                +--+

Stop code:      |                                |                |  |

..+                                +----------------+  +..

16T                      8T        T

8.8ms                   4.4ms

T = 550us

Space between transmissions: 11000us

#

File: mitsubi.info,  Node: Video,  Prev: Technical info, Up: Top

******************************************************************************

Video

******************************************************************************

Video1:

GROUP