/*���������������������������������������������������������������������������

[ c�a�l ] by Vikash R. Goel <vgoel@capaccess.org>

���������������������������������������������������������������������������*/

ABOUT CAL:

cal is a simple program for MS-DOS which allows you to exchange programs

and data between your PC and a TI-85 or a TI-82 using the "$5 parallel

link" designed by Andreas Westfeld (aw4@inf.tu-dresden.de). Until now, the

only functional programs for this link were Link85xp (a patched version of

a run-down version of TI's Link85), Connect-85 (a great Windows program

to communicate with the TI-85 and manipulate TI-85 files), and Link92xp

(a program written to use the parallel link with a TI-92). Link85xp and

Connect-85 are exclusively for the TI-85, and Link92xp is exclusively for

the TI-92. cal allows TI-82 owners to take advantage of Westfeld's ingenious

design of a calculator link. I wrote cal with two purposes in mind: to

make a better MS-DOS interface for using my TI-85 with a parallel link,

and to allow TI-82 owners to use the parallel link.

There are other programs, like tran82xp and tran83xp, which are simple

transfer programs for the parallel link, command-line based. cal isn't

supposed to be replacements for these, nor competition or anything like

that. Just use cal. It's good. =)

HOW TO USE CAL:

Without much trouble (hopefully). I tried to make a simple user interface

with access to all the features. When you start cal, you will see a list

of all *.8?? files on the left. The top one, in a different color, is the

selected one. Up, Down, PageUp, and PageDown allow you to scroll through

the list.

To send a file to the calculator, start the receive on the calculator and

then press Enter. This will send the selected file's contents.

To receive a file from the calculator, press R. Once cal says it is

waiting, start the send on the calculator. cal will allow you to give any

name to the file it receives. Since they are standard TI format files, it

would be advisable to use the extensions appropriate (ie 85G for a TI-85

group file, 82P for a TI-82 program, etc.).

When sending or receiving files, cal automagically detects whether the

calculator is a TI-85 or a TI-82. However, if you try to send a TI-82 file

to a TI-85, or vice versa, you will have no success. The Rx "light" turns on

when the computer is receiving or expecting a byte from the calculator. The

Tx "light" turns on when the computer is sending a byte. All the inquiries

and informational messages should appear in the small window near the middle

of the screen.

To capture the screen from the calculator, press S. cal will ask for the

type of calculator. Press 2 for a TI-82, or 5 for a TI-85. After displaying

the captured screen, cal will ask for the name of a file to save the image

to. This will be a standard BMP format file.

Any time cal asks for a file name to write data to, typing nothing will

simply abort the operation. Similarly, pressing Escape while cal is expecting

input from the calculator will abort that operation.

If your link isn't connected to LPT1, pressing Tab will switch between the

parallel ports LPT1, LPT2, LPT3, and LPT4. cal gets the addresses from the

BIOS.

Some extra benefits: Delete will delete the selected file and D will shell

to DOS.

BUGS, POSSIBLE BUGS, AND BUG-LIKE FEATURES IN CAL:

� Keep in mind that this is an alpha release and a lot of stuff probably

doesn't work quite right.

� I don't own a TI-82, so I can't really vouch for the TI-82 features. I've

borrowed my friends' 82s a lot so they should work all right.

� I don't know the format of BMP files. By dissecting a few made with MS

Paint from Windows 95, I think I figured them out enough.

� I'm not exactly positive about the file formats and whether or not they're

fully compatible with TI software, but I'm reasonably sure.

� Sometimes cal ends with a "Null Pointer Assignment" error, but this

doesn't seem to interfere with anything.

� Question - Does LPT4 really exist? I can't find anyone with four parallel

ports, and MSD only lists up to three. If LPT4 is simply a figment of my

imagination, I'd like to know so I can remove it from the list.

� I probably haven't written them all here.

HOPES FOR THE FUTURE:

� Originally I was hoping the TI-82 protocols would be similar enough to the

TI-83's that it would be a trivial matter to add TI-83 support. Alas, I

can't seem to get it right. But it will work. Oh yes, it will work.

� The LCD capture is admittedly kludgey. I may even be able to make it

print.

� I'm thinking of adding a UUDecode feature.

� Maybe an edit-lock feature?

� Does anybody know how to write a ASCII85P-type program? It would be nice

to have a feature that allows you to convert 85P/82P files to ascii

and vice versa.

� I'd like to make slightly better documentation.

ME:

I am Vikash R. Goel. You can reach me at vgoel@capaccess.org. If you have

questions about the link, you should contact the designer of the link,

Andreas Westfeld, at aw4@inf.tu-dresden.de. His web page contains info and

plans for the link at http://www.inf.tu-dresden.de/~aw4/ti85.html.

REGISTRATION:

This software is free. If it breaks you can keep all the pieces. However,

I disclaim all liability for what it might do to your computer, your

calculator, your social life, or yourself. If you have a problem, I would

(probably) be happy to try to help you out, but I am by no means obligated

to do so. Hey, I'm giving it away!

If you use it regularly, I'd appreciate if you would drop me a line and

tell me how it's doing. If you have suggestions, comments, or criticisms,

I'd love to hear them.

CAL HOME PAGE:

You can always get the newest version of cal at the following url:

http://www.geocities.com/SiliconValley/Pines/2242

HISTORY:

Version 0.7�

- first public release

Version 0.8�

- It turns out the BMP saving didn't work at all. Thanks to nobody for

telling me! Fixed now, at least for the TI-85. TI-82 owners, let me

know if something isn't working; I don't own a TI-82.

- BMP is no longer in inverse video.

- The bug which prevented the use of LPT1s other than 378h is fixed.

- Added support for ports LPT1 through LPT4.