ORCHID37.ZIP

---------------

The Zipped file contains the Orchid OS/2 Presentation Manager drivers for

1024x768x16 modes.  These include:

disp3745.dll:   Interlaced 45 Hz runs on

Pro/e

ProII with 4.0 series BIOS

ProIIs

disp3760.dll:   Non Interlaced 60 Hz for ProII with

Pro/e

ProII with 3.0 series BIOS

ProII with 4.0 series BIOS

ProIIs

disp3770.dll:   Non Interlaced 70 Hz

ProIIs

ORCHID29.ZIP

---------------

The Zipped file contains the Orchid OS/2 Presentation Manager drivers for

800x600x16 modes.  These include:

disp2960.dll:   60 Hz refresh runs on:

Pro/e

ProII with 4.0 series BIOS

ProIIs

disp2970.dll:   70 Hz refresh runs on:

ProIIs

Installation Procedure:

-----------------------

Before you begin, OS/2 must already be installed on your Hard Disk Drive.

1. Reboot your system from a copy of your OS/2 Installation Diskette.

2. From the IBM introduction screen, hit <ESC> to get to a DOS prompt.

3. Insert the Orchid OS/2 Driver diskette into the drive.

4. At the DOS prompt, go to the subdirectory pointed to by the LIBPATH in the

environment variable.  In most cases, this will be "C:\OS2\DLL".  Unzip

the ORCHID OS/2 display driver files.

For example,

cd \OS2\DLL                         (Hit Enter)

a:pkunzip orchid37 disp3745.dll     (Hit Enter)

5. Decide on the display driver that you wish to use.

6. Copy the original display driver into DISPLAY.ORI

Copy display.dll display.ori         (Hit Enter)

Rename the Orchid OS/2 display driver into DISPLAY.DLL

For example,

Copy disp3745.dll display.dll        (Hit Enter)

8. Remove the the Orchid OS/2 Diskette from your floppy drive.

9. Reboot the machine.