Date: 09/10/91

TVGA 8800/8900/9000 Video Driver Package

for OS/2 Version 1.2, 1.3

IMPORTANT: Before installing any of the programs in this package, make

sure that you read through this document once.

Download Info

-------------

If you download this file (PM16_256.ZIP) from Trident BBS, please

use PKUNZIP utility to unpack it (well, you may not be able to read this

file if you didn't).  Copy all the unpacked files to a 1.2 MB or 1.44 MB

diskette.  Use the diskette every time you want to install a driver.

Description:

------------

The file named BVHTVGA.DLL is an enhanced version of BVHVGA.DLL,

the OS/2 VGA driver which can be found in directory C:\OS2\DLL.  The purpose

of offering this driver is to exploit the extended features provided on TVGA

chips.  The driver takes care of setting up extended TVGA text and graphics

modes, loading TVGA fonts, 256 color DACs, save/restore screens, HiColor

support, etc.

The file W800.DLL is a customized PM display driver which supports

the 800x600-16 color mode PM session.  This file (if 800x600 16 color mode

is chosen as the desired display mode) will replace the default DISPLAY.DLL

(640x480-16 color standard VGA mode) file in directory C:\OS2\DLL.

The file W1024.DLL is a customized PM display driver which supports

the 1024x768-16 color mode PM shell.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.

The file T640.DLL is a customized PM display driver which supports

the 640x480-256 color mode PM shell.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.

The file T800.DLL is a customized PM display driver which supports

the 800x600-256 color mode PM shell.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.

The file T1024.DLL is a customized PM display driver which supports

the 1024x768-256 color mode PM shell.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.

All these PM display drivers (i.e. W800.DLL, W1024.DLL, T640,

T800 and T1024) are dependent on BVHTVGA.DLL.  That is, BVHTVGA.DLL must

be installed first.  Only one of them may be installed at a time.  Please

note that the VGA monitor and the TVGA card (8800/8900/9000 chip with

enough memory) should have the capability to support the corresponding

resolution graphics. (See "Appendix")

The driver package also provides a utility (OS2CRTC.EXE) for

setting up monitor-specific video parameters in the driver.  When OS2CRTC

is executed, it will generate a file called BVHCRTC.DLL on the TVGA OS/2

driver diskette.  BVHCRTC.DLL is a variation of BVHTVGA.DLL, in terms of

customized video parameters.  The OS2CRTC.EXE should be used in conjunction

with the TVGACRTC.EXE utility.

Another utility named SMODE.EXE is a set video mode program that

can be run in DOS mode as well as OS/2 protected mode sessions.  In DOS

mode, INT 10H is called.  In protected mode sessions, since OS/2 only

supports text mode functions, graphics mode will be reset to text mode

when SMODE terminates.

When using PM default VGA display driver (DISPLAY.DLL), you

may find a portion of PM screen is lost.  You can run PMFIX.EXE in

the DOS box once after system boots up.

BVHTVGA.DLL will provide a base for future TVGA Presentation

Manager drivers.

BVHTVGA.DLL may also be used as a substitute for PMFIX.

Installation:

------------

0. Make a backup copy of the following OS/2 drivers:

C:\OS2\DLL\BVHVGA.DLL

C:\OS2\DLL\DISPLAY.DLL

1. If you have a TVGA 8800 installed, run 8800.EXE from DOS

mode.  This program will do nothing for TVGA 8900/9000.

2. If you need to adjust video parameters to customize

BVHTVGA.DLL, run OS2CRTC.EXE from DOS mode.

3. Switch to OS/2 protected mode session and start

DDINSTAL

4. You will be prompt to insert a driver diskette in drive A.

Insert the TVGA OS/2 driver diskette, then hit the ENTER key.

5. There will be a list of driver choices shown on the screen.

Select the appropriate one (resolution and # of colors).

6. Follow the rest of the procedures.  You may be asked to insert

the OS/2 installation diskette in drive A. In this case,

do steps 7 and 8.

7. When you are prompted to press CTRL-ALT-DEL to restart the

system, do not remove the OS/2 installation disk from

drive A.

8. After system reboot, the driver installation will be completed.

Then, the system needs to re-boot once again.  This time,

remove the OS/2 installation disk from drive A.

Usage of SMODE.EXE:

------------------

The same as its DOS counterpart (from TVGA utility diskette).

Type SMODE without any parameters to view the help screen.

Usage of OS2CRTC.EXE:

---------------------

Joint operation with TVGACRTC.EXE in DOS mode.  In OS/2, please

run OS2CRTC.EXE instead of running TVGACRTC.EXE (which comes with TVGA

driver/utility diskettes).  The file will first invoke the TVGACRTC.EXE

(hence create TVGACRTC.SYS for OS/2 DOS mode), and then read the

TVGACRTC.SYS to modify the video parameters in BVHTVGA.DLL and write

the result to BVHCRTC.DLL.

Appendix

1.  Files on diskette:

README   DOC            -- Read this first

BVHTVGA  DLL            -- OS/2 Base Video Handler

W800     DLL            -- 800x600-16 color PM driver

W1024    DLL            -- 1024x768-16 color PM driver

T640     DLL            -- 640x480-256 color PM driver

T800     DLL            -- 800x600-256 color PM driver

T1024    DLL            -- 1024x768-256 color PM driver

BVHCRTC  DLL            -- Alternative BVH for OS2CRTC

BVHTVGA  DDP

W800     DDP

W1024    DDP

T640     DDP

T800     DDP

T1024    DDP

BVHCRTC  DDP

OS2CRTC  EXE            -- BVH Video Parameter Patcher

SMODE    EXE            -- OS/2 Session set mode utility

OS2CRTC  DDP

SMODE    DDP

TVGACRTC EXE            -- Called by OS2CRTC.EXE

8800     SYS            -- Video parameters for 8800

TVGACRTC SYS

TVGACRTC DAT

8800     EXE            -- Video Parameter Patcher for 8800

2.  On board video memory requirements (8800/8900/9000)

driver          resolution      Min. mem (KB)

W800.DLL        800x600 -16          256

W1024.DLL       1024x768-16          512

T640.DLL        640x480 -256         512

T800.DLL        800x600 -256        1024

T1024.DLL       1024x768-256        1024

Fail to comply with the above requirement may cause

system hangup during OS/2 boot.  In case this happens,

use a DOS diskette to boot from drive A, restore OS/2

default DISPLAY.DLL driver, then boot OS/2 again.

3.  Some useful tips for trouble shooting

. Use TVGA 8800/8900/9000 chips or newer

. Use OS/2 PM 1.2, 1.3

. Make sure your TVGA board has enough memory

. Some monitors need special parameters,

use OS2CRTC.EXE to obtain best result

. If you choose to manually copy driver files to C:,

boot DOS or boot from drive A:.

Trident Microsystems, Inc.

205 Ravendale Dr.

Mountain View, CA 94043

Phone: (415) 691-9211

BBS:   (415) 691-1016   (Original distribution site for this package)

* This package if provided free by Trident Microsystems, Inc.  It can be

uploaded to any other electronic media for user downloading, provided

that no charge is made for it.