Text file for COILBLD1.GIF, graphical instructions for

construction of high performance 1/4 wave Tesla resonators.

1) The first step in winding a coil is to select a coil form.

The coil form should be a low loss material (we are talking

about radio frequency losses in the 50 - 1000 kHz range) like

polyethylene, polystyrene, or polypropylene, polycarbonate

(Lexan), or acrylic (Plexiglas): but the most common material

used from the standpoint of cost and availability is PVC (poly-

vinyl-chloride), which is VERY HIGH LOSS. PVC may be used if the

material is properly prepared before winding on wire. Regardless

of the material selected, the thinnest possible coil form should

be used; avoid heavy walled or pressure rated tubing.

The ratio between the actual winding length and diameter is im-

portant. The ratio of the winding length to the winding diameter is

known as the ASPECT RATIO (height : diameter), where the diameter

always equals 1. Aspect ratios may be expressed by a single number

such as "3.21". Please review the simple chart below when selecting

a coil form and the proper wire gauge: (All dimensions are in U.S.A.

measurements, inches, feet, AWG, etc..)

Coil Form Diameter   Aspect Ratio     Winding Length

3 inches           6:1              18 inches

4 inches           5:1              20 inches

5 inches         4.5:1            22.5 inches

6 inches           4:1              24 inches

7 inches         3.5:1            24.5 inches

8 inches           3:1              24 inches

larger than 8 inches      3:1        multiply the coil dia. by 3

This chart is based upon the physical characteristics of the actual

winding. Do not assume that "six inch PVC drain pipe" actually

measures six inches o.d., and be sure to allow a few inches of

extra coil form length. When selecting and cutting the coil form

you should allow at least an extra inch of coil form on each end,

and I generally figure on cutting the coil form three inches longer

than the actual winding length. After determining the length of the

coil form required, measure twice, then cut. Make sure that both cut

ends are square.

2) The coil form must be free of major surface imperfections.

I wet sand my coil forms with #150 wet/dry sandpaper and water

to remove markings, oxidation, scratches and cuts. After wet sanding,

the coil form must be dried thoroughly. If PVC plastic is used the

coil form should be gently baked under a heat lamp or even placed in

a very low temperature oven overnight. PVC coil forms must then be

sealed to negate the high RF loss factors that are inherent to this

plastic. Sealing also prevents PVC plastic from reabsorbing moisture.

Using a sealer such as petroleum based polyurethane varnish, two-part

clear epoxy paint, and some acrylic spray sealers is important. Avoid

water based and milky "emulsion" type sealers.

When I am ready to seal a PVC coil form, I mount the form up on a

winding spindle. I work in a well ventilated area, and I turn on at

least one fan to keep air moving over the work. I set up a few heat

lamps or other spark-free radiant heat source. Then I begin a four

hour coating process. I prefer a high-gloss polyurethane sealer

applied with a good varnish brush. Sealer is slopped on while the

coil form is spun. I use the brush to apply and smooth the heavy coats,

and to spread out drips and runs. Coats may be applied almost contin-

uously in this fashion for an hour or two. After coating the coil form,

it should be rotated on the winding spindle for a few hours while rad-

iant heat and moving air are used to speed a cure. Using this method it

is possible to build up a high-gloss finish which is free of runs and

drips. If drips and sags do occur, then can be "grated" off with a body

putty grater, or carefully trimmed away with a knife.

3) The coil should be close wound with a good quality magnet wire. I use

double Formvar enamel coated magnet wire. Magnet wire gives the maximum

inductance per unit volume of coil form. The coil should have over 800

turns, but not too many over 1000 turns. There is a little leeway here.

Use the thickest gauge of magnet wire that will allow the correct aspect

ratio, and between 800 - 1000 turns. It has been suggested that all coil

be wound with at least No. 22 AWG, or larger, magnet wire. I would concu

with this recommendation.

I plug the ends of the coil form with a tight fitting wooden disk and ru

a dowel or threaded rod through a center hole so that it will spin. I se

up the wire spool so that it will spin at one end of a pair of sawhorses

with the coil form at the other end. I wind the wire on by hand, making

sure the windings are close-wound, tight, smooth, and even. Overlaps and

gaps will adversely affect the performance. I use a dab of hot glue or

tape to hold the first turns in place, and I make sure to leave a 3 foot

tail of wire at both ends. Do not drill holes or permanently route wire

inside of the coil form.

ROUGH FREQUENCY CALCULATIONS:

Assuming the information given in the text and chart above is used,

a rough calculation of the resonate frequency is given below. In the

chart below: OD = outside diameter in inches, Wire Gauge is in AWG,

TPI = turns per inch, Turns = total number of turns of wire on the

coil, Resonate Frequency is given in kilohertz and the figure is

approximate only.

Coil Form OD      Wire Gauge    TPI     Turns     Resonate Freq.

3                22         37       666         840 kHz

4                22         37       740         540 kHz

5                22         37       832.5       380 kHz

6                22         37       888         290 kHz

7                22         37       906         238 kHz

8                22         37       888         206 kHz

9                22         37       999         163 kHz

10                21         32       960         152 kHz

You can see by reading through the text and looking at the charts

that some design problems occur in coils with a small outside dia-

meter. Coils under five inches in diameter must make sacrifices:

either they must get long and skinny to obtain a sufficient number

of turns; or they must be wound with smaller diameter wire. When

making this decision, after determining that a larger diameter coil

will not be satisfactory, it must be understood that 90% of the

voltage produced by a 1/4 wave Tesla secondary results from pure

resonance; meaning that the "ratio of turns transformation" calc-

ulated with the primary coil is not responsible for substantial

voltage gains: go with heavier wire as I have indicated rather than

additional turns. On small diameter coils the aspect ratio may be

increased somewhat rather than sacrifice turns or wire diameter.

COILBLD1.GIF      6/7/95           Graphics and text prepared by:

Richard T. Quick II

10028 Manchester Rd

Suite 253

Glendale  MO  63122  USA