Text file for COILBLD4.GIF, graphical instructions for

construction of high performance 1/4 wave Tesla resonators.

13) If the preceding directions have been adhered to, the

completed Tesla coil (RF resonator) will closely resemble the

diagram in figure 13. The coil form is hermetically sealed. There

are no holes into the wall of the coil form. The wire never

enters inside of the coil and all connections are made externally

where they do not compromise the electrical integrity of the

construction. The base wire has been cut, peened, and connected

to a high current ground terminal. The other end of the coil, the

air terminal, has been left untrimmed.

14) This shows how the air and ground connections are made to the

completed coil. A stand-off insulator is placed on the top of the

coil. A TORIOD discharge terminal is placed on the insulator and

the wire is air-wound around until it contacts with the bottom

plate of the conductive toroid. These air wound turns are widely

spaced, but the diameter is kept as close as possible to that of

the secondary winding. Once contact has been made to the bottom

of the toriod, the wire may be held in place with a small piece

of tape, then the winding is discontinued and a bared section of

wire is connected directly to the center of the toriod with a

nut and bolt clamp, tape, etc.. Excess length may now be trimmed.

The exact length of the stand-off insulator, and therefore the

height the toriod discharge terminal sits above the secondary

resonator, can only be determined by experiment. This varies with

the size of the toriod, the size of the coil, and the input power

into the system. Due to the number of factors involved, this

insulator may require frequent adjustments/changes. For this

reason I do not permanently mount a stand-off insulator on the

coil. I keep a selection of square cut sections of PVC plastic

pipe that I use for stand-off insulators. The toriod is elec-

trically connected as indicated above, then it is simply set on

top of a PVC pipe stand-off insulator. The system is now ready to

be fired. If desired, after some experimentation, the insulator

can be permanently mounted: the end cap should be scored with a

sharp tool, not drilled; the surface should be prepared, and the

insulator should be glued in place with two-part epoxy.

The ground connection is made via the shortest available path,

using the heaviest, widest possible conductor, to a dedicated RF

ground constructed specifically for Tesla work. This ground is

referred to as the "system RF ground" or simply the "system

ground". The system ground is usually constructed, not happened

upon. I advise constructing a system ground from scratch unless

you can verify that any available grounds are electrically

isolated. Do not use a water pipe. Do not use the house ground.

Tesla rated grounds need to be extremely heavy, usually

comprising of several eight to ten foot copper pipes hammered

into the ground. The pipes should be separated in the ground by

their lengths (eight foot pipes are set eight feet apart) and

connected with one inch ground strap buried below sod level.

It should be noted that these instructions are designed to

produce a highly efficient RF resonator with exceptional

electrical strength at the lowest possible cost. Coils built to

these specifications are capable of producing, and withstanding,

discharge lengths that exceed the physical length of the coil by

a factor of 3.5 or more. These instructions are the product of

years of experimentation winding dozens and dozens of coils and

with the collaboration of others in the field of high powered

Tesla systems. This design method has been repeatedly tested and

reproduced by beginners with excellent results.

This completes the COILBLD series on construction of high

performance Tesla resonators.

COILBLD4.GIF      6/9/95           Graphics and text prepared by:

Richard T. Quick II

10028 Manchester Rd

Suite 253

Glendale  MO  63122  USA