CAML_UL.DOC

SOPWITH CAMEL ULTRALIGHT--DOCUMENT FILE

CAML_UL.AIR is an ultralight version of the Sopwith Camel which

comes with Microsoft Flight Simulator.  It is designed to work

with version 5a.

It is appropriate to redesign the Camel as an ultralight.  Some

actual ultralights are miniature "replicas" of antique aircraft.

Also, the Camel's simple instrument panel makes sense for an

ultralight.

This design performs pretty much as an ultralight should.  It has

an extremely low stall speed, short takeoff run, steep climb

angle, and a low sink rate when gliding.  It is also quite

maneuverable, and does aerobatics nicely.  This aircraft has

spoilers.

TO USE: Simply copy CAML_UL.AIR into the SIM directory.  It will

appear in the list of available aircraft.

STATISTICS:

Length*                       15'-1"

Wingspan*                     25'-2"

Height*                       8'-1"

Weight (w/pilot, no fuel)     650 Lbs

Engine                        45 HP

Wing Area                     140 Sq Ft

Fuel Capacity                 24 Gals total

* Dimensions are based on wing area relative to the default

Camel.  The size of the plane as it appears in the

simulator does not change.

PERFORMANCE

Speed

Altitude       Full Pwr  3/4 Pwr   5/8 Pwr

2000 Ft        75 Kts    67 Kts    61 Kts

5000           71        65        58

10000          66        58        54

15000          60        54        45

20000          55        45        38

25000 (!)      49        38        --

Stall Speed: 22 Kts

Takeoff Speed: 25 Kts

Rate of Climb (low altitudes)

25 Kts    672 Ft/Min

37        732

44        735

Sink Rate--Power Off (low altitudes)

25 Kts    430 Ft/Min

37 Kts    466

HINTS

TAKEOFF: At full throttle this aircraft accelerates to takeoff

speed very quickly.  Start with neutral stick--tail will rise.

As you approach 20 Kts, pull all the way back.  The plane will

leap into the air at about 25 Kts.  If you continue to hold the

stick back, it will hold 25 Kts and rise at a steep angle.

LANDING: To get this plane down, you have to throttle all the way

back and/or use spoilers (/ key toggles spoilers on and off.)  Do

not flare at touchdown--plane will rise and stall.  Do not use

wheel brakes--plane will cartwheel--use spoilers instead.

LOW SPEEDS: At near-stall speeds you need to hold the nose up a

bit to hold altitude.

CONTROL: As designed, this plane seems to have about the right

amount of maneuverability for its type.  You should set the

joystick sensitivity (under Preferences) so that the control

surfaces reach their maximum angle at full stick.

INTEREST

I would have preferred a slightly lighter, smaller, and less

powerful aircraft.  However, at a weight of 600 Lbs or less this

design has an odd tenancy to jump around on the tarmac--at 0 Kts!

I don't know why it does this--some peculiarity of the program

(?).

The aircraft color is yellow to differentiate it from the default

Camel.  Also, such a tiny, slow plane needs all the visibility it

can get.  And besides, I like how it looks.

ACKNOWLEDGEMENTS

I used Chuck Dome's AIRMOD program (version 2) to create this

aircraft.  AIRMOD is a great program for modifying FS aircraft.

You can find it in the Flight Simulation forum: file AIRMOD2.ZIP,

Library 5.

MICROSOFT FLIGHT SIMULATOR is a trademark of Microsoft.  The

default Sopwith Camel, from which this ultralight version is

derived, is part of the standard MICROSOFT FLIGHT SIMULATOR,

version 5a.

AGREEMENT FOR USE

CAML_UL.AIR is free for all to use.  It is not to be used for

profit.

The user of CAML_UL.AIR takes full responsibility for any problems

which might arise from the use of this file.

FEEDBACK

Thanks for flying my plane.  I would love to hear what you think

of this ultralight Camel.

John Stoppels (75271,2576)