ODIPKT(8)                (February 2, 1992)             ODIPKT(8)

NAME

odipkt - translate Open Data-Link Interface to Packet Driver

interface

SYNOPSIS

odipkt [ logical_board [ vector ] ]

odipktpn [ logical_board [ vector ] ]

DESCRIPTION

Odipkt.com provides a Packet Driver API over Novell's Open

Data-Link Interface, allowing a single network adapter to

service IPX, tcp/ip, and other protocol stacks which use

packet drivers.  Odipkt.com understands Ethernet, Token

Ring, and ARCnet frame types.  Ethernet and Token Ring

frames are passed unchanged to packet driver applications,

but ARCnet frames are internally mapped to Ethernet.  Thus,

packet driver applications do not need specific knowledge of

ARCnet in order to operate in such environments.

The logical_board argument indicates which logical adapter

odipkt should bind to.  Under ODI, there is one logical

adapter per frame type.  There may be multiple physical

adapters and multiple frame types per physical adapter.  The

logical adapter number starts at zero and increases by one

for each frame type.  For example, if a system contained two

physical adapters with two frame types each, there would be

four logical adapters numbered 0-3.  The frame types are

specified in the NET.CFG file which must be in the current

directory (or on the APPEND path) when the ODI drivers are

loaded.

Typically, the frame type used by default by Netware on a

given adapter will not be appropriate for tcp/ip.  The

correct frame type for tcp/ip on Token Ring is TOKEN-

RING_SNAP and the correct frame type for tcp/ip on Ethernet

is almost always ETHERNET_II.  Assuming an existing

installation with one adapter running the default Netware

frame type, it will usually be necessary to define a second

frame type (as listed above) and specify the logical_board

argument as 1.  An exception to this rule is ARCnet which

uses only one frame type.  Below is a NET.CFG entry for a

3c501 using the default address and interrupt.  In many

cases, this is the only entry required in NET.CFG.  Other

Ethernet adapters would use similar configurations:

Link driver 3c501

envelope type ETHERNET_802.3

envelope type ETHERNET_II

Given the above entry, the logical_board argument of oipkt

would be 1.  Token Ring uses different frame types:

Page 1

ODIPKT(8)                (February 2, 1992)             ODIPKT(8)

Link driver token

envelope type TOKEN-RING

envelope type TOKEN-RING_SNAP

Again, the logical_board should be specified as 1 for this

example.  ARCnet installations require no frame-type

definitions and should use a logical_board of 0.

Vector is the packet driver interrupt at which to provide

service.  The default is 105 and the parameter is decimal.

Odipkt.com should be loaded after lsl.com and the adapter-

specific driver(s).  It must, of course, be loaded before

the packet driver applications which use it.  The Netware

shell and ipxodi.com can be loaded before or after the

odipkt.com program.

Odipktpn.com supports an older version of the ODI

specification as used in Farallon's PhoneNET Talk product.

Operation of opipktpn.com is identical to that of opipkt.com

but configuration of NET.CFG is somewhat different.  No

``envelope'' statements are required and the Link Support

section's ``Buffers'' entry must specify the true maximum

frame size of the network.  Because multiple frame types are

not supported, odipktpn.com treats all Ethernet cards as

ETHERNET_II.

AUTHOR

Dan Lanciani

185 Atlantic Road

Gloucester, MA 01930