SLIPLog

USER'S MANUAL

1.0   SLIPLog

1.1   Introduction

1.2   Features

1.3   Unpacking

1.4   Installation/Setup

1.4.1 Command Line Syntax

1.4.2 Help files

1.4.3 The Structure of SLIPLOG.PWD

1.4.4 Call back

1.5   Logging In to the Server

1.5.1 Debug Mode

1.5.2 Sysop mode

1.5.3 Dial On Demand

1.5.4 Resident SLIPLog Commands

1.6   Multiline SLIP

2.0   COMTool, Introduction

2.1   Command Line Syntax

2.2   Programming Your Modem With COMTool

2.2.1 How Your Modem Should Work

2.2.2 Programming The Modem

2.2.3 Troubleshooting

2.2.4 Q&A: Troubleshooting the Modem

3.0   Assembling a SLIP Gate

3.1   CONFIG.SYS and AUTOEXEC.BAT

3.2   Starting NOS (SLIPSERV.BAT)

3.3   Configuring NOS (AUTOEXEC.NOS)

3.4   Date Patch and Logfile Processing (SETTIME.SRC)

3.5   NOS Documents

4.0   Some Notes About Windows

4.1   A SLIPgate under Windows for Workgroups

5.0   Getnews

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

1.0     SLIPLog

-------

1.1     Introduction

SLIPLog is a small (<6K) TSR that expands any SLIP packetdriver with login

and remote control features, which are needed if you want to assemble a

secure, reliable and cheap SLIP Gate with ka9q/NOS.  SLIPLog sits between the

SLIP driver and the COM port.  It intercepts incoming calls, checks the logon

rights, and then activates the SLIP connection.  All you need is an old PC

(286 or better recommended, but 8086 will work), a cheap Ethernet card, one

or more (up to 4) modems, and of course a LAN to which you are allowed to

connect.  You will need a minimum of 2 IP addresses assigned to you.

I strongly suggest that you have a good understanding of networking and

SLIP gates before you try installing SLIPLog.  An improperly configured

SLIP Gate can be annoying to people dialing in or to people already on the

local network.  At a minimum, if you are not sure of how this works, then get

your system administrator to help you so you don't make his/her life miserable

on the network.

1.2     Features

- Function control of the gate with a real reboot on error.

- Sends content of a user defined message file after connect.

- Sends the SLIP IP address after a user logs in successfully.

- Authentication of callers, with time and date written to a logfile.

- Programmable, with individual call-back possible.

- Dial on demand (dial-out) possible.

- Individual maximum daily online times for each user.

- Full remote control of the SLIP Gate (with sysop password) possible.

- Switching from SLIP session to remote control at any time.

- Integrated mini-terminal program (i.e., for dialing).

- Integrated diagnostic routines.

- Runs in a MS Windows' DOS window (including remote control and reset).

- Fixes hang-up problems during the connect phase of the modem.

- Services more then one line by simply adding another SLIPLog.

1.3     Unpacking

Contents of SLIPLOG.ZIP:

sliplog.doc     This manual

sliplog.com     SLIPLog executable

sliplog.asm     SLIPLog source code

comtool.com     COMTool utility

comtool.doc     COMTool documentation

comtool.asm     COMTool source code

getnews.exe     the Getnews utility

getnews.doc     Getnews documentation

getnews.pas     Pascal source code

coldboot.doc    Coldboot utility

coldboot.com    Coldboot executable

coldboot.asm    Coldboot source code

slipcfg.zip     Configuration files

Additionally, in SLIPKIT.ZIP:

cslipper.zip    Peter Tattam's compressed SLIP driver.

pktmx12b.zip    Graham Robinson's packet multiplexer and winpkt.

ndis3pkt.zip    Dan Lanciani's ndis/pkt shim for ndis3 protocol.

Useful if your SLIP gate is running on a WfW machine

(latest version from hsdndev.harvard.edu).

11cj3.zip       Jon Lewis' latest NOS compile.

slip.txt        Jon Lewis' installations guide with lots of tips!

yanos001.exe    My special NOS derived from jnos110g. This version does

not have the nice gopher and WWW servers we have in

nos11*, but a more sophisticated handling of overload

situations (doesn't crash when memory gets low).

JNOS is supported by mvangel.npt.navy.mil.  They have

a good jnos110.doc there.

nntpget.exe     Newsget program from the biochemistry server at Case Western

Reserve University (biochemistry.cwru.edu).

ka9q0694.txt    Text file especially for the NOS11c version

(latest version may be found on cases.pubaf.washington.edu).

nos_1229.txt    Description of the basic functions of all NOS versions.

First create the following directories on your hard drive:

\nos

\nos\spool

\nos\spool\mail

\nos\spool\mqueue

\nos\spool\temp

\nos\spool\help

\gopher\public    (root of your ftp/gopher server)

Copy all executables to the NOS directory.  Copy 'sliplog.com' to the

\nos\spool directory.

1.4     Installation/Setup

SLIPLog must be loaded after the SLIP packetdriver.  Load it as the last TSR

before starting ka9q/NOS, either from the 'autoexec.bat' or manually.  ANSI

drivers (NNANSI) or terminal emulators (VT102) may especially cause problems

if loaded after SLIPLog.  If you need help, run SLIPLog with -? (or -h) as

command line parameter.

1.4.1 Command Line Syntax

SLIPLog is loaded with the following command line.

sliplog COM-Adr COM-IRQ PKT-IRQ [IP [path[\]]]

COM-Adr is the hardware address of your COM-Port, i.e., 3e8.  COM-IRQ

is the related interrupt, i.e., 4.  Only the lower IRQs (2-7) are allowed.

PKT-IRQ is the soft IRQ of your SLIP packetdriver (in hex).  This must

be a value of >= 60.  IP is the packet number in dot notation.  This is

the IP address assigned to the clients calling your gate, which is routed from

the serial line to the Ethernet card.  After a successful login, this IP

address is sent to the user and should be captured by the client for dynamic

IP assignment.

Another form for invoking SLIPLog (if your COM-Ports have normal

address/IRQ combinations) is:

sliplog COM-nbr PKT-IRQ [<IP> [path[\]]]

where COM-nbr is a number from 1 to 4, and 'path' is a fully qualified

pathname, pointing to the directory that contains the help files described

in the next section.

1.4.2 Help Files

sliplog.log        The logfile

sliplog.pwd        Passwords and online times

sliplog.msg        Greeting message

The files 'sliplog.log' and/or 'sliplog.pwd' are automatically generated by

SLIPLog at startup.  If 'sliplog.pwd' does not exist, then the password

and online time values will default to:

sysop 60

guest 10

If you omit the path parameter, SLIPLog will use the current directory

as the directory for its help files.  I recommend copying all 'sliplog.*' files

(including 'sliplog.com') to the same directory, change to that directory and

call SLIPLog without a path parameter.

1.4.3   The Structure of 'sliplog.pwd'

Th file 'sliplog.pwd' contains the user passwords and the maximum daily online

time you allow a user to be logged to your system.

password1 time1

password2 time2

password3 time3

Passwords may be of any length, consisting of all characters.  A single blank

separates passwords from time entry.  The time entry is a four digit integer,

which is the maximum time in minutes you allow a user to be online per day.

This 'time bank' will be reset at time1 (minutes after midnight).

The first two passwords have a special meaning.  The first logs you in to

'sysop mode.'  In sysop mode you can remotely control user times, list

and edit the password file, and switch to 'terminal mode.'  In terminal mode

you can fully control your SLIP PC, as long as the programs you launch

are using INT 29 for output.  Yes, NOS does use INT 29, DOS does too, Norton

does NOT but a Windows DOS box does.  If you need an editor, working remotely:

install an ANSI driver and run Sammy Mitchell's 'em.com.'  I found this nice

Editor on Simtel.

The second password, 'guest,' gives you the full amount of online time

on every login.  You can use this password to allow guests to snoop

around a while or whatever you want.  One possible idea is to use this

password if you have another server dialing you automatically (dial on

demand).

1.4.4   Call Back

You must type a pound sign (#) followed by a dial string as part of the

password.  If you have such a dial string in the sysop password, then 'dial on

demand' is possible to this number after you enabled this feature with a

special command (see below).

Example:

joeathome#atdt1234567 90

Note:

The password in this case is only the first part of the command up to

the pound sign (#).

The dial command 'atdt' in the example, may be 'atdp' if you need pulse

dialing instead of tone dialing.  If the modem of 'joeathome' needs some

special commands (for example, to switch off compression), then you write

these commands in front of the dial command.

Example:

joeathome#at&k0&n14dt1234567 90

I needed the above commands 'AT&k0&n14' (compression off, V22 on) to

call back an old 2400 Baud modem from my Zyxel in the SLIP Gate.

1.5     Logging In to the Server

SLIPLog continually checks the modem for incoming calls.  When the first ring

is detected, SLIPLog sets the DTR signal to the active state and the modem

answers immediately.  Of course, the ring-count-register has to be programmed

to 1 (ats0=1) at startup of the PC.  If something goes wrong and the modem

does not answer after the third ring, SLIPLog resets the DTR signal.  The

modem should then reset to the command state with the autoanswer enabled.

If the carrier is established, SLIPLog sends the message you defined in

'sliplog.msg,' terminated by an unique separator '%$', which may be useful if

you write a login script.  After this separator is sent, SLIPLog waits for a

carriage return.  If this character is not received within 90 seconds, the

modem hangs up.  Otherwise the login prompt 'login:' is sent.  You now have

to enter your case-sensitive password, terminated by another carriage

return.  The echo can not be seen on the screen.  If you mistype the password,

you get another prompt.  After the third try you are out.  After typing the

correct password, SLIPLog sends a control message to you with the maximum

online time you have and the IP address dedicated for the calling client.

If you typed the very first password of the password list, you are not

switched to SLIP but rather to 'sysop mode' (see 1.5.2).

1.5.1   Debugging

If you want to see what happens without really calling your SLIP gate, you can

run the debug mode of SLIPLog.  After you installed SLIPLog, launch it using

the following command line switches.

sliplog PKT-IRQ -x1

First of all type <CTRL-R>, which simulates the ring signal.  The DTR signal

is now switched on (you can see it if you have an external modem with LED's).

Next, type <CTRL-C>, which simulates the 'carrier.'  Of course, the modem is

not affected at all, but SLIPLog proceeds with the login procedure as

described above.  Now hit <ENTER> and continue to login.  After you are logged

in, you can simulate a BREAK signal with <CTRL-B>.  SLIPLog switches back to

the login prompt and you can repeat the login.  If you logged in as sysop, you

can test all sysop commands.  However 'terminal' is not very useful since the

only thing you can do when you switched to the terminal mode is type <CTRL-B>.

This brings you back to the sysop mode again.  If you forgot all the commands,

don't worry:  a list of all commands is shown if you press <F1>.

Note:

The debug mode remains active even if you quit the debug session.  (This

is useful, if you have a multi-line configuration).  To terminate the debug

mode, you must perform another call to SLIPLog with option -x0.

1.5.2     Sysop mode

After you have logged in successfully with the first password in the list,

you are in sysop mode.  You may now obtain a list of possible commands by

pressing <ENTER>.  All commands may be abbreviated, so it's sufficient to

type 'res' instead of 'reset' to reboot your PC.  But if you type 're', the

'read' command is executed.  Commands are executed in the order shown below.

Commands

--------

help                Lists all commands (same as <ENTER>).

show                Shows the content of password file and the remaining

online times of users.

slip                Switches back from sysop mode to SLIP mode.  You can quit

SLIP mode at any time and get the login prompt if you

send a break signal to the modem.  A normal user would

do this to get the current remaining online time, which is

shown on re-login.

set 'user' 'time'   Resets the current online time of a 'user' to the new

value 'time.'

read 'pwd_file'     Loads a new 'pwd_file' (default is 'sliplog.log').  All

online times are reset to the starting values.

terminal            Switches to terminal mode (remote control).  You can

switch back to sysop mode at any time if you send a break

signal.  If you can not send a break, you have to hang

up and redial.  :-(

autodial            Switches the dial-on-demand feature on or off.  This works

only when the sysop's password contains a dial string.

login               Takes you back to the login prompt.  This also works for

any other command not listed here.

reset               Reboots the computer.  You can use it if all your programs

but SLIPLog hang.  However, 'autoexec.bat' must be setup to

reinstall the SLIP gate and SLIPLog, otherwise, your remote

activities end with this call.  Rebooting the PC is

also possible if you're running MS Windows.  All buffers

are flushed before the reset is performed.  However, you

use this command at your own risk!  The only thing I can

say is it works for _me_ without any problem.

1.5.3	Dial on Demand

Dial on Demand (DoD) is another feature of SLIPLog.  Whenever a packet is

routed over the SLIP interface and the modem has not yet established a

connection, SLIPLog dials to another SLIP server, through which the final

destination can be reached.  This works only if the first password in

'sliplog.pwd' is defined with a dial command:

sysop#atdt12345678 10

SLIPLog performs a simplified login procedure after dialing.  If the carrier

is detected, SLIPLog waits about 5 seconds, then sends a return character and

waits another 5 seconds.  Next, the sysop password is sent and after another

pause of 5 seconds, SLIPLog switches to SLIP.  If login failed and there are

more packets, another attempt is made about three minutes later.

DoD is useful if you want to connect two LANs over a dialup line.  Use the

same password files on both servers, but on the second server, the first two

entries are swapped.  You should not allow a normal user to contact one of the

SLIP servers.

1.5.4     Resident SLIPLog Commands

Once SLIPLog is loaded, it's possible to interact with the resident

program to get information about its status, to change parameters,

or to start certain actions, such as a cold boot, a test program, or a

mini-terminal routine.  These commands only require the packet vector as the

first command-line parameter followed by one or more switches.

Of course you may call all these services remotely.  But terminal mode (-t)

and test mode (-x..) make no sense and would lock up the system.

sliplog PKT-IRQ switch_1 [switch_2 ..]

Some switches require a numeric decimal parameter.  The values shown here

are the default values.

Switches

--------

-a <string>         Sends a command string to the modem, which is useful in

cases when COMTool seems not work, e.g., when running

Windows For Workgroups.

-b 0                Suppresses watchdog/pause-beep if n > 0

-d 0                Allows dial on demand if a dial string is defined in the

sysop password.  The value of n defines the maximum time

of inactivity to terminate a session.  n=0 switches off

dial on demand.

-h                  Hangs up the modem.

-i 0                SLIPLog works in the background, being triggered by the

system timer (INT 8) every 55 mS.  The real work is done

in the DOS idle loop (INT 28) by default, but may be

linked to INT 8 if your application does not call INT 28.

If n > 0, then INT 8 (timer) is used instead of INT 28

(DOS idle) to execute the SLIPLog tasks.  Select this

option if your gate software does not call INT 28 and the

watchdog reboots your computer.

-l n,t              Changes the timer of user 'n' to 't,' where n is the

number of a user in the password file, and t becomes the

remaining logon time available.  If n=0, then the time of

the currently-logged user or users is set to t.  In this

case, a message is shown if nobody is logged in.

-m 0                When n > 0, incoming calls are monitored on the screen.

-n <IP-address>     Redefines the IP address sent to the user after login

before switching to SLIP.

-p 0                SLIPLog is paused for n*10 seconds, a beep occurs every 10

seconds.  The modem is not serviced and if it was

programmed properly (see 'modem-section'), no calls are

accessed during this time.  If you don't stop this mode

with '-p 0', a coldboot is performed after the pause

timer has counted down to zero.

-q 1                Quiet mode.  With n > 0, screen outputs are suppressed if

you are in remote mode.

-r                  Reset.  On all 286/386-machines, this is a real cold boot,

by pulsing the reset-line in the keyboard-controller.  A

system-call to DOS flushes all open-disk buffers.  The

reset is done 5 seconds later.

-s 0                If n > 0, only the sysop is allowed to log to the gate.

An appropriate message is shown to all callers.  The

variable n defines the time of inactivity after the modem

terminates a sysop-session.  The default is 60 minutes.

To reprogram this to 'n' use the commands 'sliplog -s n'

followed by 'sliplog -s 0.'

-t                  'Terminal mode.'  It's a very basic terminal program

allowing you to dialup another server and do a manual

login.  The control codes are: <CTRL-Q> quits, Ctrl-B sends

a break, <CTRL-D> toggles the DTR-line, <CTRL-S> shows the

status info, and <F1> shows an online help.  A screen

capture program like Scrollit and an ANSI driver like

NNANSI greatly enhance the performance of this terminal

program.

-u                  Unloads SLIPLog from memory.  This is only possible if all

other TSR's and programs loaded after SLIPLog and using

the same interrupts are terminated.  Especially NOS and

Windows must be terminated before you can remove SLIPLog.

You get a message if unloading is successful.

-w 9                Sets the watchdog timeout.  The default is 9 (= 90 sec.).

If the system does not service SLIPLog within an interval

of 10 seconds, a beep sounds and the watchdog counter is

begins to count down.  After counting to zero (90 sec.)

a cold boot is performed.

-x 1                Switches to debug mode (get online help with <F1>).

-x 0                Switches off debug mode.

If you do not use any switch, SLIPLog will show you some useful information

about a running SLIPLog (all numbers are decimal).

Port=1000 IRQ=0002 Packet=0096 state=0000 P=0000 W=0009 VM=0000

UT=0030 TW=0000

IP address: xxx.xxx.xxx.xxx     c:\path\

1user=27.06.94  ****.**** 16:04 -nils 16:05 - 16:05 - 16:05

OK

Port/IRQ/Packet are the addresses of the COM port, interrupt of COM port and

the interrupt of the SLIP driver (decimal-numbers).  'State' is an internal

process ID, showing you what SLIPLog is doing at the moment.

Examples of states:

---------------------------------------------------------

0                   Idle

1                   Carrier detected

4                   Waiting for carriage-return character

5                   Login:

7                   SLIP

8                   Sysop mode

10                  Terminal mode (remote control)

11-15               Dial back/dial on demand

P and W are Pause and Watchdog timer (unit is 10 seconds), UT is the

remaining online time of the current (or last) logged user in minutes.  TW

is the internal Tick counter (unit is 55 mS), which is used as a base

for all timing-purposes.

The next line shows you IP address and path of this copy of SLIPLog.

The last line contains the last stored logfile entry so you can see

who is currently logged in or who was last logged to the gate.  If you are in

terminal or test mode, you can get this status message any time you type

<CTRL-S>.

1.6     Multiline SLIP

If you want to service more then one phone line, you have to load another SLIP

driver and another SLIPLog, however on a different COM port and packet vector.

SLIPLog only allows the use of the lower interrupts.  If you have 4 lines, and

your Ethernet card uses IRQ 5, then your serial card must be jumpered to IRQs

2, 3, 4 and 7.  Because 7 is used by the parallel port (LPT1), but never used

by any software, you have to disable this IRQ on the parallel adapter.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2.0     COMTool

While developing SLIPLog, I needed a small program for testing and commanding

the UART and the modem.  So I wrote COMTool.  I use it now in my 'autoexec.bat'

to clear the UART, set the baud rate and control lines of my modem, and to

send some command sequences when powering up my SLIP Gate.  I wrote a simple

autodialer with it, which is very useful to me if my gate is busy.

Furthermore, I use it as a simple terminal program for remote control of the

SLIP Gate, because it allows me to send a break signal over the line.  To make

the best use of COMTool's terminal mode, install an ANSI driver like 'NNANSI'

and a scroll utility like Scrollit before starting COMTool.

2.1     Command Line Syntax

You call COMTool just as you would SLIPLog.

comtool COM-Adr COM-IRQ [command]

comtool COM-nbr command

The commands are listed below.  Each command begins with a caret (^).  If

there is no caret, the text is sent as is.

(The '<' character on a command line is used for redirection.  That means

you can create a command file with one or more command lines in it and

redirect this to COMTool with '< cmdfile' on the command line.  The

last character in such a command file MUST be a caret and ^cr

(send return-character) is not needed at the end of a line.)

If the last character on the command line is NOT a caret, then you are in

terminal mode.  You can type AT sequences, connect to a site with 'atdt ...'

or do whatever you want.  You leave this mode with <Alt-E>.  If this doesn't

work (due to a keyboard problem that one day I'll fix), then try <CTRL-C> or

<CTRL-BREAK>.  If the last character was a caret, then the program returns to

DOS automatically.  You can recall this program whenever you want because

all interrupts and the UART state are restored carefully.  You may shell out

from another communication application, start COMTool, do something with it,

and return to your application without any effect on your application.  It

works even with Windows.  However, when SLIPLog is running, you should use

SLIPLog's own terminal mode.  Two terminal programs running in the same DOS

window and using the same hardware would conflict.

Switches:

---------

^Fn                 Programs the FIFO in an 16550 Uart.

'n' is a hexadecimal value.

0  FIFO off

1  FIFO on, Triggerlevel 1

41          Triggerlevel 4

81          Triggerlevel 8

c1          Triggerlevel 12

^tn                 Wait n/10 seconds.

^cr                 Send cr-character.

^lf                 Send lf-character.

^sp                 Send space-character.

^^                  Send 'caret.'

^d1                 Set DTR on.

^d0                 Set DTR off.

^bn                 Set baud rate, n=0 (115200), n=1 (57600) ...  to n=7 (1200).

^kn                 Send break-character, duration is n/10 seconds.  The normal

break is 3/10 seconds.

^CDn                Wait n/10 seconds for the carrier-detect signal.  Set the

errorlevel to 8 if the counter times out or the user

breaks, otherwise set errorlevel to 4.

^wn string1 string2 string3

Wait n/10 seconds for string1 (or 2 or 3), set the

errorlevel to 1, 2 or 3 if found, or to 8 if user breaks

or the counter timed out.  No partial strings are detected,

only complete words.

^0n (Enter)         Send a character with ascii-value n. n is in hexadecimal

notation. '^3 (Enter)' sends the ctrl-c character for

example. (You can't type ctrl-c directly)

Special Keys:

-------------

<ALT-E>             Exit COMTool (may not work with all keyboards).

<ALT-H>             Hang-up the modem (may not work on all modems).

<ALT-S>             Stop or Resume datatransfer from the modem to the PC.

This is done by toggling the handshakesignal RTS

<CTRL-BREAK>        Breaks COMTool (if <ALT-E> doesn't work).

<CTRL-C>            Breaks COMTool (same as <CTRL-BREAK>).

Function Keys:

--------------

The <F1> through <F10> keys, the cursor, and other special keys are sent as

combination codes over the line.  The first code is 0x00, the second one is

the function code.  SLIPLog decodes these appropriately when in terminal mode.

That means it 'presses' the appropriate function key on the remote machine

too, whenever you press a function key locally.

Here a simple batch program that dials to my SLIP gate showing how some of the

commands are used.  If busy, dialing is repeated.  If connected, the password

is sent.  I use this at home instead of trumpets internal dialer when my

gate is busy.

@echo off

comtool 5 ^b2 ^t6 at&f2&A0=0\N3L0&D0M3^cr^

:oncemore

comtool 5 ^t20 atdp07216084138^cr^w600 CONNECT BUSY NO^t10^

if errorlevel 3 goto end

if errorlevel 2 goto oncemore

comtool 5 ^t20^cr^t20 ******^cr^t20

echo you are connected now!

:end

2.2     Programming Your Modem for SLIPLog

Some users of SLIPLog have had problems getting SLIPLog to run.  The reason

was often a misconfigured COM port or modem.  Below are two possible reasons

why difficulties can arise.

1. SLIPLog DOES NOT PROGRAM YOUR MODEM.  THIS MUST BE DONE BEFORE

YOU START THE PACKETDRIVER.  THE ONLY ACTION TAKEN BY SLIPLog TO

COMMAND YOUR MODEM IS TO PULL THE DTR LINE HIGH AND LOW.

2. EACH COM PORT IN YOUR PC MUST HAVE AN INDIVIDUAL ADDRESS AND

HARDWARE INTERRUPT.  DON'T TRUST A 'DISABLED' COM PORT.  THE

INTERRUPT LINE MAY BE IN USE NEVERTHELESS.

2.2.1   How Your Modem Should Work

1. Your modem must be set to autoanswer a call on the FIRST RING.

2. Pulling the DTR line low must result in a hang-up and reset of the

modem to the command state.  While DTR is low, the modem should not

answer an incoming call.

3. Your modem must supply the RI (ring indicator) Line.  SLIPLog

checks this signal and pulls DTR high when the first ring is

detected.  If carrier is not detected after the third ring, the

DTR line is pulled low again, which should reset the modem to the

proper autoanswer state.

4. The carrier-detect line must reflect the state of the carrier.

SLIPLog needs this line to detect the line status.

2.2.2   Programming Your Modem

The best place to program your modem is in the 'autoexec.bat' before all other

programs are executed.  I use COMTool to send the appropriate AT sequences to

the modem.  This example is for a Zyxel modem connected to COM2.  Although you

can put all the modem command switches on one line, it is probably easier to

use separate lines to keep them organized.

comtool 2 ^b2^d1^t5^d0^t5^

Sets the UART to 38400 baud, resets the DTR line.

comtool 2 +++^t10 at&f^cr^t10^

The string '+++' should reset the modem to command state

if the DTR method didn't work.  Now the factory

configuration (at&f) is loaded.  If you don't get an "OK"

response, then something is wrong.

comtool 2 ats0=1^cr^t5^

Autoanswers after the first ring.

comtool 2 at&d3&c1^cr^t5^

This command programs the DTR (Data Terminal Ready; &d3)

and the CD (Carrier Detect; &c1) functions.  When the DTR

is programmed low, the factory profile "0" is loaded,

which means the modem is ready for commands, but not for

incoming calls.  The CD reflects the state of the carrier.

comtool 2 at&w0^cr^t10^

Stores our configuration as profile 0.

2.2.3   Troubleshooting

1. Your COM port should have a 16550-UART, especially on slower

machines (8086/286 or 386 sx).

2. If you have a slow 8086/80286 and you don't have a 16550 and you

see a lot of header checksum errors when you trace the IP traffic

on the serial line (use a NOS-version with 'trace' command compiled

in for debugging), then reduce the DTE speed to 19200 baud.  That's

the only way to fix this problem.

3. Do not use any additional COM driver, like Fossil.  Your packet

driver and SLIPLog do not like another resident COM-driver.

4. You don't need any special COM driver for Windows or W4WG and you

need no special entries in the system.ini file.

5. Be careful about some TSRs that deal with the keyboard, the screen,

or the timer.  If they seem to be causing a problem, remove them

all and re-install one at a time until you found the culprit.  In

most cases, you can work around the problems when you change the

order of loading this TSRs.  DON'T USE ANY KEYBOARD EXTENDER, the

only TSRs you should have are NNANSI or VT102, a scroll back utility

like Scrollit, and Smartdrive or another good caching program.  If

you have smartdrive, disable write-behind caching if it's not the

latest MS DOS 6.x version.

6. Load the SLIP driver and SLIPLog after all other TSRs are installed.

2.2.4    Q&A: Troubleshooting the Modem

>NOS has been sorted out and works great.  I can't wait to create my

>own Class C network!  However, I am still having a problem with the

>slipgate.  On the serving machine, the modem is initalized with

>Comtool, the packet drivers are loaded (Cslipper and NE1000), and

>Sliplog is loaded.  NO PROBLEMS.  NOS starts fine.  COMLOG.LOG is

>initialized.                                          ^^^^^???

Might be you have an old version of sliplog? The name of the

logfile is 'sliplog,log', the greeting-message is in sliplog.msg and

the name of the passwordfile is sliplog.pwd. Please check.

>On the client machine, I am using Trumpet Winsock 1.0B with the

>login.cmd script that you provided, modified for my password, etc.

>Trumpet dials, the server answers, and a connection is made. NO

>PROBLEMS.

First try to connect with COMTOOL from DOS without starting WINDOWS

to exclude windows related problems . If you don't get a greeting

message and there is no %$- sync message to be seen, you have

a problem with your comport or modem either at home or at the gate.

Dial to a local bbs (you should have an ansi driver). If this

works, your problem is at the gate. Go to your gate. Break the

startbatch immediately after sliplog is installed. Now invoke

sliplog with the -t Option:

sliplog PKT-IRQ -t

You are in 'terminalmode' now and should be able to send 'AT-commands'

to your modem. Type 'AT <Enter>' If you don't have an 'OK'-response,

something is wrong with either Interrupt/Portadress/Modem or all of them ;-)

If this works, try more, e.g. at&v gives you a listing of the

settings with some modems (see your modem manual). If this works: call

a local bbs. Get a long listing. You should have no garbled screen and

a smooth scrolling. Try different baudrates. 38400cps should work ok

on any PC > 8086. If everything looks fine for you, things become

difficult for me ;-) ....

Do you have more then 2 serial ports? In this case you have

apparently an interrupt conflict because COM3/COM4 are using

the same IRQ's then COM1/COM2. Don't trust a 'disabled' IRQ!

It's better to remove the conflicting serial I/O-card.

----

(to be continued)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

3.0     Assembling a SLIP Gate

To begin, you must have some information about the LAN you are connecting to.

1. You need 2 IP addresses:  one for the SLIP Gate and one for the

remote users.  These addresses must belong to the same subnet.

2. You must know the address of the local gateway that connects your

SLIP server to the rest of the world.  This gateway must be

situated in the same subnet as your 2 IP addresses.

3. You must know the hardware address of your Ethernet card.  You

usually get this information when you start the packetdriver of

your card.

4. You should have the IP address of a name server.

Next, you must know to which COM port your modem is connected, either port

number or port address/IRQ combination.  You should have some experience with

your modem, i.e., BAUD rate, protocols, default state, and so on.  Load COMTool

and try to communicate with your modem so you know what's going on.

Now, if you are sure you do not have any address/IRQ conflicts, and your modem

seems to be working properly (try to contact a local BBS with COMTool or

SLIPLog's terminal mode as a confidence test!), you can modify your

'autoexec.bat.'  The 'autoexec.bat' in section 3.1, is for a SLIP Gate running

Windows 3.1.  The SLIP Gate is running in a DOS window.  Additionally,

Trumpet Winsock and all the nice Winsock applications are installed, as well

as the NFS server, XFS.  You can run these concurrently with the SLIP software.

Finally, twice a day I start a second DOS window with the aid of Wincron and

launch another TCP/IP application 'Getnews', which collects news from our

server.  These are located in special directories, accessible by ftp by the

users.  However, you need a packet multiplexer to do all these tasks

concurrently.  The primary interrupt used here is 66, multiplexing to 60 (for

the Getnews program) and to 61 for the Winsock applications.  Cslipper is

installed at 62.  A third possible multiplex interrupt is intended to be used

by an NFS client.

3.1     CONFIG.SYS and AUTOEXEC.BAT

Because NOS needs a lot of RAM, it's very important to have all TSR's loaded

to the UMBs.  I use emm386 to free an additional 32kB of RAM in the range

b000 to b7ff.  You must have an appropriate 'emmexclude' entry in the

system.ini file.

**********

config.sys

**********

LASTDRIVE=Z

FILES=50

BUFFERS=20

FCBS=4,0

COUNTRY=049,,\DOS\COUNTRY.SYS

DEVICE=C:\DOS\SETVER.EXE

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=c:\DOS\EMM386.EXE /noems /x=cc00-cfff (line continued)

i=b000-b7ff i=d000-efff i=c800-cbff

shell=c:\dos\command.com /e:512 /p

DOS=HIGH,umb

STACKS=9,256

************

autoexec.bat

************

ECHO OFF

BREAK=OFF

SET COMSPEC=C:\DOS\COMMAND.COM

SET TMP=C:\TEMP

SET TEMP=C:\TEMP

REM  Definitions required by the 'Getnews'-process

SET TZ=MET-2

SET IP=129.13.118.9

SET GATEWAY=129.13.118.254

SET DNS=129.13.64.5

SET NETMASK=255.255.255.0

SET MTU=1500

SET WINDOW=3000

SET MSS=1460

SET CITY=KARLSRUHE/GERMANY

PROMPT $P$G

PATH=C:\;C:\WINDOWS;C:\DOS;C:\NOS;

CD \NOS

comtool 2 ^b2^t5^d1^t5^d0^t5 at&f^cr^t10^

comtool 2 at&d3x3l0s0=1s2=255^cr^t10^

comtool 2 at&w0^cr^t10^

LH wd8003e 0x66 5 0x340 0xcc00

LH pktmux 3 66

LH cslipper com2 vec=62 ether

LH c:\dos\smartdrv.exe c

LH winpkt 0x62

LH pktdrv 61 66

LH winpkt 0x61

LH pktdrv 60 66

LH winpkt 0x60

nnansi.com

CD \NOS\SPOOL

LH sliplog 2 62 129.13.118.21

win slipserv.pif

If you don't have MS Windows and only need routing, your 'autoexec.bat' could

be as simple as:

************

autoexec.bat

************

CD \NOS

comtool 2 ^b2^t5^d1^t5^d0^t5 at&f^cr^t10^

comtool 2 at&f&d3x3l0s0=1s2=255^cr^t10^

comtool 2 at&w0^cr^t10^

LH wd8003e 0x60 5 0x340 0xcc00

LH cslipper com2 vec=61 ether

CD \NOS\SPOOL

LH sliplog 2 62 129.13.118.21

slipserv.bat

However, you have to edit the IP addresses, the initialization strings of

your modem (shown here is a Zyxel 1496E), the Ethernet driver, and possibly

all ports and interrupts.

3.2     Starting NOS

Generally, NOS can be launched simply by executing the following command

for example.

nos.exe -d /nos c:/nos/autoexec.nos

However, there is often an advantage to starting NOS in an endless loop using

a batch file, such as 'slipserv.bat' given below.  This batch file also

presents a good example of how the watchdog function of SLIPLog works.

Begin by executing 'slipserv.bat,' which launches NOS.  When you exit NOS, the

batch file continues to the next line, which is a pause command that asks

"continue, y/n?"  INT 28 is not called while DOS waits for your answer, so the

watchdog counter is decreased every 10 seconds.  At this point, there are

four options:

1. Type "y" to continue, which resets the counter (default is 9=90

seconds) and loops through to restart NOS;

2. Type "n" to stop the batch file from continuing;

3. Hit <CTRL-C> and the entire COMTool session is stopped;

4. Do nothing while the watchdog counter beeps every ten seconds until

the timer reaches the ninth beep and the PC is rebooted.

A possible use for 'slipserv.bat' is to insert another batch file or executable

prior to the 'pause' command.  An example is the 'settime.src' batch file that

processes your log files.  This batch file is described in Section 3.4.  You

would exit NOS as normal, let the inserted batch file do its job, and when the

pause command is run, answer "y" to loop through and restart NOS.  This allows

the SLIP gateway software to be restarted without rebooting, which would

disconnect your modem.

************

slipserv.bat

************

@ECHO OFF

:BEGIN

CD \NOS

IF EXIST spool\mqueue\*.lck DEL spool\mqueue\*.lck

IF EXIST spool\mail\*.lck DEL spool\mail\*.lck

nos.exe -d /nos c:/nos/autoexec.nos

PAUSE

SPOOL\SLIPLOG 62 -W9

GOTO BEGIN

3.3     Configuring NOS

-------------------------------------------------------------------------------

Sample 'autoexec.nos' for a  SLIP-to-Ethernet gate, tested with NOS11c*

-------------------------------------------------------------------------------

This is my installation of NOS11c for SLIP.  If you don't have a Class C

network, you probably need to change the mask-bits in the 'route add ..'

statement.  And if the routing doesn't work, you must set the netmask and

broadcast address with the 'ifconfig' statement, which I've commented out

below.  Your system administrator should know these (but often doesn't).  ;-)

!!!!   Please change all IP addresses to your own.   !!!!

!!!!  DON'T USE ANY OF THE IP ADDRESSES LISTED BELOW  !!!!

************

autoexec.nos

************

hostname mvmpc9.ciw.uni-karlsruhe.de

ip address 129.13.118.9

domain suffix ciw.uni-karlsruhe.de.

# Memory

memory thresh 15000

memory efficient yes

memory ibufsize 1600

memory nibufs 30

# TCP/IP

tcp mss 1460

tcp window 6000

tcp irtt 15000

# attach interfaces

attach packet 0x62 sl0 5 1500

attach packet 0x61 en0 5 1500

#ifconfig en0 netmask 255.255.255.0 broadcast 129.13.118.255

#log

log \nos\spool\net.log

# Domain

domain addserver 129.13.64.5

domain addserver 129.13.96.2

domain verbose on

domain translate off

# Mailbox

attended off

mbox password topsecret

mbox motd "**** This computer is in Karlsruhe, Germany ****"

echo accept

# Ethernet- Routing....

route add default en0 129.13.118.254

route add 129.13.118.0/24 en0

# SLIP-Routing

route add 129.13.118.21 sl0

# ARP

arp publish 129.13.118.21 ether 00:00:c0:0a:0f:65

# SMTP

smtp gate 129.13.64.7

smtp maxclients 10

smtp mode route

smtp timer 600

smtp t4 120

smtp usemx yes

# Set time and date

time server 129.13.64.5

time set

# Server

ftype image

ftpmaxcli 5

ftype image

ftptdisc 600

start ftp

start telnet

start smtp

start pop3

start remote

start discard

start finger

start echo

start gopher

remote -s topsecret

lock pass topsecret

at 0001 topsecret

at 0003 "source settime.src"

lock

3.4     Date Patch and Logfile Organization

The file 'settime.src' is a NOS batch file that solves the 'midnight rollover'

problem (AKA 'date problem'), where the date is not switched at midnight.

Additionally, this batch file processes and organizes my log files.  It copies

'net.log,' 'mail.log,' 'gopher.log' and 'sliplog.log' to the file 'daily.log.'

Then this combination file is copied to the file 'main.log.'

***********

settime.src

***********

CD C:\NOS\SPOOL

! /c copy /b net.log+mail.log+gopher.log+sliplog.log daily.log

! /c copy /b main.log+daily.log tmp.log

DEL main.log

REN tmp.log main.log

LOG ""

TIME SET

! /C ECHO .> net.log

! /C ECHO .> mail.log

! /C ECHO .> gopher.log

! /C ECHO .> sliplog.log

LOG c:\nos\spool\net.log

CD C:\NOS

AT NOW+2400 topsecret

AT NOW+2402 "source settime.src"

LOCK

3.5     NOS Documents

To understand all this NOS stuff, you should look for the latest '*.doc' files

distributed by the Case Western biochemistry ftp server.  There is a file

'ka9q*.txt', which is the official manual for the NOS11c distribution.  And

you should look for the file 'nos_1229.txt,' which has more information about

the basic functions of NOS.  You may obtain this files from

biochemistry.cwru.edu, or from my own server.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.0     Some Notes About Microsoft Windows

If your SLIP server is running under MS Windows, you should write a PIF file

calling 'slipserv.bat.'  Then start Windows with this file on the command line.

The PIF settings are:

- Video Memory: Text

- Memory: Required -1  Desired -1

- Execution: background

Advanced setup:

- Background Priority: 50   Foreground Priority: 100

- Detect Idle Time

- Uses High Mem

Display

- Emulate text Mode

If NOS is working in the background, it's very slow on function calls.  The

foreground/background settings have absolutely no influence on this.  After I

removed a system call to Windows in the 'giveup' routine, the background

performance was much better and the fgnd/bgnd settings worked as they should.

Removing this call seems to have no side effects.  All new NOS11cj* compiles

made by Jon Lewis have this odd function call removed.

4.1   A SLIPGate under Windows for Workgroups

As time goes by, things are becoming easier.  When I wrote this manual, Dan

Lanciani published his excellent 'ndis3pkt.386.'  This is a piece of software

that gives you a virtual packetdriver if loaded under WfW over Microsoft's

NDIS3 driver.  This driver makes the installation of a SLIP Gate under WfW

very easy:  pktmux, pktdrv and winpkt ar no longer needed!.  The only realmode

components we have to load now from DOS are cslipper and SLIPLog.  In WfW you

have to install an NDIS3 driver for your Ethernet card and to select NETBEUI

as the default protocol.  There are drivers for a lot of cards, but if you buy

a new one, be aware if Microsoft has an NDIS3 driver for it.  Otherwise you

are out of luck.

1. Launch Your network setup program and install your ethernet board.

Select NDIS3 driver and NETBEUI protocol. Delete all other

protocols if you have.

2. Test your Workgroup. You should be able to share directories and

printers with other WfW servers on the same subnet.

3. Edit your system.ini:

386Enh

device=ndis3pkt.386

....

The following entries are generated by the network install program.  You may

need to remove an old 'dis_pkt9.dos' entry from the transport line and change

the LoadRMDrivers= entry from "yes" to "no".

network drivers

netcard=ne2000.dos

transport=*netbeui,ndishlp.sys

devdir=C:\WINDOWS

LoadRMDrivers=no

This section must be entered manually.  You should find the name

of your netcard in the bindings line of the 'protocol.ini' file

ndis3pkt

netcard0=MS$NE2CLONE

intvec0=0x60

tcpmux=true

After this is done, reboot your PC.  Now you should have a virtual

packetdriver on INT 0x60.  As a check, first try some Winsock applications

with the Trumpet Winsock.  If this works, create a PIF file that starts

ka9q/nos.  Now test the Ethernet from the NOS window.  Try ping, ftp, telnet,

etc.  This should work independently from any other Winsock application you

have running at the same time.  As a last check, you could launch an

additional DOS application, my Getnews program for example.  Of course, all

applications have to use the same software interrupt (0x60).

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

5.0     Getnews

This quick and dirty program uses the NNTPGET program I found on

bichemistry.cwru.edu.  This program collects the new news articles from a

single newsgroup and a single server.  Together with my Getnews, you may

collect all the newsgroups from all the servers you define in a file called

groups.id.  Wincron starts this program twice a day as a background task, so

you need not quit NOS while Getnews is getting the news.  Here is a sample

batch file for the users 'markus,' 'khweis,' 'frank' and 'nils.'  Each user has

his own, individual accessible directory with his own groups.id file.

***********

getnews.bat

***********

CD \NOS\SPOOL\NEWS\MARKUS

getnews.exe \nos\nntpget.exe

pkzip news news.dmp

CD \NOS\SPOOL\NEWS\KHWEIS

getnews.exe \nos\nntpget.exe

pkzip news news.dmp

CD \NOS\SPOOL\NEWS\FRANK

getnews.exe \nos\nntpget.exe

pkzip news news.dmp

CD \NOS\SPOOL\NEWS\NILS

getnews.exe \nos\nntpget.exe

pkzip news news.dmp

Getnews reads the entries in the 'groups.id' file, creates a 'group.id' file

that is needed by 'nntpget.exe,' then calls NNTPGET.  After reading the news,

the 'groups.id' file is updated with the new article pointer, then the next

entry in 'groups.id' is processed.  After reading all groups, the articles

(each one is in a separate file) are concatenated to a dump file, named

'news.dmp' together with a rnews header.  You can stop the news dump after each

group by pressing any key.

Note:

You must start Getnews from the users news directory, which is the location

where your 'news.dmp' and 'groups.id' files reside.

Invoking gentnews

-----------------

Before you start Getnews, the following environment variables should be set:

REM  Your own IP address

SET IP=129.13.118.21

REM  Your netmask

SET NETMASK=255.255.255.0

REM  Your local gateway

SET GATEWAY=129.13.118.9

REM  Your nameserver (if you use domain names)

SET DNS=129.13.64.5

Start Getnews now with this command:

getnews.exe \'full_path_to_nntpget'\nntpget.exe

You have to write the full pathname and the name of the external 'nntpget.exe'

on the command line.

The structure of 'groups.id' file is as follows:

server=xxx.xxx.xxx.xxx     Name or IP address definition of your news server.

0 new.news.group           Opens (or subscribes to) a new newsgroup or rereads

all messages in it.

n1 n2 old.news.group       Reads a currently subscribed newsgroup.  'n1' and

'n2' are respectively the old and new pointers to

the last article read.

server=xxx.xxx.xxx.xxx     Moves to next server and newsgroups.

<...  etc.  ...>             ...  and so on ...

I have added the Pascal source code of Getnews to this distribution in the

file 'getnews.pas.'  You can modify and recompile it for your own purposes

using Turbo-Pascal 6.0.

You can read the news dump with an offline reader.  The only one I tested is

Crosspoint, a very popular program.  Crosspoint isn't only an offline reader,

it's also a point program for a lot of networks, like Fidonet, Z-Net,

Maus-Net, etc.  You may also poll uucp boxes with this program, but I need

only the 'uuz' converter, which converts the news dump file to z-connect

format, which is internally used by Crosspoint.

Other offline readers might also process my news dump file.  Look at the

import functions.  If this function accepts rnews batches, you possibly

have found a working newsreader.  Please send me an e-mail message if you do

find one.

-------------------------------------------------------------------------------

**********************************************************************

*  This program executable, help file, and related text files may be *

*  distributed freely and used without fee by any individual for     *

*  non-commercial use.  This version of SLIPLog may NOT be           *

*       distributed as part of any commercial package.               *

*                                                                    *

*           USE THIS SOFTWARE AT YOUR OWN RISK                       *

**********************************************************************

Thanks to all those people who are working on ka9q/NOS, especially

Ashok Aiyar.  His server (biochemistry.cwru.edu) contains an

excellent NOS Version (NOS11c*.exe), the NNTPGET tool, and a lot of

tips for the installation of NOS and Winsock software!

Thanks to Jon Lewis at inorganic5.chem.ufl.edu for extensive testing

and the NOS-compiles he made exclusively for me.

Thanks to Ryan Simmons who corrected my 'broken' English and brought

this document to a readable form.

Report bugs and comments to: <khweis@mvmpc9.ciw.uni-karlsruhe.de>

Copyright (c) 1994 by K.H.  Weiss  All rights reserved.

Written by:

K.H.  Weiss           Internet: <khweis@mvmpc9.ciw.uni-karlsruhe.de>

Eulenweg 2            Phone   : (49)-721-608-2418

76536 Weingarten

Germany

-----