The tests/ directory contains a suite of script files used

to test the correctness of opcodes that bochs emulates.  If

the bochs program is passed the "-script" option, then it

is also passed the name of a script file from which to take

commands, with the intention that the script file tests some

aspect of the emulator.

Currently, bochs only supports this test script mode of execution,

until the opcodes and other emulated features are sufficiently tested.

Some time after testing is completed, work will begin on creating

the abililty to boot and execute real programs from a floppy image.

If you type "make" in this directory, the csh script file "testem"

is run.  It looks in the directory for files with extension ".test",

and for each one it finds, runs the bochs executable with the

"-script" option and name of the *.test file.  Output is redirected

to a file with the same base name, but with extension ".out".

Then, if a third file with same base name, but with extension ".ref"

exists, it is diff'd with your *.out file.  If it doesn't exist

it is created.  The idea is that after a test script file is created,

and a corresponding *.ref file is generated, the reference file

provides a means of checking for consistent behaviour across multiple

platforms, and after modifications to the emulator.

To summarize:

somefile.test   : contains the test script

somefile.out    : bx_8086 -script somefile.test >&! somefile.out

somefile.ref    : the somefile.out file from script file creator's run.

Please email me any *.test & *.ref files!!!

The remainder of this document describes the simple test script language.

=========================================================================

=========================================================================

--------------------------------------------------------------

- .print                                                     -

-                                                            -

-   Print the value of a register(s) or an arbitrary message -

--------------------------------------------------------------

Synopsis:

.print register [register ...]

.print message ...

Available registers:

eax edx ecx ebx ebp esi edi esp eflags eip cs ds ss es fs gs

Examples:

.print eax eip

.print eflags

.print An arbitrary message for documentive purposes

--------------------------------------------------------------

- .set                                                       -

-                                                            -

-   Set the value of a register                              -

--------------------------------------------------------------

Synopsis:

.set register lsb .. msb

Available registers:

eax edx ecx ebx ebp esi edi esp eflags eip cs ds ss es fs gs

Examples:

.set eax 11 22 33 44                    # yields EAX = 44332211 (44 is MSB)

.set eflags %10000000 %00001000 00 00   # set SF & OF

--------------------------------------------------------------

- #                                                          -

-                                                            -

-   Comment.  Text from '#' to EOL is ignored                -

--------------------------------------------------------------

Synopsis:

# arbitrary comment verbage

--------------------------------------------------------------

- .exit                                                      -

-                                                            -

-   Exit prematurely from script file                        -

--------------------------------------------------------------

Synopsis:

.exit

--------------------------------------------------------------

- .i                                                         -

-                                                            -

-   Execute an arbitrary instruction.  The bytes comprising  -

-   the instruction follow the '.i' and can include prefixes -

-   opcodes, mod+rm, s-i-b, addresses, & immediate data      -

--------------------------------------------------------------

Synopsis:

.i op1 .. opn

Examples:

.i 90                                        # NOP

.i 00 %10-000-100 %01-001-010 34 12 00 00    # add [00001234+EDX+ECX*2], AL

.i 26 00 %00-000-010                         # add AX, ES:[EDX]

--------------------------------------------------------------

- .no-report-vm-access                                       -

-                                                            -

-   Turn off automatic reporting of accesses to virtual      -

-   memory.                                                  -

--------------------------------------------------------------

Synopsis:

.no-report-vm-access

--------------------------------------------------------------

- .report-vm-access                                          -

-                                                            -

-   Turn on automatic reporting of accesses to virutal       -

-   memory. (default behaviour)                              -

--------------------------------------------------------------

Synopsis:

.report-vm-access

--------------------------------------------------------------

- .printvm                                                   -

-                                                            -

-   Print n sequential bytes of memory beginning at the      -

-   requested virtual memory address in the requested        -

-   segment.                                                 -

--------------------------------------------------------------

Synopsis:

.printvm  n  segment  addr_msb .. addr_lsb

Examples:

.printvm 1 ds ff ee 00 00  # print 1 byte @ VM DS:0000EEFF

.printvm 2 es ff ee 00 00  # print 2 bytes starting @ VM ES:0000EEFF