Commodore 64/128 Kermit User's Guide

Version 2.2

Electronic Version -- Copyright 1988  Dr. Evil Laboratories

All rights reserved

Commodore 64/128 Kermit

Version 2.0, 2.1, & 2.2 enhancements by Ray Moody

User's Guide v2.2 by Kent Sullivan

Kent Sullivan

16611 NE 26th St.

Bellevue, WA 98008

Rev. 4 - 1/1/92

i

Summary: C-64/128 Kermit Version 2.2

What Kermit Is

The term "Kermit" denotes both a specific protocol for

transferring files between computers AND those computer pro-

grams which can do the transferring.  Kermit as a protocol

is often called the Kermit Protocol to distinguish it from

an actual computer program.  Various versions of the Kermit

program for different computers also have longer, official

names to distinguish themselves.  Commodore 64/128 Kermit is

a public-domain telecommunications program that utilizes the

Kermit protocol for file transfers.

C-64/128 Kermit was originally known as Kermit-65.  Begin-

ning with version 2.0, the name was officially changed to

Commodore 64/128 Kermit to reflect the fact that support for

the C-128's 80 column screen had been added.

The "65" in Kermit-65 signifies that the Commodore 64 has as

its CPU the 6510A, a member of the 6500 series family, which

also includes the Apple II.  In fact, Kermit for the Commo-

dore 64/128 was originally translated from Apple II Kermit.

Recent Improvements

The following describes the improvements made to Kermit for

version 2.2.  If you are a first-time user of Kermit, you

may want to move directly to the Introduction section.  As

this manual was originally intended to accompany v2.1

(released in May, 1988), also included is a summary of v2.1

improvements.

V2.2 really only has one significant change from v2.1: reli-

able communication at 2400 bps is now possible (but ONLY

when using the C-128 screen driver).  There was quite a bit

of interest in this feature so we decided it was worth

adding right away, before any of the other scheduled

additions/fixes listed in the Future Plans for Kermit sec-

tion of Appendix A.

The main improvements to v2.1 are as follows:

+ Enhanced DEC VT-100 terminal emulation with support for

the VT-100 keypad.  Kermit should now work fine with

operating systems such as VMS.  V2.1 has some VT-102

features added as well: Insert/Delete Line/Character.

ii

+ Limited Tektronix 4010 graphics terminal emulation.  Ker-

mit can plot both graphics and text with a resolution of

320 x 200 (C-64) or 640 x 200 (C-128).

+ Commodore 128 grey key and numeric keypad support.  The

grey keys and the numeric keypad, not normally accessible

on a C-128 in C-64 mode, are active within Kermit.

+ A special file-type for transferring C Power (now sold as

Power C) source code files.  You can now download C

source code ASCII files as well as upload C Power source

files.  All necessary character translations are handled

automatically.

+ A screen driver for the Batteries Included BI-80 80

column card.  This device gives a display as nice as the

Commodore 128's 80-column hardware, but lacks many attri-

butes such as blinking and bold.

In addition to these major improvements, many smaller

changes were made.  Several bugs were squashed that affected

VT-100 emulation, the STATUS command, and other things.  For

related information, please see Appendix A: Implementation

Notes.

Hall of Fame

Many people have worked on Commodore 64/128 Kermit since its

first appearance in March, 1984, continually adding improve-

ments and fixing bugs.  Below is a list of the people who

have worked on the C-64/128 Kermit project over the years.

Programming:

David Dermott

Eric Lavitsky

Michael Marchiondo

Ray Moody

Frank Prindle

Documentation:

Eric Lavitsky

Kent Sullivan

C-64/128 Kermit User's Guide                        Page iii

Table of Contents

Summary: C-64/128 Kermit Version 2.2 ..................    i

What Kermit Is ......................................    i

Recent Improvements .................................    i

Hall of Fame ........................................   ii

Introduction: The Kermit User's Guide .................    1

What You Need to Know ...............................    1

Recommended Texts ...................................    1

Kermit Availability .................................    2

Syntax of Commands ..................................    2

Tutorial: Introduction to Kermit ......................    3

Booting Kermit ......................................    3

Choosing a Screen Driver ............................    3

The Three Modes of Kermit ...........................    4

Command Mode ......................................    4

Connect Mode ......................................    6

File-Transfer Mode ................................    7

Troubleshooting: If You Have Problems .................   10

Emulation: Terminal-Type Features .....................   11

VT-100 Emulation ....................................   11

VT-102 Emulation ....................................   12

VT-52 Emulation .....................................   12

Tektronix 4010 Emulation ............................   13

Alpha Text Mode ...................................   13

Point Plot Mode ...................................   14

Vector Plot Mode ..................................   14

Displays: Screen Driver Comparison ....................   15

C-128 80 Column Driver ..............................   15

C-64 80 Column Driver ...............................   15

Batteries Included BI-80 Driver (Standard) ..........   15

Batteries Included BI-80 Driver (Custom) ............   16

C-64 40-Column Driver ...............................   16

File Transfers: Transferring Files ....................   17

Choosing the Correct File-type ......................   17

ASCII File-type ...................................   17

Binary File-type ..................................   17

C-Power File-type .................................   18

PETSCII File-type .................................   18

Script File-type ..................................   18

Setting the File-byte-size ..........................   18

Using Eight-bit-quoting .............................   19

Special File-Transfer Mode Features .................   19

Reference: Command Descriptions .......................   20

Command List ........................................   20

Command Summary .....................................   20

Bye ...............................................   20

Connect ...........................................   20

Directory .........................................   21

Disk ..............................................   21

Exit ..............................................   22

Finish ............................................   22

Page iv                         C-64/128 Kermit User's Guide

Get ...............................................   22

Quit ..............................................   22

Receive ...........................................   23

Restore ...........................................   23

Save ..............................................   23

Send ..............................................   24

Set ...............................................   24

Set Baud ........................................   24

Set Bold ........................................   25

General Notes on Color ..........................   25

Set Border ......................................   25

Set Character ...................................   26

Set Dark-background .............................   26

Set Debugging ...................................   26

Set Eight-bit-quoting ...........................   26

Set Escape ......................................   27

Set File-byte-size ..............................   27

Set File-type ...................................   27

Set File-warning ................................   28

Set Flow-control ................................   28

Set IBM .........................................   28

Set Light-background ............................   29

Set Local-echo ..................................   29

Set Parity ......................................   29

Set Receive .....................................   30

Set Receive Eight-bit-quote-char ..............   30

Set Receive End-of-line .......................   31

Set Receive Packet-length .....................   31

Set Receive Pad-char ..........................   31

Set Receive Padding ...........................   31

Set Receive Quote-char ........................   32

Set Receive Timeout ...........................   32

Set RS232-registers .............................   32

Set Screen-driver ...............................   33

Set Send ........................................   33

Set Terminal-emulation ..........................   34

Set Word-size ...................................   34

Show ..............................................   34

Status ............................................   35

Appendix A: Implementation Notes ......................   36

Summary of Kermit Capabilities ......................   36

Known Restrictions ..................................   36

Future Plans for Kermit .............................   37

Appendix B: Default Initialization Settings ...........   38

Appendix C: Keyboard Layouts ..........................   39

Commodore 64 Map ....................................   39

Commodore 128 Map ...................................   40

Appendix D: UNIX (TM) Termcap Settings ................   41

Appendix E: BI-80 Replacement ROM Information .........   42

C-64/128 Kermit User's Guide                          Page 1

Introduction: The Kermit User's Guide

What You Need to Know

This manual introduces Kermit to new users as well as pro-

vides an organized reference for experienced users.  Some

basic knowledge of computers is required to use Kermit.  New

users should understand the basics of computers and telecom-

munications.  Please see Recommended Texts below for a short

list of books recommended for background knowledge.

Don't become frustrated if you have problems initially using

Kermit.  Careful reading of this manual will clear up most

problems that you might have.  If you have questions, don't

be afraid to ask around.  Kermit is a very popular program,

so many people know how to use it.  You may direct any ques-

tions to Ray or me at the address on the front of this

manual as well.

This manual is available in both laser-printed and elec-

tronic form.  This, the electronic version, comes as part of

the standard Kermit disk from most distribution sites

(including Kent Sullivan).  The electronic version

can be printed out using most any sequential file printer or

word processor since all special features like underlining

and bold have been removed.  The laser-printed version of

the manual is available from Kent Sullivan for $7.50

(US funds ONLY; WA residents please add 8.2% sales tax).

Recommended Texts

The following is a short list of recommended background

texts.  The area covered is listed first, followed by the

title of the text and publishing information.

+ Basic Commodore Computing: Commodore 64/64C/128 User's

Guides.

+ Basic Telecommunications: Compute!'s Telecomputing on

the Commodore 64, Compute! Books.

+ The Kermit Protocol: Kermit: A File-Transfer Protocol

for Universities, 2 parts, Byte magazine, June & July,

1984, & Kermit, A File Transfer Protocol, Digital Press

(1987).

Page 2                          C-64/128 Kermit User's Guide

Kermit Availability

All Kermit distribution for versions that Ray and I have

worked on (versions 2.0 and later) is now handled by Kent

Sullivan.  In the past, Dr. Evil Laboratories handled

distribution, but that company is now out of business.

If you have any questions or recommendations

concerning Kermit, please contact Kent Sullivan.  Kermit

is also available for download on many local BBS systems

and national information services.

Syntax of Commands

Most Kermit commands are simple English phrases.  Commands

are represented in this document in capital letters (i.e.

CONNECT) to set them off from any surrounding text.  Many

standard representations for keys are used.  Below is a

table showing the representations and the corresponding

keys:

Representation       Actual key

<C=>                 Commodore

<CAPS LOCK>          Caps Lock *

<CLR>                Clear (Shifted Home)

<CTRL>               Control

<DEL>                Delete

<ESC>                Escape *

<F1>,<F2>,...,<F8>   Function key 1,2,...,8

<HELP>               Help *

<HOME>               Home

<INST>               Insert (Shifted Delete)

<LINEFEED>           Linefeed *

<RETURN>             Return

<SPACE>              Spacebar

<STOP>               Stop

<TAB>                Tab *

(*) These keys are available on the C-128 keyboard only.

For a list of keys that have been re-defined to implement

special characters, please see Appendix C: Keyboard Layouts.

C-64/128 Kermit User's Guide                          Page 3

Tutorial: Introduction to Kermit

Booting Kermit

Kermit is written for the Commodore 64 and 128 (in 64 mode).

One Commodore disk drive (or compatible) is required, and a

modem.  Almost any modem will work except the Commodore

1660.  To boot Kermit, follow these easy steps:

For Commodore 64:

1. Turn on monitor, drive, and computer.

2. Insert the Kermit disk, label side up, into device 8,

drive 0, and close the drive door.

3. Type: LOAD ""BOOT"",8,1 and press <RETURN>.

For Commodore 128:

1. Insert the Kermit disk, label side up, into device 8,

drive 0, and close the drive door.

2. Turn on the monitor, drive, and computer.

3. Kermit will automatically boot on power-up.

Once loaded, you should see the following in black on a grey

screen with a blue border:

Commodore 64/128 Kermit version 2.2 (70)

Type '?' for help

Kermit-65>

A blinking block cursor should be just to the right of the

">".  If you don't see this on your screen, turn off all

your equipment and try again.  If you still have problems

booting Kermit, please see the Troubleshooting section.

Choosing a Screen Driver

Kermit can use many kinds of displays.  You can use either

40 columns or 80 columns (software-driven) on a standard C-

64, or, if you have a Batteries Included BI-80 80-column

card, you can use it too.  All of these options are avail-

able on a C-128, plus the C-128's built-in 80-column

display.  Kermit defaults to the setting found in its ini-

tialization file.

Each of the screen drivers has its advantages and disadvan-

tages.  Not all of the drivers, for example, are capable of

supporting features such as underlining or bold.  Please see

the Screen Driver Comparison section for further details.

Page 4                          C-64/128 Kermit User's Guide

If you want to switch to a different screen driver now,

please see the example given in the Command Mode section

below.

The Three Modes of Kermit

Kermit has three basic operational modes: Command, Connect,

and File-Transfer.  In Command mode, you can communicate

directly with Kermit to change various program settings and

also enter either of the two other modes.  In Connect mode

you can communicate with another computer (known in many

other terminal programs as Terminal mode).  In File-Transfer

mode you may send to and receive files from the computer to

which you are connected.  Let's begin by learning about Com-

mand mode and the Kermit parser.

Command Mode

When Kermit first boots, you are placed automatically in

Command mode.  From here you can communicate with the pro-

gram by typing commands at the "Kermit-65>" prompt.  Command

mode is really the program's command "parser", which is that

part of a computer program which takes your command input

and tries to interpret it and then execute the appropriate

action.

Kermit's command parser is very easy to use.  Let's begin

with a look at the main level commands.  At the top of the

screen you should see "Type '?' for help".  So let's do just

that: Type a "?".  A list of Kermit's main commands (there

are 16 of them) should appear on the screen in two columns

like this:

Kermit-65>? Keyword, one of the following:

bye         connect

directory   disk

exit        finish

get         help

quit        receive

restore     save

send        set

show        status

The "?" is a very helpful feature of the command parser.

You can type a "?" at ANY point in the middle of a command

to receive a list of the valid ways to complete the command.

A complete list of commands may be found in the Command

Descriptions section.

Let's try a slightly more difficult command to give you

practice with the parser.  For example, let's say we want to

C-64/128 Kermit User's Guide                          Page 5

change which screen display we are using.  After looking at

the various main level commands, we deduce that the SET com-

mand might be the way to get the job done.  Type: SET fol-

lowed by a space and then a "?".  Kermit should respond with

a list that looks like this:

Kermit-65>set ? Keyword, one of the following:

baud                 bold

border               character

dark-background      debugging

eight-bit-quoting    escape

file-byte-size       file-type

file-warning         flow-control

ibm                  light-background

local-echo           parity

receive              rs232-registers

send                 screen-driver

terminal-emulation   word-size

After looking through this list, we can determine we need

the SCREEN-DRIVER sub-option.  Notice that the parser con-

veniently reprinted the SET command so that we can continue

where we left off.  So, type: SCREEN-DRIVER followed by a

space and a "?".  Now we have another list, looking like

this:

Kermit-65> set screen-driver ? Keyword, one of the following:

40-columns     80-columns

bi-80          commodore-128

custom-bi-80

It should now be obvious what to do to change to a different

screen driver.  Try it out.  Now, let's go back to 80

columns.

One thing you are probably wondering is, "Do I have to type

ALL of that out?" Luckily, no, you don't.  The left arrow

key (upper left corner of the keyboard) is the "Escape" key

in Kermit.  This key will spell out a command one level at a

time for you if you provide it with the first few characters

of the command.  Note: if you are using a C-128, you can

also use <ESC>.  To try this out, type: SET SC and hit the

left arrow key.  Kermit should spell out SCREEN-DRIVER and

put a space at the end.  Notice that we had to use "sc" and

not just "s".  Consulting the list above, we can see why.

Both SEND and SCREEN-DRIVER begin with an "s".  If we had

just typed "s", Kermit would have chosen SEND because it is

the first command in the list that meets the specified

requirements.  The same applies for why we spelled out SET.

SEND is before SET in the main command list, making the left

arrow useless.  So keep in mind that you must always use

Page 6                          C-64/128 Kermit User's Guide

enough characters to uniquely identify the command before

hitting the left arrow.  Ok, let's finish our command.  Type

"8" and hit the left arrow.  Kermit should finish the com-

mand.  Now press <RETURN>.

Presto!  We're back to 80-column mode, saving 19 character's

worth of typing.  Not all commands are nearly this long, but

the left arrow (along with the "?") can be very helpful.

NOTE: Once you are familiar with the parser, you can save

even more keystrokes by using just the abbreviations for the

commands.  In other words, you don't HAVE to use the left

arrow key.  For example, you could just type "SET SC 8" to

set the screen driver to C-64 80 columns.

There are a few other useful tricks to know about the

parser.  They are:

1) Pressing <CLR> or <CTRL> - <L> instead of <RETURN> to

execute a command will clear the screen first and then

execute the command.

2) Pressing <CTRL> - <W> while typing a command will back

the cursor up to the previous word, erasing any associ-

ated text.

3) Pressing <CTRL> - <U> while typing a command will can-

cel the command by backing the cursor up to the

"Kermit-65>" prompt.

Connect Mode

Connect mode is the mode in which you will spend most of

your time while using Kermit because it is in this mode that

you can communicate with another computer (often known as

the remote host).  Now let's prepare to connect to another

computer.  Type: CONNECT.  This command will take us out of

Command mode and into Connect mode.  If you type CONNECT

then hit <CLR>, the screen will be cleared before entering

Connect mode.  Every time you enter Connect mode, you will

see the message:

[Connecting to host: Type ^Y c to return]

This tells us that we are in Connect mode and that to return

to Command mode we should type <CTRL> - <Y> then type "c".

Try it out.  Then return to Connect mode.

Now we will call the other computer.  A Commodore 1670 or

other Hayes compatible modem can dial directly from Connect

mode via its built-in dialing commands, but with most other

modems you will have to dial manually.  In other words, Ker-

mit does NOT have the capability to auto-dial a modem

C-64/128 Kermit User's Guide                          Page 7

directly.  Also please note that Kermit currently does NOT

work with the Commodore 1660 modem.  It will, however, be

supported in a future version.  Please see Appendix A:

Implementation Notes for further information.

We need to check a few things before dialing.  Each computer

you connect to may have different baud, parity, and word-

size requirements.  Kermit defaults to the settings found in

the initialization file: 1200 bps transmission speed, eight

bit word length, no parity, and VT-100 terminal emulation.

These settings may be different if you did not get your copy

of Kermit from Kent Sullivan.  To adjust any of these param-

eters, escape back to Command mode and use the SET command

as we did above to change the screen-driver. For example, to

change to 300 bps transmission speed, type: SET BAUD 300.

Remember to use the "?" to get help at any level.

If you wish to save any changes you make as new defaults,

please see the SAVE command explanation in the Command

Descriptions section.

Once you have set up your communications parameters

correctly, all that's left to do is dial the phone and con-

nect to the remote host.

File-Transfer Mode

Once you have connected to a remote host you may wish to

send and/or receive files.  With Kermit you can send and

receive most any kind of file, as Kermit has automatic

translation features for many types of files.  For example,

you can automatically convert a PETSCII (Commodore's special

ASCII) text file to ASCII when sending it to another

machine, and vice versa.  For a list of the various file-

types supported, see the SET FILE-TYPE command explanation

in the Command Descriptions section.

The procedures for transferring files with Kermit can differ

widely depending on the system to which you are connected.

This tutorial can't possibly cover every possible situation.

Instead, we will walk through a simple transfer and point

out some things of which you should be aware.  The MOST

important thing when attempting to transfer files with Ker-

mit is to have your file-transfer parameters set correctly!

You may want to skip ahead to the Transferring Files section

for more information.

Now, we will show two examples: 1) uploading a PETSCII

sequential text file, converting it to standard ASCII, and

2) downloading a C language program source code file for use

with the C Power (now sold as Power C) C compiler for the

64/128.

Page 8                          C-64/128 Kermit User's Guide

Text File:

We are sending and the other end is receiving.  In note

form, here is what we need to do:

1) Check file-transfer parameters!!  Make sure the FILE-

TYPE is set to PETSCII.

2) Start Kermit on the host machine using the appropriate

command.

3) Tell the host Kermit to RECEIVE the file.

4) Escape to Command mode in our Kermit (<CTRL> - <Y> then

<"c">).  Type SEND followed by the filename of the Com-

modore text file you wish to send.

When the two Kermits have finished transferring the file,

you will be returned to Command mode.  Type: CONNECT and

continue with your login session.

NOTE: The recommended parameters (work most of the time)

are:

File type is PETSCII

File byte size is EIGHT-BIT

Everything else should retain its default value.

C Power File:

We are receiving and the host is sending.  In note form,

here is what we need to do:

1) Check parameters!!  Make sure the FILE-TYPE is set to

C-POWER.

2) Start Kermit on the host machine using the appropriate

command.

3) Tell the host Kermit to SEND the file.

4) Escape to Command mode in our Kermit (<CTRL> - <Y> then

<"c">).  Type RECEIVE followed by the filename of the C

source code file you wish to receive in C Power format.

5) When the two Kermits have finished transferring the

file, you will be returned to Command mode.  Type: CON-

NECT and continue with your login session.

NOTE: The recommended parameters (work most of the time)

are:

C-64/128 Kermit User's Guide                          Page 9

File type is C-POWER

File byte size is EIGHT-BIT

Word size is EIGHT-BIT

Everything else should retain its default value.

If you have persistent problems with transferring files,

don't hesitate to ask someone for help.  Also try VERBOSE

debugging to see more of what is happening during transfers.

Kermit is very flexible, but flexible can mean complicated!

We now have completed the introductory tutorial.  The next

section of this manual, Troubleshooting, may help if you

have some problems using Kermit.  The Command Descriptions

section describes in detail each of Kermit's commands and

will prove to be invaluable to successfully using this pro-

gram.  Good Luck with Kermit!

Page 10                         C-64/128 Kermit User's Guide

Troubleshooting: If You Have Problems

Here are some common problems and their most likely solu-

tions.  The best troubleshooting usually comes from someone

who has experience using Kermit, but this isn't always

available.

1) Problem: Kermit won't boot.

Solution: Make sure all of your equipment is hooked up

correctly.  If the red error light blinks on your

disk drive, you might a) have a bad disk or b) be

missing the initialization file, KERMIT.INI.  See the

Known Restrictions section of Appendix A for more

information.

2) Problem: I can't get out of Connect mode.

Solution: Remember to type <CTRL> - <Y>, let up, then

hit the letter "c".  The escape character might also

be set to something other than <CTRL> - <Y>.  You can

check it with the SHOW ESCAPE command and change it

with the SET ESCAPE command.

3) Problem: When I connect to a host machine, all I get is

garbage.

Solution: Check the baud, parity and word-size parame-

ters.  You may have to hang up and try again if you

change some of the parameters.

4) Problem: I can't see what I am typing, but I see what

the host machine is sending.

Solution: Check to see if the host machine is supposed

to echo characters back to you.  You might need to

SET LOCAL-ECHO ON.

5) Problem: I downloaded a binary file but the file I

received was garbage.

Solution: Remember to SET EIGHT-BIT QUOTING ON.  Also,

check the file type on the host Kermit.  It should be

set to BINARY as well as on your end.

6) Problem: All of a sudden output stopped appearing on

the screen and nothing I type shows up either.

Solution: You may have hit the <C=> key accidently,

which pauses all data transmission temporarily.

Press it again to re-start transmission.

C-64/128 Kermit User's Guide                         Page 11

Emulation: Terminal-Type Features

Kermit is a very powerful terminal emulator.  For text

displays, it can emulate both a VT-100 (with or without

added VT-102 features) and a VT-52.  For graphics work, Ker-

mit has limited Tektronix 4010 emulation.  Emulation can be

turned completely off as well.

It is beyond the scope of this manual to present each of the

escape sequences that these terminals use.  This section

will instead describe in general terms what each of the emu-

lations can do, and provide a table of which Tek commands

are supported.

VT-100 Emulation

Most people will use Kermit's VT-100 emulation more than any

other.  The DEC VT-100 terminal is an industry standard, and

so is supported on virtually every medium-to-large-sized

operating system.  The VT-100 terminal provides you with

many powerful features that make communicating with a remote

host (especially at speeds under 9600 bps) much more effi-

cient and therefore, quicker.

The major VT-100 features are summarized below:

1) Scrolling regions.  This feature of the VT-100 allows

selected parts of the screen to be scrolled, cutting

down on character transmission greatly-- making

telecommunicating faster.

2) Text attributes.  Text on a VT-100 can be displayed in

normal or bold, and be vanilla, underlined, or blinking

(or any logical combination), and displayed on either a

dark or light background.  Kermit has the added feature

of selectable colors for text, bold text, dark-

background, light-background, and border for most

screen drivers.

3) Numeric keypad.  A VT-100 has a numeric keypad that

includes four personal function (PF) keys.  Kermit has

this feature on both a C-64 & 128.

4) Graphics characters.  A VT-100 has a wide range of

graphics characters that can be used for graphs and

charts.

5) Status line.  Kermit has an extra 25th screen line that

can be used as a system status line on operating sys-

tems such as UNIX (TM).  Note that a standard VT-100

terminal does NOT have a status line.

Page 12                         C-64/128 Kermit User's Guide

For a chart showing what keys have been re-defined in Ker-

mit, please see Appendix C: Keyboard Layouts.  Also, if you

are using Kermit on a machine running UNIX (TM), please see

Appendix D: UNIX (TM) Termcap Settings, for information on

how to customize your termcap to take full advantage of

Kermit's features.

Not all of the VT-100's features were feasible to support in

Kermit.  Below is a list of the restrictions to the VT-100

emulation:

1) No 132 column mode

2) No double high/double wide characters

3) Limited terminal reports (only the important ones are

emulated)

4) No foreign character sets

5) No smooth scrolling

6) No terminal LEDs

For related information, please see the Screen Driver Com-

parison section.

VT-102 Emulation

Only the four most important VT-102 terminal features are

present in Kermit.  These are Insert Character, Delete Char-

acter, Insert Line, and Delete Line.  The Insert Character

function, as on a real VT-102 terminal, is really a switch-

able Insert/Replace mode.

These additional features allow MUCH faster, more efficient

communication, since the C-64/128 handles much of the time-

consuming work (inserting and deleting lines & moving char-

acters around on a line).  If you are able to utilize the

VT-102 features, you will be pleasantly surprised at how

much quicker certain operations are, such as editing text

files.

VT-52 Emulation

Kermit emulates a VT-52 terminal as a subset of its VT-100

emulation, just as a real VT-100 terminal does.  It is a

complete emulation, except that the graphics character set

is the VT-100's, not a true VT-52's.  All of the real VT-100

terminals we have seen also use the VT-100 character set in

C-64/128 Kermit User's Guide                         Page 13

place of the true VT-52 set in VT-52 mode.

Tektronix 4010 Emulation

Kermit supports a limited kind of Tektronix 4010 emulation.

Two resolutions are supported: 320 x 200 (C-64) and 640 x

200 (C-128 80 col.).  You can use the Tektronix emulation

even if you are using a BI-80 card, as Kermit will automati-

cally switch between the appropriate graphics screen and the

BI-80.  All plots and text are plotted using the current

character color and are plotted on the current background

color.  Both the 320 x 200 and 640 x 200 screens are a lit-

tle too wide horizontally scale-wise compared with a real

Tektronix 4010 terminal.  We thought it was better to use

the full width of the screen given the limited resolution

than to have perfect scaling.

Kermit supports three Tek modes: Alpha Text, Point Plot, and

Vector Plot, for a total of eight different Tek commands.

The commands are:

Sequence                Action

<ESC> FF (Form Feed)    Activate Tek emulation and clear screen

RS (Record Separator)   Switch to incremental plot mode

US (Unit Separator)     Switch to alpha text mode

GS (Group Separator)    Switch to vector plot mode

CAN (Cancel)            Deactivate Tek emulation and clear screen

Tek mode is also exited when many VT-100/52 escape sequences

are received.  These include any sequences beginning with

<ESC> "[" or <ESC> <upper-case letter>.  Also, once Tek mode

is activated, any further <ESC> FF sequences received will

clear the current graphics screen.  Finally, the current

text screen is not saved when Tek mode is entered, and the

current Tek screen is not saved when Tek mode is exited.

Alpha Text Mode

Alpha Text mode uses the standard ASCII character set and

can display 79 columns by 24 lines of text if Tek mode is

entered from any of the screen drivers except C-64 40

columns.  If Tek mode is entered from the C-64 40 column

screen driver, up to 40 columns of text can be displayed.

This mode is used generally to label graphs.  No on-screen

Page 14                         C-64/128 Kermit User's Guide

editing should be attempted while in this mode.

Point Plot Mode

In this mode the drawing pen is given a direction to move

and whether to have the pen up (no line drawn) or down (line

drawn) when moving.  This allows considerable freedom in

drawing odd-shaped objects.

Vector Plot Mode

In Vector Plot Mode, just the endpoints of a line to be

drawn are specified.  This allows lines to be drawn quickly,

since not all the points on the lines must be specified

individually.

C-64/128 Kermit User's Guide                         Page 15

Displays: Screen Driver Comparison

Each of Kermit's screen drivers has different capabilities.

Below is a discussion of the various limitations of the dif-

ferent screen drivers.

C-128 80 Column Driver

This driver is the best that Kermit has.  All video attri-

butes are supported with no limitations, and the highest Tek

resolution (640 x 200) is available. The only feature not

available with this driver, that is on some others, is a

border.  This is because the C-128's 80 column chip (the

8563 VDC) does not support a border.

One note: many people who use the 80-column mode on the C-

128 do so on a monochrome monitor.  With this setup, only

THREE colors are possible: bright, normal, and black.  This

can pose a problem when trying to set up readable light- and

dark-background screens.  One solution to this is to buy a

grey scale attachment for the 80-column video cable to pro-

vide a wider range of grey shades.

C-64 80 Column Driver

This screen driver is traditionally the most popular, mainly

because more people have C-64s than C-128s.  When using this

driver, even on a C-128, the Tek 320 x 200 mode will be used

for graphics.  This driver is full-featured except for one

minor limitation.

Because of the way hi-res graphics screens are created on a

C-64, if two characters with different attributes are placed

adjacent horizontally, the first character sometimes will

not have the correct attribute.  For example, if a bold

character is printed immediately followed by a flashing

character, the bold character may change to a flashing char-

acter.  This can easily be avoided by inserting a space

between the two characters.

Batteries Included BI-80 Driver (Standard)

The BI-80, an excellent 80-column card for the C-64 (also

C-128 in 64 mode), is no longer in production and generally

very hard to find used.  We added support for it because it

offers the same text resolution as the C-128's 80 column

screen, making it clearer than the standard C-64 80 column

screen.

There are several limitations when using this driver.  The

Page 16                         C-64/128 Kermit User's Guide

main limitation is that the BI-80 character set is not rede-

finable through software, meaning that some characters like

the braces, tilde, underscore, etc., are not available.

Also, the VT-100/52 graphics character set is not available.

A closest match was made on many of the characters, but some

of them look pretty unrecognizable.

Also, the only character attribute available is reverse.

All other modes, such as blinking, bold, and underlined, are

mapped into reverse, so you can at least tell when the text

is not plain vanilla.

Finally, no color change options are available as the BI-80

card operates only in black and white mode with no border.

Batteries Included BI-80 Driver (Custom)

This driver is for use with the custom BI-80 character ROM

available from Kent Sullivan.  This driver makes use

of the re-defined characters in the replacement ROM.  The

full ASCII character set is available, as well as the VT-

100/52 graphics character set.  All other restrictions from

the standard BI-80 driver apply.

Please see Appendix E: BI-80 Replacement ROM Information for

details on how to obtain a replacement character ROM.

C-64 40-Column Driver

This driver supports all character attributes and screen

functions.  Its main limitation is its width-- 40 columns is

generally not very useful when communicating with most large

computer systems.

C-64/128 Kermit User's Guide                         Page 17

File Transfers: Transferring Files

Transferring files with Kermit isn't really any more diffi-

cult than with most other Commodore telecommunications

packages/protocols.  In fact, Kermit is much more flexible--

but flexibility can be confusing.

There are three major parameters to be concerned with when

transferring files.  They are: 1) File-type, 2) File-byte-

size, and 3) Eight-bit-quoting.  Let's discuss each of them

in turn, beginning with File-type.

Choosing the Correct File-type

Kermit currently supports five different file types. They

are:

a) ASCII

b) Binary

c) C-Power

d) PETSCII

e) Script

Each of these file types has a distinct use.  When to use

one file-type over another can be confusing initially.  So,

we'll discuss each of them separately, starting with ASCII.

ASCII File-type

This file-type is used a lot less often than you might think

at first.  Because Commodore computers (except for the Ami-

gas and PCs) use their own, modified character set arrange-

ment (PETSCII), most text files you transfer to and from

your Commodore will be PETSCII files, not standard ASCII.

You will use this file-type only when you a) wish to down-

load a standard ASCII file and NOT convert it to a form

usable on your Commodore (PETSCII), or b) you have, for some

reason, a standard ASCII file that you wish to upload to

another computer.

If you happen to choose this file-type when you really

needed PETSCII, you will be able to tell immediately, as the

case of all the text will be reversed.

Binary File-type

The Binary file-type is not used as often as some of the

other file-types.  This file-type is used to transfer Commo-

dore programs that are executable (i.e. a BASIC program or a

machine language game), or memory images such as graphics

screens.  In other, more technical, terms, this file-type

Page 18                         C-64/128 Kermit User's Guide

allows you to transfer files without performing any charac-

ter translation.  Note that it's generally a good idea to

have Eight-bit-quoting set on when transferring binary

files, but more about that below.

C-Power File-type

This file-type is designed specifically to transfer source

code files for use with the C Power (now sold as Power C) C

compiler for the C-64 and C-128.  C Power files have, in

addition to the standard PETSCII file-type base, a few re-

defined characters that are needed for C programming, such

as braces, the underscore, the pipe, etc.

When you use this file-type, any standard C source code

files you download to your Commodore will be converted to C

Power format.  On the other side of the coin, any C Power

files you upload to another computer will be converted to

standard ASCII source code files.

PETSCII File-type

This file-type is the most common file-type, and so it is

the default Kermit file-type.  When you use this file-type,

any Commodore text file you upload to another computer will

be converted to standard ASCII text.  Going the other direc-

tion, when you download a standard ASCII text file to your

Commodore, it will be translated to PETSCII.

If you choose this file type when you really meant ASCII,

the case of all the letters in the text will be reversed.

Script File-type

This is a special file-type for use when transferring files

for the Commodore Speedscript word processor (Compute! Pub-

lications).  Speedscript files have a special file format.

If you want to download a standard text file from another

computer for Speedscript, you must use this format.  The

same applies for sending a Speedscript file to another com-

puter that uses standard ASCII text files.  Note that no

attempt is made to translate any of Speedscript's special

text formatting or control codes-- you will have to make the

appropriate changes manually.

Setting the File-byte-size

This parameter is often confusing because it sounds a lot

like the Word-size parameter.  These two parameters are

actually quite different.  Whereas Word-size describes the

width of the data path, File-byte-size describes the size of

the data words within files being transferred.

C-64/128 Kermit User's Guide                         Page 19

This parameter can always be left set to EIGHT-BIT.  Techni-

cally, it is really only necessary to be set to EIGHT-BIT

when you are transferring Binary or PETSCII files.

Using Eight-bit-quoting

This feature allows you to transfer Binary or PETSCII files

to and from computers that would normally be unable to han-

dle them.  Eight-bit-quoting allows you to send files with

eight-bit data words over a seven-bit data path.  This

parameter can always be left set to ON.

For instance, I often communicate with a machine where my

communications parameters have to be Word-size of Seven-bit

and Even Parity.  In this kind of environment I would nor-

mally be unable to send a program file to a friend on that

machine, as program files are Binary (eight-bits).  But if I

make sure Eight-bit-quoting is ON, everything will be fine.

Technically, Eight-bit-quoting prefixes each byte of data

that is over seven bits long (greater than ASCII 127 decimal

in value) so that it can be transferred along a seven-bit

data path.

Special File-Transfer Mode Features

There are three helpful features available when transferring

files.  These features are standard in most versions of Ker-

mit.  They are:

Key Sequence   Action

<RETURN>       Force local Kermit to re-send current packet

<CTRL> - <X>   Cancel transmission of current file

<CTRL> - <Y>   Cancel transmission of entire batch of files

All of these commands are available when receiving a file or

files.  Because C-64/128 Kermit can't send more than one

file per SEND command (a feature called "wildcard send"),

the <CTRL> - <Y> feature is not applicable when sending a

file.

Page 20                         C-64/128 Kermit User's Guide

Reference: Command Descriptions

Command List

Kermit v2.2 has 16 major commands, many of which have sub-

options.  Below is a list of the major commands.

Bye    Connect   Directory   Disk      Exit      Finish

Get    Help      Quit        Receive   Restore   Save

Send   Set       Show        Status

Command Summary

Each of the major commands are described below in alphabeti-

cal order.  Following the description is one or more exam-

ples using the command previously described.  Please note

that all commands may be entered in either lower or upper

case-- case distinction is not made by the Kermit parser.

Bye

This command causes C-64/128 Kermit to tell the remote Ker-

mit, which must be in server mode, to exit and, if applica-

ble, terminate its job (or process, etc.).  The remote Ker-

mit then closes the connection.  C-64/128 Kermit finishes by

exiting to BASIC.  Bye has no arguments, with the format:

Kermit-65>bye

Connect

The CONNECT command puts Kermit into Connect Mode (see the

Introduction to Kermit section for more information on Con-

nect Mode).  Kermit can then begin communication with

another computer as a terminal (as defined by the SET

TERMINAL-EMULATION command).  Connect has no arguments, with

the format:

Kermit-65>connect

To return to Command Mode (see the Introduction to Kermit

section for more information on Command Mode), type the

escape character followed by a <"c">.  The default escape

character is <CTRL> - <Y>.  The escape character may be fol-

lowed by characters other than <"c">.  The other valid

options and what they do can be viewed by substituting a

<"?"> for the <"c">, giving the following display:

C-64/128 Kermit User's Guide                         Page 21

One of the following:

?             - this help message.

b             - send a break signal.

c             - close the connection.

s             - status of connection.

0             - send a null.

escape-char   - transmit the escape character.

Directory

The DIRECTORY command returns a disk directory of the speci-

fied files on device 8 (the normal device number for the

first serial Commodore disk drive).  DIRECTORY with no argu-

ments returns a complete disk directory.  The directory may

be paused by pressing <SPACE> and resumed by pressing any

key, and aborted by pressing <STOP>.  The format is:

Kermit-65>directory [filename]

where [filename] is a valid CBM DOS filename.  Some exam-

ples:

Kermit-65>directory        (directory of all files

on device 8)

Kermit-65>directory ker*   (directory of files be-

ginning with "ker")

Kermit-65>directory k?w    (directory of files 3

characters long, begin-

ning with "k" and ending

with "w")

Disk

The DISK command allows you to send DOS commands directly to

the disk drive controller.  This command is useful for ini-

tializing the drive, formatting a disk, and deleting

unwanted files.  This command supports all the standard Com-

modore DOS commands, except that NO spaces in filenames are

allowed.  This is actually a "feature" of the parser and

will be corrected in a future version.  DISK with no argu-

ments will return the drive error in standard DOS format if

the drive's red light is blinking.  The format is:

Kermit-65>disk [string]

where [string] is a valid CBM DOS command.  Some examples:

Page 22                         C-64/128 Kermit User's Guide

Kermit-65>disk s0:k*         (delete all files begin-

ning with "k" on device

8)

Kermit-65>disk i0:           (initialize device 8)

Kermit-65>disk n0:files,01   (format disk in device 8,

giving it name "files"

and id "01"-- note the

lack of spaces in the

disk name chosen)

Exit

The EXIT command causes C-64/128 Kermit to stop execution

and return control of the machine to BASIC.  This command is

the same as the QUIT command.  Kermit may be restarted

immediately with the command RUN.  If you type NEW, Kermit

can still be restarted most of the time with the command SYS

2064.  Exit has no arguments.  The format of this command

is:

Kermit-65>exit

Finish

This command causes C-64/128 Kermit to tell the remote Ker-

mit, which must be in server mode, to exit.  When C-64/128

Kermit receives the acknowledgement that this is being done,

you are prompted for another command.  Finish has no argu-

ments, with the format:

Kermit-65>finish

Get

This command causes C-64/128 Kermit to request that the

other Kermit (which must be running in server mode) to

transmit the specified file to the C-64/128 Kermit.  The

format of this command is:

Kermit-65>get [file-specification]

where [file-specification] is a valid file specification for

the system on which the other Kermit is running.

Quit

This command causes C-64/128 Kermit to stop execution and

return control of the machine to BASIC.  QUIT is identical

to the EXIT command (see EXIT above for more information).

QUIT has no arguments.  The format this command is:

C-64/128 Kermit User's Guide                         Page 23

Kermit-65>quit

Receive

The RECEIVE command causes C-64/128 Kermit to wait for the

remote Kermit to initiate a file transfer (using a command

like SEND).  The format for this command is:

Kermit-65>receive [filename]

where [filename] is any valid CBM DOS file name not contain-

ing spaces.  If no file name is given, C-64/128 Kermit will

wait for a file transfer initialization sequence from the

other Kermit.  If a file name is given, that file name will

be used to store the received file(s), regardless of what

name is supplied by the other Kermit.  If more than one file

is received, the second file will have a ".00" appended to

it.  The third file will have a ".01", and so on.  The file

name will be truncated appropriately to accommodate the

".XX" suffix.  If FILE-WARNING is turned OFF, the ".XX" suf-

fix scheme will not be used-- transmission will stop with

disk error 63, FILE EXISTS.

Some examples:

Kermit-65>receive

Kermit-65>receive myfile

Restore

The RESTORE command is used to re-initialize Kermit parame-

ters that have been previously saved with the SAVE command.

Kermit searches the disk in device 8 for a file named

KERMIT.INI.  If this file is present on disk, Kermit loads

the parameters contained therein and restarts itself with

those new parameters.  If no KERMIT.INI file is found, Ker-

mit crashes, exiting to BASIC.  You must insert a disk con-

taining a KERMIT.INI file in order to restart Kermit.

RESTORE has no arguments, with the command format:

Kermit-65>restore

Save

The SAVE command saves to the disk in device 8 the current

Kermit settings and parameters in the file KERMIT.INI.  If a

KERMIT.INI already exists on the disk, no attempt is made to

write over the existing file.  The blinking red light on the

disk drive indicates error #63, FILE EXISTS.  It can be

scratched using the DISK command, however.  The parameters

may later be recalled with the RESTORE command.  SAVE has no

arguments, with the format:

Page 24                         C-64/128 Kermit User's Guide

Kermit-65>save

Send

The SEND command allows you to send a file to the remote

Kermit.  The command format is:

Kermit-65>send [filename]

where [filename] is any valid CBM DOS filename.  Please note

that the C-64/128 currently does not have a wildcard send

capability.  In other words, the entire file name must be

specified-- no pattern matching is allowed.  Also, only one

file may be sent per SEND command.

Set

The SET command is used to alter various Kermit parameters.

The format of this command is:

Kermit-65>set [option] [suboption]...

where [option] is one of the following:

Baud                Bold               Border

Character           Dark-background    Debugging

Eight-bit-quoting   Escape             File-byte-size

File-type           File-warning       Flow-control

IBM                 Light-background   Local-echo

Parity              Receive            RS232-registers

Screen-driver       Send               Terminal-emulation

Word-size

Many of the above options have sub-options.  A description

of the options and sub-options follows.

Set Baud

The SET BAUD command is used to set the speed at which com-

munications will take place.  The format is:

Kermit-65>set baud [value]

where [value] is one of the following:

300   1200   2400

Please note that communication speeds above 1200 are possi-

ble ONLY on a C-128 when using the COMMODORE-128 screen

driver.  Because of hardware limitations, speeds above 1200

baud are not possible on a C-64.  Also, please note that

this command does not support speeds under 300 bps.  You

C-64/128 Kermit User's Guide                         Page 25

will need to use the SET RS232-REGISTERS command instead.

See the SET RS232-REGISTERS section and the Known Restric-

tions section of Appendix A for further information.

Set Bold

The SET BOLD command sets the bold character color, which

can be used for highlighting text in VT-100 emulation mode.

An example of bold text is the "Type ? for help" message

when Kermit boots.  The default bold color is white.  The

format of this command is:

Kermit-65>set bold [color]

where [color] is one of the following:

Black        Blue          Brown

Cyan         Dark-grey     Green

Light-blue   Light-green   Light-grey

Light-red    Medium-grey   Orange

Purple       Red           White

Yellow

General Notes on Color

The above colors correspond to the C-64 VIC chip colors.  A

closest mapping was done for the C-128 8563 VDC (the 80-

column chip) BUT: 1) orange is really a dark purple while 2)

medium-grey has a greenish tint.  Also, note that:

1) The C-128 display has no border (just like a real VT-

100).

2) The BI-80 has no color options or border-- and, any

highlighted text is printed as reverse.

3) None of the SET commands for color currently have a

corresponding SHOW command.

A real VT-100 terminal doesn't have any color options at

all-- simply light text on a dark background, or vice versa.

So, most of Kermit's screen drivers give you an added capa-

bility with their color options.

Set Border

The SET BORDER command sets the border color, which is

available only when using the C-64 40 or 80 column screen

drivers.  The default border color is blue.  The format of

this command is:

Kermit-65>set border [color]

where [color] is one of the colors listed under the "Set

Page 26                         C-64/128 Kermit User's Guide

Bold" section.

Set Character

The SET CHARACTER command sets the normal character color.

The default character color is black.  The format for this

command is:

Kermit-65>set character [color]

where [color] is one of the colors listed under the "Set

Bold" section.

Set Dark-background

The SET DARK-BACKGROUND command sets the VT-100 dark back-

ground color (the one normally used).  This is also the

background color used in VT-52 mode or when no terminal emu-

lation is used.  The default dark-background color is

medium-grey.  The format for this command is:

Kermit-65>set dark-background [color]

where [color] is one of the colors listed under the "Set

Bold" section.

Set Debugging

The SET DEBUGGING command is used to set the type of debug-

ging generated by C-64/128 Kermit during a file transfer.

The format is:

Kermit-65>set debugging [keyword]

where [keyword] is either OFF, TERSE, or VERBOSE.  OFF

causes all debugging output to be suppressed.  TERSE directs

a copy of each packet to the screen, printing any control

characters as <CTRL> - <"char">.  VERBOSE prints identifying

text for each field in the header and prints the actual data

section of the packet in the same manner TERSE uses.

Set Eight-bit-quoting

This command turns on or turns off the eight-bit-quoting

feature.  Eight bit quoting is used to transmit eight-bit

bytes over a seven-bit data path.  If the WORD-SIZE is

SEVEN-BIT, eight bit quoting allows full eight-bit data to

still be transmitted.  If the WORD-SIZE is EIGHT-BIT, the

quoting is ignored as it is not needed.  The format of the

command is:

Kermit-65>set eight-bit-quoting [keyword]

C-64/128 Kermit User's Guide                         Page 27

where [keyword] is ON or OFF.

Set Escape

This command sets the escape character, which is the charac-

ter used to "escape" back to Command Mode from Connect Mode.

The command takes a hexadecimal value of the chosen charac-

ter to use as the escape character.  The default character

is <CTRL> - <Y> ($19).  It is usually a good idea to set

this character to something which is not used (or at least

not used very often) on the system to which C-64/128 Kermit

is connected.  The format of the command is:

Kermit-65>set escape [hex value]

where [hex value] is the ASCII value, in hex, of the charac-

ter to use as the escape character.

Set File-byte-size

This command sets the byte size C-64/128 Kermit will use for

files being received or transmitted.  The file byte size can

almost always be EIGHT-BIT, as this setting will work for

most SEVEN-BIT and EIGHT-BIT data files.  The format of this

command is:

Kermit-65>set file-byte-size [size]

where [size] is SEVEN-BIT or EIGHT-BIT.

Set File-type

This command tells Kermit what type of file is being sent or

received.  It is important that this parameter be set

correctly since it is used to determine the exact file

length when sending files in addition to determining special

processing which may have to be done (i.e. when receiving a

text file, write all characters in PETSCII).  The format of

this command is:

Kermit-65>set file-type [type]

where [type] is ASCII, BINARY, C-POWER, PETSCII, or SCRIPT.

An ASCII type file is any standard ASCII text file.  A

BINARY type file is a memory image (i.e. machine language

program, graphics screen, etc.) and is normally transferred

in eight-bit mode.  A C-POWER type file is a C language

source file for use with the C Power C compiler from Pro-

Line (now sold as Power C by Spinnaker).  A PETSCII type

file is a CBM ASCII text file.  A SCRIPT type file is a file

produced by the Speedscript word processor published by Com-

Page 28                         C-64/128 Kermit User's Guide

pute! Publications.

Set File-warning

This command determines what C-64/128 Kermit will do when it

receives a file with a filename that already exists.  If

FILE-WARNING is turned ON, C-64/128 Kermit will not

overwrite the old file.  Instead, it will create a file with

the original file name with a ".00" appended to it.  It will

increment the ".XX" number until it succeeds in creating a

unique file name.  If FILE-WARNING is turned OFF, C-64/128

Kermit will stop with disk error 63, FILE EXISTS.  The for-

mat of this command is:

Kermit-65>set file-warning [keyword]

where [keyword] is either ON or OFF.

Set Flow-control

The SET FLOW-CONTROL command enables or disables XON/XOFF

flow control.  With flow control on, C-64/128 Kermit will

automatically send a <CTRL> - <S> to the host computer when

it can't keep up (i.e. buffer full) to prevent loss of char-

acters.  Kermit will then send a <CTRL> - <Q> when it is

ready to receive more data.  This option is recommended for

communication speeds of 1200 baud or higher.  The format of

this command is:

Kermit-65>set flow-control [keyword]

where [keyword] is either ON or OFF.

Set IBM

The SET IBM command sets up C-64/128 Kermit to communicate

with certain IBM mainframes.  This command is needed on a

limited number of systems.  The format of this command is:

Kermit-65>set ibm [keyword]

where [keyword] is either ON or OFF.  With IBM mode on, C-

64/128 Kermit waits for the IBM turnaround character (XON,

<CTRL> - <Q>) before sending any characters to the other

Kermit.  It also forces the parity type to be mark, the word

length to be seven-bit, turns on local echo for the CONNECT

command and turns off flow-control.  All of these changes

are temporary.  When IBM mode is turned off, any parameters

C-64/128 Kermit User's Guide                         Page 29

altered are returned to their previous values.

Set Light-background

The SET LIGHT-BACKGROUND command sets the VT-100 light back-

ground color (the one not normally used).  The light back-

ground is generally used only by programs to catch your

attention (usually by flashing the background color

quickly).  The default light-background color is light-grey.

The format for this command is:

Kermit-65>set light-background [color]

where [color] is one of the colors listed under the "Set

Bold" section.

Set Local-echo

The SET LOCAL-ECHO command specifies whether characters

should be echoed locally when connected to another computer.

If LOCAL-ECHO is set ON, any character typed on the C-64/128

will be echoed immediately to the C-64/128 as well as being

sent to the other computer.  This setup is useful when you

are connected to another personal computer.  If LOCAL-ECHO

is set OFF, the characters typed on the C-64/128 are sent

only to the other computer (which would normally echo the

characters).  This arrangement is used by most mainframe

hosts.  The format of this command is:

Kermit-65>set local-echo [keyword]

where [keyword] is either ON or OFF.

Set Parity

This command determines the type of parity to use during

communication.  Kermit normally uses characters which con-

sist of eight data bits with no parity bit.  For systems or

transmission media which require a specific parity type,

Kermit can send characters as seven data bits plus a parity

bit.  The format of this command is:

Kermit-65>set parity [keyword]

where [keyword] is one of the following:

Page 30                         C-64/128 Kermit User's Guide

NONE (default)   -   eight data bits and no

parity bit.

MARK             -   seven data bits with the

parity bit set to one.

SPACE            -   seven data bits with the

parity bit set to zero.

EVEN             -   seven data bits with the

parity bit set to make

the overall parity even.

ODD              -   seven data bits with the

parity bit set to make

the overall parity odd.

Set Receive

It is possible to set various parameters associated with the

receiving of data from the Kermit on the other end of the

connection (the remote Kermit).  SET RECEIVE will enable you

to set the various Receive parameters.  The format of this

command is:

Kermit-65>set receive <suboption>

The suboptions of the SET RECEIVE command are:

Eight-bit-quote-char   End-of-line   Packet-length

Pad-char               Padding       Quote-char

Timeout

These suboptions are described below.

Set Receive Eight-bit-quote-char

This command sets the character to be used to quote charac-

ters when EIGHT-BIT QUOTING is ON.  The default value is

hexadecimal 26 (decimal 38, ASCII "&").  Please see Set

Eight-bit-quoting for further information.  The format of

this command is:

Kermit-65>set receive eight-bit-quote-char [value]

where [value] is the ASCII value of the character (in hex)

C-64/128 Kermit User's Guide                         Page 31

to use for eight-bit quoting.

Set Receive End-of-line

This command sets the end of line character that C-64/128

Kermit expects to receive from the remote Kermit.  This is

the character which terminates a packet of data during file

transmission.  The default value is hexadecimal 0D (ASCII

<CTRL> - <M>, which is <RETURN>).  The format of this com-

mand is:

Kermit-65>set receive end-of-line [value]

where [value] is the ASCII value of the character to use for

the end of line character (in hex).

Set Receive Packet-length

This command sets the receive packet length.  The value for

this parameter must be between hexadecimal 0A (decimal 10)

and $60 (decimal 96).  Packet lengths outside of this range

are illegal.  The default value is $5E (94 decimal).  The

format of this command is:

Kermit-65>set receive packet-length [value]

where [value] is the ASCII value of the receive packet

length (in hex).

Set Receive Pad-char

This parameter is the padding character that is expected

from the remote Kermit.  The parameter must be a hex number

between 0 and $1F or $7F.  All other values are illegal.

The default value is 0 (ASCII NUL).  The format of this com-

mand is:

Kermit-65>set receive pad-char [value]

where [value] is the ASCII value of the character to be used

as a pad character (in hex).

Set Receive Padding

This command sets the number of padding characters to

request before each packet coming from the other Kermit.

The default value is 0.  The format of this command is:

Kermit-65>set receive padding [value]

where [value] is the number (in hex) of padding characters

to use.  NOTE: This command, along with SET RECEIVE PAD-

CHAR, is only used under very unusual circumstances.  At

Page 32                         C-64/128 Kermit User's Guide

this time, no Kermit program for any machine is known to

need padding, and if one did, it would request it without

intervention.

Set Receive Quote-char

This command sets the quoting character that C-64/128 Kermit

will expect on incoming files.  This is the character used

to quote control characters.  The default value is $23

(ASCII "#").  The format of this command is:

Kermit-65>set receive quote-char [value]

where [value] is the ASCII value of the quoting character

(in hex).

Set Receive Timeout

This command sets the number of seconds that C-64/128 Kermit

will wait for acknowledgement before it will retry sending

or receiving the current packet of data (during a RECEIVE,

SEND, or GET).  The default value is 15 seconds (hex 0F).

The format of this command is:

Kermit-65>set receive timeout [value]

where [value] is the number of seconds to wait for a message

from the remote Kermit (in hex).

Set RS232-registers

This command sets the 2 RS-232 registers in the C-64/128

that control the baud rate, parity, etc.  The 2 registers

are treated as a 16 bit hex integer.  The first 2 digits are

the SECOND register and the last 2 digits are the FIRST

register.  If only 2 digits are entered, the second register

is cleared.  The format of this command is:

Kermit-65>set rs232-registers [value]

where [value] is a four-byte long hex integer.  Some exam-

ples:

C-64/128 Kermit User's Guide                         Page 33

Kermit-65>set rs232-registers 0006   sets the registers

to 06, 00 for 300

baud, 8 bits, no

parity

Kermit-65>set rs232-registers a026   sets the registers

to $26, $A0 for 300

baud, 7 bits, mark

parity.

See the Commodore 64 or 128 Programmer's Reference Guides

for a complete description of the RS-232 registers.  Please

note that this command is rarely used-- it is a holdover

from the earliest versions of Kermit.  You can use the SET

BAUD and SET PARITY commands to set the baud and parity for

most cases.  See the Known Restrictions section of Appendix

A for exceptions.

Set Screen-driver

This command switches the display mode.  Kermit is capable

of displaying text in 40 and 80 columns, using the Batteries

Included BI-80 80 column card, and also using the Commodore

128's 80 column screen.  Not all of the displays are capable

of completely accurate displays.  For example, the standard

Batteries Included 80 column character set does not include

certain standard ASCII characters.  In all cases the best

compromise was made to provide the most accurate display

(Please see the Screen Driver Comparison section for further

information).  The format of this command is:

Kermit-65>set screen-driver [keyword]

where [keyword] is 40-COLUMNS, 80-COLUMNS, BI-80,

COMMODORE-128, or CUSTOM-BI-80.

Set Send

It is possible to set various parameters associated with the

sending of data to the remote Kermit.  SET SEND enables you

to set the various SEND parameters.  These parameters should

not normally be changed since, as part of the transfer ini-

tialization process, the two Kermits exchange their RECEIVE

parameters.  The capability of setting these parameters is

provided so that the transfer initialization can be com-

pleted even if the default parameters are not correct.  The

format of this command is:

Kermit-65>set send <suboption>

The suboptions of the SET SEND command are:

Eight-bit-quote-char   End-of-line   Packet-length

Pad-char               Padding       Quote-char

Timeout

The suboption descriptions for the SET SEND command are the

same as those for the SET RECEIVE command (except that they

affect sending, not receiving, parameters). So, to avoid

Page 34                         C-64/128 Kermit User's Guide

redundancy, please refer to the appropriate suboption

descriptions under the Set Receive section for further

information.

Set Terminal-emulation

Kermit is capable of emulating two popular terminals, as

well as act as a dummy (glass) terminal.  The SET TERMINAL-

EMULATION command changes the type of terminal Kermit emu-

lates.  The format for this command is:

Kermit-65>set terminal-emulation [type]

where [type] is NONE, VT-52, or VT-100.  Kermit provides

very accurate emulation of the DEC VT-52 and VT-100 termi-

nals.  In VT-100 mode, Kermit can perform most all functions

of a VT-100 terminal, including displaying a graphics char-

acter set as well as emulating certain VT-102 sequences.

See Terminal-Type Features for more info.

Set Word-size

This option sets the data word length for communication with

another computer.  Although word lengths below seven bits

exist, C-64/128 Kermit only supports seven and eight bit

word lengths.  The format of this command is:

Kermit-65>set word-size [length]

where [length] is either SEVEN-BIT or EIGHT-BIT.

Show

The SHOW command allows you to examine the status of many of

the SET parameters.  The format is:

Kermit-65>show <option> <suboption>...

The options available to SHOW are:

All                  Baud              Debugging

Eight-bit-quoting    Escape            File-byte-size

File-type            File-warning      Flow-control

IBM                  Local-echo        Parity

Receive              RS232-Registers   Send

Terminal-emulation   Word-size

The SHOW ALL command will display the settings for most of

the parameters that may be SET.  The other SHOW options and

suboptions will display the current value of just that par-

ticular parameter.  Note: The 4 color commands and the

C-64/128 Kermit User's Guide                         Page 35

SCREEN-DRIVER command are not currently supported.

Status

STATUS displays the current values of various counts associ-

ated with the most recent file transfer.  This includes the

number of characters that have been sent and received from

the remote Kermit with the amount of characters which were

actual data as well as the overhead for both send and

receive.  Also included is the last error encountered (if

any) and the disk drive error status (absent if no error).

STATUS has no arguments with the format:

Kermit-65>status

Page 36                         C-64/128 Kermit User's Guide

Appendix A: Implementation Notes

This appendix contains information mainly of interest to

people who are familiar with Kermit-- experienced users and

programmers.  First time users may not find too much useful

information here.

Summary of Kermit Capabilities

C-64/128 Kermit v2.2 Capabilities at a Glance:

Local operation:                  Yes

Remote operation:                 Yes

Transfer text files:              Yes

Transfer binary files:            Yes

Wildcard send:                    No

^X/^Y interruption:               Yes

Filename collision avoidance:     Yes

Can time out:                     Yes

8th-bit prefixing:                Yes

Repeat count prefixing:           No

Alternate block checks:           No

Terminal emulation:               Yes

Communication settings:           Yes

Transmit BREAK:                   Yes

IBM communication:                Yes

Transaction logging:              No

Session logging (raw download):   No

Raw upload:                       No

Act as server:                    No

Talk to server:                   Yes

Advanced server commands:         No

Local file management:            Yes

Handle file attributes:           No

Command/init files:               Yes

Printer control:                  No

Known Restrictions

Kermit is a comprehensive telecommunications package.

Because it is continually under development, however, some

features are not fully implemented.  These incomplete

features therefore have some restrictions as to their use.

They are:

+ The SET BAUD command does not support baud rates under

300 bps.  To set speeds of 50, 75, 110, or 150 bps, you

C-64/128 Kermit User's Guide                         Page 37

will have to set the RS-232 registers directly.  Please

see the SET RS232-REGISTERS section for more informa-

tion on this procedure.

+ Any changes made using the SET RS232-REGISTERS command

will not be reflected in the BAUD, PARITY, or WORD-SIZE

parameters.

+ If the initialization file, KERMIT.INI, isn't on the

same disk as Kermit when you boot, Kermit will crash,

exiting to BASIC.  You can recover from this condition

by inserting a disk containing a KERMIT.INI and typing

RUN.  You MUST have a disk with a KERMIT.INI file on it

in order to boot Kermit.

+ Kermit currently does not work with the Commodore 1660

modem.  This modem is backwards in terms of picking up

and hanging up the phone.  We will fix this in a future

version of Kermit.

Future Plans for Kermit

New features can always be added, it seems, to a program.

We are currently considering implementing the following

features for a future version of Kermit:

+ Modem support.  Kermit needs to be able to work with

the Commodore 1660 modem and other "oddballs".  This

would require a SET MODEM-TYPE command.

+ Long packets.  Long packets allow more data to be sent

per packet, speeding up file transmission considerably.

+ Remote server commands.  With this command Kermit would

be able to answer the phone automatically and

send/receive files without anyone present.  Kermit

would function as a very simple bulletin board in a

way.

+ New parser.  The current parser is VERY large, ineffi-

cient, and riddled with numerous bugs.

+ TAKE command.  This command would allow multiple ini-

tialization files.  In other words, you could have an

initialization file for every different configuration

you use.

Page 38                         C-64/128 Kermit User's Guide

Appendix B: Default Initialization Settings

This appendix is of use mainly to experienced Kermit users.

Kermit loads various default settings from disk on power-up

(or when the command RESTORE is issued).  Kermit reads in

the settings from a file called KERMIT.INI.  Below are the

default settings for all of the parameters contained in

KERMIT.INI.

Parameter                       Setting

Debugging                       Terse

Terminal Emulation              VT-100

IBM Mode                        Off

Local Echo                      Off

Baud Rate                       1200

Parity                          None

Word Size                       Eight

Flow Control                    On

Eight Bit Quoting               On

File Warning                    On

Escape Character                ^Y

File Type                       PETSCII

File Byte Size                  Eight Bit

RS232 Registers                 $0000

Receive

Padding Character             ^@

Amount of Padding             00

Eight Bit Quoting Character   &

End of Line Character         ^M

Packet Length                 $5D

Quote Character               #

Timeout Character             0A

Send

Padding Character             ^@

Amount of Padding             00

Eight Bit Quoting Character   &

End of Line Character         ^M

Packet Length                 $5D

Quote Character               #

Timeout Character             0A

C-64/128 Kermit User's Guide                         Page 39

Appendix C: Keyboard Layouts

Commodore 64 Map

Kermit redefines a few keys on the Commodore 64 keyboard

both to send ASCII characters that are not found in the

standard Commodore character set and to send some special

VT-100/52 key sequences.  These redefined keys are most use-

ful when communicating with a mainframe host.  The following

is a list of the redefined keys:

If You Type This                    You Send This

<Left Arrow>                        <ESC>

<Up Arrow>                          <Hat>

<Shifted "="> or <F1>               <Underscore>

<Shifted British Pound> or <F2>     <Pipe>

<Shifted "@"> or <F3>               <Grave accent>

<Shifted Up Arrow> or <F4>          <Tilde>

<Shifted "+"> or <F5>               <Left Brace>

<Shifted "-"> or <F6>               <Right Brace>

<British Pound>                     <Backslash>

<STOP>                              <CTRL> - <C>

<CLR>                               <CTRL> - <L>

<HOME>                              <CTRL> - <H>

<DEL>                               <DEL>

<INST>                              <BREAK>

Cursor Keys                         VT-100/52 Cursor Key

Sequences

<CTRL> - <F1>                       VT-100/52 PF 1

<CTRL> - <F3>                       VT-100/52 PF 2

<CTRL> - <F5>                       VT-100/52 PF 3

<CTRL> - <F7>                       VT-100/52 PF 4

<CTRL> - <0> through <CTRL> - <9>   VT-100/52 Num.

Keypad 0 through 9

<CTRL> - "-"                        VT-100/52 Num.

Keypad "-"

<CTRL> - "."                        VT-100/52 Num.

Keypad "."

<CTRL> - ","                        VT-100/52 Num.

Keypad ","

<CTRL> - "+"                        VT-100/52 Num.

Keypad "," (dupli-

cate)

<CTRL> - <RETURN>                   VT-100/52 Num.

Keypad <ENTER>

Also, the <C=> key will temporarily stop character display.

It acts as a toggle, and will start the display again when

Page 40                         C-64/128 Kermit User's Guide

pressed a second time.  This works only in Connect mode.

Commodore 128 Map

The Commodore 128 has an expanded layout that includes all

of the Commodore 64 redefined keys.  In other words, the

Commodore 128 keyboard layout is a superset of the Commodore

64 layout.  The following is a list of the special C-128

keys.

If You Type This             You Send This

<ESC>                        <ESC>

<HELP>                       <"?">

<LINEFEED>                   <CTRL> - <J>

<TAB>                        <CTRL> - <I>

Grey Cursor Keys             VT-100/52 Cursor Key

Sequences

Numeric Keypad 0 through 9   VT-100/52 Num.

Keypad 0 through 9

Numeric Keypad "-"           VT-100/52 Num.

Keypad "-"

Numeric Keypad "."           VT-100/52 Num.

Keypad "."

Numeric Keypad "+"           VT-100/52 Num.

Keypad ","

Also, <CAPS LOCK> is functional.  All lowercase letters are

changed to upper case when this key is in the down position.

C-64/128 Kermit User's Guide                         Page 41

Appendix D: UNIX (TM) Termcap Settings

Kermit emulates a DEC VT-100 terminal with a complete VT-52

mode, graphics character set, and extra VT-102 editing

features.  On UNIX (TM), you can use most of Kermit's

features by setting your terminal type to VT-100.  However,

to use the extra features such as the VT-102 editing

sequences, a visual bell, and a sysline (25th line used for

system status programs), you will need to use the following

as your termcap instead:

Kc|vt100|Commodore 64/128 Kermit in VT-100 mode:\

:ae=\E(1:AL=\E[%dL:al=\E[L:as=\E(0:bc=^H:bl=^G:bs:\

:cd=\E[J:ce=\E[K:cl=\E[H\E[2J:cm=\E[%i%d;%dH:co#80:\

:cr=^M:cs=\E[%i%d;%dr:DC=\E[%dP:dc=\E[P:\

:DL=\E[%dM:dl=\E[M:DO=\E[%dB:do=^J:\

:ds=\E7\E[25H\E[2K\E8:ei=\E[4l:es:fs=\E8:ho=\E[H:hs:\

:im=\E[4h:k1=\EOP:k2=\EOQ:k3=\EOR:k4=\EOS:\

:kb=\177:kd=\E[B:ke=\E>:kl=\E[D:kn=4:kr=\E[C:\

:ks=\E=:ku=\E[A:l1=PF1:l2=PF2:l3=PF3:l4=PF4:\

:LE=\E[%dD:le=^H:li#24:ll=\E[24H:mb=\E[5m:\

:md=\E[1m:me=\E[m:mi:mr=\E[7m:ms:nd=\E[C:pt:\

:rc=\E8:RI=\E[%dC:rs=\E<\Ec:sc=\E7:se=\E[m:\

:so=\E[7m:sr=\EM:ta=^I:te=\E[?7h:ti=\E[?7l:\

:ts=\E7\E[%i25;%dH\E[m\E(1\E[1K:ue=\E[m:\

:UP=\E[%dA:up=\E[A:us=\E[4m:vb=\E[?5h\E,\E[?5l:\

:vt#3:xo

This custom termcap also fixes several minor bugs and omis-

sions in the standard VT-100 termcap entry found in Berkeley

4.3 BSD UNIX.

Note that any British Pound signs in the above termcap entry

should really be backslashes.  Commodore computers map the

backslash onto the British Pound sign, as they don't have

the backslash in their character sets.

Page 42                         C-64/128 Kermit User's Guide

Appendix E: BI-80 Replacement ROM Information

Kent Sullivan is now making available a replacement

character ROM for the BI-80 card.  This replacement ROM adds

the ASCII characters that are missing from the Commodore

character set (backslash, braces, underscore, tilde, grave

accent, pipe, and hat) plus the VT-100 line graphics charac-

ter set.

To add these characters, some sacrifices had to be made.  As

you may know, the Commodore 64/128 and the BI-80 have two

separate character sets: upper/lower case and upper

case/graphics.  The upper/lower case set remains as is in

the new ROM, so most commercial programs that work on a BI-

80 are not affected (such as the Paperclip word processor

from Batteries Included).  The upper case/graphics character

set has been changed quite a bit, so some programs may be

affected (such as a BASIC program that uses extensive char-

acter graphics).

The new ROM is $7.50 (US funds only; WA residents please add

8.2% sales tax) and comes with complete installation

instructions.