To:       SCSI-2 CAM COMMITTEE          October 4, 1989

From:     Patrick J. Manley   SyQuest Technology

Subject: Proposal to resolve Physical to logical Conversion

Problem: Logical to Physical Conversion.

1.  	Two road blocks to agreement on a new  algorithm

are that

a.  each vendor has an install base  to  maintain

compatibility with , and

b. they like the one their using.

Proposal: # 1

The  one  sector  that is independent of  the  algorithm  is

sector 00 . Under Dos  and Many other Operating Systems this

Sector is used for the bootsector and contains the partition

table for a fixed disk.

If  the partition table is done according to MSDOS  and

IBMDOS standards, partitions end on cylinder boundaries.

Example:

offset from start of partition table entry

00h  Boot Indicator                80h

01h  Beginning or start head       01h

02h  beginning or start sector     01h

03h  Beginning or start cylinder   00h

04h  System indicator              04h

05h  Ending head                   07h

06h  Ending sector                 91h

07h  Ending cylinder               7Ch

08h  Starting sector(relative to begining of Disk )

0Ch  Number of sectors in partition

The ending head 07h indicates a device with 8 heads (0 to 7)

The ending sector 91h contains 2 bits of high cylinder so it

has to be masked to obtain ending sector = 11h or 17 dec.

To verify these values calculate Logical the Ending sector

from the Beginning	Head Cylinder and Sector and compare it

to the (Starting sector + Number of sector in Partition)

This leaves one unresolved parameter , number of cylinders.

By dividing the Read Capacity value by heads * sectors

the number of cylinder is obtained .

All of this can be done by the Bios in Rom or Ram .

To  be  capable  of Booting from  any  drive  or  cartridge

regardless of the algorithm used to partition and format the

media  ,the bios would need to respond to int 13 function  8

the  head  sector  and cylinder values  obtained  from  this

information  . In addition the bios would need to use  those

values in its calculation from physical to logical sectors.

EXAMPLE OF Psuedocode

For each Drive

Read Boot Sector (LBA 0)

Validate The Signature at end of Sector (55AA)

Find Partion with largest Logical Start Cyl

If No Partitions found

Use Defaults

Exit

SECS = Ending Sector (from partition table)

Heads = Ending Head+1  (from partition table)

Logical_End = End_cyl( End_head+1 * End_Sector)+

( End_head+1 * End_Sector)+ End_sector

Comp   Logical_End to Starting_sec + Number_sec

If not equal

Use Defaults

Exit

Cyls = Capacity /(End_head+1 * End_sector)

Proposal: # 2

Another method is to modify the code of the boot sector

to   accomplish  bootability   ,however  access   to   other

partitions       is dependent on the device driver to  do  a

translation.

This method is a patch just prior to jumping to code

loaded in memory at seg 00 offset 7c00h

PUSH   AX		 ; Save regesters used in patch

PUSH   DX

MOV    AH,08        ; set function code = 8 get drive parameters

INT    13           ; do INT 13 call

INC    DH           ; inc head number to convert from zero based

MOV    [7C1A],DH    ; fix value of heads in BPB table

AND    CL,3F	 ; Mask off non sector information

MOV    [7C18],CL    ; fix value of sectors in BPB table

POP    DX

POP    AX		 ; Restore  regesters used in patch

JMP    7C00         ; jump to partition boot loader

01B0  00 00 00 00 00 00 00 00-00 00 00 00 00 00 80 01

01C0  01 00 06 07 91 7C 11 00-00 00 57 52 01 00 00 00

01D0  00 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00

01E0  00 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00

01F0  00 00 00 00 00 00 00 00-00 00 00 00 00 00 55 AA

|  Name		    | Bytes/ |Sectors/ |Reserved

jump nop  I	B  M	    4  .  0 | sec    |Cluster  |sectors

7C00  EB 3C 90 49 42 4D 20 20-34 2E 30 | 00 02  |  04     | 01 00

7C18     7C1A

Num of| Num of    | Num of | Media  | Num of  | Num of  | Num of

Fats | Dir Entyrs| Log Sec| Descrip| Fat Sec | Sectors | Heads

7C10  02 | 00 02     | 00 00  | F8     | 55 00     11 00   | 08 00