if (room in clist and process is sleeping waiting for room)

clear flag indecating user process is sleeping

wake up sleeping process

if (clist is empty)

clear device busy flag

if (someone is sleeping waiting to close )

wake up process

else if (timeout pending flag is clear )

request that lprestart be called in a tenth of a second

set timeout pending flag

enable device interrupts

intr entry point :

if (device busy flag is clear )

return (i.e. interrupt is spurious)

if (there is at least one byte in the clist buffer)

call lpwork to try to print it

lp restart routing :

clear the timeout pending flag

raise processor  priority (disable interrupts)

call lpwork

restore processor priority