A Trivial Ethernet Analyzer

Vance Morrison

Very often a network administrator has to debug networking problems

given only the raw symptoms of the problem (my program hangs).  Trying

to debug given such sparse information is difficult at best and impossible

at worst.  Often the ability to 'snoop' on the network and watch the

packet dialog is VERY helpful.  Basically I have written a program

called 'analyzer' that watches packets go by on the network and prints

them out in a human readable form.   At present, this program only

understands TCP/IP protocols (and not all of them), and for security

reasons analyzer only prints what it understands.  Thus at least at

present, analyzer will not help with DECNET, XNS, ETHERTALK traffic.

WHAT YOU NEED

In addition to the analyzer.exe executable (available from

accuvax.nwu.edu (129.105.49.1) in pub/pcroute/analyzer), you will also

need a PC with a network card, as well as the clarkson packet driver for

that  card.  The clarkson packet driver is a piece of software that allows

analyzer.exe (as well as other programs) to access the network card in

a device independent way.  A driver exists for most of the common networking

cards, and is available (among other places)  sun.soe.clarkson.edu

(128.153.12.3) in the directory 'pub/packet-drivers'.  Note that some

packet drivers do not support 'promiscuous' mode (in particular I know the

ni5010 does not).  In that case analyzer.exe will work, but will not be

of much use (since it will only see packets destined for the itself and

the broadcast address).  I do know that the wd8003e driver DOES work.

RUNNING THE SOFTWARE

Before you can run analyzer.exe, you must first load the packet driver.

This is usually just executing the driver command with arguments that

tell it about the networking card.  Note that analyzer.exe is pretty dumb

and always expects the driver to be accessible though interrupt 60H.  This

is the default, and the driver will usually tell you where it installed

itself, so if it is not 60H, analyzer.exe will not work.

The syntax for the analyzer command is simple, namely.

analyzer <ethernet address 1> <ethernet address 2> ...

That is you simply run the command with the ethernet addresses that

you are interested in seeing.  Up to 10 addresses can be given.  Analyzer

will then ONLY print those packets whose source or destination match one

of the given ethernet addresses. For example the command

analyzer 0:0:C0:EF:12:34  FFFFFFFFFFFF

will print only packets from the broadcast address or to/from the address

0:0:C0:EF:12:34.  Notice that the colons in the ethernet addresses are

optional.

If not ethernet addresses are given then analyzer will try to print EVERY

packet.  If the network is at all busy, analyzer will not be able to keep

up and packets will undoubtedly be lost, but sometimes you don't care.

Analyzer does buffer 16 packets (at interrupt level), so even if you freeze

the screen, analyzer will probably capture the next 16 packets correctly.

To stop analyzer simply press the <ESC> key.  Also you you want to place

the data in a file instead of on the screen, simply redirect the output

like in the following example.

analyzer FFFFFFFFFFFF > analyzer.out

FINDING OUT ETHERNET ADDRESSES

You undoubtedly do not know the ethernet address of your hosts off the top

of your head.  If you have a unix system on you net, you can use the 'arp'

command to display the IP - Ethernet address translation.  Often, however,

it is easier to simply to run analyzer without argument and then look at

the packets.  Each IP packet will have its source IP address as well as

the source ethernet address in the packet and IF that IP host is on the

local net, then you have just found out its ethernet address.

SHORTCOMINGS

Admittedly, analyzer is not a very flexible program.  The philosophy here

is that something is a LOT better than nothing, and once you have the

output as ASCII output it can be filtered and beautified to your hearts

content.  For example, if you have a batch editor (like sed), you can

do a global search an substitute to replace ethernet addresses with a

more mnemonic one.   Also, I did NOT what to spend a lot of time on this,

analyzer in its present form is about 3 days work.

SECURITY ISSUES

In an ideal network, it shouldn't matter that a program like analyzer

exists.  However, life is not ideal, and passwords and other sensitive

data are routinely sent in unencrypted form across nets.    I have therefore

gone to a little trouble to make analyzer 'safe'.  It only prints out

parts of packets that should not contain sensitive data.   While this is

not foolproof, I believe is does open any security loophole any wider

than it already is (which is actually pretty wide).

EXTENSIONS

Usually I provide the source code to my work so that if anyone wants

to make extensions they can do it themselves (and not bother me).  In

the case of analyzer, this would be particularly nice since analyzer

does not print (in any form) what it does not know, and analyzer is

a pretty ignorant program (:-).  Unfortunately releasing source would

be a security problem, since it would be VERY easy to modify analyzer

to do mischief.  Thus I propose a compromise.  If analyzer does not

print output packets that you want to see, simply write some C code

that takes a pointer to that packet and prints it out.  I have included

the file 'ip.c' in the distribution to give you an example of how to

do it.  If you send this file to me, I will look it over for security

holes, compile it, and send you an executable.  This arrangement is

less than perfect, but it will give you an option if you wish to exercise

it.

INFO ON NETWORKING PROTOCOLS

In order to understand analyzers output, you need to understand the

meanings of fields in the packets.  Please don't ask me.  For the

beginner there are a variety of good books on TCP and networking in

general.  Douglas Comer's "internetworking with TCP/IP" is good for

beginners to the TCP/IP world.  After that, the RFC's are the place

to look.  They are available among other places nic.ddn.mil (10.1.1.51)

and merit.edu (35.1.1.42).

BUGS ETC

Basically, I want to spend little/no time supporting analyzer.  Thus

if the bug requires debugging effort, or a non-trivial amount of time

to fix, write it yourself.  Thus if analyzer does not work with your

particularly PC/packet-driver I don't want to hear about it.  I don't

mean to sound harsh, but there is only one of me, and many of you, and

frankly I have better things to do.  I released analyzer in the first

place because I believe it will be useful to people.

COPYRIGHT

Please notice the copyright.  This is shareware.  You are allowed to

use this software on a trial basis for one month.  If after that time

you find analyzer and wish to keep using it, send a $10 registration

fee to the address below.  If you send me an Internet E-mail address

along with your registration fee, I will give you my E-mail address,

and will notify you via E-mail of updates to analyzer.   Just for your

information, this fee will go toward a fund for purchasing LAN cards

and other hardware I need in order to continue to write/debug neat

code like analyzer.

The above payment requirement expires AUGUST 1993.  After that time

you may use analyzer without registration.

Vance Morrison

301 E. White St  Apt 4.

Champaign, Il.  61820