Availability of cards for PCroute or PCbridge.

PCroute and PCbridge were first designed to work with the Western Digital

WD8003E card.   Later Western Digital came out with 16 bit versions

WD8013EBT that PCroute and PCbridge also support.   Unfortuately both

of these cards are no longer available.  The have been replaced with the

WD8003EP and WD8013EP (as well as others).  Unforunately these cards are

not exact replacements and SOME people have experienced problems getting

them to work.   Thus if you are buying new cards you have a problem.

Some solutions are:

1) Try it and see.  As I said most people do not have problems.  For

all I know it is not the cards at fault but some other aspect

of installation.  It is also possible that certain early batches

of cards were slightly different and this caused the bug.  Thus

you stand a good chance of it working by simply using the

WD80xEP cards.  Be aware that you MAY have problems however.

2) Use 3c507 card.  PCroute also supports this card.  This card IS

still available.  Unforunately it is more expensive.  On the other

hand, it IS a very good, high performance card, so you are not

spending your money for nothing.

3) Use packet drivers.  PCroute (but NOT PCbridge) can used packet

drivers (available from sun.soe.clarkson.edu in packet-drivers

directory).  This allows most ethernet cards to be used.

Unfortuately this solution slows the router down by more than

half.

4) Use the COMPEX card.

It has been reported that a WD8013EBT clone exists and works

well with PCroute and PCbridge.  It is made by COMPEX.  It

is also quite cheap (cheaper than the original!).    The

model number is #ENET-16-COMBO (supports UTP/AUI/BNC).

Revision D has been confirmed to work.  Cost is $115 (3/93).

There seems to be a version that is slightly less expensive

($95) that does not have UTP support.    A possible vendor

is given below.  Note that this information is supplied simply

as a courtesy, it is not a recomendation for this card or

this particular vendor.

Advance Computer Support

5515 North 7th Street

Suite 5-504

Phoenix, AZ 85014

PH: 602-305-1345 (Hagan Fox)

If you are buying Western Digital cards (now owned by SMC), or 3Com cards

(andy maybe even COMPEX cards), the best place to get a good price is probably

the back of PC Shopper Magazine.  If you don't have this most other PC

oriented magazines have a shopping section (PC Magazine, Byte etc).

Good Luck