ARCEther Packet Driver:

Novell 3.11 servers now have the possibility to route IP packets and to send

IP traffic over ARCnet. The protocol numbers and framing types these servers

use on ARCnet are described in RFC 1201. Because there is actually no packet

driver which supports this framing, we decided to change the existing ARCNET

packet driver so that  it supports the standard described by RFC 1201.

Since there exists hardly any free software which supports ARCnet type packet

drivers, we decided two show up for applications as Ethernet class 1 driver

and transform Ethernet to ARCnet packets and vice versa within the driver.

This includes splitting and reassembling packets on ARCnet since Ethernet

packets can be 1500 bytes long while the maximum size for single ARCnet

packets is 504 bytes.

Since you certainly don't want to log off from your network and reboot the PC

to use Telnet of FTP, the driver has to support Novell IPX packets as well. So

if you have the BYU IPX Shell, ARCEther allows you to work within your Novell

Network and at the same time use TCP/IP products written for Ethernet packet

drivers.

The usage of ARCEther is straight forward. If you start the program without

any command line parameters, it tells you

usage: arcether [-n] [-d] [-w] <packet_int_no> <int-no> <io_addr> <mem_base>

The "packet_int_no" is the interrupt number through which apllications can

access the packet driver.

"int_no" is the IRQ your ARCnet card uses. Don't be afraid if you enter 2 and

the driver tells you that is uses int_no 9, thats okay.

"io_addr" is the port through which the packet driver can communicate with the

ARCnet card and is determined by the jumper settings on the card.

"mem_base" is the segment of memory which the ARCnet card uses.

The option -w and -d are the same as for all packet drivers.

-d tells the driver to delay the initialization of the ARCnet card until

the first request arrives. You must start the driver with this this

option if you boot your PC from the network because otherwise the Boot-ROM

cannot load the IPX-Shell.

-w tells the driver to use the window option

The -n option is for ARCEther a very special option and has not the same

effect as for standard Ethernet packet drivers. If you use the -n option,

ARCEther assumes that it should build and expect Novell 802.3 packets instead

of Novell Bluebox packets. You should therefore use this option with the 1991

version of the IPX shell which sends and receives 802.3 packets. If you have an

older version of the IPX Shell, do not use the -n Option.

Another effect of the -n option is that the driver does not refuse requests

for a class 11 driver when you use this option since the new IPX-Shell

requires such a driver. Please remember that ARCEther is not really a class 11

driver and may hang your PC if other programms than the IPX-Shell access it

via class 11.

There is one possible difficulty which the driver can't handle: The Server

does not access the ARCnet hardware through ARCEther but has its own TRXNET

driver. So Novell Servers may send packets which consist of more than 3

fragments or are longer than 1500 bytes. ARCEther cannot handle such packets

because it cannot request such a buffer from a client and simply does not

deliver such packets to any client application.

We tested ARCEther on our own Network and noticed that our Novell 3.11

Server never used such long packets, but maybe there are situations in which

Novell very uses long packets since Novell's TRXNET Driver supports a maximum

frame sze of 4602 bytes.