IPXPKT

The standard packet drivers allow applications such as Novell and TCP/IP

to share a single ethernet card in a workstation. A user logged onto

a Novell fileserver may run TCP/IP to connect to a TCP/IP host on the

same network. However, there are a number of circumstances in which

this cannot be done:

1. If the workstation is connected to a Novell fileserver or bridge

rather than the network backbone. Packet drivers are not available

for most non-ethernet topologies, and Novell servers and bridges will

only forward IPX packets, not TCP/IP packets which they do not

recognise.

2. Packet drivers are not available for all ethernet cards e.g.

Univation (we still have a few which haven't yet died).

A solution to the above problems has been developed by Paul Kranenburg of

Leiden University in Holland. His IPXPKT program is a packet driver which sends

and receives packets via Novell's IPX.COM rather than directly from the network

card. It receives packets from an application, adds the IPX headers and passes

them to IPX for transmission. It also does the converse i.e. receives packets

>from IPX, strips off the IPX header and forwards the packet to the application.

IPXPKT allows any application configured with a packet driver interface

(e.g. NCSA Telnet) to function from any workstation anywhere on a Novell

network regardless of the network topology.

A typical network setup might be:

+-------------+        +----------+   +--------+

| IPX <-> IP  |        | Unix Box |   |   VAX  |

|router using |        |  TCP/IP  |   | TCP/IP |

|   IPXPKT    |        |          |   |        |

+-------------+        +----------+   + -------+

Ethernet          |     |                  |             |

----------------------------------------------------------------------

|

+----------+

|  Novell  |    Arcnet, token ring, ethernet or whatever

|fileserver|---------------------------------------------------

|          |             |                         |

+----------+       +------------+             +------------+

|    work    |             |    work    |

|   station  |             |   station  |

|      1     |             |      2     |

+------------+             +------------+

Because the packets are transmitted by IPX.COM, IPXPKT works on any

topology supported by Netware. Version 2 of IPXPKT allows packets to

be transmitted through Novell fileservers and bridges.

The IPX <-> IP router on the network backbone receives the IPX packets, strips

off the IPX headers and retransmits Ethernet II packets which can be picked up

by TCP/IP hosts. Additional software is required in the router to route the

packets between the two interfaces. Phil Karn's NET.EXE is commonly used, but

PCROUTE configured for one or two packet driver interfaces is an alternative.

NET.EXE is available via anonymous FTP from THUMPER.BELLCORE.COM (128.96.41.1).

An autoexec.bat for the IPX <-> IP router is:

wd8003e 0x60 0x7 0x360 0xd000

ipx

ipxpkt 0x61

net

NET is configured to use two Packet driver interfaces. The first sends and

receives Ethernet II packets via the wd8003e packet driver which uses the WD

Ethernet card configured to IRQ 7 and I/O Base 360h. The second sends and

receives Ethernet II packets via IPXPKT using software interupt 0x61. IPXPKT

sends and receives packets via IPX.COM, wrapping IPX headers around packets to

be transmitted before passing them to IPX.COM, and stripping off IPX headers

>from packets received from IPX.COM before passing them to NET. IPX.COM is

configured to match the second Ethernet card in the router.

Note that NET could be configured to communicate with the first WD card

directly. I got it working first using the packet driver, and haven't

bothered to change it.

The configuration I'm using for the second ethernet card is:

LAN Option: WD Star/Ethercard PLUS v2.11.3 (111188)

Hardware configuration: IRQ=3, I/O Base=280h, ram at C400:0 for 8k, no DMA

NET requires a configuration file named AUTOEXEC.NET. An example is:

ip address [138.75.30.1]

hostname ipxrouter

#

# Our VAX cluster and link to Canterbury use the network 138.75.10.0

# Workstations using WD cards on the network backbone also use 138.75.10.0

# All other Novell w/s (GNET, Univation etc) use 138.75.30.0

#

# This router uses the address 138.75.10.11 on the VAX side and 138.75.30.1

# on the IPXPKT side

#

attach packet 0x60 wd0 15  1500 [138.75.10.11]

ifconfig wd0 ipaddress 138.75.10.11 netmask 0xffffff00 broadcast 138.75.10.255

#

attach packet 0x61 pkt 15 1500 [138.75.30.1]

ifconfig pkt ipaddress 138.75.30.1 netmask 0xffffff00 broadcast 138.75.30.255

#

route add [138.75.30]/24 pkt

route add [138.75.10]/24 wd0

#

arp add [138.75.30.255] ether ff:ff:ff:ff:ff:ff

arp add [138.75.10.255] ether ff:ff:ff:ff:ff:ff

#

start rip

#

rip add [138.75.30.255] 30

rip add [138.75.10.255] 30

#

ip ttl 16

#

tcp window 1024

#

tcp mss 1024

#

start echo

start discard

As mentioned above, version 2.0 or higher of IPXPKT must be used to

establish connections with TCP/IP hosts through Novell servers or

bridges. If version 2.0 is not in the packet driver distribution set

(it wasn't in the 6.0a set), it can be obtained from Leiden University.

Send a message to archive-server@cs.leidenuniv.nl containing

'send source ipxpkt.asm' as the first and only line.

To assemble IPXPKT, you will need the packet driver distribution and

Borland's Turbo Assembler. Issue the following commands

tasm head           ;head.asm is in the packet driver distribution

tasm ipxpkt         ;requires defs.asm from the packet driver distribution

tasm tail           ;tail.asm is from the packet driver distribution

tlink head ipxpkt tail

exe2bin head.exe ipxpkt.com

Once the router is functioning, any application which uses a packet driver

may be run from anywhere on the network. The bat file I use to run

NCSA's FTP is:

@echo off

map w:=sys:ncsa

w:ncsacfg

if not errorlevel 1 goto skip

w:marknet f:marknet.mrk

w:ipxpkt 0x60

:skip

w:ftpbin %1

del config.tel

if exist f:marknet.mrk w:relnet f:marknet.mrk

The program ncsacfg creates config.tel in the users current directory. It

uses the workstations network address and physical station number to

determine whether or not ipxpkt is needed, and concocts a unique IP

address accordingly. Note that this program is specific to Lincoln's setup.

Marknet and Relnet are used to remove ipxpkt from memory.

John Baird

j.baird@lincoln.ac.nz