P O R T   L O C K I N G   H E L P

���������������������������������

The following information is provided by PRE-Mail Systems - DIGITAL

X-Connect BBS to assist Sysops of WILDCAT! Bulletin Boards to set up

external protocols with the PORT LOCKING feature turned on in the

WILDCAT! MAKEWILD utility.

This also applies to users of QModem who lock their serial ports!  This

can, and should be done when using high speed modems with V.32 or V.42

capability to increase throughput capability.

DISCLAIMER:

�����������

Sorry folks...gotta say the next couple of words...

We have made every effort possible to test all the options of all the

protocols listed in this file.  We do not guarantee that these externals

will work BUG FREE with your modem and hardware set-up. In fact,

PRE-Mail Systems, hereby wherefore hitherto, is not liable for any

damage caused by the recommendations or information contained herein.

If you feel brave enough to try what I'm going to tell you - please do

yourself a BIG favor....write down all your settings before you start

messing with them....that way you can at the very worst return to normal

operation.

The PORT LOCKING feature presents some unusual problems in modem setup that

must receive adjustments for the external protocols to operate correctly.

INTRODUCTION

������������

If oversimplifies for some of you experts, please bear with me.  There is so

much misunderstanding out there about modem setups and all....I wanted to

get to the basics.

RULE #1:

Port Locking is not for everyone.  In fact, the primary users of this feature

are the Courier HST or V.32 modem owners who have the ability to upspeed

to 14.4Kbps on a 9600bps CONNECT.

[see enclosed modem 101 text file for further technical details]

If you do not own this type of modem, you will NEED a modem that can

buffer the computers data to transmit to the connected modem.

If you have neither of these - DO NOT USE PORT LOCKING - it does not buy you

anything and is not worth the trouble!!!

RULE #2:

You have to understand the language I'm going to use.

Here is a simple diagram that explains the whole shooting match:

����������Ŀ          ��������Ŀ         ��������Ŀ         �����������Ŀ

� (DTE)    �          � (DCE)  �         �  (DCE) �         �    (DTE)  �

�   #1     �<-------->�   #1   �<=======>�   #2   �<------->�      #2   �

�          �  Cable   ����������  Phone  ����������   Cable �           �

������������                       Line                     �������������

DTE: Data Terminating Equipment

DCE: Data Communication Equipment

The DTE equipment above are the computers - the DCE's are the modems...for

the sake of further discussion - your computer and your modem are DTE#1 and

DCE#1....the caller to your board is DTE#2 and DCE#2.

BASIC APPLICATION

����������������-

When you "port Lock" WILDCAT! or QMODEM - the DTE#1 to DCE#1 connection

is always going to be at the speed you choose WILDCAT! or QMODEM to

default to. HST users will simply modify the MAKEWILD utility to reflect

19200 or 38400 (for HST DS) in the port lock speed entry.

Now, you will have to adjust your modem to the understand that DTE is set at

one speed and the DCE connection (i.e. to DCE#2) will be variable, depending

on the caller speed.

To do this with the Courier HST modem, you must change the &B switch from "0"

to "1".  Normal setting is &B0 which is the DCE follows the DTE connection

rate (i.e. if CONNECT 2400, the computer to modem, DTE to DCE, rate is 2400,

if the CONNECT 1200, the DTE to DCE rate is 1200 -- how wasteful)!

Do this from WCMODEM terminal or your comm package by typing:

AT &B1

Also, I suggest you set the compression OFF...this is the &K switch.  This

is optional as the normal &K1 setting auto sets to the callers setting, but

to get faster speeds - why compress already compressed ZIP or ARC files?

Again, from your terminal:

AT &K0

HANDSHAKING:

�����������-

Now, the last step is to make sure the correct and most reliable "handshaking"

is going to occur.  Let's spend a sentence or two on handshaking:  basically,

if the DTE to DCE rate is 19200 and locked at that speed, and a caller

connects at say 2400, there is a mismatch.  The modem has to do the buffering

between the cable and the phone line!  All modems have some kind of ability

to buffer and they all have a limited size.  So they want to be able to tell

the DTE that it should stop - hold for a bit - then keep sending more when

the buffer is empty enough.  Think of it as a traffic cop!

There are two ways for the modem or DCE to do this:  Hardware or Software.

The most reliable (to me) is the Hardware...this is accomplished by turning off

the CTS pin voltage...the DTE is stopped!  So the modem must be told to do so.

This is done on the HST by assumring the &H is changed to "1".  Normally, this

setting is turned off completely as you will never need the buffering or

traffic management between the DTE and DCE.  Using your terminal:

AT &H1

The &I switch allows receive data control also....although since the DCE to DTE

rate is 19200, this should never be needed, but I suggest setting this switch

to &I2 just in case:

AT &I2

That should do it for the modem setup if you have an HST.  If there are other

modem setup's that sysops would like to give me for furthering the depth of

this document, I welcome them.  I hope this is a good starter and helps those

who are new, understand the process.

EXTERNAL PROTOCOLS & PORT LOCKING

���������������������������������

Some protocols require you add special commands to the command line to

advise of port locking, for example, some versions of TModem requires

you install a COM1=19200 in your environment (i.e. autoexec.bat file) in

order that it knows the port is locked at 19200. Others, like MPt [old

PUMA] which requires the '-L 19200' addition to the command line.  While

others, like DSZ, require no additional changes to their command string.

Unfortunately, without these changes, protocols may be unaware of the

difference in port speed versus modem connect speed which provides for

erroneous transfer time estimations.  WILDCAT! does provide the %4

parameter which passes the DCE connect speed rather than the DTE connect

speed (the DTE will always be the same since it is locked!).  QMODEM

does the same with %1 as the serial port speed and %3 as the connect

speed.

I suggest you carefully read both the external protocol documentation

provided in the WILDCAT! and QMODEM manuals and each individual protocol

manual and choose what best suits your needs....but first, read on....

AndyW  <<Sysop>>  The Digital X-Connect WILDCAT!

214-517-8443  9600 V.32