5.12 Macros - a way of simplifying logic network descriptions

When simulating complex logic  systems  it  is  essential  to  have  a  MACRO

facility which allows a previously defined logic network to be treated  as  a

module in its own right.  Three basic ideas are central to the macro concept,

these are:-

(1) A macro is essentially a stand alone network description with the relevant

input and output nodes specified in the definition of the macro.

(2) Once defined, a macro can be treated as a functional BLACK BOX and can be

used over and over again just by referencing it by name.   The LSYSTEM

compiler expands each macro which generates a list of the internal logic

components, ready for simulation.

(3) All circuit nodes defined within a macro definition are LOCAL, that is,

the actual node names used will not conflict with those used in the main

logic network or other macro definitions.

Each time a macro is invoked in the main logic network ( called like a

program subroutine ), UNIQUE versions of its internal node names are

automatically generated by the LSYSTEM compiler.

An unlimited number of macros can be defined for inclusion in  logic  network

descriptions.   Macros can be nested up to 20 levels deep, that is macros can

use other macros, which use other macros, which use other macros etc ....  up

to 20 levels.

5.12.1    Defining and using macros

Once a macro has been defined its use is identical to that of a primitive.

Four essential rules MUST be adhered to when defining and using macros:-

(1)    A macro must be defined before it is used.

(2)    A macro cannot be defined inside a macro definition.

However, other macros can be invoked from inside a macro definition.

(3)    Clock primitives are NOT allowed within a macro definition.

(4)    Command statements, for example .PRINT or .TIME, are not allowed

within a macro definition.

To define a macro the internal logic network description  is  placed  between

the keywords .MACRO and .ENDM.  The macro start directive .MACRO is  followed

by a macro name, this is the macro reference name, and a  list  of  interface

nodes.   The macro definition  ends  with  the  end  directive  .ENDM.    For

example, a very simple macro defining a AND_OR functional element is:-

+----+

A ---I  & I

B ---I    I--------+    AND_OR

+----+        I

+----+        I    +----+

C ---I  & I        +----I >= I

D ---I    I-------------I  1 I---------- F

+----+             +----+

I

I

I

+----------+

A -----I AND_OR   I

B -----I          I

C -----I          I----- F

D -----I          I

+----------+

In LSYSTEM syntax this macro could be written as:-

.MACRO AND_OR A B C D F

G1 A B N1 AND2

G2 C D N2 AND2

G3 N1 N2 F OR2

.ENDM

In this example definition nodes N1 and N2 are LOCAL  internal  nodes.    The

AND_OR macro just defined could then be used in the main network  description

or in another macro definition in exactly the same way as a  primitive  logic

component type would be.

Please note that when a macro is invoked (  called  ),  interface  nodes  are

parsed to it by POSITION and not by NAME.

For example, the main network description  for  a  logic  network  using  the

AND_OR macro twice, such as:-

+----------+

IN1 -----I AND_OR   I

IN2 -----I          I  N1

IN3 -----I          I-----+

IN4 -----I          I     I

+----------+     I      +-----+

+------I  &  I

+----------+     +------I     I------- OUTF

IN5 -----I AND_OR   I     I      +-----+

IN6 -----I          I     I

IN7 -----I          I-----+

IN8 -----I          I  N2

+----------+

would be described as:-

M1  IN1  IN2 IN3  IN4 N1  AND_OR

M2  IN5  IN6 IN7  IN8 N2  AND_OR

G1  N1  N2  OUT  AND2

The network description defined above, using the AND_OR  macro  twice,  could

itself have been defined as a macro,  so  that  it  could  be  treated  as  a

functional black box with eight inputs and one output.  Hence, a macro called

AND_OR_2, which uses the macro AND_OR twice:-

+-----------+

A -----I AND_OR    I

B -----I           I-------+   AND_OR_2

C -----I           I  N1   I

D -----I           I       I

+-----------+       I    +-----+

+----I  &  I

+-----------+       +----I     I-------  O

E -----I AND_OR    I N2    I    +-----+

F -----I           I-------+

G -----I           I

H -----I           I

+-----------+

would be defined as:-

.MACRO AND_OR_2  A B C D E F G H O

M1 A B C D N1 AND_OR

M2 E F G H N2

G1 N1 N2 O

.ENDM

NOTE how the AND_OR macro has been NESTED within the AND_OR_2  macro.    This

nesting process can be continued to 20 levels deep ( this example is  just  1

level deep ) which should be more than adequate for most purposes.

The logic network description using the AND_OR macro  twice,  given  earlier;

can now be defined in just one line, using the AND_OR_2 macro:-

M1 IN1 IN2 IN3 IN4 IN5 IN6 IN7 IN8 OUT AND_OR_2

Once expanded out by the LSYSTEM compiler, the resulting network  description

will be no different to that generated by not using  macros   and  the  logic

network repetitively described with logic primitives.

Macro nested definitions or recursive calls are NOT allowed., for example:-

CORRECT                             WRONG

.macro one a b                      .macro two a b

g1 n1 n1 a nand2                         .macro one c d

g2 a b n1 nor2                             g1 n1 n1 c nand2

.endm                                        g2 a b n1 nor2

.macro two a b                             .endm

g3 a b n xor                             g3 a b n xor

g4 a b one                               g4 n b one

.endm                                .endm

A final feature of macros worth remembering when defining  a  macro  concerns

the rise and fall time of different paths through the internal logic network.

In general each path will have a different rise and fall time, for example in

a D flip flop the D to Q path is likely to be  different  to  the  PRESET  or

CLEAR to Q path.  The individual delay times can be set by including,  within

a macro structure, non-inverting buffers with their rise and fall  times  set

to give the correct overall path delays.    The following example illustrates

the technique:-

* Asynchronous BCD counter.

.LIB default.LIB

*

.macro jkff_cp_ne j clock k clear qn q preset

g1 clock clockn INV (delay = 3)

ic1 clockn clear preset j k q qn jkff_cp_pe

.endm

.MACRO DECODE Q1 Q2 Q3 Q4 DOUT

G1 Q1 O1 INV DELAY=1

G3 Q3 O3 INV DELAY=1

G5 O1 Q2 O3 Q4 DOUT NAND4 (DELAY=6)

.ENDM

*

GEN1  COUNT CLK0 100

GEN2  CLOCK CLK0 PERIOD(200)

GEN3  PRESET CLK1 PERIOD(50000)

*

IC1  COUNT CLOCK COUNT CLR NQ1 Q1 PRESET JKFF_CP_NE

IC2  COUNT Q1    COUNT CLR NQ2 Q2 PRESET JKFF_CP_NE

IC3  COUNT Q2    COUNT CLR NQ3 Q3 PRESET JKFF_CP_NE

IC4  COUNT Q3    COUNT CLR NQ4 Q4 PRESET JKFF_CP_NE

IC5 Q1 Q2 Q3 Q4 CLR DECODE

*

.TIME 7000 7000

.PRINT COUNT PRESET CLR CLOCK Q1 Q2 Q3 Q4

.END

5.13 Comments

Comments can be embedded into the logic network description file by beginning

the line they occupy with an asterisk ( * ).   Comments can also be  appended

to any line by preceding them with a semi-colon ( ; ).   Note  that  appended

comments are not passed on to any output files, whereas * delimited  comments

are.  Comments are simply ignored by the compilation stage of the  simulation

cycle.

A comment line, after the  delimiter,  may  contain  any  valid  alphanumeric

string, for example:-

*--- Network description of a 16bit shift register.

*--- Using D type flip flops.

;++  THIS COMMENT WILL NOT BE PASSED TO THE OUTPUT FILES.

*--- Default primitive library loaded.

.lib default;      This is an appended comment.

*--- Macro library loaded via include statement.

.include default.mac

*--- Excitation clocks.

clo1 clock clk0 period(100) ; system clock with 100ns period.

clo2 reset clk1 523 577 ; short pulse to initialize shift register stages.

clo3 enable clk0 ; tri state output enable.

*--- Network description

.

.

etc etc

6.   Compiling a logic network description

6.1  The compile command

Once the network description file has been created, using a text editor,  the

description must be COMPILED into a pure data format ready for input  to  the

LSYSTEM simulator.   Compilation, as detailed in section 4.2, is initiated by

entering:-

C:\LSYSTEM> compile file[.NWK] [switches......] <ENTER>

6.2  Error and warning messages reported during compilation

Some  fatal  errors,  such  as  inability  to  open  the  specified   network

description file or insufficient memory  available,  just  display  an  error

message saying as  such  and  abort  compilation.    Most  minor  errors  and

warnings however, do not abort compilation immediately.   Warnings and errors

are added up during the first pass of the compiler and a decision is made  on

whether to execute the second ( final ) pass or not.

Errors and warnings are reported on the  video  screen  and  written  to  the

".LST" file if requested by:-

- Printing the line number and the file error found.

- Printing the line in error, with the error highlighted.

- Printing a arrowhead underneath the error for clarity.

An example error is:-

0115 <4SHIFT.NWK> "Numerical value invalid"

G10 OE OE1 INV RISE=1O FALL=8

This error has been reported because the O used for the  1O  is  actually  an

letter O ( as in N, O, P, Q etc ) and not a number zero ( as in 0, 1 ,2 etc).

All error and warning message output is duplicated in the same format in  the

".LST" error listing, when requested by a -L or a .LST command.

6.3  The information file -- file.INF

An information file with the extension ".INF" is always generated during  the

compilation phase of the LSYSTEM simulation cycle.  This  output  file  gives

information about the logic network statistics, including the number of nodes

, logic components, number of primitives loaded and so forth.

After an error free compilation the information file contains:-

- Network description name.

- Number of source lines compiled.

- Number of nodes in network.

- Number of trace nodes.

- Number of logic components in network.

- Number of excitation clocks.

- Simulation length.

- Simulation display period.

- Number of primitives loaded.

- Number of macros loaded.

- Number of used macros.

- Highest macro nesting.

- Complete node name table, with internal node numbers.

- Table of used primitives, with number of times used.

- Table of used macros, with internal reference numbers.

The information file can be used in conjunction with any  warnings  generated

at the simulation phase of the LSYSTEM simulation cycle to  trace  contending

nodes,  and  undriven  (  floating  )  inputs.    Warnings  reported  at  the

simulation stage reference nodes and logic elements by their internal numbers

and not by name, so this file is invaluable.

7.   Simulating a digital logic network

7.1  Prerequisites

A logic network description file must have been created  and  an  error  free

compilation performed to generate the necessary  data  files  needed  by  the

simulation stage of the LSYSTEM simulation cycle.

Simulate the network as specified in section 4.3 by entering:-

C:\LSYSTEM> simulate file <ENTER>

7.2  Interpreting the tabular results file -- file.RLT

The primary output file generated by a simulation has  the  extension  ".RLT"

and contains tabulated information representing each trace node change,  with

reference to time in nS.   Node states are output in the results file EITHER:

at the simulation display period time specified in the  network  description,

OR whenever the logical state of a trace  node  changes,  see  section  5.11.

Trace nodes are shown with four possible states:-

0 -> Logical LOW.

1 -> Logical HIGH.

Z -> High impedance state.

X -> Indeterminate state ( startup or contention condition).

Warnings generated at simulation time are  embedded  within  the  node  state

table as and when they occur.

7.3  Errors and warnings reported at simulation time

Errors and warnings generated at simulation time are minimal, since  many  of

the  problems  causing  errors,  such  as  undriven  nodes,  are  removed  at

compilation stage in the LSYSTEM simulation cycle.

The warnings, with a typical example,  that are reported are:-

(1)  Output contention

* WARNING [17nS] - Output contention: Node  40 , f49 f67

Reports that two logic components are trying to drive  node  number  40  into

opposing states.

(2)  Floating input node

* WARNING [100nS] - Floating input node:  f71 {JKFF_CP_NE}, N40

Reports that node 40 has gone into high impedance state  and  has  caused  an

input on a JK flip flop to float high.

(3)  Simulation manually aborted

* WARNING - Simulation manually aborted at time : 3053nS

Reports simulation aborted by pressing <ESC>.  The results up to  this  point

are still valid.

(5)  A node never assumed a logical state

* Warning - Node number 5 never assumed a logical state

This warning, for every node applicable, is given at the end of  the  results

listing.  It means  that  the  node  stayed  in  the  indeterminate  state  X

throughout the entire simulation period.   The logic element(s)  driving  the

node were either never excited by a  changing  input  node,  or  remained  in

contention the whole time.   Contention though would have  been  reported  as

above, when it occurred.

(6) Network instability

In the case of a logic network which is unstable, LSYSTEM reports the time at

which the instability occurred, on screen in a separate window, then finishes.

8.   Post simulation graphics processing of trace node waveforms

8.1  Prerequisites

The network description must have been compiled  and  simulated  successfully

and your computer must include one of  the  graphics  adapters  specified  in

section 2.  The waveform graphics post processor, as described in  detail  in

section 4.4, is initiated by entering:-

C:\LSYSTEM> waveform file [switches....] <ENTER>

8.2  Split screen display

The graphical display starts in split screen mode.    The  top  half  of  the

screen contains a timing diagram format display of the results  data,  whilst

the bottom contains a window displaying  the  tabular  results  file.    This

allows direct comparison of both  forms  of  data  presentation.   The  split

screen mode can be toggled on and off using  function  key  F2  whilst  being

displayed.   The tabular results file is loaded by waveform into a buffer  at

the start of a display session.   If the results file is greater in size than

this buffer only those results which will fit in the  buffer  are  loaded  by

waveform.   The parameters in the .TIME statement should be changed  if  this

occurs.

8.3  Waveform logic state presentation

Four logical states are shown on the graphical display, they are:-

....XXXXXXX........    Pulse of HIGH Impedance state [RED].

....XXXXXXX........    Pulse of Indeterminate state [GREEN].

.......

....     ..........    Pulse of HIGH state.

....    ...........    Pulse of LOW state.

......

these state notations are equivalent to the Z, X, 1 and 0 notation  used  for

the tabulated results file.   Trace node states  are  represented  using  the

above notation in a graph format,  plotting  time  (  in  for  example  nS  )

horizontally, against specified trace nodes vertically.

Several  user  commands  are  available  interactively  whilst  viewing   the

graphical results.  They are are listed in the following screen dump  of  the

waveform help screen.

SEE PRINTED USER GUIDE FOR SCREEN DUMP

[ OR YOUR COMPUTER SCREEN ]

9.   A further example simulation

To demonstrate the processes discussed in earlier sections of this manual,  a

commented example network is presented next.  The  network  description  file

for this example is also included on the LSYSTEM master disks.   The  example

has been chosen to demonstrate as wide  a  range  of  the  available  LSYSTEM

functions as possible.   Functionally, the sample network describes an 8  bit

serial data transmitter.  Eight data bits are loaded and the  transmitted  as

an 11 bit serial data stream which has one start bit and two stop bits.

+--------------------+

B1 ---------I                    I

B2 ---------I                    I---------  OUTM

B3 ---------I                    I

B4 ---------I                    I

B5 ---------I                    I

B6 ---------I                    I

B7 ---------I                    I

B8 ---------I                    I

I                    I

LOAD  --------OI                    I

TRANS --------OI                    I

RESET --------OI                    I--------- BUSY

CLOCK ---------I                    I

+--------------------+

The operation of the transmitter is as follows:-

1- Reset by holding RESET low for a short period.

Reset state for transmitter output is logic 1.

2- Latch parallel data in by taking LOAD low for at least one clock

cycle (loads on rising edge).

3- Pulse TRANSMIT low to send 11 bit serial data, at bit rate = clock / 2,

made up as follows :-

0XXXXXXXX11

where 0 = start bit,

X = data bits,

and   1 = stop bits.

4- BUSY remains high whilst serial data is being transmitted.

5- TRANSMIT ignored during BUSY period.

6- LOAD ignored during BUSY period.

7- RESET clears asynchronously whenever applied.

8- A new LOAD/TRANSMIT sequence can occur as soon as BUSY

returns to an inactive (logic 0 ) state.

9.1  The transmitter network file -- SAMPLE.NWK

The sample network description file should be studied until all the important

factors are understood.   Note the way in which the logic  network  has  been

broken down into manageable portions with the use of macros.

* SAMPLE.NWK example file -- 8 bit serial data transmitter.

*

.lib default.lib; ** Standard SHAREWARE PRIMITIVES **

*

elc clocks CLK0 PERIOD(100) ; Main system clock.

elc reset  CLK1 323 379 ; Single RESET pulse to initialise counter.

elc trans clk1 1023 1523 2720 2803

*------------------------------------- transmit input pulse -- for sending

*------------------------------------- data.

elc load CLK1 577 777 2423 2533

*-----------------------------  Load pulse -- for loading parallel data.

elc B1 clk1 2400 ; Parallel data -- bit 1

elc B2 clk1 2400 ;                  bit 2

elc B3 clk0 2400 ;                  bit 3

elc B4 clk0 2400 ;                  bit 4

elc B5 clk0 2400 ;                  bit 5

elc B6 clk1 2400 ;                  bit 6

elc B7 clk1 2400 ;                  bit 7

elc B8 clk1 2400 ;                  bit 8

elc vcc  clk1 ; V+ power rail.

elc gnd  clk0 ; zero volt power rail.

*-- 4 bit ring counter. Resetting to 1000

*-- sequence:

*  1000

*  0100

*  0010

*  0001

*  1000

*  etc....

*

.macro 12ring clock reset qa qb qc qd qe qf qg qh qi qj qk ql vcc

el_ring clock vcc reset  l  a na dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  a  b nb dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  b  c nc dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  c  d nd dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  d  e ne dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  e  f nf dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  f  g ng dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  g  h nh dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  h  i ni dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  i  j nj dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  j  k nk dff_cp_pe (rise=12 fall=13)

el_ring clock reset vcc  k  l nl dff_cp_pe (rise=12 fall=13)

el_ring nl a  qa and2 (delay=9)

el_ring na b  qb and2 (delay=9)

el_ring nb c  qc and2 (delay=9)

el_ring nc d  qd and2 (delay=9)

el_ring nd e  qe and2 (delay=9)

el_ring ne f  qf and2 (delay=9)

el_ring nf g  qg and2 (delay=9)

el_ring ng h  qh and2 (delay=9)

el_ring nh i  qi and2 (delay=9)

el_ring ni j  qj and2 (delay=9)

el_ring nj k  qk and2 (delay=9)

el_ring nk l  ql and2 (delay=9)

.endm

*

.macro 12BURST clock reset XMIT a b c d e f g h i j k l active vcc

*-- Generates set of enable pulses for conversion

*-- of parallel data to serial stream.

el_12burst reset n1 n2 and2 (delay = 9)

el_12burst xmit n5 inv (delay = 9)

el_12burst n5 n4 n7 or2 (delay = 9)

el_12burst clock nclock inv (delay = 9)

el_12burst n4 nclock n3 and2 (delay = 9)

el_12burst L n1 inv (delay = 9)

el_12burst clock n2 vcc n7 n4 not_n4 dff_cp_pe (delay = 9)

el_12burst N3 N4 a b c d e f g h i j k l vcc 12ring

el_12burst a active inv (delay = 9)

.endm

.macro 8LATCH clock reset Nload a b c d e f g h  fa fb fc fd fe ff fg fh vcc

*-- Nload LOW causes data to be latched in at next rise clk edge.

*-- Data can be read always from outputs.

el_8latch nload not_1 inv (delay = 9)

el_8latch not_1 clock clk and2 (delay = 9)

el_8latch clk reset vcc a fa not_fa dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc b fb not_fb dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc c fc not_fc dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc d fd not_fd dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc e fe not_fe dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc f ff not_ff dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc g fg not_fg dff_cp_pe (rise=12 fall=13)

el_8latch clk reset vcc h fh not_fh dff_cp_pe (rise=12 fall=13)

.endm

*

.macro TriSbuf in enable out

el_TriSbuf enable Nenable inv

el_TriSbuf nenable in out tbuf

.endm

*

*-- Transmitter

*

el clockS reset trans +1 +2 +3 +4 +5 +6 +7 +8 +9 +10 +11 +12 ACT vcc 12burst

el gnd +2  out triSbuf

el a +3  out triSbuf

el b +4  out triSbuf

el c +5  out triSbuf

el d +6  out triSbuf

el e +7  out triSbuf

el f +8  out triSbuf

el g +9  out triSbuf

el h +10  out triSbuf

el vcc +11  out triSbuf;  ** Stop 1 **

el vcc +12  out triSbuf;  ** Stop 2 **

el vcc +1  out triSbuf

*

el clockS NclockS inv (delay = 1)

el NclockS vcc reset out outM not_outM dff_cp_pe; REALLY "SET" not reset.

el clockS reset vcc active busy not_busy dff_cp_pe

el vcc out res;   ** Pull up TRi-STATE o/ps **

el clockS NCw inv (delay = 0)

el NCw NCu inv (delay = 1)

el NCw NCu ref and2 (delay = 1)

el clockS reset load b1 b2 b3 b4 b5 b6 b7 b8 a b c d e f g h vcc 8latch

*

*

.expand

.time 10000 10000

.print clockS reset ! load ! transmit ! ref ! busy !!!! outm !!!!!!

.print bit1 bit2 bit3 bit4 bit5 bit6 bit7 bit8

.end

Appendix A   Installing LSYSTEM on floppy disks

LSYSTEM is a disk intensive suite of computer programs.  If the computer  you

are using has  a hard disk or a high  density  floppy  disk  fitted  then  NO

problems with lack of disk space are likely to occur.  For  users  with  only

360K DSDD floppy drives installed in their computer it is still  possible  to

run LSYSTEM but care must be taken over the available disk space.

To install LSYSTEM on a 360K floppy  disk  system  the  following  steps  are

recommended:-

(1) Format a new 360K floppy disk.

(2) Make a directory called LSYSTEM on the formatted disk.

(3) Copy the following files from the LSYSTEM master  disks  to  the  LSYSTEM

directory on the 360k floppy.

COMPILE.EXE       LSM.BAT       CGA.BGI

SIMULATE.EXE      LS.BAT        EGAVGA.BGI

WAVEFORM.EXE                    HERC.BGI

DEFAULT.LIB

You should find these files require slightly over 191K bytes of disk

space.

(4) Add to these files the example network file SAMPLE.NWK.

The 360K floppy is now a working LSYSTEM disk which can be used in either the

A or B drives.  To use the LSYSTEM in the B  drive,  make  the  B  drive  the

current drive and change directories to the LSYSTEM directory.   This  allows

LSYSTEM to be run by entering, for example

B:\LSYSTEM> LSM SAMPLE, or

B:\LSYSTEM> LS SAMPLE

The example file SAMPLE.NWK, see page 38, generates a number of  files  which

require roughly 58K of disk space, leaving room on a 360K floppy for a number

of other library and macro files.  Take care not to exceed the available free

disk space by requesting large quantities of output results with badly chosen

.TIME statement parameters.

Appendix B   The LSYSTEM primitive library file structure

ONLY AVAILABLE  TO REGISTERED LSYSTEM USERS

Appendix C   LSYSTEM RAM primitives

ONLY AVAILABLE TO REGISTERED LSYSTEM USERS

Appendix  D  ROM primitives

ONLY AVAILABLE TO REGISTERED LSYSTEM USERS

#######################################################################################################