Documentation for NKFSA v0.96 User-Written Drivers...

NKFSA can be interfaced to any A/D hardware via a User-Written Driver Program.

This document describes the interface between NKFSA and the driver, and

describes the example driver that comes with NKFSA.

NKFSA calls the driver via a software interrupt.  You get to choose which

software interrupt NKFSA will use for this purpose.  The default is int 0F1H.

It is assumed that the driver is written in the form of a TSR.  TSR is DOS

nomenclature for a program that uses the DOS Terminate-and-Stay-Resident

function.

Once your driver is written and working, you simply execute it, which will

cause it to load, initialize itself, then, of course, terminate-and-stay-

resident.  After that, anytime NKFSA is run, it can use the driver and access

your A/D.

An example driver SAMTSR.COM is provided.  Because it is only an example, it

does not require any A/D converter hardware.  When called by NKFSA, it simply

returns a fixed waveform it has stored inside.  To try SAMTSR...

>   SAMTSR           <---causes sample user-written driver to load

>   NKFSA -su        <---fires up NKFSA specifying the user-written driver

When NKFSA issues software interrupt 0F1H to call the driver, the registers

have the following meanings...

AX = -1 means that this call is to set the sample rate

ES:BX = address of doubleword containing the sample rate in Hz.

AX >0   means that this call is to fetch a buffer of samples

AX contains NUMSAM, the number of samples desired

ES:BX = address of sample buffer.  Driver must transfer NUMSAM

samples into the buffer.  Each sample is a 16-bit number

in ordinary 2's-complement form, in the range [-8192,+8191].

To make your own driver, make a copy of SAMTSR.ASM, and add the code required

to talk to your A/D converter hardware.  (Give your file a new name!)

The comments in SAMTSR should be adequate to guide you, even if you have never

written a TSR before.

The sample TSR provided uses interrupt 0F1H.  If this interrupt is already

used by some other software you use, you may change the interrupt by editing

the value of the user_supplied_driver_interrupt_number in the NKFSA.INI file,

and altering the corresponding parameter in your driver TSR.  If you decide to

use a different interrupt, it is strongly suggested that you choose one of the

interrupts that Microsoft & IBM have "reserved for user program interrupts".

These are 60H thru 66H and 0F1H thru 0FFH.

TSRs can be a little tricky to debug, so it is suggested that you debug the

code that talks to your A/D hardware in a normal (not TSR) program before you

integrate that code into SAMTSR.

To generate the executable form of your TSR...

Use MASM to make a .OBJ file

Use LINK to make a .EXE file (it will complain that there's no stack

segment, but that's normal when generating a .COM file)

Use EXE2BIN to make the .COM file

Alternatively, if you don't want to write in assembler, i'm told some compilers

now are able to generate TSRs.  If you have one, great, but don't ask me

questions about that.  I've never tried it.

73,

Franklin Antonio

N6NKF

06/17/88