+-------------------------------------------+

| N6NKF Real Time Spectrum Analyzer Program |

|           Version 0.97  06/19/88          |

|   Copyright (c) Franklin Antonio, 1988    |

+-------------------------------------------+

Required Hardware/Software...

Computer: Any IBM PC/XT/AT/PS2 or clone

Memory: At least 256k memory

Graphics: CGA or EGA or VGA graphics board

System: DOS 2.0 or newer, or OS/2 1.0

Signal Source: A signal source is required, which can be any one of

the following...

a.  A binary disk file of integer data samples

b.  A formatted disk file of real data samples

c.  A/D converter on a Dalanco/Spry Model 10 board

d.  Any A/D converter, with a user-written driver.

Recommended Optional Hardware..

An EGA or VGA graphics board is especially recommended.

Microsoft mouse, or Mouse-Systems Corp. mouse, or compatible.

Things that are NOT required...

Coprocessor:  8087 or 80287 numeric co-processor is NOT required, but

will be used if present.

DSP Processor:  A DSP board is NOT required.

ANSI.SYS:  This program does NOT require ANSI.SYS

Copyright Statement...

Copyright (c) Franklin Antonio, 1988.  All Rights Reserved.

This program is provided at no cost for amateur non-profit use

only.  Non-profit distribution of the unmodified program and

documentation is allowed.  Commercial use or distribution of

this program is prohibited unless license has been obtained

from the author.  Copyright is claimed on both the program and

the visual images the program generates.

The remainder of this document consists of the following sections...

0.  Getting Started

1.  Commands

2.  Command line options

3.  NKFSA.INI file options

4.  Speed of Execution

5.  Notes re Dalanco-Spry board

6.  Notes re using file signal source

7.  Characteristics with various display adapters

8.  Limitations

9.  Testing, Bugs, Etc.

10.  Bibliography

Getting Started...

------------------

If you are not already familiar with use of a spectrum analyzer, some of the

controls may seem quite alien.  To gain familiarity with the effect of

windowing, averaging, resolution, etc controls, I suggest you fire up the

program using the internally generated "demo" signal source.  This signal

consists of a sum of five sinusoids at different amplitudes and frequencies.

One is frequency modulated, and one is amplitude modulated.  To start NKFSA

with the demo signal source, give the command..

>  NKFSA -sd

The program will begin set for highest resolution, and a Blackman window.

You may now alter any control settings using either the mouse or keyboard.

Commands...

-----------

HELP:

Type ? or click mouse on "HELP" for on-screen help display.

In some cases, the on-screen help contains additional information.

Source:

Change by clicking mouse on "Sig Src", or type "s".  This selects the

source of the signal to be analyzed.  Present options are...

1. Demo (a dummy signal provided for demo)

2. A/D on Dalanco/Spry Model 10 board

3. Binary 16-bit Integer Disk file

4. Formatted Real Disk File

5. User-written A/D converter driver

As shipped, the Demo signal is the default signal source, because

that's the only signal source i'm sure everybody has.

Signal source may also be specified on the command line, and the

default signal source may be specified in the NKFSA.INI file.  See

descriptions of these below.

More info about #2, #3, and #4 above can be found toward the end of

this document and in the online help screens.  More info about #5 can

be found in the file NKFDRV.DOC.

Function:

Change by clicking mouse on "Function", or typing "f".

This selects the function to be computed & displayed.  Present options

are "Waveform" (ie like an oscilloscope), "Spectrum" (like a spectrum

analyzer), and "Cepstrum".  This program is basically a spectrum anal-

izer.  The waveform display was added as an afterthought, and is not

elaborate.

Frequency Resolution:

Change by clicking the mouse on "Frq Res" or type "r".

This selects the transform size, from 1024 pts to 64 pts, which

at 6kHz sample rate corresponds to 5.8 Hz to 93.8 Hz resolution.

This corresponds to the resolution bandwidth knob on an analog

spectrum analyzer.

Wdw:

Select the spectral window by clicking on "Wdw:", or typing "w".

This selects among three fairly standard windows...

Rectangular     -- Maximum detectability.  -13 db first sidelobe

Hamming         -- Compromise.             -43 db first sidelobe

Exact Blackman  -- Lowest Leakage          -68 db first sidelobe

If you are not familiar with the effect of windowing, then you may

wish to leave this control set on "Blk" (the Exact Blackman window,

which is the default.  There is no corresponding knob on an analog

spectrum analyzer.

Avg:

Selects the # of periodograms which are averaged prior to display.

Click on "Avg", or type "a".  Options are "off" and "x3".  Averaging

increases the stability of a spectral estimate, but may blur rapidly

changing spectra.  This corresponds to the video averaging control on

an analog spectrum analyzer.

Hld:

Turns the "max hold" feature on and off.  When this switch is on, the

program remembers the maximum value of the spectrum at each frequency,

and displays the maximum in a different color.  (only available on EGA

or VGA displays.)

scL:

Sets the vertical display scale.  In this version, 10db/division

logrithmic display is the only option.

Grat:

Selects which graticule appears behind the spectrum.  Choices are none,

dot, line, and grid.  On CGA displays, there is no graticule.

EXITing:

Click on "EXIT", or type "e", or the F10 key.  This corresponds to the

off switch on an analog spectrum analyzer.

Annotation:

Press the mouse button while the mouse cursor is in the spectrum

display area, and the frequency and amplitude at that point will be

displayed.  I've used the standard mouse cursor, which has it's hot-

spot located three pixels to the left and one above the mouse tip.

PrtSc:

Typing the PrtSc key (not Shift-PrtSc) at any time will produce a

screen dump to printer.  Epson-like printer command set assumed, and

normal DOS printer redirection in effect.  This built-in screen dump

does not prevent you from using your own screen dump TSR.  See next

paragraph.

Typing Shift-PrtSc will, as usual, invoke the default DOS print screen

function, which is not graphics capable, unless you have installed a

TSR for this purpose.

Command Line Options...

-----------------------

You may specify any one of the following options on the command line when you

invoke this program.  Options are case sensitive, so must be typed in lowercase

as shown..

-su	Set signal source to "user-supplied driver"

-sf{f}	Set signal source to 16-bit integer binary disk file {f}

-sg{f}	Set signal source to real formatted data file {f}

-s3	Set signal source to "Dalanco-Spry"

-sd	Set signal source to "Demo"

-wr	Set window to "rectangular"

-wh	Set window to "Hamming"

-wb	Set window to "Blackman"

-sr{n}	Set sample-rate to n samples per second.

-dc	Force use of CGA display mode.

Normally, this program attempts to automatically use an EGA if

available, otherwise it uses CGA.  -dc forces the display to

use CGA mode.  I use this for debugging the CGA mode.

Example command line with lots of options...

>  nkfsa -dc -sfa:\dir\file.dat -sr6000 -wh

NKFSA.INI file options...

-------------------------

Each time NKFSA runs, it reads a parameter file, NKFSA.INI.  NKFSA searches

first in the local directory.  If there is no NKFSA.INI file in the local

directory, the top level directory on the current drive is searched.  Commands

in this file specify the colors NKFSA uses (on EGA or VGA displays), and

default values for many of the parameter settings described above.

Documentation on commands in the .INI file is contained in comments in the file

itself.

Speed of Execution...

---------------------

There is a commonly held belief that you can't do real-time signal processing

without using special DSP chips.  It isn't true!

At it's highest resolution setting, NKFSA computes one spectral estimate in

about 140 ms (for any integer data source, i.e. any except the "formatted real

data file")...

times on standard 8 MHz IBM-AT are...

1024 sample Windowing+FFT                  = 109 ms

512 frequency Power+Averaging+Logrithm     =  19 ms

512 frequency display on CGA/EGA/VGA       =  11 ms

-------

Total   = 139 ms

Max hold display, when enabled, adds another 10 ms.

Assuming a sample rate of 6kHz, 1024 samples = 171 ms, so this does run in

REAL-TIME, examining 100% of the data, for a < 3kHz (voice) bandwidth.  This

example is for the highest resolution (1024 pts).  At lower resolution

settings, of course, it goes faster!

When running on a slower machine such as an 8088-based PC or XT, the program

will obviously run more slowly.  Much of the speed can be regained, however,

by reducing the resolution setting (Frq Res).

Examining 100% of the data may seem "only reasonable" (It does to me.), but

this is an amazingly larger portion of the signal than would be processed by a

traditional analog spectrum analyzer with similar settings on its controls.

To put this in perspective, consider how a traditional analog spectrum

analyzer works.  It estimates the spectum by sweeping a single filter across

the band of interest.  To be comparable to the NKFSA settings described above,

our example analog spectrum analyzer would be sweeping a 5.9 Hz wide filter

over the range [0 Hz, 3 kHz].  This filter can only be in one place at a time,

and that 3 kHz range is 512 times wider than the 5.9 Hz wide analysis filter.

This means that this single analysis filter spends roughly 1/512th of it's time

at any particular frequency.  This means that at each frequency, the filter is

only able to examine about 1/512th of the input data!

Using Fast-Fourier-Transform techniques, digital spectrum analyzers, such as

NKFSA, compute the output of a large number of filters simultaniously.  In the

example above, with 1024 input points, NKFSA would be computing 512 analysis

filters in parallel, every 140ms.  Using 512 filters in parallel, a filter at

each frequency is able to examine all the input data, all of the time.

What about sample rates significantly greater than 6kHz?  NKFSA can deal with

these too, although the computation can no longer examine 100% of the data that

flys by.  In particular, I have used NKFSA with my Dalanco/Spry board at 80kHz

sample rate, which is the maximum sample rate that boards supports.  As

described above, NKFSA takes about 140 ms to compute & display one spectral

estimate based on 1024 points of input.  At 80kHz, 11,200 samples will fly by

in that 140ms.  This means NKFSA can examine 1024 out of every approx 11,200

samples.  That's 1/11th of the data.  Still 46 times better than the analog

spectrum analyzer!

Notes re Dalanco-Spry Model 10 Board..

--------------------------------------

The driver for the D/S board assumes that D/S shared memory is mapped to D000H,

D/S i/o is mapped to 300H, jumper JA is in the "lower" position, and jumper J5

is in the "ext" position.

NKFSA loads the appropriate software into the D/S board & sets the sample rate.

You don't have to run any ancillary programs.

The D/S board is used only for it's A/D converter.  All calculations are done

in the PC.

The author has no affiliation with Dalanco/Spry.  Someone gave me one of their

boards, so it seemed convenient to write a driver.

Notes re Using the "Integer Disk File" Signal Source

----------------------------------------------------

NKFSA assumes that the disk file you specify is a binary file that contains a

stream of 16-bit numbers, and nothing else (no formatting, record marks, etc).

Each of these 16-bit numbers should be an ordinary 2's-complement number in

the range [-8192,+8191].  If you write one of these files in a FORTRAN program,

be sure to specify FORM='BINARY' in the open statement.

If the disk file is longer than NKFSA's spectral analysis window, NKFSA will

read thru the file, (like a movie) crunching on blocks of samples in a fashion

similar to the way it operates on samples in real-time.  When it hits the end-

of-file, it repeats.  If the disk file is shorter than the analysis window,

NKFSA pads the window with zeros, and processes it as usual.

Notes on Using the "Real Disk File" Source:

------------------------------------------

The "Real Disk File" source reads a formatted file of numbers in an "e" or "f"

type format (ie like nnn.nnnnnn or nnn.nnnEnnn), one number per line.  If the

file is shorter than the analysis window, the window is padded with zeros.

The analysis is done in floating-point arithmetic, and the results are auto-

matically scaled so that the highest point in the spectrum appears at a fixed

point on the screen.

This mode was specifically included to evaluate the frequency characteristics

of FIR filters.  Some versions of the McLellen-Parks FIR filter design program

produce a formatted output file containing the impulse response of the designed

filter in a format as described above.  <<The widely distributed version from

the old IEEE distribution tape, does not generate such a file.  I have included

the program MACPARK.EXE with this distribution.  This is an updated McLellen-

Parks from the book "Digital Filter Design" by Parks & Burrus.  It has an inter-

active input section which is much easier to use, and it generates an output

file of impulse response data.  If you need documentation for using MACPARK,

please buy a copy of Parks & Burrus' excellent book.>>  When the input signal

has a natural beginning and end within the analysis window, as an FIR filter

impulse response does, you should always use a rectangular analysis window.

I have also included a file FIR.DAT which contains an example FIR bandpass

filter impulse response.  To view the corresponding frequency response...

>  NKFSA -sgFIR.DAT -wr

You may also specify a sample rate on the command line, using the -sr option.

This will make the frequency annotation units on the screen agree with your

intended use of the filter.

Also included in the distribution is the file SUNSPOT.DAT.  This file contains

184 years of January mean sunspot numbers.  The sample rate is 1 sample/year.

Because NKFSA doesn't understand sample rates that low, you have to lie.  Tell

it the sample rate is 1000 samples/second.  Then, scale the resulting anno-

tation in your head.  For example, 500Hz --> 0.5 cycles/year,  100Hz --> 0.1

cycles/year.  The highest point in the spectrum appears at approximately

90Hz --> 0.090 cycles/year, and is known as the 11 year sunspot cycle.

> NKFSA -sgSUNSPOT.DAT -sr1000

Processing real input is much slower than any other calculation done by NKFSA,

because it's the only calculation done in floating-point arithmetic.  A 1024

point calculation takes about 2.5 seconds on an AT w/287, about 3 seconds on a

PC w/8087.

Characteristics with Various Display Adapters:

----------------------------------------------

There are unfortunately a great many different display adapters on IBMPC

systems.  This program is designed to work with many of them.  A brief summary

of what this program supports follows..

EGA = Enhanced Graphics Adapter

The EGA is the lowest-cost adapter that allows full functionality

of NKFSA. I run this adapter in the 16-color 640x350 graphics mode.

User can choose which colors NKFSA uses, by specifying them in the

NKFSA.INI file.

VGA = Video Graphics Array (also known as IBM PS/2 Display Adapter)

I run this adapter in the 16-color 640x350 graphics mode.

Everything works identical to EGA at the present time.  I am

considering adding new features to NKFSA which will only be

accessible to users with VGA cards however.

CGA = Color Display Adapter

I run this adapter in the black & white 640x200 graphics mode.

Everything is supported on CGA, except that only one spectrum or

waveform can be displayed in the central window.  (ie the graticule

and the max-hold feature are invisible).  Of course, the program

loses much of it's pizzaz when viewed in black & white, but it is

functional.

HGC = Hercules Graphics Card

I intend to support HGC in a later version of this program.

It's about half done now.  Should also work on HGC+.  Everything will

be supported except the max-hold feature.  I will run this adapter in

its 720x348 black & white graphics mode, so it will have resolution as

good as the EGA, but no color.  Meanwhile, Joe Mack, NA3T, has reported

that he uses NKFSA successfully on a PC with HGC by running a CGA->HGC

conversion utility called HGCIBM.  He reports that this slows NKFSA by

a factor of 2.

HCC = Hercules Color Card

This card is a clone of the CGA, so should work just like CGA, but

has not been tested.

MDA = Monochrome Display Adapter

This display adapter doesn't support graphics, so we can't use it

to display spectra.  If you have two display adapters on your system,

either MDA+CGA, or MDA+EGA, or MDA+VGA, that's ok.  If you do have a

two display system, you may fire up NKFSA when either display is

current.  If the MDA is the current display when you run NKFSA, it

will leave the spectrum analyzer picture on the CGA/EGA/VGA when it

exits.

XXX = Any other display adapter

I don't know.  I haven't tried it.

Limitations:

------------

Probably not presently very compatible with DoubleDOS, TopView, or DesqView,

or other multi-processing utilities.  No attempt is made to do any special

programming to be DD or TV or DV friendly.

Appears to run ok under OS/2 1.0, in the DOS-compatibility window.

Has been tested under DoubleDOS 4.0, and worked ok in two-screen mode, (ie

DISPLAY=BOTH command in ddconfig.sys file) *IF* i did not load a mouse driver.

If I do load the mouse driver under DoubleDOS, the mouse cursor eats characters

on the display.  The official response from SoftLogic was...

Msg #  4509 Dated 06-17-88 10:56:34

From: SYSOP

To: FRANKLIN ANTONIO

Re: (R)MICE VS DDOS

Franklin, unfortunately I can offer no other suggestions. I think this

is about as good as you are going to get running a mouse under

DoubleDOS.  jack-sysop

I have not tried TopView or DesqView.

Command line arguments require at least DOS 3.0 or newer.  All other features

should work under any version of DOS.

Testing, Bugs, Etc:

-------------------

I have tried to test this program to a reasonable degree.  Unfortunately,

users have a variety of hardware and software environments, and this provides

an incredible testing challenge.  With the aid of Paul Williamson, KB5MU,

Tom Lafleur, KA6IQA, & Mike Brock, WB6HHV, this program has been tested on..

Computers:	A real IBM-AT with 287, a real IBM-7552 "industrial

computer" without 287, a COMPAQ-386 with 287, a 'turbo'

XT clone with an NEC V20 CPU with 8087, a 'turbo' XT clone

with 8088 CPU without 8087, an Atega 386 with 287.

Display Adapters: A real IBM CGA, a real IBM EGA, a Quadram EGA clone,

a Vega VGA+ clone, an ATI VGA clone.

Mice:		A Microsoft serial mouse using MOUSE.COM 6.11 driver, a

Microsoft bus mouse using MOUSE.COM 6.13, a Mouse-Systems

serial mouse using MSMOUSE.COM 5.03 driver.

System:		DOS 3.1, 3.2, 3.3 and the DOS-compatibility window of OS/2 1.0

Your environment will undoubtedly be different.  Comments are welcome.  My

address can be found in the online help text.

Known problems:

1.  One user has complained of a "lost mouse clicks" problem.

2.  If certain errors occur while processing (problems with the signal

source, for example) NKFSA displays a message on the screen which

contains the statement "type any character to continue".  If this

is a hard error (you forgot to load the TSR A/D driver, or you're

having trouble reading a disk file, etc) then the error message

will reappear after you type that character (because NKFSA will

trying again to read the same signal source).  You can break out of

this loop by typing the F10 key twice in rapid succession.

Bibliography

------------

Technical information on spectrum analysis (from a digital computation point

of view) can be found in...

R.B.Blackman & J.W.Tukey, The Measurement of Power Spectra,

Dover Publications, New York, 1958.

SBN (Paperback edition) = 486-60507-8

A.V.Oppenheim & R.W.Schafer, Digital Signal Processing,

Prentice-Hall, New Jersey, 1975

ISBN = 0-13-214635-5

(chapter 11 -- Power Spectrum Estimation)

S.M.Kay & S.L.Marple, "Spectrum Analysis -- A Modern Perspective",

Proceedings of the IEEE, V69, #11, Nov. 1981, pp 1380-1419,

(contains a bibliography with 278 entries!)

C.S.Burrus & T.W.Parks, DFT/FFT and Convolution Algorithms,

John Wiley & Sons, 1985,

ISBN 0-471-81932-8