MAPPER86/87 .. A DX MAP DRAWING and Propagation Program

for IBM PC Compatibles with graphics capability.

Version 7.1

by

Dennis P. Murray

11165 McDonald Street

Culver City, Ca. 90230

CompuServ 72770,2765

10 August, 1991

This program, written for BASIC compilers such as MICROSOFT QUICK BASIC or

BORLAND TURBO BASIC, displays a map of the world centered on a

user-selectable QTH, and does propagation predictions based upon one of 4

possible methods based upon modified MINIMUF 3.5 or MAXIMUF 3.0 methods. It

shows the regions of sunlight and darkness around the world, and updates

these regions automatically at user- specified time intervals. It can also

accept arbitrary times and dates at the user option. The program accepts

DXCC prefixes or latitude/longitude values for the DX location, draws the

path from the home QTH to that location, and displays the range, azimuth

and wave launch elevation angle from the home QTH. The user can select

either short or long paths; the sunspot number; antenna height, gain, and

polarization for both home QTH and DX station; transmit power; and the

minimum wave launch angle to be used for propagation prediction. The

propagation prediction gives the MUF (maximum usable frequency), LUF

(lowest usable frequency) based upon E-layer screening, and the minimum

number of F-layer hops for the minimum launch angle. Predictions of signal

strength are made for Ham Band frequencies which include D-layer

absorption, reflection losses, and transmit and receive antenna losses due

to ground plane reflection at low elevation angles.

This program is written for IBM PC compatible computers with a Graphics

capability and color display. The program runs quite quickly on an AT clone

with a hard disk and 640k of RAM, but it might be a 'dog' on a floppy-only

machine because it must read some long files for the map data.

The program is being uploaded in two versions, named MAPPER86 and MAPPER87.

MAPPER86 is compiled using 'alternate math' option of the Microsoft BASIC

7.1 Professional Development System in order to use floating point

computations under the Microsoft Binary Format which run much faster on

machines without 80x87 math co-processors than do computations under the

IEEE format supported by QBasic version 4.0 and later. MAPPER87 can be

compiled under QBasic v 4.5 or BC7.1 using the IEEE floating point math

option. It will run on machines with or without a Math Coprocessor, but it

is very slow without one because of the lousy software emulation of the

80x87 routines which Microsoft has provided. The MAPPER86 version is

strongly recommended if you don't have an 80x87.

Both versions now support all graphics adapters ( CGA, EGA, EGA/Monochrome,

MCGA, VGA, and HERCULES). If you want to use the Hercules mode you must

first load the Microsoft Hercules driver MSHERC.COM which I have included

in the package courtesy of Microsoft.

Version 7.0 differs from earlier releases in that it has an extended DX

Prefix list, uses a more detailed world map data file with a different file

format than previous versions, and is somewhat more 'user friendly' about

accepting input data. It also provides a facility for printing out a 24

hour summary of propagation conditions for a given path, together with a

plot of the NUF over that period. An 80 column printer which recognizes a

Form Feed command is needed for this option.

Most importantly, Version 7 has added an entirely new method for

propagation predictions based upon MAXIMUF 3.0, and allows the user to

select whether the Maximum Useable Frequency ( MUF ) uses the ionospheric

hops nearest to the transmitter and the receiver for its determination, or

takes the worst MUF among all the hops along a multi-hop path.

The author does not intend to support this software, but is simply

releasing it into the public domain for non-commercial use by HAMS who know

what they are doing with their computer. The BASIC source code is included

for that reason.

The following files are a part of the MAPPER system of files:

1- MAPPER86/87.BAS  The Program Source Code Specific for Either

Q-BASIC v4.5  or BASIC 7.1 PDS

2- MAPPER1.INC      An 'INCLUDE' File Containing the beginning

source code common to both versions.

3- MAPPER2.INC      An 'INCLUDE' File containing most of the

mapping source code common to both versions.

4- MAP-SUBS.BAS     Source code for the propagation prediction

calculations.

5- MAP-COM.INC      An 'INCLUDE' File containing some COMMON and DIM

statements needed by both code modules.

6- MAP-DEF.INC      An 'INCLUDE' file containing function definitions

needed by both modules.

7- MAPPER86/87.EXE  the Q-BASIC 4.5 or BASIC 7.1 executable code.

8- MAPPER.ATL       An Atlas of DXCC countries and their

latitudes, longitudes, and DX prefix.

This is an ASCII file ,sorted by prefix,

which can be added onto by the user with

a word processor or text editor.

9- PREFIX.ATL       An Extended Atlas of country prefixes and

their latitudes, longitudes. This file uses

the official internationally designated

prefix ranges to identify the country and

permits identification of some of the more

exotic DX call signs which have gotten more

common. This is an ASCII file ,sorted by

prefix, which can be added onto by the user

with a word processor or text editor.

10- MAPPER.DEF       A short file used to store some default

values used by the program.

11- MAPPER.DOC       This instruction file.

12- WORLD.All        A large file containing the lat/lon of the

world's borders. This is used to recompute

the map whenever the map center point is

redefined.

13- MSHERC.COM       A Microsoft program to permit the use of a

Hercules video adapter with MAPPER87. It must

be executed prior to executing MAPPER87. This

option is not available for MAPPER86 because

that version is compiled under Quick Basic

Version 3 which did not support Hercules

video.

14- MAPPER1/2.SCR    Large files which are used to store and

retrieve the screen data for a world map

centered on the user's location. These files

are not supplied in the .ARC files but

created by the program when it is first used

and they are rewritten whenever the user

changes the 'Home' location. ( It is

interesting to view the world as seen from DX

locations.) This data is read into a large

array in memory to permit rapid redrawing of

the map as the sun moves. The file is written

into the integer array NSTORE by BLOAD and

BSAVE commands. Because the program uses a

lot of memory to speed things up, this array

is DYNAMIC and does not generally reside in

the normal BASIC data segment.

15- SEA-MAP.DAT       A data file which contains a raster map of

the world's oceans in a bit-map format in 1

degree intervals of latitude and longitude.

This file is read into an array when the

program is started. It is used to determine

whether propagation 'bounce' points are on

land or sea in order to compute the correct

terrain reflection loss for that hop.

To accommodate different uses I have broken the file set into three ARC

distribution files:

1- MAPR86.ARC     is intended for those who only want to

operate MAPPER86 and who don't have math

coprocessors. It contains the following

files:

MAPPER.ATL

MAPPER.DEF

MAPPER.DOC

MAPPER86.EXE

MSHERC.COM

PREFIX.ATL

WORLD.ALL

SEA-MAP.DAT

2- MAPR87.ARC     is intended for those who only want to

operate MAPPER87 and who do have math

coprocessors. It contains the following

files:

MAPPER.ATL

MAPPER.DEF

MAPPER.DOC

MAPPER87.EXE

MSHERC.COM

PREFIX.ATL

WORLD.ALL

SEA-MAP.DAT

3- MAPPER.ARC     is intended for those who are interested in

studying, modifying, customizing, or

recompiling the MAPPER program to suit their

own purposes. It contains the source code

files for the MAPPER system:

MAPPER.DOC

MAPPER1.INC

MAPPER2.INC

MAPPER86.BAS

MAPPER87.BAS

MAP-SUBS.BAS

MAP-COM.INC

MAP-DEF.INC

The author encourages the distribution of this program to anyone who might

find it useful, or onto bulletin boards which deal with HAM radio or

shortwave listening. Please make sure to include ALL the required files for

one of the ARC distribution sets described above in any uploading however.

INSTRUCTIONS:

To run this program simply enter 'MAPPER86' or 'MAPPER87'. After an initial

message explaining the program is displayed, and you press any key to

proceed, you will be presented with a 8-choice initial menu which allows

you to change the default values of:

1- Sunspot number

2- Home QTH latitude and longitude

3- The time interval at which the daylight/darkness areas are

redrawn.

4- Transmitter power output.

5- Minimum elevation angle for propagation predictions.

6- Graphics Mode

7- Method for Propagation prediction.

8- Change or View the Set of Frequency Bands, and the antenna

heights, gains, and polarizations for each band. This will drop

you into a submenu which permits your custom definition of up to

10 Bands of interest, together with antenna parameters for both

home and DX stations. SWLs may want to use this feature to

change the frequency set to their favorite HF Broadcast bands.

Pressing the appropriate number key will put up prompts to allow changes to

these variables, and will return you to this menu after a change is

entered. (Important: Make sure you select the proper display type for your

machine at this menu when you first run it. This package will have a file

of my own default values for the parameters in this menu, and they are

unlikely to be the correct values for your machine.)

After you have made the appropriate changes pressing <ENTER> at the menu

prompt will store these new values in the MAPPER.DEF file and proceed on to

the map drawing.

The map drawing will first put up a blank box, then calculate the

position of the sun at the time and date currently held by the DOS, draw

the solar terminator ( it looks like a satellite ground trace ), and color

the sun-lit portions of the world in red. It will then go on to draw the

country boundaries, color the oceans blue, and color the USA green. The

land areas which are in darkness will show up in black. The program may

miss coloring part of the oceans blue because of the position of the solar

terminator blocking the PAINT commands used in the subroutine at line 8000

... . I got tired of adding ocean points to this routine to cover all the

contingencies.

After the map is drawn a Menu bar will appear on the bottom line of

the screen, with 9 choices selected by the function keys:

f1-   Select DX Prefix.

You will be prompted to enter a DX prefix ( eg. ZS for

S. Africa ). The program will then search the DX atlas

for an occurrence of this prefix. It will display all

of the entries ( ie ZS1,ZS2,... ) that contain this

string and ask you to select one. If the number of

candidate entries is large you can simply <ENTER> to

continue the search.

When a selection is made the program will fetch

the location of the prefix, make a propagation

prediction for the currently selected date and time,

draw the path from your QTH to that location, and

display sundry useful information at the top of the

screen. On the left of the screen the signal strength

predictions will be shown for all Ham bands for which

the frequency is more than 80% of the lowest usable

frequency (LUF), less than 120% of the maximum usable

frequency (MUF), and whose predicted signal strength

is higher than -20dB relative to 0.5 microvolts into a

50 ohm receiver input.

For each band which meets these criteria I display the

frequency, the absorption losses (Labs), the ground

reflection losses (Lref), the transmit and receive

antenna losses due to ground reflection ( Ltx, Lrx ),

and signal strength ( Prcv ). The losses are given in

dB. The signal strength is given in dB relative to 0.5

microvolts into 50 ohms. This is about an S-1 signal

level for most Ham receivers, and S-9 usually

corresponds to 50 microvolts at the antenna terminals.

Thus a signal strength of 40 dB would correspond to a

S-9 prediction.

The user should be cautioned that the signal strength

predictions are very uncertain. I only calculate paths

via the F2 layer, and there a great number of other

propagation modes possible which may be better or

worse than the pure F2 mode. Don't blame the author if

this program says that you can't possibly work

Australia when you are hearing VK's with S-9 signals.

These are only crude indications of the likelihood of

a path being open.

f2-Specify Country Name.

Same as option 1 except that you are prompted for the

name of a country ( or a subset of that name such as

'AUS', which will find both Austria and Australia ).

The search process is insensitive to the case of your

entry.

f3-Specify Lat/Lon.

Same as the above option except you will be prompted

to enter the latitude and longitude of the desired DX

location.

f4-Select Date/Time.

Prompts for a new date and time to be used to

calculate solar terminator and make propagation

predictions. <ENTER> responses to either of these

prompts will fetch the current DOS value of date or

time. This option will also turn off the automatic

updating of the terminator and map redraw.

f5-Use Real Time.

This option will reset the above option to the real

(DOS) date and time and enable the auto update

feature.

f6-Toggle Between Short and Long Path.

This toggles the path from the long path option back

to short path and vice-versa

f7-Change Parameters.

This returns you to the 7 choice initial menu to

change the various default parameters such as sunspot

number, antenna heights, power, etc.

f8-Toggle Printed Output of 24 hour Propagation Predictions

on or off. When F8 is hit for the first time it 'toggles'

this function on and subsequent hits turn the option on and

off in sequence. When this option is 'on' it prints

out the same information on propagation that is displayed

on the screen, but does it for an entire day at one hour

intervals, and plots a graph of the MUF versus time. The

printing starts when you select a new DX location, not

immediately after the function is toggled on.

f9-Quit.

Exits the program and returns you to DOS.

The program is entirely menu-driven so you don't have to remember

any special commands to use it.

Good luck!!

Instructions for recompiling:

Some of you may wish to customize this program in some way, so I have

included the BASIC source code. If you make changes you will have to

recompile and LINK the code. The following instructions may help you.

If you have a math coprocessor or if you don't care how slow the program

runs then compile using BASIC PDS 7.1 or Quick-Basic 4.0 or later:

Compile and LINK Using the following command lines:

BC MAPPER87/D/X/O/V; <Rtn>

BC MAP-subs/D/X/O;   <Rtn>

LINK MAPPER87+MAP-SUBS; <Rtn>

If you don't have a coprocessor and you want faster execution then compile

using BASIC PDS 7.1 with the /FPa (alternate math package option).

BC MAPPER86/D/X/O/V/FPa; <Rtn>

BC MAP-SUBS/D/X/O/FPa;   <Rtn>

LINK MAPPER86+MAP-SUBS; <Rtn>

This program is totally incompatible with the BASIC Interpreter which IBM

and Microsoft supply with their DOS. (i.e. BASICA of GWBASIC ). There is

not a prayer of making it run under the interpreter.

Some Technical Information:

The propagation calculations are based upon either the MINIMUF 3.5 method

or the MAXIMUF 3.0 method, but with many modifications by this author. I am

indebted to Robert R. Brown, NM7M, for providing me with the MAXIMUF 3.0

method for MUF calculation. I had been trying (unsucessfully) since Version

2 of this program to obtain this method because it has been evaluated as

the most accurate PC-based method in comparison with IONCAP ( The elaborate

Ionospheric model developed by the Institute of Telecommunication Sciences

in Boulder, CO. which is normally used as a standard of comparison). The

method is due originally to Raymond Fricker of the BBC External Services.

Its key element is a mathematical model for the vertical incidence sounding

frequency obtained by noting that the ionosphere mainly depends upon the

solar zenith angle and geomagnetic latitude. He then did a curve-fit of the

the CCIR Report 340 vertical sounding frequency to the function

Fv= A*Cos( B*(T-C) )* Cos ( Z+D))^0.5

where T is the local time, Z is the geomagnetic latitude, and A,B,C,D are

'constants' which depend upon Sunspot number and season of the year. Having

obtained the main dependence in this form, he the went on to derive a set

of 20 'correction' terms to improve the fit. The method also uses a

curve-fit to the virtual height of the F2 layer. The method's equations can

be obtained from examination of the Subroutine CRIT.FREQ in the

MAP-SUBS.BAS source code.

I have made a number of enhancements to the 'standard' propagation

calculations:

1- The 'standard' CCIR method for hand-calculating MUF uses only the

first and last ionospheric bounce points ( the two control point

method ), This method is available in MAPPER. In addition, I have

provided the user with the option to let MAPPER evaluate the MUF at

every hop and select the worst-case condition out of all of them.

This results in more pessimistic MUF and LUF estimates than the

endpoint method because conditions over parts of the total path may

be much different than conditions at only the ends of the path. A

path with a hop near the near the North pole is an important example.

I leave it to the user to try the various methods and see which works

best for you. I would be interested in any results you care to share.

2- I have included the E-layer screening frequency and lowest-

usable frequency ( LUF ) estimation technique given in chapter 4

of 'Amateur Radio Software ', by John Morris GM4ANB , RSGB

Books, Lambda House, Cranborne Road, Potters Bar, Hertfordshire

EN6 3JW. The subroutine starting at line 45000 contains this

calculation.

3-I have used what apparently is the HAM convention for longitudes

being positive-West for the input and output in this program, in

the PROPAGATE subroutine, and in the DX Atlas file MAPPER.ATL.

However, in the internals of the program I immediately reverse

the sign of the longitude. Being an engineer by training I think

in terms of right-handed coordinate systems! Therefore those who

want to modify the program should be wary of the sign of the

longitude being used.

4-Note that I have changed the names of the variables in the

MINIMUF subroutine from the standard values to names which are

descriptive of what the variable actually is.

5-I have augmented the propagation predictions with a signal

strength prediction. For each ionospheric control (bounce) point

an estimate of the D-layer absorption is made according to the

method in chapter 5 of Davies, "Ionospheric Radio Propagation",

NBS monograph 80, 1965. The total D-region absorption is summed

up for all control points. The subroutine at line 46000 contains

this calculation.

6-In addition, a calculation of the loss due to ground reflection is

made for each ground reflection point along the path. At each of

these points the array SEA-MAP is decoded for the reflection point

location and if it is a sea point then the reflection coefficient for

sea water is calculated ( dielectric constant of 80, conductivity of

4 mho/m ). If the reflection point is over land then a dielectric

constant of 4 and conductivity of 0.01 mho/m are used. The Fresnel

reflection coefficients for both horizontal an vertical polarization

are power-averaged to account for the random polarization of the wave

due to Faraday rotation at HF frequencies. I have also modified the

reflection coefficients to account for surface roughness. A RMS

surface roughness of 4m is used on the sea, and 15m for land points.

The subroutine REFLECT at line 41000 of module MAP-SUBS makes the

reflection calculations.

7-To account for the fact that most Hams have fairly low antenna

heights, thus suffer large losses at low elevation angles, I

explicitly calculate the multipath cancellation losses for each

antenna in the subroutine MULTIPATH at line 42000 in MAP-SUBS.BAS.

This is the standard Fresnel reflection calculation without surface

roughness included. Most urban locations will be worse than this

calculation suggests because buildings in the foreground will

actually shadow the low angle line-of-sight and modify the reflection

area. If you know you have poor line-of-sight you should modify the

minimum launch angle setting at the initial menu appropriately.