The WEAK Link

Version 1.2a

(Shareware version)

(c) Copyright 1987-1992  Caranet Corporation and Information Modes

Caranet Corporation

Postal Outlet Box 46001

1725 Kingston Road,

Pickering, Ontario,

Canada.    L1V 6P6

This product may be distributed freely as long as nothing is added

to, altered, or removed from either the disk or the zip file as downloaded

from a Bulletin Board. (The only exception to this is that the Bulletin

Board operators themselves may add a file to the ZIP containing a promotional

message for their Bulletin Board itself.)  This is the shareware version of a

commercial product.  It is fully functional but does not include source code.

A registered copy of this product can be purchased from Caranet Corporation.

PAGE 1

INTRODUCTION

This is a simple master/slave serial link-up program.  Two standard

IBMs or clones can be linked together via RS232 ports and a null modem cable.

COM1 thru COM4 may be used.  The master program is a device driver which must

be included in the CONFIG.SYS file.  It is always resident and always "on".

The slave is a .COM program which, when run, allows the master to use any block

device installed on that slave.   For instance, if the master has 1 floppy as

A: and 1 hard disk as drive C:, then  drive D: can be the slave's drive A:, and

drive E: can be the slave's drive C:, and drive F: can be the slave's RAMDRIVE.

Mapping of the slave's drives to the master can be in any order and can include

or exclude any of the slave's block devices. Mapping is done via a

SHARE:abcd...  parameter on the slave program's command line.  When the slave

program is running the master has complete control of the computer.  This

utility is primarily intended to allow file transfers between two machines with

different or non-removable media.  You could transfer from a portable's 3 1/2

inch drives to a 5 1/4 inch drive on another machine. Or you could transfer an

entire directory of one hard disk to another on a different machine.  Transfer

speed is selectable from 1200 to 115k baud.  Transfer is extremely simple since

the master thinks the slave is a collection of drives.  Standard COPY and DIR

commands work equally well on the master or slave drives.

It is necessary to make or purchase a null modem cable to connect your

computer serial ports together. A diagram is included at the end of this

documentation for making your own cable for 25 pin serial ports. The wiring

is different for 9 pin serial connectors.

NOTE: Some later model PCs use 82C50 UARTs which have a bug.  NEC Multispeed

is one example.  This bug causes The WEAK Link to run sporadically.

A fix is in the works.

_______________

[               ]

[   IMPORTANT   ]

[               ]

[  PLEASE READ  ]

[_______________]

* BE EXTREMELY CAREFULL YOU DO NOT MIX DOS VERSIONS ON HARD DISK SYSTEMS.

DOS 2.x, 3.x, 4.x, AND 5.x  WILL NOT PROPERLY ACCESS EACH OTHERS FORMATTED

HARD DISKS. FILE CORRUPTION COULD RESULT!! DOWNWARD COMPATIBILITY IS

USUALLY SAFE. (ie: A BOOT DISK CREATED WITH DOS 5.x CAN USUALLY SAFELY

ACCESS A HARD DISK FORMATTED WITH DOS 2.x, 3.x, OR 4.x.)

* TEST THE READ INTEGRITY OF REMOTE HARD DISKS.  FOR SAFETY, TRANSFER

A LONG TEXT FILE AS YOUR FIRST TEST AND THEN DO A "TYPE" ON THAT

TEXT FILE TO MAKE SURE TEXT IS COMPLETELY THERE.

* NEVER "WRITE" TO A REMOTE HARD DISK UNTIL YOU ARE TOTALLY SURE OF "READ"

INTEGRITY. (Do a test READ of a file on a drive before you try writing

a file to that drive.)

* NEVER WRITE TO A HARD DISK WITHOUT FIRST

MAKING A BACKUP OF YOUR VALUABLE DATA.

PAGE 2

INSTALLATION

The WEAK Link will work on any IBM PC/XT/AT which has a serial card

installed.  The card must follow the IBM specifications as outlined in the

Technical Reference manual.  If it uses a 8250 or 16450 communications chip

then it is probably ok.  The memory requirements are minimal.  DOS 2.0 or

higher is required.

SIMPLIFIED STEPS:

Connect the serial ports of the two computers together using the null modem

cable.

COPY CONFIG.S                  to the slave's boot disk.

REN  CONFIG.S  CONFIG.SYS      on the slave's boot disk.

COPY NET00000.SYS              to the slave's boot disk.

COPY PS.COM                    to the slave's boot disk.

reboot the slave computer

COPY CONFIG.M                  to the master's boot disk

REN  CONFIG.M  CONFIG.SYS      on the master's boot disk

COPY PM.SYS                    to the master's boot disk.

reboot the master computer

run PS.COM                     on the slave

The master can now use the slave's drives as it's own.  These

directions assume both systems have at least one 5 1/4 inch drive to read

this disk.

TRANSFERING TO OTHER MEDIA:

Included on this disk are two short BASICA programs which can be used

to transfer the slave's startup files via a serial link. TMT.BAS should be run

on the machine which can read this disk.  RCV.BAS should be run on the other

machine, and will have to be typed in at the keyboard.  The program is short

and shouldn't take much time to enter.  Once RCV.BAS has been entered on the

other machine, run it under BASICA or GWBASIC.  Make sure your nul cable is

hooked up.  Then run TMT.BAS under BASICA on the main machine.  TMT.BAS will

transfer (slowly) the three files you will need to set up the other machine as

a slave.  Then you can transfer the rest of this disk mush faster using The

WEAK Link.

If all else fails, other media can be supplied.

PAGE 3

SLAVE INSTALLATION:

The config.sys file must include the line:

device=net00000.sys

The net00000.sys character device is used to setup the internal device

driver list.  If net00000.sys is not properly installed the slave program

cannot find the addresses of the DOS device drivers.  It should be the first

device in the list so should be the last line in the config.sys file.

Programmers....  any 4 byte read of net0 (the device name is net0, the file

name is net00000.sys ) will return the dword pointer to the net0 device

driver.  This dword address can be used to search through the DOS device

driver chain.

Once net00000.sys is installed,  you can run ps.com to put the computer

into the slave mode.  If the communication port you are using is not COM1 then

you must indicate which port you are using with the COMx parameter on the

command line.  If the baud rate you want is not 56k baud then you must indicate

your baud rate on the command line with the /nn parameter.  You can modify the

order slave drives are presented to the master by using the SHARE:xxx

parameter.  These parameters are described below.  When the master/slave link

is no longer needed, just press "q" and the slave will return to normal

operation.

EXAMPLE:

A> PS COM1 /2 SHARE:ACDE

_________________________________________________

MASTER INSTALLATION:

The config.sys file must include the line:

device=pm.sys

When the system reboots, the master is installed and will display its

sign-on message.  Then the master can access the slave whenever the slave is

running ps.com.  Necessary parameters are entered on the config.sys command

line.  Parameters are just as in the slave device with the addition of an @nn

parameter which tells DOS how many devices the slave will support.

EXAMPLE:

DEVICE=PM.SYS COM2 /2 @4

COMBINATION MASTER/SLAVE:

You can combine the master and slave configuration files.  This will

allow you to run the slave program PS.COM on either machine then access that

machine from the other.  Both machines would have the master installed:

DEVICE=NET00000.SYS

DEVICE=PM.SYS

PAGE 4

COMMAND LINE PARAMETERS

COMx            You can use COM1 thru COM4 as the communications port on

either machine.  If this parameter is not given then COM1

is used.  If the port specified does not exist then the

program cannot continue and will abort.

/nnn            This will specify the baud rate divisor.  The "nnn" is any

decimal number from 0 to 199.  The default is "/2" which is 56k

baud.  Both computers must use the same baud rate, so if this

parameter is entered on one machine, the same value must also

be entered on the other.  Some selected values are:

/1      115,200 baud

/2       57,600 baud   <-- default

/3       38,400 baud

/6       19,200 baud

/12       9,600 buad

/16       7,200 baud

/24       4,800 baud

/32       3,600 baud

/48       2,400 baud

/96       1,200 baud

/192        600 baud

SHARE:abc...    The slave can specify which drives the master may control and

in which order they are presented.  "SHARE:ca" would allow the

master to access drives C and A drives only, and in that order.

If the Master had drives A, B, and C installed locally, then

its logical drive D would be the slave's drive C because C was

the first drive in the slave's SHARE: list.  The master's drive

E would be the slave's drive A because A was the second drive

in the SHARE: list.  There can be no space between the colon

and the drive letters.  Up to 16 drive letters can be

specified.  The drive letters can be in any order and can be

any valid block device including ram drives.  The only

limitation is that the device must use 512 byte sectors.  If

SHARE: is not specified then the default used is

"SHARE:abcdefghijklmnop".  This parameter is valid for the

slave only.

@nn             The master needs to know how many drive units to allocate to

the slave.  If this parameter is unused then the master assumes

4 drive units reside on the slave.  In most cases this is just

fine,  but if the master has too many drives of its own, or if

the master must define some more drives after the slave, or if

the slave has more than 4 drives, then the "@nn" parameter can

be used to limit or extend the number of logical drives used up

by the slave.  "nn" can be any decimal number from 1 to 16.

The default is "@4".  This parameter is valid for the master

only.

NOTE:   The command line parameters may be entered in any order and

in either upper or lower case.

PAGE 5

*** CABLE ***

NULL modem cable,  only 5 lines are used but you MUST use all 5:

Computer 1                            Computer 2

----------                            ----------

13*                                    13*

*25                                    *25

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

*-DTR------\  /----------------DTR-----*

GROUND *----------->>  \/  <<-----------------* GROUND

*           /\                         *

DSR *--------------/  \----------------DSR-*

*                                      *

*                                      *

*                                      *

*                                      *

*                                      *

RCV DATA *-----------------\ /------------------* RCV DATA

*              X                       *

TMT DATA *-----------------/ \------------------* TMT DATA

*14                                    *14

1*                                     1*

NOTE: The GROUND is important. Do NOT leave it out.

When using 9 pin serial connectors use pin 2 for RCV DATA, pin 3 for TMT DATA,

pin 4 for DTR, pin 5 for GROUND and pin 6 for DSR.

Page 6

01/20/91

REGISTRATION AND OTHER IMPORTANT INFORMATION

--------------------------------------------

If you find this program to be of value please register your copy for $20.

The WEAK Link I, is not crippled in any way. You also will not see any

repetetive "Beg" screens when you use it. We believe that The WEAK Link I

is as good as or superior to similar products which would cost you $100-$150

and that you will feel it truly is worth the $20.  Your money will help

fund the development of new products which may be even more valuable to

you than this one. We specialize in high quality software at low prices.

When you send us your cheque or money order for $20, a registered copy of

The WEAK LINK II will be sent to you within 24 hours. The WEAK LINK II contains

the registered version of the programs from the shareware package and also

includes all source code for the files within it.

Your $20 cheque or money order should be sent to:

Caranet Corporation

Postal Outlet Box 46001

1725 Kingston Road,

Pickering, Ontario,

Canada.    L1V 6P6

You can call us at 1-416-427-2886. Our hours are from 9:30 AM to 5:30 PM,

Monday to Friday.  You can also receive evening or weekend technical support

but this should be prearranged during our regular hours if possible.

You'll find the support you will get far exceeds much of the so-called

support you get from some of the major software companies. Our product

line is limited so we know how to support the products we have.

Please note that in order to keep our prices low, we do NOT return long

distance technical support calls. However, we do maintain technical support

via bulletin board access. See details at the end of this file.

WEAK LINK II PRICES:

The WEAK LINK II programs and source on 5 1/4 inch media: ....$ 20.00

The WEAK LINK II programs and source on 3 1/2 inch media add:..+ 5.00

25' Modular cable and two DB9 or DB25 adaptors (Specify): ....$ 25.00

Shipping in Canada: ..........................................$  6.00

C.O.D. charges if shipped that way - add to shipping: ........  +4.00

Although we recommend our cable, you are not obligated to purchase it. You

may purchase only the registered version of the program if you like. Please

remember there is GST of 7% on this for almost everyone in Canada and there

is Provincial Sales Tax of 8% for those who live in Ontario. The taxes are

NOT included in the above prices. Please add them where required.

--------------------------------------------------------------------

Page 7

The WEAK LINK III

(Parallel)

For those who want faster file transfers, The WEAK Link is also available

in a PARALLEL version which we call The WEAK Link III. In most cases,

transfer speeds are considerably faster when using parallel ports. When

using two fast "AT" class computers or 386 machines it can be up to 3 times

faster. (If you are using original PC's running at 4.77 Mhz, the Parallel

ports will actually be SLOWER than using Serial ports.)

The price for The WEAK Link III (Parallel) is the same as for the serial

version. It also includes the source code for the programs. If you wish to

purchase BOTH the Serial and Parallel versions, call us for pricing. The

Parallel version uses a special 11 wire cable. A diagram for making your own

is included with the package. Ready made Parallel cables are not available

from Caranet at this time. (We hope to have a "ready-made" solution to offer

in the near future.)

The WEAK Link III (Parallel) is not available in a shareware version.

________________________________________________________________________

The ALSO-LAN

(Zero-Slot Network)

The WEAK Link was actually a spinoff product generated while working

on a far superior program called:

��������������������������Ŀ

�                          �

�  **** The ALSO-LAN ****  �

�                          �

����������������������������

"I had long wanted a network for my two computers but the cost of even the

cheapest setup was prohibitive.  I decided to write my own network program

using the RS232 cards I already had. Since networking software was relatively

complex, I thought it would be best to tackle a simpler project first.  The

WEAK Link was a "study" to make sure I understood DOS block device drivers. It

took me about 2-3 weeks.  I then used some of the concepts I had learned to

develop The ALSO-LAN.  It was far more complex because I wanted to be able

to use both computers at any time, regardless of the activity on the other.

This initial project took about 3 months.  But I now have a two or three

computer network on which I can share any device - including disks and printers

- at any time.  Its speed is equivalent to The WEAK Link.  I am also working on

a larger network which will support even more machines, possible 20-30. It

will support computer to computer links via serial cards, parallel cards, or a

cheap network card of my own design.  I also intend to support modem links."

Don Jindra

So said the programmer almost 3 years ago. Since that time the Also-LAN has

undergone tremendous improvements and upgrades. It is possible to connect 3

computers together quite easily with it. In fact, using specialized serial port

boards it is possible (although not really practical) to connect 5 or more

computers together. It has many utilities added to it and will work with all

versions of MS/PC-DOS from 2.1 to 5.0 including Compaq DOS 3.31.

Page 8

One of the most beneficial features of The ALSO-LAN is its extremely low

RAM requirements. In a two computer installation, the ALSO-LAN requires

only an average of 15K of RAM on each computer!

The Also-LAN works strictly with serial communications ports. It operates

completely in the background and both (or all 3 if you connect 3) computers

remain usable while the network is in operation.

We certainly hope you will register your copy of The WEAK Link. But if you

like the WEAK Link I, and are considering registering it, you may want to

buy The ALSO-LAN instead as it renders The WEAK Link obsolete.

Of course, if you want source code then The WEAK Link will provide you with

a good example of how to write DOS device drivers.  The source code for

The ALSO-LAN will NOT be made available. The ALSO-LAN is not available

in a shareware version either.

If you are interested in more information on The Also-LAN, a fully detailed

brochure is available for the asking. For pricing information on The

ALSO-LAN, see further on in this file.

------------------------------------------------------------------------------

03/01/92

ANNOUNCING.......

The evolution continues. As mentioned above, the plan when developing

The ALSO-LAN was to come up with a network system that would use Serial

ports, Parallel ports, or perhaps inexpensive network cards. Well, the

time finally arrived!

The latest offering from Information Modes and Caranet Corporation has

been released. It is new and it is different as far as networks go.

It is the:

��������������������������Ŀ

�                          �

� **** Little Big LAN **** �

�                          �

����������������������������

Little Big LAN (LBL) goes well beyond even the features of The Also-LAN and

is what many would consider to be a "True" network.

The Little Big LAN is a master of interconnection methods.

With LBL you can interconnect your computers using Serial ports, Parallel ports

or standard off-the-shelf Arcnet cards. And you can MIX the interconnect

methods within the same network! It is an extremely FLEXIBLE Peer-To-Peer

network operating system for use on IBM and 100% compatible computers.

When properly installed, the things you should be able to share include

disk drives (both hard and floppy), printers, software, databases,

plotters, and CD-ROMs. If you have another manufacturer's network then

the Little Big LAN should be able to use those network drives as well.

Page 9

With Little Big LAN, every drive letter is available to you to refer to

any drive on the network. All drive letters can be redirected and this

can be done "on-the-fly" without having to reboot the system after each

time you do a redirection.

Little Big LAN offers flexible growth. You can start with a two computer

network linked through serial ports. This will cost a fraction of what

most other networks cost for the software and even less for the cabling

costs. Thus, for less than you might spend on a single EtherNet adapter

card for another network, you can have your first LAN up and running.

Next month if you go out and buy another computer you can just link it

into the first two machines through another serial port and cable. There

is no additional software cost involved. Your purchase of The Little Big

LAN Network Operating System software covers one network. Not just one node.

If you later decide that the serial port connections are too slow you

can switch to using parallel ports instead. Parallel port connections

are about 2.5 to 3 times faster than serial port connections if you are

using AT class computers. If you are really hungry for speed, you can

upgrade to standard, non-proprietary, off-the-shelf Arcnet cards which

offer 5 to 10 times the performance of serial ports. Arcnet Cards are

typically less expensive than EtherNet cards and still give good

performance.  Again, no matter which interconnection scheme you choose,

Little Big LAN supports it all at the same software cost. You don't have

to come to us to buy a new driver, or a higher performance version of

our software. We support all connections methods for the one low cost.

ONE SIZE FITS ALL!

Not only can you choose different methods to link machines on the

network, you can even mix those methods on the same network. For

instance, some of your machines could be linked via Arcnet cards, and

others could be linked into the network via serial ports and/or paralled

ports. This kind of flexibility is offered by few other LAN products,

and if offered is usually a high priced option. It's standard on ours.

You should be aware that there are a few things you may not be able to

share. Modems present a problem - not because it is theoretically

impossible to share modems, but because in practice most commonly

available communications software such as TELIX*, PROCOMM*, and QMODEM*

read and write to the hardware directly. It is therefore not possible

for any network to trap those hardware requests and redirect them.

Sharing MICE or Digitizers poses the same sort of problem. Please note

that we are currently working on developement of our own communications

software package which will allow full background sharing of modems on

the Little Big LAN. When it is completed it will be included with Little

Big LAN at no extra cost. Inexpensive upgrades will be offered to

previous owners of LBL.

Also note that if you have copy protected software, you will probably be

forced to buy a legitimate copy for any computer that software is to run

on. This is especially true if the copy protection is hardware based -

such as via some gizmo that attaches to your printer port. It is not

possible - nor is it our desire - that the network should defeat this

method of copy protection.

Page 10

LBL supports DOS file and record locking for true multi-user use of network

compatible databases and other software.

LBL works under MS/PC-DOS* versions 2.1 and above. (3.1 or above is

preferred.) It also works with DR-DOS* version 3.41 and up. With LBL it

is possible (although not recommended) to mix the versions of DOS within

the network.

LBL requires approximately 35K of RAM on each computer. Most of the network

files can be loaded into high memory using appropriate software such as

QEMM. (Not included with Little Big LAN.)

PRICING FOR THIS AND OTHER CARANET PRODUCTS:

1)  The Also-LAN.................................................. $40

(This package is similar to the U.S. product but contains expanded

documentation and both 5.25" and 3.5" diskettes.)

2)  Cabling for The Also-LAN - See the cable prices for The WEAK Link.

3)  Little Big LAN - Documentation and Software: ................. $99

4)  Serial cabling for LBL - See the cable prices for The WEAK Link.

5)  Mentor Accountant - Accounting software for the non-accountant.

Includes full GST tracking: ............. $169

6)  Menu Express - A colour high resolution graphics based menu

system for those who don't like the C:> prompt

but don't want a full GUI either: ............  $59

7)  The UNIX Tutor - Menu driven UNIX tutorial package that runs

under DOS - includes UNIX simulations: ..... $109

Note: Caranet carries a full line of serial connection products for The

WEAK Link, The Also-LAN, and The Little Big LAN. Serial cables of any

length up to 100 feet are available along with modular adaptors for

connection to the computer serial ports. For the Little Big LAN only, we

carry Arcnet cards in four different flavours. Eight bit and sixteen bit

versions of each of Coax/Star topology boards and also combination

Coax/Twisted Pair/Star/Bus topology boards. A full selection of Passive

and Active hubs, Coax cable, Twisted Pair cable and connectors is

available. Call for current pricing.

--------------------------------------------------------------------

BULLETIN BOARD SUPPORT:

For your convenience we offer support by a number of channels through

dial-up public access Bulletin Board Systems. This access is as follows:

1. Our home base support is through The Midnight Express BBS. This is a

Toronto area BBS. By "home base" we mean that this is where you will

have access to upload and download of files, program updates, bug

fixes, and any demonstration software we have available. There is

also a message base for Caranet on this board for technical support

questions and answers. The message conference on Midnight Express is

Page 11

not echoed so you must call the board directly for access. The

telephone number is: (416) 650-5411. When you log on, you will be

presented with a questionaire to gain access to the board. After

filling this out you will be then offered the option of requesting

access to the Caranet Conference. After answering the questions you

should have immediate access to the Caranet Conference.

2. We have a "Technical Support ONLY" conference on the ECHONET netmail

system. If you log on to any bulletin board that carries ECHONET

conferences you should have access to a conference called "ALSOLAN".

If it is not available on your local board ask your Sysop if he/she

would carry the conference for you. As this is a Tech Support Only

conference, only a message base is available through ECHONET. Files

and upgrades are not available. You will also not normally receive

news of new product releases via this conference.

3. Finally, we monitor the NANET "Vendor Support Conference" on a regular

basis. NANET conferences are based through the very large Canada

Remote Systems bulletin board system in Mississauga, Ontario. NANET

conferences are "echoed" to dozens of bulletin boards across Canada

and also into the United States. Again, if your local BBS Sysop

carries NANET conferences but does not carry the Vendor Support

conference, please ask them to. If you wish you can also call Canada

Remote Systems directly at: 629-7000, 629-7044, 629-7020, or their

demo line at 629-9263. All numbers are in the 416 area code. This

will give access to over 60 lines into CRS. Please note that a

charge applies for full access to Canada Remote Systems.

NOTE: Caranet Corporation is not affiliated with Canada Remote Systems.

__________________________________________________________________________

[                                                                          ]

[                         European customers                               ]

[                                                                          ]

[    As of 3/3/88, the registered version of The WEAK Link and German      ]

[    language versions of The Also-LAN (Called Kirschbaum Link) and the    ]

[    Little Big LAN is available from:                                     ]

[                                                                          ]

[                      Kirschbaum Software GmbH                            ]

[               Kronau 15, d-8091 Emmering, West Germany                   ]

[             Tel. (0 80 67) 12 20     Fax: (0 80 67) 10 53                ]

[      ------------------------------------------------------------        ]

[                                                                          ]

[  As of 5/06/91,                                                          ]

[                                                                          ]

[  In the U.K., The Also-LAN, WEAK Link and LBL are available from:        ]

[                                                                          ]

[                      E.Q. Consulting                                     ]

[                      New Gilston, Leven Fife,                            ]

[                      Scotland, U.K.  KY8 5TF                             ]

[                      Tel. (0334) 84248                                   ]

[__________________________________________________________________________]

Prices are subject to change without notice. *All trademarks mentioned

are the property of their respective owners. WEAK Link, Also-LAN, Little

Big LAN, and Menu Express are Canadian Registered Trademarks of Caranet

Corporation. WEAK Link and Little Big LAN are U.S. Registered Trademarks

of Information Modes.