1.   LSYSTEM - Overview

The LSYSTEM digital logic simulation software package is  a  CAD  tool  which

allows users to (1) describe the  composition  and  structure  of  a  digital

system comprising combinational logic elements, sequential logic elements and

memory devices, (2) verify the system's logic function, and (3) simulate  and

display the timing diagram  for  specified  logic  signals.   LSYSTEM  allows

complete flexibility in  the  use  and  generation  of  logic  element  model

libraries.   Logic elements can be modelled as gate level primitives or at  a

higher functional level.   Gate and functional level  models  can  be  freely

mixed in both the model libraries and logic network simulations.

- Logic networks are described in a form similar to SPICE.

- Node names can be alphanumeric up to 16 characters long.

- 10k Maximum nodes.

- Maximum logic elements is dependent on system memory.

- 500 maximum user defined gate and functional primitives.

- 500 user defined macros.

- 256 maximum internal states for sequential primitives.

- Comprehensive default primitive library supplied.

- Range of TTL and CMOS macros supplied.

- Zero propagation delays allowed with gate and functional elements.

- Resolution of 1nS during simulation.

- Primitive element delays may be set from 0 to 4S.

- Primitive element strengths can be set from 0 to 100%.

- Macro nesting up to 20 levels deep.

- Unlimited number of include files.

- Include file nesting to 20 levels deep.

- Full network description syntax checking.

- Rigorous declaration checking for correct number of nodes etc..

- Errors reported in a user friendly manner.

- Fast and accurate simulation cycle.

- Flexible digital waveform post-simulation graphics processor.

PLEASE NOTE the LSYSTEM specification presented above referees to the

Production version of the software.  Files LS1.DOC, LS2.DOC and LS3.DOC

describe the production software - the SHAREWARE version of LSYSTEM is a

full working version of the package BUT is restricted in the size of logic

network it can simulate.  SEE file README.1 for the SHAREWARE details.

2.   Specification of the computer system required to run LSYSTEM

Computer:                IBM PC/XT/AT family.

Memory:                  640k ram required.

Floppy disk:             at least two 5.25" DSDD 360k capacity.

Hard disk:               optional.

Numeric coprocessor:     not required.

Printer:                 9 or 24 pin dot matrix.

Operating system:        MS/PC DOS 2.0 upwards.

Mouse:                   optional.

Graphics adapters, one of the following or 100% clones:-

IBM Colour Graphics Adapter (CGA).

IBM Enhanced Graphics Adapter (EGA).

IBM Video Graphics ARRAY (VGA).

Hercules Monochrome Graphics Adapter.

3.   Installing LSYSTEM

Ensure that the CONFIG.SYS file on your boot disk ( whether hard or floppy )

contains the lines :-

FILES     = 20

BUFFERS   = 20

Copy all files from the set of LSYSTEM disks into your chosen directory.  The

LSYSTEM software is now ready to be used for digital  logic  simulation.   In

this publication it is  assumed  that  the  LSYSTEM  software  resides  in  a

directory called LSYSTEM on the C drive.    If  you  are  using  floppy  disk

drives see Appendix A for installation instructions.

4.   Using LSYSTEM

4.1  The Simulation cycle

Three programs are used to complete the simulation cycle:-

(1) COMPILE.EXE, (2) SIMULATE.EXE and (3) WAVEFORM.EXE.

To simulate and study the performance of  a  digital  logic  network  over  a

specified time period, several steps are required, these are:-

- Describe the structure and logic elements which form the  digital  network,

specify all input signals and output test points, in file FILE.NWK using a

suitable NON-DOCUMENT text editor.

- Compile the network description using COMPILE.EXE.

- Simulate the network using SIMULATE.EXE.

- View the logic network signal waveforms using WAVEFORM.EXE.

- Inspect tabular results in file FILE.RLT.

Various files are required (or  generated)  at each  stage,  they  are  as

follows, where FILE is the logic network name chosen:-

COMPILE.EXE        FILE.NWK --+   FILE.INF --   Information

I   FILE.EXP --   Macro expansion

I   FILE.LST --+

I   FILE.MOD   I- DATA files

I   FILE.TRA   I

I   FILE.NAM---+

+---------------- Network description

SIMULATE.EXE       FILE.FNC --+

FILE.MOD   I

FILE.TRA   I

FILE.NAM   I---------------- Data

I

WAVEFORM.EXE       FILE.GRA   I

FILE.NAM --+

INPUT FILES    OUTPUT FILES

Fig. 1 outlines the complete simulation sequence and the relationship between

the input and output files.

SEE PRINTED USER GUIDE FOR DIAGRAM

A minimum simulation cycle, assuming a digital network description  had  been

generated, using for example the PCWRITE word processor,  in  file  test.NWK,

would proceed as follows ( ignore  the  DOS  prompt,  this  is  included  for

completeness ):-

C:\LSYSTEM> compile test <ENTER>

- Correct any errors ( using a text editor or word processor ) and

repeat.

C:\LSYSTEM> simulate test <ENTER>

- Inspect the "test.RLT" file of tabular output results.

- May stop the simulation cycle here if the tabular data is sufficient.

C:\LSYSTEM> waveform test <ENTER>

- Inspect graphical output of trace data.

- Make modifications to the network description file "test.NWK".

- Repeat simulation cycle from the compilation stage or finish.

4.2  Compiler options

Once a network description file ( for example file.NWK )  has  been  created,

the compiler can be invoked from the DOS command line by entering:-

C:\LSYSTEM> compile file[.NWK] [switches......] <ENTER>

where the switches and the file extension  ".NWK"  are  optional  parameters.

The currently available switches are:-

-l Generate a listing file, reporting any errors, (output file.LST).

-e Generate an expanded file, macros fully expanded, (output file.EXP)

-h Display a help screen, output highlights basic syntax rules.

Entering

C:\LSYSTEM> compile <ENTER>

alone, will display the basic invocation instructions on the screen.

4.3  Simulator options

Once the digital network description file has been compiled, and found to  be

error free, the simulator is invoked from the DOS command line by entering:-

C:\LSYSTEM> simulate file <ENTER>

No command line switches are available.

4.4  Waveform options

After simulation, the digital network simulation  results  can  be  displayed

graphically from the DOS command line by entering:-

C:\LSYSTEM> waveform file [switches......] <ENTER>

where  the  switches  are  optional  parameters.    All   graphical   display

parameters,  such  as  window  sizing,  are  calculated,  from  automatically

detected graphics adapter information, if no switches are used to  specify  a

given display adapter.     The  waveform program also assumes that a mouse  (

Microsoft mouse or a 100%  compatible  clone  )  is  not  installed  on  your

computer system.   The available switches are:-

-CGA_BL   Set graphics to CGA 320 x 200 black and white mode.

-CGA_C    Set graphics to CGA 320 x 200 colour mode.

-CGA_BH   Set graphics to CGA 640 x 200 black and white mode.

-EGA_C    Set graphics to EGA 640 x 350 colour mode.

-EGA_B    Set graphics to EGA 640 x 350 black and white mode.

-VGA_C    Set graphics to VGA 640 x 480 colour mode.

-VGA_B    Set graphics to VGA 640 x 480 black and white mode.

-HERC     Set graphics to Hercules  black and white mode.

-MOUSE    Activate MOUSE.  Must only be used if a MOUSE driver is

installed.

Note all the graphics mode change switches can be interactively operated from

the keyboard when the waveform graphics post processor program is running.

4.5  LSYSTEM batch files LS.BAT and LSM.BAT

Two batch files are provided on the LSYSTEM master disks.  These batch  files

automate the simulation cycle.  Batch file LS.BAT assumes that your  computer

does NOT have a mouse installed.  The reverse is true for batch file LSM.BAT.

The complete LSYSTEM cycle can be  invoked  from  the  DOS  command  line  by

entering:-

C:\LSYSTEM> LS  file <ENTER>     or

C:\LSYSTEM> LSM file <ENTER>     if you have a mouse installed.

5.   Creating a digital logic network description file

5.1  The LSYSTEM logic network description language

The LSYSTEM logic network description  language  is  modelled  on  the  SPICE

language and has a similar format.   The minimum  information  needed  to  be

able to simulate the function and timing of a digital  system  is  a  set  of

statements which:-

- Define the logic network topology

- Give the the logic element types and parameters

- Describe the primitive element models and MACRO structures

- Define the signals driving the logic system

- Specify what signals to monitor during the simulation

- Control the simulation task

Only one statement per line is allowed by LSYSTEM

The LAST statement in the logic network  description  file  must  be  a  .END

entry; NOTE NO space between the . and the E character.

Any statement that begins with a * is assumed by  LSYSTEM  to  be  a  COMMENT

statement.

Statements may be entered in either lower or upper case letters, and indeed a

mixture of both forms is  allowed,  but  please  note  LSYSTEM  converts  all

letters to upper case so names entered, for example, as GATE1, gate1 or GaTe1

are identical.

Logic element DELAY or RISE or FALL times  must  be  entered  as  an  INTEGER

number of nano seconds.

Logic element STRENGTH values or HIGH  or  LOW  values  must  be  entered  as

INTEGER percentages in the range 0% to 100%.

The logic network description is a nodal representation of the digital system

to be simulated.   Each connection point between elements, these  are  called

nodes, is allocated a unique name ( up to 16 characters long ).    The  total

network is described in terms of the functional logic elements, from which it

is built, and how these elements interconnect.   The  interconnecting  points

or nodes represent the functional elements input/output  pins,  for  example,

the following network has 4 nodes: A, B, C and D:-

+-----+         +------+

I  &  I         I   1  I

A  ----I     I    C    I      I

I     I---------I      I-------- D

B  ----I     I         I      I

I     I         I      I

+-----+         +------+

This network would be described by saying that the two  input  AND  gate  has

inputs A and B, with output C connected to the  input  of  the  buffer  gate.

The output of the buffer gate is D.

5.2  An example simulation

At this stage in the description  of  the  LSYSTEM  CAD  package  an  example

simulation is introduced.   The example has been chosen to illustrate a range

of basic features provided by the software.

5.2.1     A synchronous BCD mod 16 counter

SEE PRINTED USER GUIDE FOR DIAGRAM

5.2.2     The network description file - BCD16.NWK

* Example 1 --- SYNCHRONOUS BCD MOD 16 COUNTER

*               This example illustrates the "FILE.NWK" structure

*               and a range of basic features provided by the

*               LSYSTEM CAD software.

*

.LIB DEFAULT.LIB ; Load the SHAREWARE default logic primitive library.

*

GEN1 CLOCK  CLK0 PERIOD(500)  ; Periodic clock generator.

GEN2 PRESET CLK1              ; PRESET set to logic 1.

GEN3 CLEAR  CLK0 300          ; CLEAR changes from logic 0 to 1 at 300nS.

GEN4 COUNT  CLK0 600          ; COUNT changes from logic 0 to 1 at 600nS.

GEN5 J1K1   CLK1              ; J1K1 input set to logic 1.

*

.macro jkff_cp_ne j clock k clear qn q preset ; JKFF negative edge triggered

g1  clock clockn INV (delay=3)            ; MACRO.

ic1 clockn clear preset j k q qn jkff_cp_pe

.endm

*

G1 CLOCK COUNT CLK AND2 (DELAY=6)                 ; Start of logic network

IC1 J1K1 CLK J1K1 CLEAR QN1 Q1 PRESET jkff_cp_ne  ; description.

G2  Q1 QN4 J2K2 AND2 (DELAY=6)

IC2 J2K2 CLK J2K2 CLEAR QN2 Q2 PRESET jkff_cp_ne

G3  J2K2 Q2 J3K3 AND2 (DELAY=6)

IC3 J3K3 CLK J3K3 CLEAR QN3 Q3 PRESET jkff_cp_ne

G4 J3K3 Q3 J4 AND2 (DELAY=6)

IC4 J4 CLK Q1 CLEAR QN4 Q4 PRESET jkff_cp_ne

*

.TIME 6500 6500            ; Simulation duration.

*

.PRINT PRESET CLEAR COUNT CLOCK Q1 Q2 Q3 Q4 J1K1 J2K2 J3K3 ; Trace node list

*

.END

5.2.3     File BCD16.LST

***************************** LSYSTEM V2.0 ***************************

Logic network listing for compilation of BCD16.NWK

0001: * Example 1 --- SYNCHRONOUS BCD MOD 16 COUNTER

0002: *               This example illustrates the "FILE.NWK" structure

0003: *               and a range of basic features provided by the

0004: *               LSYSTEM CAD software.

0005: *

0006: .LIB DEFAULT.LIB;

0007: *

0008: GEN1 CLOCK  CLK0 PERIOD(500);

0009: GEN2 PRESET CLK1;

0010: GEN3 CLEAR  CLK0 300;

0011: GEN4 COUNT  CLK0 600;

0012: GEN5 J1K1   CLK1;

0013: *

0014: .macro jkff_cp_ne j clock k clear qn q preset;

0015:     g1  clock clockn INV (delay=3);

0016:     ic1 clockn clear preset j k q qn jkff_cp_pe

0017: .endm

0018: *

0019: G1 CLOCK COUNT CLK AND2 (DELAY=6);

0020: IC1 J1K1 CLK J1K1 CLEAR QN1 Q1 PRESET jkff_cp_ne;

0021: G2  Q1 QN4 J2K2 AND2 (DELAY=6)

0022: IC2 J2K2 CLK J2K2 CLEAR QN2 Q2 PRESET jkff_cp_ne

0023: G3  J2K2 Q2 J3K3 AND2 (DELAY=6)

0024: IC3 J3K3 CLK J3K3 CLEAR QN3 Q3 PRESET jkff_cp_ne

0025: G4 J3K3 Q3 J4 AND2 (DELAY=6)

0026: IC4 J4 CLK Q1 CLEAR QN4 Q4 PRESET jkff_cp_ne

0027: *

0028: .TIME 6500 6500;

0029: *

0030: .PRINT PRESET CLEAR COUNT CLOCK Q1 Q2 Q3 Q4 J1K1 J2K2 J3K3;

0031: *

0032: .END

***** NODE and MACRO related warnings *****

5.2.4     File BCD16.EXP

***************************** LSYSTEM V2.0 ***************************

* Expansion listing for compilation of BCD16.NWK

.LIB DEFAULT.LIB;

GEN1 CLOCK  CLK0 PERIOD(500);

GEN2 PRESET CLK1;

GEN3 CLEAR  CLK0 300;

GEN4 COUNT  CLK0 600;

GEN5 J1K1   CLK1;

*-------------------- .macro jkff_cp_ne j clock k clear qn q preset;

*--------------------     g1  clock clockn INV (delay=3);

*--------------------     ic1 clockn clear preset j k q qn jkff_cp_pe

*-------------------- .endm

G1 CLOCK COUNT CLK AND2 (DELAY=6);

*IC1 J1K1 CLK J1K1 CLEAR QN1 Q1 PRESET jkff_cp_ne;

G1_0  CLK $1_CLOCKN INV DELAY 3

IC1_1  $1_CLOCKN CLEAR PRESET J1K1 J1K1 Q1 QN1 JKFF_CP_PE

G2  Q1 QN4 J2K2 AND2 (DELAY=6)

*IC2 J2K2 CLK J2K2 CLEAR QN2 Q2 PRESET jkff_cp_ne

G1_2  CLK $2_CLOCKN INV DELAY 3

IC1_3  $2_CLOCKN CLEAR PRESET J2K2 J2K2 Q2 QN2 JKFF_CP_PE

G3  J2K2 Q2 J3K3 AND2 (DELAY=6)

*IC3 J3K3 CLK J3K3 CLEAR QN3 Q3 PRESET jkff_cp_ne

G1_4  CLK $3_CLOCKN INV DELAY 3

IC1_5  $3_CLOCKN CLEAR PRESET J3K3 J3K3 Q3 QN3 JKFF_CP_PE

G4 J3K3 Q3 J4 AND2 (DELAY=6)

*IC4 J4 CLK Q1 CLEAR QN4 Q4 PRESET jkff_cp_ne

G1_6  CLK $4_CLOCKN INV DELAY 3

IC1_7  $4_CLOCKN CLEAR PRESET J4 Q1 Q4 QN4 JKFF_CP_PE

.TIME 6500 6500;

.PRINT PRESET CLEAR COUNT CLOCK Q1 Q2 Q3 Q4 J1K1 J2K2 J3K3;

.END

5.2.5     File BCD16.INF

***************************** LSYSTEM V2.0 ***************************

Information file for network BCD16.NWK

Number of source lines compiled:                  32

Number of nodes in network:                       21

Number of trace nodes:                            11

Number of logic components in network:            12

Number of excitation clocks:                       5

Simulation length:                              6500

Simulation display period:                      6500

Number of primitives loaded:                      63

Number of macros loaded:                           1

Number of used macros:                             1

Highest macro nesting:                             1

--- S Y M B O L    T A B L E ---

{0001} $1_CLOCKN                        {0002} $2_CLOCKN

{0003} $3_CLOCKN                        {0004} $4_CLOCKN

{0005} CLEAR                            {0006} CLK

{0007} CLOCK                            {0008} COUNT

{0009} J1K1                             {0010} J2K2

{0011} J3K3                             {0012} J4

{0013} PRESET                           {0014} Q1

{0015} Q2                               {0016} Q3

{0017} Q4                               {0018} QN1

{0019} QN2                              {0020} QN3

{0021} QN4

--- U S E D   P R I M I T I V E   T A B L E ---

0004 off AND2

0004 off INV

0004 off JKFF_CP_PE

--- U S E D   M A C R O   T A B L E ---

{0000} JKFF_CP_NE

5.2.6     Tabular results - file BCD16.RLT

******************************* LSYSTEM V 2.0 ******************************

Simulation Results for network file BCD16.FNC

PCCCQQQQJJJ

RLOL1234123

EEUO    KKK

SANC    123

ERTK

Time..... (nS)  T

Startup condition

0:      0111XXXX0XX

NC        1:      1000XXXX1XX

NC        2:      1000XXXX1XX

NC        3:      1000XXXX1XX

NC        4:      1000XXXX1XX

NC        5:      1000XXXX1XX

NC        6:      1000XXXX1XX

NC        7:      1000XXXX1XX

NC        8:      1000XXXX1XX

NC        9:      1000XXXX1XX

NC       10:      1000XXXX1XX

NC       11:      1000XXXX1XX

NC       12:      1000XXXX1XX

NC       13:      1000XXXX1XX

NC       14:      1000XXXX1XX

NC       15:      1000XXXX11X

NC       16:      1000XXXX11X

NC       17:      1000XXXX11X

NC       18:      1000XXXX11X

NC       19:      1000XXXX11X

NC       20:      1000XXXX11X

NC       21:      1000XXXX111

NC       22:      1000XXXX111

NC       23:      1000XXXX111

NC       24:      10000000111

NC       30:      10000000100

NC      250:      10010000100

NC      300:      11010000100

NC      500:      11000000100

NC      600:      11100000100

NC      750:      11110000100

NC     1000:      11100000100

NC     1018:      11101000100

NC     1024:      11101000110

NC     1250:      11111000110

NC     1500:      11101000110

NC     1518:      11101100110

NC     1524:      11100100111

NC     1530:      11100100101

NC     1536:      11100100100

NC     1750:      11110100100

NC     2000:      11100100100

NC     2018:      11101100100

NC     2024:      11101100110

NC     2030:      11101100111

NC     2250:      11111100111

NC     2500:      11101100111

NC     2518:      11101110111

NC     2524:      11100010111

NC     2530:      11100010100

NC     2750:      11110010100

NC     3000:      11100010100

NC     3018:      11101010100

NC     3024:      11101010110

NC     3250:      11111010110

NC     3500:      11101010110

NC     3518:      11101110110

NC     3524:      11100110111

NC     3530:      11100110101

NC     3536:      11100110100

NC     3750:      11110110100

NC     4000:      11100110100

NC     4018:      11101110100

NC     4024:      11101110110

NC     4030:      11101110111

NC     4250:      11111110111

NC     4500:      11101110111

NC     4518:      11101111111

NC     4524:      11100001111

NC     4530:      11100001100

NC     4750:      11110001100

NC     5000:      11100001100

NC     5018:      11101001100

NC     5250:      11111001100

NC     5500:      11101001100

NC     5524:      11100000110

NC     5530:      11100000100

NC     5750:      11110000100

NC     6000:      11100000100

NC     6018:      11101000100

NC     6024:      11101000110

NC     6250:      11111000110

6500:      11101000110

5.2.7     A screen dump of the BCD16 Network timing diagram

SEE PRINTED USER GUIDE FOR DIAGRAM

5.3  The structure of the LSYSTEM network description file

The  following  template  outlines  the  structure  of  the  LSYSTEM  network

description file:-

*      Title and header information

*      - entered as a series of comments.

.LIB    DEFAULT.LIB  ;   Logic primitive library files loaded

*                        using the .LIB directive.

.INCLUDE  DEFAULT.MAC ;  Macro library file loaded using the

*                        .INCLUDE directive.

*  Other macro definitions next.

.MACRO    ............

............

.ENDM

* Clock and logic function statements next.

...........................

.TIME .............

.PLOT or .PRINT statements.

.END

5.4  Logic primitive libraries

Access to a library  of  primitive  logic  element  models  is  available  in

LSYSTEM.   To load a library, the statement

.LIB   library_name

is entered near the beginning of the LSYSTEM network description  file.    At

least one model library must be loaded for LSYSTEM to function.   Please note

a primitive library must be loaded before any models in the library are used.

Only one library can be loaded per .LIB  command.    However,  as  many  .LIB

statements may be specified as required, for example:-

.LIB default.LIB    ; load the SHAREWARE default model library.

.LIB gates.LIB      ; only supplied to registered users.

.LIB memory.LIB     ; again only supplied to registered users.

5.4.1     Using primitives

All primitive model definitions have the same structure, although some of the

information is redundant, depending on  the  primitive  type.    Three  basic

types of primitive model currently exist, they are

- Combinational logic elements.

- Clocked sequential logic elements.

- memory elements.

Their  use  is  strictly  controlled  by,  for  example,  nodal  positioning,

triggering edges, and asynchronous input polarities.   Primitive  models  are

based on a look-up table.   Inputs specified in  the  model  definition  form

part of the look-up table index.   Hence, it is important that input nodes be

used in the correct order.   The structure of a  primitive  library  file  is

described in detail in Appendix B.   Information is  also  provided  to  help

LSYSTEM users construct their own primitive libraries.

5.4.2     Combinational primitives

Combinational model primitives have one type of input, this is one  or  more,

non-clocked, logic inputs.  A primitive's outputs have a direct  relationship

to to these inputs since  the  model  is  purely  a  combinational  function.

Information specifying a primitive ALWAYS  has the following sequence:-

- Input nodes, in the order given in the primitive definition.

- Output nodes, again in the order given in the primitive definition.

- The model name.

Hence, a functional element description for a primitive type called NAME  has

the format:-

Element IN1 IN2 IN3 IN4 .... OUT1 OUT2 OUT3........ NAME

.

........... Reference Element name to identify the logic component.

where each item in the list is separated by one or more spaces.

5.4.3     Sequential primitives

Sequential primitives are a little  more  complex  than  combinational  ones.

Using them  is  very  advantageous  though,  since  it  allows  a  degree  of

FUNCTIONAL SIMULATION, which simplifies and  speeds  the  simulation  process

enormously.   When modelling a complex digital system, it is often sufficient

to just use a functional box to represent an element or  a  sub-unit  of  the

digital network, for example a D-TYPE flip flop, or an 8bit up/down  counter,

instead of having to code the entire logic system as gate level primitives.

The maximum internal state bits for a sequential primitive is 8, yielding 256

maximum internal states.   This is quite sufficient  for  most  purposes  and

restricts look-up tables to a moderate size ( a practical  consideration  due

to the DOS 640k memory limit).

Sequential primitives have FOUR types of input, these must  be  specified  in

the following order:-

- Clock input node.

- Combinational input nodes, these DO NOT effect the internal state,

for example an OUTPUT ENABLE which switches a logic element into

tri state output mode.

- Asynchronous ( active low ) input nodes, for example CLEAR and PRESET

on a flip flop.

- Synchronous clocked ( rising edge triggered ) input nodes, for example

J and K on a flip flop.

The input node names are then followed by the output node list,  and  finally

the name of the logic primitive.   Hence. for example a 4 bit  counter  would

be described as:-

IC1 CLK OE CLEAR LOAD A B C D Qa Qb Qc Qd TC  4CountLoad

5.4.4     RAM  primitives

INFORMATION ONLY PROVIDED TO REGISTERED LSYSTEM USERS

5.4.5     ROM primitives

INFORMATION ONLY PROVIDED TO REGISTERED LSYSTEM USERS

5.4.6     Default primitive library list.

1.  EL IN1 IN2 OUT AND2 (DELAY=9 LOW=50 HIGH=40)

2.  EL IN1 IN2 OUT OR2 (DELAY=9 LOW=50 HIGH=40)

3.  EL IN OUT RES (DELAY=1 STRENGTH=10)

4.  EL IN OUT INV (DELAY=9 LOW=50 HIGH=40)

5.  EL CLK CLR PR J K Q NQ JKFF_CP_PE (RISE=9 FALL=15 LOW=50 HIGH=40)

6.  EL CLK CLR PR D Q NQ DFF_CP_PE (RISE=9 FALL=15 LOW=50 HIGH=40)

7.  EL IN1 IN2 IN3 IN4 OUT NAND4 (DELAY=9 LOW=50 HIGH=40)

8.  EL OE IN OUT TBUF (RISE=13 FALL=17 LOW=50 HIGH=40)

9.  EL I1 I2 I3 I4 I5 OUT AND5 (DELAY=9 LOW=50 HIGH=40)

THE DEFAULT LIBRARY PROVIDED TO REGISTERED LSYSTEM USERS

CONSISTS OF 65 PRIMITIVES ---- SEPARATE GATE, FLIP FLOP,

OPEN CIRCUIT COMPONENT AND TRI STATE DEVICE LIBRARIES ARE

ALSO INCLUDED ON THE LSYSTEM PRODUCTION MASTER DISKS.

5.5  The include file command - .INCLUDE

Include files are standard text files that are opened  and  read  by  LSYSTEM

compiler into the main network description file as it is being compiled.   As

many files as required can be defined for inclusion, although  only  ONE  per

.INCLUDE directive.   Include files can be nested to 20 levels deep, i.e., an

include file can call another include file which  can  call  another  include

file ...... up to 20 levels.

The syntax for the include command is:-

.INCLUDE FILE_1.EXT

.INCLUDE FILE_2.EXT

The main use for this directive is for loading previously defined MACROS.   A

file containing some useful macros. prepared for direct  inclusion  into  any

network description, has been included on the LSYSTEM disks.   The macro file

is called DEFAULT.MAC.

###################################