5.6  The expanded listing output command - .EXPAND

When macros are expanded during the  compilation  of  a  network  description

file many internally generated node  names  are  produced.    To  generate  a

complete listing file with all the macro expansions  documented  include  the

.EXPAND directive in the network description file.   Alternatively, the  same

expansion file can  be produced by using the compiler command line option -e.

5.7  The error listing output directive - .LIST

Error reporting during the compilation  phase  of  the  LSYSTEM  software  is

primarily directed towards the video screen.   However, including  the  .LIST

directive in a network description file will cause a copy of the input  lines

( numbered ), plus all errors detected during compilation, to be output to  a

file with extension LST.  Alteratively, the same error listing  file  can  be

produced by using the compiler command line option -l.

5.8  Logic component descriptive statements

A logic component is described by a statement which  contains  the  following

information:-

- The component name.

- The component connecting node names.

- The component primitive or macro type.

+-----+

I  &  I

A  ---I     I

I     I--- F     G1 A B  F  AND2

B  ---I     I

I     I

+-----+

The component name (G1 in the above example) can be any  alphanumeric  string

with up to 16 characters, as long as it begins with a letter, rather  than  a

number.  The component name does not have to be unique.   The same  name  can

be used as many times as the user likes.   The component name is  mainly  for

reference but it can also be used to group blocks of components  together  to

split up a network description into identified modules.

The input and output connecting  node  names  must  be  alphanumeric  with  a

maximum of 16 characters.   They may start with any extended ASCII  printable

character except $ which is used for internally generated node  names  during

macro expansion.   Each node within a logic network description must  have  a

unique name.

Several optional parameter  keywords  may  be  tagged  at  the  end  of  each

component description, these are:-

- RISE     Logic 0 to 1 state propagation time in nS.

- FALL     Logic 1 to 0 state propagation time in nS.

- DEALAY   Set both fall and rise time to delay time in nS.

- HIGH     Logic 1 state strength of component outputs in %.

- LOW      Logic 0 state strength of component outputs in %.

- STRENGTH Set both high and low strengths to strength in %.

Hence, to define the previous two input AND component with  a  rise  time  of

11nS, a fall time of 16nS and both high and low strengths of 76%,  the  logic

component description becomes:-

G1 A B F AND2 (RISE = 11 FALL = 16 STRENGTH = 76 )

5.8.1     Node contention

The digital logic simulator LSYSTEM uses four logic states  and  a  range  of

output signal strengths, set as percentages between 0% and 100%, to represent

signal activity in a logic system.   The four possible logic states are:-

0 -> Logical LOW.

1 -> Logical HIGH.

Z -> HIGH IMPEDANCE state (to represent tri state or open circuit

device outputs).

X -> INDETERMINATE state (for startup or contention conditions).

Node contention, often referred to as bus contention, occurs when two or more

logic component outputs, or of course, excitation clock signals,  are  trying

to drive a node into differing states, at the same time.   This can occur for

a variety of  reasons.    The  most  obvious  reason  is  when  two  standard

components are defined as driving the same node, for example:-

+-----+

A  ----I  &  I

I     I-------+

B  ----I     I       I

+-----+       I

+------------- F

+-----+       I

C  ----I  &  I       I

I     IO------+

D  ----I     I

+-----+

The situation shown above, assuming that the devices have totem pole  outputs

pulling both up and down  to  achieve  their  output  states,  would  produce

continuous node contention, since when  one  gate  is driving  (  trying  to

anyway) the output node F into a high state, the other is driving it low.  In

practice ( in a physical network ) the devices would get very  warm  and  one

of them would be destroyed.

A  much  more  subtle  contention  situation  may  arise  due  to   differing

propagation delays in two or more gates.   For  example,  if  two  tri  state

devices were driving a common node, in a multiplexed display driver:-

+---------+

I   TRI   I

A  ----I  STATE  I----------+

I         I          I

+----+----+          I

I               I

CONTROL ---------+               +--------- OUT

+---------+          I

I   TRI   I          I

B  ----I  STATE  I----------+

I         I

+----+----+

-------          I

CONTROL ---------+

Then, if the logic state to tri state delay of the gates was 10nS whilst  the

tri state to logic state delay was 9nS, an overlap period of 1nS would  occur

every time control was switched from one gate to the other,  yielding  a  1nS

node contention condition.

The strength of a logic component  output  is  used  by  LSYSTEM  to  resolve

contention conflicts.   If only one component is seen to be driving  a  given

node no action is taken.   However, if more than one component is  driving  a

node, LSYSTEM checks to see if their outputs conflict.   Node  contention  is

determined by calculating two strength values, firstly the  highest  state  1

strength, and secondly the highest state 0 value.   Once these  two  opposing

maximum values have been found, they are subtracted.   The result  from  this

subtraction is normalised, i.e. the sign of the result  is  disregarded,  and

then compared to a threshold value.   This threshold value is actually set to

25% in the simulator.   If the value is found to be less than the  threshold,

a contention warning is embedded into the results file, at the time at  which

it occurred.

This conflict resolution technique allows the use of 'pull up'  (  or  indeed

'pull down' ) resistors on tri state and open collector (  or  open  drain  )

gate outputs.   By  assigning  a  resistor  a  low  value  for  the  strength

parameter, and tying its input to a permanent logic 1, the calculated figure,

when the gate is pulling the node low and the resistor is pulling it high, is

greater than the  threshold  value,  and  therefore  does  not  give  a  node

contention error.

5.9  Clocks - external generators

Excitation clocks, or driving nodes, are signal generators  external  to  the

logic network which drive the logic network inputs.   They  can  only  assume

two states, logic 0 and logic 1.  At these logic levels the clock  generators

are assumed to be able to supply or sink an infinite signal  and  hence  have

their strengths set to 100%.

An  excitation  clock  specification  must  always  contain   the   following

information:-

- CLOCK NAME,  [ 16 characters maximum. ]

- NODE NAME,   the name of the node to which the clock output is connected.

- CLOCK TYPE,  Two types are available:-

CLK0  -  initial state logic 0 at 0nS.

CLK1  -  initial state logic 1 at 0nS.

The State Change Times list can be a simple  list  or  a  more  complex  form

generated using the keywords PERIOD, REPEAT  or  TABLE.   For  instance,  the

following non-repeating pulse sequence, connected to a node called NRESET:-

............                    .....................

.          .                    .

..........          ......................

0        70         200                  500    Time (nS)

would be defined as:-

CLOCK1  NRESET  CLK0  70 200 500

Permanent logic levels can  be  assigned  to  a  clock  output  node  by  NOT

specifying any state change times.   For example, a node  called  ENABLE,  at

permanent state of logic 1 would be defined:-

CLKP1   ENABLE CLK1

5.9.1     The PERIOD clock keyword

Often a logic network is synchronized to  an  external  clock.   The  keyword

period allows periodic clock signals to be simple and efficiently  generated.

This type of excitation clock is specified by:-

GENP0  CLK_IN CLK0 PERIOD( TP ) , or

GENP1  CLK_IN CLK1 PERIOD( TP )

....  ....  ....  ....  ....  ....

........  ....  ....  ....  ....  ....  ......

5.9.2     The REPEAT clock keyword

A common requirement when simulating logic system operation is the need for a

clock input which is constructed from a complex repeating pattern that starts

at some time after the start of the simulation, for example:-

:                             :

:    .....       .........    :    .....        ...........

...........:.....   .........       .....:.....   ..........         ......

0          :    T   T       T       T    :

<-- TD  -->:     1   2       3       4   :      Time (nS)

<--        TS             -->

This type of excitattion clock is specified by:-

GENR CLK_IN CLK0 REPEAT( TD No_repeats T T T  ... TS )

1 2 3

or

GENR CLK_IN CLK1 REPEAT ( TD No_repeats T T T ... TS )

1 2 3

Here, No_repeats is the number of times the pattern is to repeat.  The  value

assigned to N0_repeats must be such that the total number state change  times

generated by the REPEAT keyword is less or equal to 1000.    The logic  level

at the end of each pattern ( i.e at time T ) must be the same  as  the  start

of the pattern, otherwise  each  alternate  group  of  patterns  will  change

polarity.   After  No_repeats of the waveform pattern are completed the clock

output remains at logic 0 or logic 1  till  the  simulation  time  period  is

complete.

5.9.3     The TABLE clock keyword

A second common requirement when simulating combinational logic  networks  is

the need for a series of clock inputs which have outputs  that  generate  all

the possible values in a truth table.   The  TABLE  keyword  is  included  in

LSYSTEM for this purpose.  This type of excitation clock is specified by:-

CLKT clk_node CLK0 TABLE( TS TPD 0,1,1,1,0,0,0,1,0,1, ....) , or

CLKT clk_node CLK1 TABLE( TS TPD 0,0,0,1,1,1,0,1,0,1, ....)

For example, a four bit truth table becomes:-

CLKB0 BN0 CLK0 TABLE(200 100 0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1)

CLKB1 BN1 CLK0 TABLE(200 100 0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,1)

CLKB2 BN2 CLK0 TABLE(200 100 0,0,0,0,1,1,1,1,0,0,0,0,1,1,1,1)

CLKB3 BN3 CLK0 TABLE(200 100 0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1)

At times beyond the time representing the last bit in the binary pattern  the

clock output remains at the logic level set by the last bit in the pattern.

5.9.4     An example showing the different clock generators

* Example 2 --- Demonstration of different clock generators.

.LIB DEFAULT.LIB

*

CLOCK1 NRESET CLK0 70 200 500

CLKP1 ENABLE CLK1

GENP0 CLK_IN0 CLK0 PERIOD(200)

GENP1 CLK_IN1 CLK1 PERIOD(300)

GENR  CLK_IN2 CLK0 REPEAT(400 20 50 70 200 400 700)

CLKB0 BN0     CLK0 TABLE(200 100 0,1,0,1,0,1,0,1,0,1,0,1,0,1,0,1)

CLKB1 BN1     CLK0 TABLE(200 100 0,0,1,1,0,0,1,1,0,0,1,1,0,0,1,1)

CLKB2 BN2     CLK0 TABLE(200 100 0,0,0,0,1,1,1,1,0,0,0,0,1,1,1,1)

CLKB3 BN3     CLK0 TABLE(200 100 0,0,0,0,0,0,0,0,1,1,1,1,1,1,1,1)

*

G1 NRESET ENABLE  G1_OUT AND2

G2 CLK_IN1 CLK_IN2 G2_OUT AND2

G3 ENABLE BN0 BN1 BN2 BN3 G3_OUT AND5

*

.TIME 2000 2000

.PRINT NRESET ENABLE G1_OUT ! CLK_IN1 CLK_IN2 G2_OUT

.PRINT ENABLE BN0 BN1 BN2 BN3 G3_OUT

*

.END

******************************* LSYSTEM V 2.0 ******************************

Simulation Results for network file CLKGEN.FNC

NEG CCGEBBBBG

RN1 LL2NNNNN3

EA_ KK_A0123_

SBO __OB    O

ELU IIUL    U

TET NNTE    T

Time..... (nS)      12

Startup condition

0:      10X 01X01111X

NC        1:      01X 10X10000X

NC        2:      01X 10X10000X

NC        3:      01X 10X10000X

NC        4:      01X 10X10000X

NC        5:      01X 10X10000X

NC        6:      01X 10X10000X

NC        7:      01X 10X10000X

NC        8:      01X 10X10000X

NC        9:      010 100100000

NC       70:      110 100100000

NC       79:      111 100100000

NC      150:      111 000100000

NC      200:      011 000100000

NC      209:      010 000100000

NC      300:      010 100110000

NC      400:      010 100101000

NC      450:      010 010101000

NC      470:      010 000101000

NC      500:      110 000111000

NC      509:      111 000111000

NC      600:      111 110100100

NC      609:      111 111100100

NC      700:      111 111110100

NC      750:      111 011110100

NC      759:      111 010110100

NC      800:      111 000101100

NC      900:      111 100111100

NC     1000:      111 100100010

NC     1050:      111 000100010

NC     1100:      111 000110010

NC     1150:      111 010110010

NC     1170:      111 000110010

NC     1200:      111 100101010

NC     1300:      111 110111010

NC     1309:      111 111111010

NC     1350:      111 011111010

NC     1359:      111 010111010

NC     1400:      111 010100110

NC     1500:      111 100110110

NC     1600:      111 100101110

NC     1650:      111 000101110

NC     1700:      111 000111110

NC     1709:      111 000111111

NC     1800:      111 100111111

NC     1900:      111 110111111

NC     1909:      111 111111111

NC     1920:      111 101111111

NC     1929:      111 100111111

NC     1950:      111 000111111

2000:      111 000111111

5.10 Trace nodes - the .PRINT and .PLOT commands

Trace nodes are points in the digital network whose activity the user  wishes

to examine.   Obviously, it is  not  practical  or  desirable  in  a  complex

network, to trace all nodal activity.

No nodal activity tracing is performed by default.   Omission of  the  .PRINT

or .PLOT statements will result in no simulation results being produced.

Node activity output, recorded in the  results  file  (  name.RLT  )  can  be

formatted by including a ! character where a space is to be inserted  between

output items.   For example, a 4 bit  counter  network,  with  inputs  RESET,

CLOCK and outputs A, B, C, D, shown in  the  following  diagram  as  a  black

box:-

+-------------------------------------+

-----       I       4 BIT BINARY COUNTER          I------ A

RESET  -----I       NETWORK.    INTERNAL          I------ B

I       NODE   ACTIVITY  NOT          I------ C

CLOCK  -----I              TRACED                 I------ D

+-------------------------------------+

could have the input and output nodes specified as:-

.PRINT  CLOCK ! RESET !!!!! A B C D

Multiple .PRINT statements are allowed by LSYSTEM, so for example, the  above

trace directive could have been written:-

.PRINT CLOCK

.PRINT ! RESET

.PRINT !!!!!

.PRINT A B C D

and would  have  generated  the  same  results  file  as  the  single  .PRINT

statement.

The .PRINT statement also allows users  to  trace  selected  nodes  within  a

macro.   The internally generated names for these local nodes is given in the

expansion listing file.   Taking the expansion file shown on  page  8  as  an

example  we  could  add  the  following  .PRINT  statement  to  the   network

description file:-

.PRINT $1_CLOCKN $2_CLOCKN $_CLOCKN $4_CLOCKN

The LSYSTEM directive .PLOT has the  same  function  as  the  .PRINT  command

except the results file has the logic 0 and logic 1  entries  offset  by  one

column yielding a basic line printer plot of  the  trace  results.    LSYSTEM

does NOT allow .PRINT and .PLOT statements to be mixed in the same simulation

network description file. The results for example 2  on  page  22  are  given

below, with the output generated by a .PLOT statement:-

******************************* LSYSTEM V 2.0 ******************************

Simulation Results for network file CLKGEN.FNC

N  E  G   C  C  G  E  B  B  B  B  G

R  N  1   L  L  2  N  N  N  N  N  3

E  A  _   K  K  _  A  0  1  2  3  _

S  B  O   _  _  O  B              O

E  L  U   I  I  U  L              U

T  E  T   N  N  T  E              T

Time..... (nS)            1  2

Startup condition

0:       1 0  X   0   1 X  0   1  1  1  1 X

NC        1:      0   1 X    1 0  X   1 0  0  0  0  X

NC        2:      0   1 X    1 0  X   1 0  0  0  0  X

NC        3:      0   1 X    1 0  X   1 0  0  0  0  X

NC        4:      0   1 X    1 0  X   1 0  0  0  0  X

NC        5:      0   1 X    1 0  X   1 0  0  0  0  X

NC        6:      0   1 X    1 0  X   1 0  0  0  0  X

NC        7:      0   1 X    1 0  X   1 0  0  0  0  X

NC        8:      0   1 X    1 0  X   1 0  0  0  0  X

NC        9:      0   1 0    1 0  0   1 0  0  0  0  0

NC       70:       1  1 0    1 0  0   1 0  0  0  0  0

NC       79:       1  1  1   1 0  0   1 0  0  0  0  0

NC      150:       1  1  1  0  0  0   1 0  0  0  0  0

NC      200:      0   1  1  0  0  0   1 0  0  0  0  0

NC      209:      0   1 0   0  0  0   1 0  0  0  0  0

NC      300:      0   1 0    1 0  0   1  1 0  0  0  0

NC      400:      0   1 0    1 0  0   1 0   1 0  0  0

NC      450:      0   1 0   0   1 0   1 0   1 0  0  0

NC      470:      0   1 0   0  0  0   1 0   1 0  0  0

NC      500:       1  1 0   0  0  0   1  1  1 0  0  0

NC      509:       1  1  1  0  0  0   1  1  1 0  0  0

NC      600:       1  1  1   1  1 0   1 0  0   1 0  0

NC      609:       1  1  1   1  1  1  1 0  0   1 0  0

NC      700:       1  1  1   1  1  1  1  1 0   1 0  0

NC      750:       1  1  1  0   1  1  1  1 0   1 0  0

NC      759:       1  1  1  0   1 0   1  1 0   1 0  0

NC      800:       1  1  1  0  0  0   1 0   1  1 0  0

NC      900:       1  1  1   1 0  0   1  1  1  1 0  0

NC     1000:       1  1  1   1 0  0   1 0  0  0   1 0

NC     1050:       1  1  1  0  0  0   1 0  0  0   1 0

NC     1100:       1  1  1  0  0  0   1  1 0  0   1 0

NC     1150:       1  1  1  0   1 0   1  1 0  0   1 0

NC     1170:       1  1  1  0  0  0   1  1 0  0   1 0

NC     1200:       1  1  1   1 0  0   1 0   1 0   1 0

NC     1300:       1  1  1   1  1 0   1  1  1 0   1 0

NC     1309:       1  1  1   1  1  1  1  1  1 0   1 0

NC     1350:       1  1  1  0   1  1  1  1  1 0   1 0

NC     1359:       1  1  1  0   1 0   1  1  1 0   1 0

NC     1400:       1  1  1  0   1 0   1 0  0   1  1 0

NC     1500:       1  1  1   1 0  0   1  1 0   1  1 0

NC     1600:       1  1  1   1 0  0   1 0   1  1  1 0

NC     1650:       1  1  1  0  0  0   1 0   1  1  1 0

NC     1700:       1  1  1  0  0  0   1  1  1  1  1 0

NC     1709:       1  1  1  0  0  0   1  1  1  1  1  1

NC     1800:       1  1  1   1 0  0   1  1  1  1  1  1

NC     1900:       1  1  1   1  1 0   1  1  1  1  1  1

NC     1909:       1  1  1   1  1  1  1  1  1  1  1  1

NC     1920:       1  1  1   1 0   1  1  1  1  1  1  1

NC     1929:       1  1  1   1 0  0   1  1  1  1  1  1

NC     1950:       1  1  1  0  0  0   1  1  1  1  1  1

2000:       1  1  1  0  0  0   1  1  1  1  1  1

5.11 The simulation cycle - .TIME

The LSYSTEM command .TIME provides the simulator with  two  time  parameters.

The first is the time period over which the simulation is to take place,  and

the second is the output interval at which a  forced  output  of  trace  node

results is to occur.   The simulator always outputs a set  of  trace  results

when a network trace node changes state - the code letters NC are  placed  at

the left hand side of a line in the results file  to  signify  a  change  has

taken place.

PLEASE note setting the output interval time parameter to a small value ( 1nS

is smallest allowed ) will generate very large results files which may exceed

your computers disk capacity.   This is likely to cause a problem if you  are

using 360k floppy disks.

The syntax for the .TIME statement is:-

.TIME Simulation_finish_time Trace_output_interval

For example, if a network is to be simulated over a time period  of  500000nS

and node state data is required at   intervals of 1000nS, the directive would

be:-

.TIME 500000 1000

this would cause the trace nodes to be output every 1000nS as well  as  every

time a trace node changed state.

At the start of a simulation when the initial states of the network nodes are

being determined by LSYSTEM the trace nodes are often output as indeterminate

( the X state ).   Due to the uncertainty as to the true state of the network

nodes a trace node at state X is output as a changed node.