Supplemental Information (README.DOC file)

The "Switchers Made Simple" program is an expert system that

automates the design of Simple Switcher (TM) based basic switching

regulators.  The following topologies can be designed with the

present version: buck and buckboost (invert) regulators with the

LM2575 family, and boost and multiple-output flyback regulators

with the LM2577 family.

The first module of the program lets you input the specification

of the converter you wish to design.  It is done via a menu, with

editing capability.  The feasibility of the specification is

checked and provision is made for editing input parameters should

the program alert the user to an unsuitable parameter set.

The second module calculates the limit values of the external

components of the circuit based on the specification and loop

stability requirements.  The stability analysis calculation is

based on the state-space-averaged model introduced by

Dr. Middlebrook.

Next, the program chooses the actual components, according to the

calculated limits, from a built in data-base of standard

components.  There are three manufacturers providing standard

inductors and tranformers for the Simple Switcher (TM) product family:

AIE, Pulse Engineering and Renco.  The capacitor data-base is built up

using the Sprague, Nichikon, Cornell Dubilier and Panasonic lines.

You can override (i.e. edit) most component values of the list.

Some values can not be edited: e.g. the ESR of the capacitors,

R1, the upper resistor in the voltage divider when adjustable

regulators are used, the transformer's turns ratio, etc.

Some values are adjusted automatically with another component;

e.g. R1 if R2 is changed etc.

After the components selection is completed, the program

calculates the control loop's crossover frequency and phase

margin.  The design procedure ensures that the resulting circuit

is stable in worst-case conditions and has adequate phase margin.

Should you require higher phase margin, you can change component

values (increase Co in LM2575-based designs and increase Co and

Cc in LM2577-based designs) and rerun the stability analysis to

verify the improvement.

Finally, a junction temperature check is performed, based on our

proprietary thermal model.  It calculates the junction

temperature of the chip in your application circuit at the

specified maximum ambient temperature.  If the calculated

junction temperature exceeds the thermal shutdown limit (160degC)

the program specifies the maximum allowable thermal resistance of

the heatsink that must be used.

After completing the design, provision is made for saving the file,

printing the schematic, and/or printing the components list complete

with manufacturers' part numbers. To print the schematic, you must

have an Epson compatible printer.  You may view the schematic with

either Hercules, CGA, EGA, or VGA grahics capabilities.

WARNINGS

The program displays warnings to alert you to certain conditions:

1.   Current or voltage-limit exceeded.

This message appears after the feasibility check if either

the maximum current or voltage rating of the power switch in

the IC is exceeded.   The specifications of the circuit have

to be modified (e.g. load or Vin max decreased) or, in case

of the flyback converter, the turns ratio of the transformer

may have to be modified to stay within the IC's ratings.

2.   Burst mode operation.

This flag calls attention to the fact that due to the light

load, the regulator skips switching cycles to maintain

regulation.  Strictly speaking, the stability analysis is

not valid in this operating mode.  Practice shows, however,

that the regulators are stable under these conditions.  The

only possible problem is the indeterminate emmitted spectrum

of the circuit, due to the non-constant operating frequency.

The burst mode warning will always appear if minimum load

is not specified, because the default value is zero.

3.   Short Circuit Runaway.

This condition can occur in flyback regulators.  The problem

is generic to flybacks and not unique to the LM2577.  It

indicates that due to the non-zero minimum on-time of the

power-switch the current limit can not be maintained in an

output short circuit condition. Using a fast-recovery diode

instead of a Sckottky sometimes solves the problem.  In general,

either Vin max or N (the transformer's turns ratio) has to be

decreased to avoid the problem and to make sure that the switch

current is safely limited in case of an output short circuit.

4.   Possible Subharmonic Oscillation.

In current-mode controlled DC/DC converters, like  all  the

LM2577-based converters, subharmonic oscillation can occur if

the compensating ramp slope is not high enough. The built-in,

fixed compensating ramp of the LM2577 is designed so that it

ensures subharmonic oscillation free operation in most practical

applications. However, in some cases the stability criteria

may be violated. In these rare cases the program issues a

warning.  Although the regulator maintains control of the output

voltage even under these circumstances, you may want to avoid

this operating mode, because of increased noise and output

ripple. The value of the inductor or the primary inductance of

the transformer has to be increased to avoid this condition.

5.   Maximum Duty-cycle Exceeded.

The duty-cycle of the LM2577 is limited to 90%. If the

specification requires higher duty-cycle, the program issues this

warning.  The output - input voltage difference has to be

decreased, or the transformer's turns ratio increased to avoid

this error condition.

ESR AND OUTPUT VOLTAGE RIPPLE

The ESR, equivalent series resistance, of the output filter

capacitors is a very important parameter in switching regulator

applications.  It introduces a zero in the regulator's control

loop.  To maintain good phase margin, the ESR has to be smaller

than a limit value in boost and flyback converters and has to be

between a high and a low limit in buck and buck-boost regulators.

The program outputs these limits in the "Limit Values" section

of the screen.  They can be met using so called "high frequency"

or "low ESR" capacitors, like the ones used in the internal

database.  If you choose your own capacitor from a different

manufacturer, make sure you use a product line that specifies

the ESR at higher frequency than 120Hz, e.g. 10KHz or 100KHz.

Capacitors without high frequency ESR specifications are

not intended for use in switching regulators.  Also, ESR is

temperature dependent, it can increase substantially at low

temperature.  If you design a circuit that has to operate below

0 degC, you should consult a detailed capacitor data-sheet.

It is good practice to use a solid tantalum capacitor (at

least 10% of the output capacitor's value) in parallel with the

output capacitor if operation below 0 degC is required.

The ESR has also a strong effect on the output ripple voltage of

the converter.  The ripple voltage can be calculated as the

product of the current ripple of the inductor or transformer

winding feeding the capacitor, and the ESR.  In boost and flyback

converter applications the output ripple (Vripple) can be

specified as an input parameter.  The program ensures that the

specification is met by the final circuit.  The program both alerts

and disallows user choice of Vripple less than 0.005Vout.  This is

because very low values of ripple can only be achieved in basic

converter designs via impractically large output capacitors with

extremely low ESR.  If Vripple is not specified, the program assumes a

ripple voltage of 0.01Vout by default.

In buck and buck-boost applications the ripple voltage can not be

explicitly specified.  It will be calculated by the program and

listed under "Limit Values".  The ripple voltage can be

decreased, if needed, by optionally selecting larger inductors in

these applications.

STANDARD INDUCTORS AND TRANSFORMERS

In each regulator you have the choice to use standard

inductors/tranformers or to design your own.

The standard inductors are chosen for 30% current ripple.  The

standard transformers for flybuck converters cover the input

voltage range 4.5V to 15V and the output voltage range of +/-10V

to +/-15V.

If you decide to use nonstandard inductors, you can specify

either the inductance or the required maximum current ripple

(in Amperes).  The program automatically calculates the other

parameter.

In the case of nonstandard flyback transformers you have two

choices: either you can use a "full custom transformer", which means

you can input the value of the primary inductance (Lp), and turns

ratio of the main output (N1), or you can let the program design your

custom transformer. In the latter case the transformer is optimized to

deliver the maximum output power in the given application.

PRIME OUTPUT AND TURNS RATIO IN MULTIPLE OUTPUT FLYBACK CONVERTERS.

The output designated by "1" subscript (V01) is always the "prime"

or directly regulated output of the flyback converter. The turns ratio

of the auxiliary secondary (output) windings are designated by N1, N2,

and N3.  N1 = the number of secondary turns of output #1 divided by

the number of primary turns; N2 and N3 are defined similarly. N2 and

N3 are not variables you can change, they are always chosen by the

program, based on N1 and the output voltages given in the input

specifications section.

IC OPTIONS

If the specified output voltage is 5, 12 or 15V in a buck

converter or -5, -12 or -15V in an buck-boost converter, the

program automatically chooses and displays the appropriate fixed

output voltage versions of the Simple Switcher family.

The same is true for boost and flyback regulators with 12V or

15V output voltage.

For other output voltages the adjustable versions are specified

with appropriate voltage divider resistors of 1% tolerance.

If the specified ambient temperature limits are within the

temperature range: -40degC to 125degC, the program

automatically specifies the LM2575 or LM2577 families in 5-pin

TO-220 packages.  For wider temperature range the LM1575 and

LM1577 families in 4 lead TO-3 package are indicated and

specified. It should be noted, however, that the IC itself

is guaranteed to meet specifications in the -40 to 125 degC (LM2575 &

LM2577) and the -55 to 150 degC (LM1575 & 1577) JUNCTION temperature

range, with an approximate shutdown temperature of 160 degC.

OUTPUT  ACCURACY

The accuracy of the output voltage of the buck and boost

converters using the fixed output voltage versions of the LM2575

and LM2577 families is guaranteed to be better than +/-5% over

the entire line and load range, worst case component variations

and over the full temperature range.  If the adjustable versions

are used, the accuracy can be somewhat worse depending on the

tolerance of the external divider resistors used.  If "perfect"

divider resistors are used, the adjustables actually have an accuracy

specification slightly superior to that of the fixed versions.

In buck-boost and flyback converters the accuracy of the main

(directly regulated) output can be expected to meet the same

specification.  The auxiliary outputs of flyback converters are

not regulated and usually have lower output voltage accuracy due

to the load-regulation being much worse than on the prime

regulated output.

CROSSOVER FREQUENCY AND PHASE MARGIN

The program calculates the unity - gain crossover frequency and phase

margin of the converter's control loop. They characterize the loop's

stability. The calculation is done using the final (edited) component

values. This feature enables the user to fine tune the design and/or

experiment with different component values.

The phase margins of converters using the LM2577 are typically high

(60 - 80 deg), due to the chip's current mode control design.

The phase margin calculations are performed under worst case

conditions, i.e. minimum temperature (-55 degC), and maximum error

amplifier transconductance. As a consequence, the crossover frequency

is typically twice as high and the calculated phase margin is lower

than the value you can measure using typical parts at room

temperature.

The buck and buck-boost converters, using the LM2575, have typically

lower phase margins. It should be noted however, that their loop gain

is extremely stable, practically constant over process variations

and operating temperature. This means that the usual design reserves

that account for loop gain variations need not be applied, and

a phase margin of 20 to 40 deg is perfectly acceptable. Remember that

the value the program calculates is worst case.  The phase

margin can be increased, if needed, by increasing the value of the

output capacitor, or in the case of the buck converter, also by

increasing the value of the inductor.  This can be done when editing

in the "Component Values" column. The program calculates the crossover

frequency and phase margin using the edited component values.

Whenever Cout is edited this calculation is performed using the ESRmax

value in the "Limit Values" column.

STANDARD UNITS

Throughout the program the following standard units are used:

V, A, OHM, H, F. Any input quantity entered without unit designation

is understood in these units. E.g. a current "300" is understood as

300A, if you want to input 300mA, you should type "300m" or "300mA" or

".3".  "Micro" is understood by using the suffix, "u".

CONTINOUS/DISCONTINOUS OPERATION

Every DC/DC converter can operate in either continous or discontinous

operating mode. In continous mode the inductor current (or the

Magneto-Motive Force in transformers) never falls to zero. The

converters operating in continous mode are able to deliver higher

output power with the same power switch limits. However, they also

generate more radiated noise and and need more input filtering as well

as more careful layout.

For flyback converters you may elect to implement your

specifications with a discontinous mode converter. The program

warns you if this is not feasible, and switches to continous mode

automatically.

The program designs the boost converters ensuring continous mode

operation, so as to maximize the available output power. You can force

discontinous operation by choosing  a low value custom inductor.

Whether in buck or buck-boost converters, the program checks the input

parameters to decide whether the converter operates in continous or

discontinous mode. The model, used to calculate crossover frequency

and phase margin, is automatically adjusted to fit the operating mode.

You can recognize whether the converter is running in continous or

discontinous operating mode from the Limit Value list: if L<"value"

is shown, the converter operates in discontinous mode,  L>"value",

on the other hand, indicates continuous mode operation.

COMPONENTS NOT FOUND IN THE DATA-BASE.

Components, not found in the data-base, are printed without vendor

callout in the component list. In the case of output capacitors, the

limit values are printed.

EXTREME LOW POWER CONVERTERS

If very low output power is specified (Po<0.1W), discontinous mode

operation yields superior results. In these cases it is recommended

that you use custom inductors or full custom transformers with

adequately low primary inductance. Should you not use this preferred

mode, the resulting continous mode design (although functional and

stable) will have inductors and capacitors with extremely high, and

thus impractical, values.

MULTIPLE ITERATIONS

If you iterate by changing component values many times in the same

circuit, you may  reach your memory limit, and could be dumped back

to DOS.  To avoid this, it is recommended that after 5 changes you

save your circuit, and recall it if you wish to experiment more

with it.

A WORD OF CAUTION:

Although we made every effort to ensure that the program is

bug-free and yields circuits that correspond to the

specifications, we strongly urge you to build the regulators and

test them thoroughly before using them in production.  National

offers a small PCB for evaluation purposes to assist you in evaluating

actual hardware.

QUESTIONS OR SUGGESTIONS

Our Design and Application staff have put a great deal of effort into

creating both a true Expert System and a user friendly tool.  We

sincerely hope that you will find this diskette useful in applying our

new Simple Switcher (TM) family of high performance, yet easy-to-use

voltage regulators.

"Switchers Made Simple" represents a breakthrough in design-aid tools

offered by semiconductor vendors. Unlike modeling tools that ANALYZE

circuits the user must first create, this program undertakes the more

difficult task of SYNTHESIZING the design and then calculating

performance parameters.  It then takes the additional step of

identifying the actual components needed to physically realize the

circuit.  "Switchers Made Simple" is but one part of a complete

package of support that National offers you, our customers.

Should you have any questions or suggestions regarding this

program call Linear Applications at 408-721-5608.  You can also

reach us by Fax: 408-732-7549.  Please call your local National

Semiconductor sales office or authorized distributor for samples

and evaluation boards.

SUGGESTED READING

1.   LM2575 and LM2577 data-sheets.

2.   T. Szepesi, J. Bittner, H. Suzuki: Simple Switchers: A new

breed of power + control IC's for DC/DC converters.

PCIM '89 Proceedings, 1989 Long Beach, pp:437-449

3.   T. Szepesi, J. Bittner, H. Santo: On card DC/DC converters

with the new "Simple Switcher" regulator family.

PCIM '90 Europe Proceedings, 1990 Munich.

To get a copy, call Linear Applications at the phone number above.