This page covers notes for PrintGL  1.26  that are not included in the

v1.24 manual.

The GEM IMG output format (/FG) has been dropped.

Low resolution (180 dpi) Canon BJ and BJC drivers have been dropped.

The O suboption  of /L  (pen overwrite  mode) has been dropped.  Pen

overwrite mode is now the default.  Translucent pens may be selected

with the /C option.

/F0 drives HP-GL/2 raster devices. The nominal output resolution may

be appended to the /F option, for example /F0 300. The actual device

resolution may also be appended, for example /F0 300,303,299

/FG drives the Pacific Data Products ProTracer.

/FV* drives VESA VGA  screens  at  1024x768.  /FV+, the VESA 800x600

driver has been fixed.

The nominal output resolution of PostScript or HP-GL may be appended

to the /F option, for example /FS 600.  HP-GL  with dashed lines, at

other than 1016 dpi, requires HP 7550 enhanced IW mode.

The PostScript XP macro selects a paper tray. To use it set a prefix

code such as /J48 32 88 80 for tray 1 or /J49 32 88 80 for tray 2.

Line width N turns a pen off.

/F7+  is  a  two pass BJ-330  driver,  which gives  dark  print  and

eliminates indexing gaps.

/FB+ is a BJC driver with sparse line to line overlap.

/F8 adds support for the HP 7220.

The H suboption of /X has been dropped.

Esc breaks  out  of PrintGL, leaving the printer ready for new data.

Ctrl-Break (as before) stops PrintGL immediately and a printer reset

is usually needed in this case.

Print  size  correction  can be specified  as  a multiplier  to  the

nominal resolution (.5 to 2), for example /F7 1.0,1.01 is equivalent

to /F7 360,363.6 for a 360 dpi printer.

The E suboption of /Y sets HP 7550 enhanced IW command mode.