WARNING DOT MATRIX PRINTERS ONLY

INTRODUCTION

The Program is intended for the STAR LC10 Colour Printer and operates

by way of drop down menus. Because most of the codes used by the star

Printer are common With Most EPSON or IBM dot matix printers the program

should work OK with printers such as the Epson LX-800,FX,EX and Ibm

Proprinters.

HARDWARE

The Program requires a mouse or keyboard, Any moniter a Harddisk and DOS.

STARTUP

Copy the program Star.exe to a Directory set up in the path such as DOS

this allows the program to be called up as required from any Directory.

On start up the program checks the printer if the printer is switched off

a Error window appears Asking the user to turn on the printer.  If error

message appears first CORRECT error and press Any key to continue.

The Printer starts up in Epson mode To change mode select options menu.

The Standard or IBM mode does not change the mode of operation of the

printer itself. With the Star LC10 this is dependent on Switch 1-6 to

select the printer mode, ON (Switch towards back of printer) places the

printer in EPSON mode while OFF (Switch towards front of printer) sets

the mode to IBM.

The user is now able to move the mouse to the Bar menu across the

top of the screen, again pressing the left button causes drop down

menus to appear move mouse up and down and press left button to

select item and make choice.

Menus

Multi

This Menu allows the user to set Colours, Character Sets, Justification,

Line Spacing, Margins, and the Automatic Sheet Feeder Setting.

Selecting any Of the Above makes a window appear on the screen with small

button next to the Choices.

For instance to change Character set move mouse to bar menu

select Multi by press left button, move mouse down to character set within

menu press left button, From the window select the required character set

by moving mouse and pressing left button. To send codes to printer move to

Ok button and press enter.

Some of the other menus also open up a window on the screen from which

a number can be entered to set required value.

The Escape button Closes the menu,.

The automatic paper feed menu requires that the Switch 1 - 4 is OFF

(towards front of printer). ASF Active

Style

This allows varies choices to be made Moving mouse to required choice and

pressing left button sends the code to printer. The Font Choice can not

be selected until Letter Quality is selected. The font Choice than becomes

active and clicking on this with left button selects a further small

dropdown menu from which the required font can be selected.

Style2,Paper and Size

These menus operate the same as style menu above.

Any item on the menu in a grey shading can not be selected either until

further choices have to be made as Letter Quality For FONT, or because

Printer is not in correct mode for item. Set top of paper only works in

IBM mode and becomes active only if IBM mode is selected from opening window.

Print

Open drop down window to print from User file, Input line, Or Print source

file with line numbers, date file list modified, date printed, size of file.

Print from user file

Opens window on screen, small left window contains list of files

Move mouse to Left window and select file required for printing with left

mouse button. If required File is not visable in window move slider button

Up or Down as Required. The selected file name appears in the Filename

window. Pressing Ok sends file to printer and Closes window If it

is not required to print press Cancel button to close window.

It is nessary to wait until file is printed before making further

selections from the menus. Also while the file selection window is

active further menus can not be selected until the window is closed.

Print User line

Opens up window from which a typed line of data can be sent to the printer.

Move to small window next to words "input", press left button colour of

window changes from black to grey. If line of data is already in window

move mouse to end of line and press left mouse button small bar cursor

appears use backspace key to clear data type required line and move

mouse to print button and press left mouse button to print line.

Escape button closes window.

Print Source File.

Print source code files with header at top of every page

comprising of Name of File and source directory, Size of file in Bytes,

Page number, Date source file last modified, Date of print out,

and line numbers. For best results limited columns to around 70 characters.

Change Directory

Use to change drive or Directory.

Options

Quick Set Menu

Quick set menu to select Elite, Proportional, Condensed, Emphasized,

Double Strike, Expanded, Italic, Underline in combination or single.

Switch Mode

Select Epson (default) or Ibm Mode of Operation.

Standard Character set

Selects standard character set.

Ibm Character set

Open menu from which Set 1 or 2 can be selected.

The above two menus are only active when Standard mode of operation is

selected from the opening menu. They DO NOT change mode of operation But

just change the Character set to IBM 1,2 or standard.

Immediate mode

When selected Paper moves up about 2" inches between printing to allow

user to read line. Further printing takeing place the printer moves the

paper up and down.

Cancel by selecting Cancel Immediate mode.

Exiting

By selecting exit and pressing return key. Or by pressing Alt-X key.

General

As the menu items toggle on and off as selected the printer is reset on

entering the program. Also a lot of programs reset the printer and will

not print in the manor set up from the menus.

As each code is sent to the printer it beeps to show the code has been correctly

received. If the printer does not beep make sure the correct mode has been

selected. Exit program and select mode again from opening menu.

The left and right margins column correct settings varies with FONT

type, and no check is made for correct settings left margin less then

right margin e.t.c. Refer to printer handbook, Page 76 in the case of

the Star LC10 for correct settings.

To Select required Top and Bottom margins first select required paper

length and then the required top and bottom margins. (Paper length resets

top, bottom margin settings).

See Update.doc .

Please Register the program By Sending 10 pounds to:-

MR E.W.DOBSON

12 WANSBECK CLOSE

SPENNYMOOR

CO.DURHAM

DL16 6XH

It may be possible to alter the program to different types of printers

if a Photo copy of required codes are sent.

(c) MR E.W.DOBSON 01/93

It is the users responability to operate the printer correctly.