�����������������������������������������������������������������Ŀ

�    This Program brought to you through the facilities of        �

�               ***  SIMPLE SOFTWARE COMPANY  ***                 �

�                        P.O BOX 8184                             �

�                    NEW ORLEANS, LA.  70182                      �

�             >>>>>>>> Custom Programming <<<<<<<<                �

�          Voice (504) 288-6550   Data (504) 283-9785             �

�   Database - Accounting - Telecommunications - DOS Utilities    �

�        Terry Tekippe, Ph.D, Gary M. Raymond, Programmers        �

�������������������������������������������������������������������

by Gary M. Raymond

Here is a summary of technical modem information collected from

many sources over the past few years. Hopefully it will serve you

as it has helped us. The information lacks great detail but has

all the elemental information necessary for code writers.

Standard IBM Default Values shown here.

Vector Address = IRQ# plus 8

PORT  DEC   HEX  ISR   VEC ADD   IRQ OCW1 MASK

COM1  1016  3F8  IRQ4  12d(0xC)  AND 0xEF (or) AND (NOT 0x10)

COM2  760   2F8  IRQ3  11d(0xB)  AND 0xF7 (or) AND (NOT 0x8)

COM3  1000  3E8  IRQ4  12d(0xC)  AND 0xEF (or) AND (NOT 0x10)

COM4  744   2E8  IRQ3  11d(0xB)  AND 0xF7 (or) AND (NOT 0x8)

IRQ1  AND 0xFD  (various bit mask operations for OCW1 register

IRQ2  AND 0xEF   depending on Interrupt Request Line in use)

IRQ3  AND 0xF7

IRQ4  AND 0xEF

IRQ5  AND 0xDF

IRQ6  AND 0xBF

IRQ7  AND 0x7F

NOTE: AT PC's may address many

additional serial port

addresses not mentioned here.

REGISTER   (EXAMPLE: MCR for COM1 = 3F8+4 or 3FC)

---------------------------------------------

IER  =  BASE  + 1  ;  Interrupt Enable Register

IIR  =  BASE  + 2  ;  Interrupt Identification Register

LCR  =  BASE  + 3  ;  Line Control Register

MCR  =  BASE  + 4  ;  Modem Control Register

LSR  =  BASE  + 5  ;  Line Status Register

MSR  =  BASE  + 6  ;  Modem Status Register

IER  BIT MAPPING

---------------------------------------------

BIT     Activates             Action

0    =  Data Received         Read RDR

1    =  THR empty             Output to THR

2    =  Data error/break      Read LSR

3    =  MSR change            Read MSR

4    =  zero

5    =  zero

6    =  zero

7    =  zero

IIR  BIT MAPPING

---------------------------------------------

BIT

0    =  more than 1 intr has occured

1-2  =  00  change in MSR        10    data received

01  THR empty            11    recv error/break

3    =  zero

4    =  zero

5    =  zero

6    =  zero

7    =  zero

LCR  BIT MAPPING

---------------------------------------------

BIT

0-1  =  Data Length      00=5bits           10=7bits

01=6bits           11=8bits

2    =  Stop Bits        0=1bit             1=2bits

3-5  =  Parity           000=Ignore         101=mark

100=ODD            111=space

110=EVEN

6    =  Break Condition  0=disabled         1=enabled

7    =  Port Toggle      0=Use THR/RDR/IER  1=Use BRDL/BRDH

LSR  BIT MAPPING

---------------------------------------------

BIT

0    =  Set DTR line active

1    =  Set RTS line active

2    =  USER BIT #1

3    =  USER BIT #2

4    =  UART loopback

5    =  zero

6    =  zero

7    =  zero

LSR  BIT MAPPING

---------------------------------------------

BIT

0    =  byte in RDR (received)

1    =  overrun in RDR

2    =  Parity error detected

3    =  Framing error (out of sync, no stop bit received)

4    =  Break Detect

5    =  THR empty

6    =  TSR empty (removes char from THR)

7    =  Time Out

MSR  BIT MAPPING

---------------------------------------------

BIT

0    =  change in [C]lear [T]o [S]end  CTS

1    =  change in [D]ata [S]et [R]eady DSR

2    =  change in ring indicator RI

3    =  change in [D]ata [C]arrier [D]etect  DCD

4    =  Clear to send ..... [CTS] set high

5    =  Data set ready .... [DSR] set high

6    =  Ring Indicator .... [RI]  set high

7    =  Data Carrier detect [DCD] set high

===================================================================

AT        ATTENTION COMMAND

A/        REPEAT COMMAND

A         ANSWER MODE COMMAND

B/B0      SELECT CCITT V.22 IN 1200 BPS

B1        SELECT BELL 212A

Dn        DIAL COMMAND (n = DIAL STRING, 0-9, *, #, !, W, R, AND,

E         DISABLE ECHO

E1        ENABLE ECHO

H         ON HOOK (HANG UP)

H1        OFF HOOK

I/I0      IDENTIFY

I1        DISPLAY 000

I2        DISPLAY OK

I3        DISPLAY MESSAGE PROGRAM ROM

L/L0      LOW VOLUME

L1        LOW VOLUME

L2        MEDIUM VOLUME

L3        MAXIMUM SPEAKER VOLUME

M/M0      DISABLE SPEAKER

M1        SPEAKER ON UNTIL CARRIER

M2        SPEAKER ALWAYS ON

M3        DISABLE SPEAKER WHEN DIALING OR CARRIER

O         GO ON LINE

O1        RETRAIN (2400 BPS MODE ONLY)

Q/Q0      ENABLE RESULT CODES

Q1        DISABLE RESULT CODES

Sn=X      S REGISTER WRITE (n = VALID S REG, X = ALLOWED VALUE SEE

&F COMMAND)

Sn?       READ SPECIFIED S REGISTER (n = ANY VALID S REGISTER)

V0        SELECT DIGIT RESULT CODE

V1        SELECT WORD RESULT CODE

X0        BLIND DIAL, DISPLAY "CONNECT"

X1        BLIND DIAL, DISPLAY "CONNECT 1200/2400"

X2        DETECT DIAL TONE, DISPLAY "CONNECT 1200/2400"

X3        DETECT BUSY TONE, DISPLAY "CONNECT 1200/2400"

X4        DIAL AND BUSY TONE DETECT, DISPLAY "CONNECT 1200/2400"

Y0        DISABLE LONG SPACE DISCONNECT

Y1        ENABLE LONG SPACE DISCONNECT

Z         SOFTWARE RESET

+++       ESCAPE CODE (GO TO COMMAND MODE IF ON LINE)

&C0       DCD ALWAYS ON

&C1       DCD FOLLOWS REMOTE CARRIER STATUS

&D0       IGNORE DTR

&D1       GO TO COMMAND WHEN DTR ON TO OFF

&D2       HANGUP AND GO TO COMMAND WHEN DTR ON TO OFF

&D3       POWER UP RESET WHEN DTR ON TO OFF

&F        FETCH FACTORY CONFIGURATION for S REGESTERS

&G0       NO GUARD TONE

&G1       550 HZ GUARD TONE

&G2       1800 HZ GUARD TONE

&J0       SELECT RJ-11/RJ-41S/RJ-45S

&J1       SELECT RJ-12/RJ-13

&M0       SELECT ASYNCHRONOUS MODE

&P0       MAKE BREAK RATIO FOR US (40/60)

&P1       MAKE BREAK RATIO FOR FOREIGN STANDARD (33/66)

&R0       CTS TRACKS RTS

&R1       CTS ON WHEN MODEM READY IN DATA MODE

&S0       DSR ALWAYS ON

&S1       DSR FOLLOWS RE232 SPECIFICATION

&T0       STOP TEST

&T1       LOCAL ALB TEST

&T3       LOCAL DLB

&T        ENABLE DLB REQUEST

&T5       DISABLE DLB REQUEST

&T6       REMOTE DLB

&T7       REMOTE DLB AND SELF TEST

&T8       LOCAL ALB AND SELF TEST

&V        VIEW PROFILE

S-REG   ADDRESS  DEFAULT  VALID RANGE   DESCRIPTION

S0      228      0        0-255         RING TO ANSWER ON

S1      229      0        0-255         RING COUNT

S2      230      43       0-127         ESCAPE CODE CHARACTER

S3      231      13       0-127         CR CHARACTER

S4      232      10       0-127         LF CHARACTER

S5      233      8        0-127         BS CHARACTER

S6      234      2        0-255         WAIT FOR DIAL TONE

S7      235      30       1-30          WAIT FOR DATA CARRIER

S8      236      2        0-255         PAUSE TIME FOR COMMA

S9      237      6        1-255(.1SEC)  CARRIER DET RESP. TIME

S10     238      14       1-255(.1SEC)  LOST CARRIER TO HANG UP

DELAY

S11     239                             TOUCH TONE DURATION

S12     240      50       0-255         ESCAPE CODE GUARD TIME

(1/50SEC)

S13     241                             NOT USED

S14     242      AAH                    BIT MAPPED OPTION REG

S15     243                             NOT USED

S16     244      0                      MODEM TEST OPTIONS

S17     245                             NOT USED

S18     246      0        0-255         TEST TIMER

S19     247                             NOT USED

S20     248                             NOT USED

S21     249      0                      BIT MAPPED OPTIONS REG

S22     250      76                     BIT MAPPED OPTIONS REG

S23     251      7                      BIT MAPPED OPTIONS REG

S24     252                             NOT USED

S25     253      5        0-255(.01SEC) DELAY TO DTR (SYNC ONLY)

S26     254      1        0-255(.01SEC) RTS TO CTS DELAY (HALF

DUP)

S27     255      40H                    BIT MAPPED OPTION REG

RESULT CODES

DIGIT     WORD            RESULT

0         OK              Command executed without error.

1         CONNECT         Carrier detected (300 bbs or 1200 bbs if

X0 is set.)

2         RING            Phone is ringing (will answer only if

auto answer is enabled.)

3         NO CARRIER      Carrier lost or never detected.

4         ERROR           Error in command line, line too long, or

invalid character format.

5         CONNECT 1200    Carrier detected at 1200 bbs if X1 set.

6         NO DIALTONE     Displayed if no dialtone is detected

within 5 seconds and X2 or X4 is set.

7         BUSY            Displayed when a busy signal is

detected and Z2 or X4 is set.

8         NO ANSWER       Displayed when "@" dial modifier is used

and phone continues to ring.

10        CONNECT 2400    Carrier detected at 2400 bbs. X1 or X2

must be set.

MODEM          BELL 103    BELL 212    V.22        V.22 bis

STANDARDS      300         1200        1200        2400

BELL 300       300         300         300**       300**

BELL 1200      1200*       1200        1200        1200

CCITT 1200     1200*       1200        1200        1200

CCITT 2400     2400*       1200        1200        2400

CCITT 2400     300*        1200         1200         2400

COMMAND             DESCRIPTION

AT                  COMMAND LINE PREFIX (ATTENTION CODE);

PRECEDES COMMAND LINES EXCEPT +++ (ESCAPE)

AND A/ (REPEAT) COMMANDS.

A/                 RE-EXECUTE LAST COMMAND LINE; (A/ IS NOT

FOLLOWED BY A CARRIAGE RETURN).

A                  OFF-HOOK IN ANSWER MODE.

5

B                  SELECTS CCITT V.22 OPERATION WHEN

COMMUNICATING AT 1200 BPS.

B1  (Default)      SELECTS BELL 212A OPERATION WHEN

COMMUNICATING AT 1200 BPS.

D                  DIAL NUMBER WHICH FOLLOWWD IN THE COMMAND

LINE.

E                  MODEM DOES NOT "ECHO" COMMANDS BACK TO

TERMINAL.

E1  (Default)      MODEM SECHOS COMMANDS BACK TO TERMINAL.

H  (Default)       ON-HOOK (HANG-UP) COMMAND.

H1                 GO OFF-HOOK.

I                  REQUEST PRODUCT IDENTIFICATION CODE.

I1                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

CHECKSUM.

I2                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

OK OR ERROR.

M                  SPEAKER OFF.

M1  (Default)      SPEAKER ON UNTIL CARRIER DETECTED.

M2                 SPEAKER ALWAYS ON.

M3                 SPEAKER ON UNTIL CARRIER DETECTED EXCEPT

DURING DIALING.

O                  RETURN TO ON-LINE STATE.

O1                 RETURN TO ON-LINE STATE AND INITIATE RETRAIN.

Q  (Default)       MODEM RETURNS RESULT CODES.

Q1                 MODEM DOES NOT RETURN RESULT CODES.

SRT=N              SET REGISTER R TO VALUE N.

SR?                REQUEST CONTENTS OF REGISTER R.

V                  SHORT FORM NUMERIC RESULT CODES.

V1  (Default)      FULL WORD RESULT CODES.

X                  300 BPS COMPATIBILITY MODE; CONNECT RESULT

6

CODE ENABLED.

X1                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; BUSY SIGNAL NOT DETECTED.

X2                 MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; MODEM

SEND BUSY RESULT CODE IF BUSY DETECTED.

X3                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; MODEM SENDS BUSY CODE IF BUSY

DETECTED.

X4  (Default)      MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; BUSY

SIGNAL DETECTED.

Z                  LOAD STORED CONFIGURATION PROFILE.

&C                 DCD ALWAYS ON.

&C1  (Default)     DCD ON INDICATES PRESENCE OF DATA CARRIER.

&D   (Default)     MODEM IGNORES DTR.

&D1                MODEM ASSUMES COMMAND STATE WHEN ON-TO-OFF

TRANSITION DETECTED ON DTR.

&D2                MODEM HANGS UP & ASSUMES COMMAND STATE.

AUTO ANSWER NOT DISABLED UNLESS EXTERNALLY

IMPLEMENTED BY (RI- # DTR-)= A6-.

&D3                MODEM ASSUMES INITIALIZATION STATE UPON

DETECTING AN ON-TO-OFF TRANSITION ON DTR.

&F                 LOAD FACTORY CONFIGURATION PROFILE.

&P  (Default)      PULSE DIAL MAKE/BREAK RATIO = 39/61 (UK/HK).

&P1                PULSE DIAL MAKE/BREAK RATIO = 33/67 (UK/HK).

&S                 DSR ALWAYS ON.

&S1                DSR OPERATES IN ACCORDANCE WITH EIA RS-232-C

SPEC.

&T                 TERMINATE TEST PROGRESS.

&T1                INITIATE LOCAL ANALOG LOOPBACK TEST.

&T8                INITIATE LOCAL ANALOG LOOPBACK WITH SELF TEST.

&Z                 STORE TELEPHONE NUMBER.

EXPLANATIONS

A     Go off hook and answer phone immediately. The Model 2400

immediately goes off hook, transmits the answer tone and

waits for a carrier from the remote mode.

Commands entered after the A command are ignored.

A/    Repeat previous command. The last command executed by the

Model 2400 will execute again. This command does not require

the "AT" prefix or <cr> termination.

Bn    Bell/CCITT mode select.

B0 - Selects CCITT V. 22 operation at 1200 bps.

B1 - Selects BELL212A operation at 1200 bps (Default).

Most 1200 bps modems in the United States operate using the

BELL212A specification. In order to communicate with a modem

conforming to the international CCITT V.22 standard, issue

the B0 command before dialing.

Dn    Dial the number immediately following this command where

n = digits 0 through 9 together with the dial modifiers

listed below.

Dial Modifiers:  P  Pulse dial

T  Tone dial

R  Originate call in answer mode

W  Wait for dial tone before dialing

,  Pause before dialing next digit

;  Return to command mode after dialing

@  Wait for quiet answer before dialing

!  Flash-hook

S  Dial the number stores in RAM

DIAL MODIFIERS

P                   PULSE DIAL

T                   TONE DIAL

,                   PAUSE

!                   FLASH

@                   WAIT FOR SILENCE

W                   WAIT FOR SECOND DIAL TONE

;                   RETURN TO COMMAND STATE AFTER DIALING

R                   ORIGINATE CALL IN ANSWER MODE

D                   DIAL STORED NUMBER

ALL S REGISTER COMMANDS ARE ENTERED IN THE AT COMMAND LINE.

EXAMPLE: AT S8=2 S2=43 S7=30

TO READ THE VALUE IN ANY REGISTER USE

AT S#? <CR>

THE MODEM SHOULD ECHO THE VALUE FOUND IN THE REGISTER #

REG  Values      Register function

S0   0           AUTO ANSWER OFF

1-255       AUTO ANSWER ON, ANSWER ON INDICATED # OF RINGS

S1 **0-255       RING COUNT (THIS IS A READ-ONLY REGISTER)

S2   0-127       ESCAPE CHARACTER  NORMALLY SET TO 43 OR "+"

127-255     ESCAPE FUNCTION DISABLED

S3  0-127        END OF LINE CHARACTER  NORMALLY SET TO 13 (CR)

S4  0-127        LINE FEED CHARACTER  NORMALLY SET TO 10 (LF)

S5  0-127        BACKSPACE CHARACTER  NORMALLY SET TO 8 (BS)

S6  2-255  SEC   PAUSE BEFORE DIALING (NORMALLY 2 SEC)

S7  1-255  SEC   WAIT FOR CARRIER (NORMALLY SET TO 30 SEC)

S8  0-255  SEC   PAUSE FOR COMMA IN DIAL STRING (NORMALLY SET TO 2 SEC)

S9  1-255  .1SEC CARRIER VALIDATION TIME NORMALLY 6 (.6 SEC)

S10 1-255  .1SEC LOSS OF CARRIER DISCONNECT DELAY NORMALLY 14 (1.4 SEC)

S11 50-127 .1SEC TOUCH TONE DURATION. THE DEFAULT IS 90.

S12 20-255 .02SE ESCAPE SEQENCE PAUSE  NORMALLY 50 (1.0 SEC)

S13 *            POWER UP ASYNC DATA FORMAT (USE CHART BELOW)

S14 *            OPTION STATUS DEFULT AA (HEX CODE) (USE CHART BELOW)

S16 *            DEFULT 0. TEST STATUS (USE CHART BELOW)

S18 0-255 SEC    DEFULT 0. TEST TIMER (REMOTE TEST)

S21 *            DEFULT 0.OPTION STATUS  (USE CHART BELOW)

S22 *            OPTION STATUS, DEFULT 76. (USE CHART BELOW)

S23 *            OPTION STATUS  NORMALLY 07 (USE CHART BELOW)

S25 0-255 SEC    DTR DETECT DELAY NORMALLY 5 SEC

ASYNC MODE, SYIC AUTO-DIAL AND SYIC MANUAL DIAL MODE .05 SEC

S26 0-255 MS     RTS TO CTS DELAY (1 MS)

S27 *            OPTION STATUS, DEFULT 40 (HEX CODE)  (USE CHART BELOW)

* REGISTERS USED BY MODEM PROCESSOR. DO NOT ALTER THESE

* READ ONLY REGISTERS

S13 DATA FORMAT:

POSITION  VALUE  FUNCTION

BIT 0      -    NOT USED

BIT 1      -    NOT USED

BIT 2      0    PARITY OFF

1    PARITY ON

BIT 3      0    ODD PARITY

1    EVEN PARITY

BIT 4      0    7 DATA BITS

1    8 DATA BITS

BIT 5      0    BIT 8=0

1    BIT 8=1

BIT 6      -    NOT USED

BIT 7      -    NOT USED

S14 DATA FORMAT:

BIT 0      -    NOT USED

BIT 1      0    LOCAL CHARACTER ECHO OFF

1    LOCAL CHARACTER ECHO ON

BIT 2      0    STATUS DISPLAYS ON

1    STATUS DISPLAYS OFF

BIT 3      0    STATUS CODES SENT

1    STATUS MESSAGES SENT

BIT 4      0    MODEM RESPONDS TO COMMANDS

1    MODEM IGNORES COMMANDS

BIT 5      0    TONE-DIALING

1    PULSE-DIALING

BIT 6      -    NOT USED

BIT 7      0    ANSWER MODE

1    ORIGINATE MODE

S16 DATA FORMAT:

BIT 0      0    ANALOG LOOP OFF

1    ANALOG LOOP ACTIVE

BIT 1      -    NOT USED

BIT 2      0    DIGITAL LOOP OFF

1    DIGITAL LOOP ACTIVE

BIT 3      0    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS OFF

1    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS ACTIVE

BIT 4      0    REMOTE DIGITAL LOOP OFF

1    REMOTE DIGITAL LOOP ACTIVE

BIT 5      0    SELF TEST RDL OFF

1    SELF TEST RDL ACTIVE

BIT 6      0    SELF TEST ANALOG LOOP OFF

1    SELF TEST ANALOG LOOP ACTIVE

BIT 7      -    NOT USED

S21 DATA FORMAT:

BIT 0      0    RJ11/RJ41/45 JACK TYPE

1    RJ12/FJ13 JACK TYPE

BIT 1      -    NOT USED

BIT 2      0    CTS FOLLOWS RTS

1    CTS FORCED ON

BIT 3&4    0    MODEM IGNORES DTR

1    MODEM ASSUMES COMMAND STATE WHEN DTR TURNS OFF

2    MODEM ASSUMES COMMAND STATE AND DISABLES AUTO-ANSWER

WHEN DTR TURNS OFF

3    MODEM RESETS WHEN DTR TURNS OFF

BIT 5      0    DCD FORCED ON

1    DCD FOLLOWS CARRIER

BIT 6      0    DSR FORCED ON

1    DSR BEHAVES NORMALLY

BIT 7      0    LONG SPACE DISCONNECT DISABLED

1    LONG SPACE DISCONNECT ENABLED

S22 DATA FORMAT:

BIT 0&1    1    LOW SPEAKER VOLUME

2    MEDIUM SPEAKER VOLUME

3    HIGH SPEAKER VOLUME

BIT 2&3    0    SPEAKER ALWAYS OFF

1    SPEAKER ON UNTIL CARRIER DETECTED

2    SPEAKER ALWAYS ON

3    SPEAKER OFF DURING DIALING THEN ON UNTIL CARRIER DETECTED

BIT 4&5&6  0    X COMMAND IN EFFECT

4    X 1 COMMAND IN EFFECT

5    X 2 COMMAND IN EFFECT

6    X 3 COMMAND IN EFFECT

7    X 4 COMMAND IN EFFECT

BIT 7      0    39/61 MAKE/BREAK RATIO (PULSE DIALING)

1    33/67 MAKE/BREAK RATIO

S23 DATA FORMAT:

BIT 0      0    REMOTE DIGITAL LOOP REQUEST DENIED

1    REMOTE DIGITAL LOOP REQUEST GRANTED

BIT 1&2    0    0-300 BPS

1    600 BPS

2    1200 BPS

3    2400 BPS

BIT 3      -    NOT USED

BIT 4&5    0    EVEN PARITY

1    PARITY BIT ALWAYS 0

2    ODD PARITY

3    PARITY BIT ALWAYS 1

BIT 6&7    0    NO GUARD TONES

1    550 HZ GUARD TONE

2    1800 HZ GUARD TONE

S27 DATA FORMAT:

BIT 0&1    0    ASYNCRONOUS MODE

1    ASYNC/SYNC MODE

2    SYNC AUTO-DIAL

3    SYNC MANUAL DIAL

BIT 2      0    PUBLIC PHONE LINE

1    LEASED LINE

BIT 3      -    NOT USED

BIT 4&5    0    INTERNAL CLOCK USED

1    EXTERNAL CLOCK USED

2    SLAVED CLOCK USED

BIT 6      0    CCITT V.22 PROTOCOL (1200 BPS)

1    BELL 212A (1200 BPS)

BIT 7      -    NOT USED

OTHER MODEM COMMANDS:

AT       STANDARD LEAD IN SEQUENCE (ATTENTION)

ATA      FORCE ANSWER

ATD      DIAL (ATD ###-#####)

P     PULSE (ATDP ###-####)

T     TONE  (ATDT ###-####)

ATE1     ENABLE LOCAL CHARACTER ECHO

ATE0     DISABLE LOCAL CHARACTER ECHO

ATH1     FORCE LINE CONNECT

ATH0     FORCE LINE DISCONNECT

ATI      ECHO VERSION TYPE

ATL1     SPEAKER VOLUME LOW

ATL2     SPEAKER VOLUME MEDIUM

ATL3     SPEAKER VOLUME HIGH

ATM0     SPEAKER ALWAYS OFF

ATM1     SPEAKER ON EXCEPT WHILE CARRIER PRESENT

ATM2     SPEAKER ALWAYS ON

ATM3     SPEAKER ON EXCEPT WHILE DIALING AND CARRIER PRESENT

ATO      GO ONLINE AFTER ESCAPE OR WHILE IN COMMAND MODE

ATQ0     STATUS DISPLAY ON

ATQ1     STATUS DISPLAY OFF

ATV0     STATUS MESSAGES SENT

ATV1     STATUS CODES SENT

ATX0     "CONNECT" (CODE 1)  ALL SPEEDS NO DIAL TONE DETECTION

ATX1     "CONNECT" (CODE 1)  0-300 BPS OR "CONNECT 1200" FOR 1200 OR

"CONNECT 2400" FOR 2400 BPS NO DIAL TONE DETECTION

ATX2     WAIT FOR DIAL TONE (CONNECT SPEEDS SHOWN AS FOR X1)

ATX3     DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATX4     WAIT FOR DIAL TONE, DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATZ      RESET TO POWER UP CONDITION (CLEARS ALL REGISTERS)

2400 MODEM SPECIAL COMMANDS:    (* = FACTORY SETTING STANDARD)

ATB0     CCITT V.22 AT 1200 BPS

ATB1     BELL 212A AT 1200 BPS *

AT &F    RESTORE TO FACTORY C0NFIGURATION

AT &G    NO GUARD TONE *

AT &G1   550 HZ GUARD TONE

AT &G2   1800 HZ GUARD TONE

AT &J    RJ11/RJ41S/RJ45 JACK *

AT &J2   RJ12/RJ13 JACK

AT &L    REGULAR PHONE LINE *

AT &L1   LEASED LINE

AT &M    ASYNC OPERATION *

AT &M1   ASYNC/SYNC OPERATION

AT &M2   SYNC AUTO DIAL

AT &M3   SYNC MANUAL DIAL

AT &P    39/61 PULSE MAKE/BREAK RATIO *

AT &P1   33/67 PULSE MAKE/BREAK RATIO

AT &T4   GRANT RDL TEST REQUEST *

AT &T5   DENY RDL TEST REQUEST

AT &V    VIEW STATUS OF ALL MODEM SETTINGS. SEE ALL S REGISTER SETTINGS AND

INTERNAL MODEM COMMAND SETTINGS ON ONE SCREEN!

AT &X    SYNC CLOCK INTERNAL *

AT &X1   SYNC CLOCK EXTERNAL

AT &X2   SYNC CLOCK SLAVED

AT &Z    STORE DIAL COMMAND LINE

DATA INTERFACE (2400)

AT &C    DCD ALWAYS ON

AT &C1   DCD ON WHILE CARRIER PRESENT *

AT &D    DTR IGNORED *

AT &D1   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE)

AT &D2   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE, TURN

AUTO ANSWER OFF)

AT &D3   DTR FAIL DISCONNECT ENABLED (RESET MODEM)

EOF()