Interrupt Address Function                   BIOS Entry ( * = see table )

Vector    (Hex)

0      00-03  Divide by Zero             Unused (D11 A-81)

1      04-07  Single Step                Unused (D11 A-81)

2      08-0B  Nonmaskable                NMI_INT (A-72)

3      0C-0F  Breakpoint                 Unused (D11 A-81)

4      10-13  Overflow                   Unused (D11 A-81)

5      14-17  Print Screen               PRINT_SCREEN (A-81)

6      18-1B  Reserved                   Unused (D11 A-81)

7      1C-1F  Reserved                   Unused (D11 A-81)

8      20-23  Time of Day                TIMER_INT (A-79)

9      24-27  Keyboard                   KB_INT (A-28)

A      28-2B  Reserved       [      ]    Unused (D11 A-81)

B      2C-2F  Communications [ 8259 ]    Unused (D11 A-81)

C      30-33  Communications [      ]    Unused (D11 A-81)

D      34-37  Disk                       Unused (D11 A-81)

E      38-3B  Diskette                   DISK_INT (A-43)

F      3C-3F  Printer                    Unused (D11 A-81)

10 *    40-43  Video                      VIDEO_IO (A-46)

11 *    44-47  Equipment Check            EQUIPMENT (A-72)

12 *    48-4B  Memory                     MEMORY_SIZE_DET (A-71)

13 *    4C-4F  Diskette/Disk              DISKETTE_IO (A-35)

14 *    50-53  Communications             RS232_IO (A-22)

15      54-57  Cassette                   CASSETTE_IO (A-72)

16 *    58-5B  Keyboard                   KEYBOARD_IO (A-24)

17 *    5C-5F  Printer                    PRINTER_IO (A-45)

18      60-63  Resident BASIC             F600:0000

19      64-67  Bootstrap                  BOOT_STRAP (A-20)

1A *    68-6B  Time of day                TIME_OF_DAY (A-79)

1B      6C-6F  Keyboard Break             DUMMY_RETURN (A-81)

1C      70-73  Timer Tick                 DUMMY_RETURN (A-81)

1D      74-77  Video Initialization       VIDEO_PARMS (A-48)

1E      78-7B  Diskette Parameters        DISK_BASE (A-44)

1F      7C-7F  Video Graphics Characters  0

20 *    80-83            General program termination

21      84-87            DOS function request

22      88-8B            Called program termination address

23      8C-8F            Control Break termination address

24      90-93            Critical error handler

25      94-97            Absolute disk read

26      98-9B            Absolute disk write

27      9C-9F            Terminate but stay resident

Interrupt Address Function                   BIOS Entry

Vector    (Hex)

0      00-03  Divide by Zero             Unused (D11 A-81)

1      04-07  Single Step                Unused (D11 A-81)

2      08-0B  Nonmaskable                NMI_INT (A-72)

3      0C-0F  Breakpoint                 Unused (D11 A-81)

4      10-13  Overflow                   Unused (D11 A-81)

5      14-17  Print Screen               PRINT_SCREEN (A-81)

6      18-1B  Reserved                   Unused (D11 A-81)

7      1C-1F  Reserved                   Unused (D11 A-81)

8      20-23  Time of Day                TIMER_INT (A-79)

9      24-27  Keyboard                   KB_INT (A-28)

A      28-2B  Reserved       [      ]    Unused (D11 A-81)

B      2C-2F  Communications [ 8259 ]    Unused (D11 A-81)

C      30-33  Communications [      ]    Unused (D11 A-81)

D      34-37  Disk                       Unused (D11 A-81)

E      38-3B  Diskette                   DISK_INT (A-43)

F      3C-3F  Printer                    Unused (D11 A-81)

�������������������������������������������������������������������������Ŀ

�  10      40-43  Video                      VIDEO_IO (A-46)              �

�������������������������������������������������������������������������Ĵ

�   � AH=0 Set Mode                                                       �

�   ���������������������������������������������������������������������Ĵ

�   �  � AL=0 40X25 BW (POR Default)   �   AL=4 320X200 Color             �

�   �  � AL=1 40X25 Color              �   AL=5 320X200 BW                �

�   �  � AL=2 80X25 BW                 �   AL=6 640X200 BW                �

�   �  � AL=3 80X25 Color              �   AL=7 80X25 Monochrome Card     �

�   ���������������������������������������������������������������������Ĵ

�   � AH=1 Set Cursor Type                                                �

�   ���������������������������������������������������������������������Ĵ

�   �  � CH= Bits 4-0 = Start Line for Cursor                             �

�   �  � CL= Bits 4-0 = End Line for Cursor                               �

�   ���������������������������������������������������������������������Ĵ

�   � AH=2 Set Cursor Position                                            �

�   ���������������������������������������������������������������������Ĵ

�   �  � DH,DL= Row,Column (0,0 is upper left)                            �

�   �  � BH= Page Number (Must be 0 for graphics modes)                   �

�   ���������������������������������������������������������������������Ĵ

�   � AH=3 Read Cursor Position                                           �

�   ���������������������������������������������������������������������Ĵ

�   �  � BH= Page Number (Must be 0 for graphics modes)                   �

�   �  � On Exit DH,DL= Row,Column                                        �

�   �  �         CH,CL= Cursor Mode Currently Set                         �

�   ���������������������������������������������������������������������Ĵ

�   � AH=4 Read Light Pen Position                                        �

�   ���������������������������������������������������������������������Ĵ

�   �  � On Exit AH=0 Light Pen Switch Not Down/Not Triggered             �

�   �  �         AH=1 Valid Light Pen Value in Registers                  �

�   �  �         DH,DL= Row,Column of Character Light Pen Position        �

�   �  �         CH= Raster Line (0-199)                                  �

�   �  �         BX= Pixel Column (0-319 or 639)                          �

�   ���������������������������������������������������������������������Ĵ

�   � AH=5 Select Active Display Page                                     �

�   ���������������������������������������������������������������������Ĵ

�   �  � AL= New Page (0-7 for Modes 0&1, 0-3 for Modes 2&3)              �

�   ���������������������������������������������������������������������Ĵ

�   � AH=6 Scroll Active Page Up                                          �

�   ���������������������������������������������������������������������Ĵ

�   �  � AL= Number of Lines, Input Lines Blanked at Bottom of Window     �

�   �  �     0 Means Blank Entire Window                                  �

�   �  � CH,CL= Row,Column of Upper Left Corner of Scroll                 �

�   �  � DH,DL= Row,Column of Lower Right Corner of Scroll                �

�   �  � BH= Attribute to be used on blank line                           �

�   ���������������������������������������������������������������������Ĵ

�   � AH=7 Scroll Active Page Down                                        �

�   ���������������������������������������������������������������������Ĵ

�   �  � AL= Number of Lines, Input Lines Blanked at Top of Window        �

�   �  �     0 Means Blank Entire Window                                  �

�   �  � CH,CL= Row,Column of Upper Left Corner of Scroll                 �

�   �  � DH,DL= Row,Column of Lower Right Corner of Scroll                �

�   �  � BH= Attribute to be used on blank line                           �

�   ���������������������������������������������������������������������Ĵ

�   � AH=8 Read Attribute/Character at Current Position                   �

�   ���������������������������������������������������������������������Ĵ

�   �  � BH= Display Page (Valid for alpha modes only)                    �

�   �  � On Exit AL= Character Read                                       �

�   �  �         AH= Attribute of Character Read (alpha modes only)       �

�   ���������������������������������������������������������������������Ĵ

�   � AH=9 Write Attribute/Character at Current Position                  �

�   ���������������������������������������������������������������������Ĵ

�   �  � BH= Display Page (Valid for alpha modes only)                    �

�   �  � CX= Count of Characters to Write                                 �

�   �  � AL= Character to Write                                           �

�   �  � BL= Attribute of Character (alpha)/Color of Character (graphics) �

�   �  �     If Bit 7 = 1 color value is exclusive or'd with current value�

�   ���������������������������������������������������������������������Ĵ

�   � AH=10 Write Character Only at Current Position                      �

�   ���������������������������������������������������������������������Ĵ

�   �  � BH= Display Page (Valid for alpha modes only)                    �

�   �  � CX= Count of Characters to Write                                 �

�   �  � AL= Character to Write                                           �

�   ���������������������������������������������������������������������Ĵ

�   � AH=11 Set Color Palette                                             �

�   ���������������������������������������������������������������������Ĵ

�   �  � BH= Palette Color ID being set (0 or 1)                          �

�   �  � BL= Color Value to be used with that Color ID                    �

�   �  �     Color ID = 0 selects background color (0-15)                 �

�   �  �     Color ID = 1 selects palette to be used                      �

�   �  �     Palette = 0: Green(1)/Red(2)/Yellow(3)                       �

�   �  �     Palette = 1: Cyan(1)/Magenta(2)/White(3)                     �

�   ���������������������������������������������������������������������Ĵ

�   � AH=12 Write Dot                                                     �

�   ���������������������������������������������������������������������Ĵ

�   �  � DX= Row Number                                                   �

�   �  � CX= Column Number                                                �

�   �  � AL= Color Value                                                  �

�   �  �     If Bit 7 = 1 color value is exclusive or'd with current value�

�   ���������������������������������������������������������������������Ĵ

�   � AH=13 Read Dot                                                      �

�   ���������������������������������������������������������������������Ĵ

�   �  � DX= Row Number                                                   �

�   �  � CX= Column Number                                                �

�   �  � AL= Returns the Dot Read                                         �

�   ���������������������������������������������������������������������Ĵ

�   � AH=14 Write Teletype to Active Page                                 �

�   ���������������������������������������������������������������������Ĵ

�   �  � AL= Character to write                                           �

�   �  � BL= Foreground color in graphics mode                            �

�   ���������������������������������������������������������������������Ĵ

�   � AH=15 Current Video State                                           �

�   ���������������������������������������������������������������������Ĵ

�   �  � On Exit AL= Mode currently set                                   �

�   �  �         AH= Number of character columns on screen                �

�   �  �         BH= Current active display page                          �

���������������������������������������������������������������������������

11      44-47  Equipment Check            EQUIPMENT (A-72)

���������������������������������������������������������������������Ŀ

� AX is set to indicate attached I/O                                  �

���������������������������������������������������������������������Ĵ

�  � Bit 15,14= Number of printers attached                           �

�  � Bit 13   = Not used                                              �

�  � Bit 12   = Game I/O attached                                     �

�  � Bit 11,10,9= Number of RS232 cards attached                      �

�  � Bit 8    = Not used                                              �

�  � Bit 7,6  = Number of diskette drives (only if Bit 0 = 1)         �

�  �    00=1, 01=2, 10=3, 11=4                                        �

�  � Bit 5,4= Initial video mode                                      �

�  �    00= Unused                                                    �

�  �    01= 40X25 BW using color card                                 �

�  �    10= 80X25 BW using color card                                 �

�  �    11= 80X25 BW using BW card                                    �

�  � Bit 3,2= Planar RAM size (00=16K,01=32K,10=48K,11=64K)           �

�  � Bit 1  = Not used                                                �

�  � Bit 0  = IPL from diskette (there are drives present in system)  �

�����������������������������������������������������������������������

12      48-4B  Memory                     MEMORY_SIZE_DET (A-71)

���������������������������������������������������������������������Ŀ

� AX= Number of contiguous 1K blocks of memory                        �

�����������������������������������������������������������������������

13      4C-4F  Diskette/Disk              DISKETTE_IO (A-35)

���������������������������������������������������������������������Ŀ

� AH=0 Reset the Diskette System                                      �

���������������������������������������������������������������������Ĵ

� AH=1 Read the Status of the system into AL                          �

���������������������������������������������������������������������Ĵ

� AH=2 Read the desired sectors into memory                           �

���������������������������������������������������������������������Ĵ

� AH=3 Write the desired sectors from memory                          �

���������������������������������������������������������������������Ĵ

� AH=4 Verify the desired sectors                                     �

���������������������������������������������������������������������Ĵ

� AH=5 Format the desired track                                       �

���������������������������������������������������������������������Ĵ

�  �For Read/Write/Verify/Format                                      �

�  ������������������������������������������������������������������Ĵ

�  �  � Input                                                         �

�  �  ���������������������������������������������������������������Ĵ

�  �  �  � DL= Drive Number (0-3) (value checked)                     �

�  �  �  � DH= Head Number (0-1)                                      �

�  �  �  � CH= Track Number (0-39)                                    �

�  �  �  � CL= Sector Number (1-8) (not used for format)              �

�  �  �  � AL= Number of sectors(1-8) (not used for format)           �

�  �  �  � ES:BX= address of buffer (not required for verify)         �

�  �  ���������������������������������������������������������������Ĵ

�  �  � Output                                                        �

�  �  ���������������������������������������������������������������Ĵ

�  �  �  � AH= status of operations                                   �

�  �  �  ������������������������������������������������������������Ĵ

�  �  �  �  � 80H= attachment failed to respond                       �

�  �  �  �  � 40H= seek operation failed                              �

�  �  �  �  � 20H= NEC controller has failed                          �

�  �  �  �  � 10H= Bad CRC on diskette read                           �

�  �  �  �  � 09H= attempt to DMA accross 64K boundary                �

�  �  �  �  � 08H= DMA overrun on operation                           �

�  �  �  �  � 04H= requested sector not found                         �

�  �  �  �  � 03H= write attempted on write protected diskette        �

�  �  �  �  � 02H= address mark not found                             �

�  �  �  �  � 01H= bad command passed to diskette I/O                 �

�  �  �  ������������������������������������������������������������Ĵ

�  �  �  � CY= 0 successful operation, AH=0                           �

�  �  �  � CY= 1 Failed operation                                     �

�����������������������������������������������������������������������

14      50-53  Communications             RS232_IO (A-22)

AH=0 Initialize communications port

AL=Parms:

7       6       5       4       3       2       1       0

------Baud ------       -Parity--    StopBit    --WordLength

000 - 110               X0 - none     0 - 1     10 - 7 bits

001 - 150               01 - odd      1 - 2     11 - 8 bits

010 - 300               11 - even

011 - 600

100 - 1200

101 - 2400

110 - 4800

111 - 9600

On Exit, registers set as in AH=3

AH=1 send the character in AL over the commo line

On Exit, AH bit 7 = 1 if unable to transmit byte

AH=2 receive a character in AL from commo line

AH=3 return the commo port status in AX

AH contains the line status

Bit 7 = time out

Bit 6 = trans shift register empty

Bit 5 = tran holding register empty

Bit 4 = break detect

Bit 3 = framing error

Bit 2 = parity error

Bit 1 = overrun error

Bit 0 = data ready

AL contains the modem status

Bit 7 = received line signal detect

Bit 6 = ring indicator

Bit 5 = data set ready

Bit 4 = clear to send

Bit 3 = delta receive line signal detect

Bit 2 = trailing edge ring detector

Bit 1 = delta data set ready

Bit 0 = delta clear to send

DX= parameter indicating which RS232 card (0,1)

15      54-57  Cassette                   CASSETTE_IO (A-72)

16      58-5B  Keyboard                   KEYBOARD_IO (A-24)

AH=0 read the next ASCII character struck from the keyboard

return the result in AL, scan code in AH

AH=1 set the Z flag to indicate if an ASCII character is

available to be read ZF=0 code is available

AH=2 return the current shift status in AL

Bit 7= insert state is active

Bit 6= caps lock state has been toggled

Bit 5= num lock state has been toggled

Bit 4= scroll lock state has been toggled

Bit 3= alternate shift key depressed

Bit 2= control shift key depressed

Bit 1= left shift key depressed

Bit 0= right shift key depressed

17      5C-5F  Printer                    PRINTER_IO (A-45)

AH=0 print the character in AL

On Exit, AH=1 if character could not be printed (time out)

AH=1 initialize the printer port

AH=2 read the printer status into AH

Bit 7= 1=not busy

Bit 6= 1=acknowledge

Bit 5= 1=out of paper

Bit 4= 1=selected

Bit 3= 1= I/O error

Bit 2,1 unused

Bit 0= 1=Time Out

DX= Printer to be used (0,1,2)

18      60-63  Resident BASIC             F600:0000

19      64-67  Bootstrap                  BOOT_STRAP (A-20)

1A      68-6B  Time of day                TIME_OF_DAY (A-79)

AH= 0 read the current clock setting

On Exit CX= high portion of count

DX= low portion of count

AL= 0 if timer has not passed 24 hours since last read

AH=1 set the current clock

CX= high portion of count

DX= low portion of count

1B      6C-6F  Keyboard Break             DUMMY_RETURN (A-81)

1C      70-73  Timer Tick                 DUMMY_RETURN (A-81)

1D      74-77  Video Initialization       VIDEO_PARMS (A-48)

1E      78-7B  Diskette Parameters        DISK_BASE (A-44)

1F      7C-7F  Video Graphics Characters  0

20      80-83            General program termination

21      84-87            DOS function request

22      88-8B            Called program termination address

23      8C-8F            Control Break termination address

24      90-93            Critical error handler

25      94-97            Absolute disk read

26      98-9B            Absolute disk write

27      9C-9F            Terminate but stay resident

Function Call Summary   ( * = not documented )

Number  Description

0     Program Terminate

1     Keyboard input.

2     Display output.

3     Auxiliary (Async) input.

4     Auxiliary (Axync) output.

5     Printer output.

6     Direct console I/O.

7     Direct console input without echo.

8     Console input without echo.

9     Print string.

A     Buffered keyboard input.

B     Check standard input status.

C     Clear keyboard buffer and invoke a keyboard function.

D     Disk reset.

E     Select disk.

F     Open file.

10     Close a file

11     Search for the first entry.

12     Search for the next entry.

13     Delete file.

14     Sequential read.

15     Sequential write.

16     Create file.

17     Rename file.

18    *Unused.

19     Current disk.

1A     Set disk transfer address.

1B     Allocation table information.

1C     Allocation table information for specific drive.

1D    *Unused.

1E    *Unused.

1F    *Get the default DOS Parameter Block in DS:BX

20    *Unused.

21     Random read.

22     Random write.

23     File size.

24     Set random record field.

25     Set interrupt vector.

26     Create a new program segment.

27     Random block read.

28     Random block write.

29     Parse filename.

2A     Get date.

2B     Set date.

2C     Get time.

2D     Set time.

2E     Set/reset verify switch.

2F     Get DTA.

30     Get DOS version number.

31     Terminate process and remain resident.

32    *Get the Dos Parameter Block of disk # in DL

33     Ctrl-Break check.

35     Get vector.

36     Get disk free space.

37    *Get switch character

38     Return country dependent information (international).

39     Create a subdirectory.

3A     Remove a directory entry.

3B     Change the current directory.

3C     Create a file.

3D     Open a file.

3E     Close a file handle.

3F     Read from a file or device.

40     Write to a file or device.

41     Delete a file from a specified directory.

42     Move file read/write pointer.

43     Change file mode.

44     I/O control for devices.

45     Duplicate a file handle.

46     Force a duplicate of a handle.

47     Get current directory.

48     Allocate memory.

49     Free allocated memory.

4A     Modify allocated memory blocks.

4B     Load or execute a program.

4C     Terminate a process.

4D     Retrieve the return code of a sub-process.

4E     Find first matching file.

4F     Find next matching file.

50    *Set PSP

51    *Get PSP

52    *????

53    *Set initial Dos Parameter Block

54     Get verify state.

55    *Duplicate PSP and make the new one active

56     Rename a file.

57     Get/Set a file's date and time.

#################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################