Program STSORBIT PLUS Satellite Orbit Simulation                   Page 1

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

(Enhanced Version for 286/386/486 Computers)

Version  9331

July 25, 1992

by David H. Ransom, Jr.

Rancho Palos Verdes, CA  USA

Bulletin Board Systems

----------------------

RPV ASTRONOMY BBS

(310) 541-7299 @ 14400/9600/2400 Baud

RPV HOTLINE BBS

(310) 544-8977 @ 2400/1200 Baud

BRIEF PROGRAM INSTRUCTIONS

--------------------------

This file is intended to help the experienced user get started with

program STSORBIT PLUS (referred to as STSPLUS hereafter). It is NOT

intended to be a substitute for the full program documentation!

STSPLUS is a program to track the NASA Space Shuttle and other

satellites. The program requires NASA/NORAD 2-line elements for orbital

data and may be used with any satellite for which 2-line data is available.

The following steps should get the program "up and running" with as

little fuss and bother as practical.

1.   A hard disk is required. Copy all files to a subdirectory called

STSPLUS.

2.   Log into the STSPLUS subdirectory using the following commands (which

assume drive C:):

C:

CD \STSPLUS

If the program has been received as a set of self-extracting archive

files with names like SOP9331A.EXE, SOP9331B.EXE and EARTH3.EXE,

execute each of these programs in turn; this will extract the actual

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 2

program and data files. (The self-extracting archive files may be

deleted if they have been saved on a floppy disk.)

3.   If you have used a previous version of STSPLUS, note that STSPLUS may

not read data from file STSPLUS.INI if it was written by a prior,

incompatible program version. This will eliminate problems if the

format of the .INI file has changed and will reset all features to

their default conditions. If difficulties are encountered, DELETE FILE

STSPLUS.INI AND RESET ALL OPTIONS MANUALLY!

4.   Enter the following command to start the program:

STSPLUS            (for all CGA/EGA/VGA color systems)

STSPLUS /M         (for all EGA/VGA monochrome systems)

5.   The STSPLUS.EXE file is stored on disk in compressed form. It is

decompressed at load time; this may cause a brief delay depending upon

the computer type used. Once the program is properly loaded, a screen

will appear which requests that you set the UTC Offset for your time

zone. Follow the on-screen instructions and enter the UTC Offset and

the Daylight Flag.

6.   The distribution package includes a set of US Space Command 2-line

elements with a name like TLE232.TXT. This contains orbital data on

over 170 different satellites. Press F2 to load orbital information.

The screen will display a list of files having the extension .TXT. The

TLEnnn.TXT file should be one of them. Enter the file name at the

prompt and press ENTER.

7.   The program will ask what satellite you wish to view with a default

response of "HST" (meaning the Hubble Space Telescope). Press ENTER to

accept the default or enter another satellite name (such as "MIR" or

"UARS"). If the name of the data file begins with the letters "STS",

the default satellite name will be "STS". You may also enter the

satellite's NORAD Number, such as "#16609" for MIR.

8.   The program will search the data file for the requested satellite and

display its orbital data if found. If this is the satellite you wish,

press ENTER when asked. Press SPACE to continue searching.

9.   After the satellite has been selected, STSPLUS will display a map of

the world and the ground track of the satellite. The satellite itself

will be a white symbol along the ground track. EGA and VGA systems

will display in color; CGA and HGC systems will display in monochrome.

10.  Press ENTER at any time to return to the Main Menu.

11.  STSPLUS has many features which are fully described in the program

documentation. Please read that documentation!

12.  The latest version of STSPLUS and current US Space Command 2-line

elements are always available on my bulletin board system (BBS) noted

at the beginning of this document. Eash BBS uses a power controller;

if it hasn't answered after the THIRD RING, hang up and call back in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 3

two minutes. The systems are often busy, so please be patient.

USING MISSION ELAPSED TIME WITH 2-LINE ELEMENTS

-----------------------------------------------

The standard NASA/NORAD 2-line orbital element format does not include

the launch time or launch date. However, this information is obviously

required if Mission Elapsed Time (MET) is to be displayed. That information

must therefore be obtained independently and manually entered into the

program. The following steps are required:

1.   Start program STSPLUS (see above).

2.   Now load the desired 2-line elements using Function Key F2 on the Main

Menu. Press ENTER to return from the ground track display to the Main

Menu.

3.   Press Function Key F5 to enter Launch Time and Launch Date. Enter the

appropriate information. Note that if you enter UTC or GMT Launch Time

(by adding the letter "U" or "G" at the end of the time, upper or

lower case), STSPLUS assumes that the date you enter will also be the

UTC/GMT Launch Date.

4.   The information will be displayed for approval. Press ENTER to accept

the data, or press SPACE BAR to repeat the process.

5.   Once the data have been accepted, press Function Key F6 to display

Mission Elapsed Time. Press F6 again to return to Time Elapsed Since

Epoch. While the ground track is displayed, Function Key F5 may be

pressed to switch between the two elapsed time calculations. (Press

Function Key F1 for help when the map is displayed with rectangular

projection. Help is NOT available with the orthographic projection.)

If a launch time and date have not been entered, these commands will

have no effect and time since epoch will always be displayed.

6.   The launch time and launch date will be saved in file STSPLUS.LTD and

need only be entered once unless it must be changed for some reason.

NOTE: Estimated Pre-Launch 2-Line Elements for the Space Shuttle

always use a "dummy" NORAD Number (such as "00051" for STS-51). Once

US Space Command has assigned the actual NORAD Number, the launch time

and date must be re-entered.

The simulated time, taking into account any fast time that may have

been in effect up to that point, is shown at the lower left of the screen.

For reference, the launch or epoch time and the actual local time (as

determined by the computer's internal DOS clock) are also displayed. The

MET (Mission Elapsed Time) or T+Epoch display at the lower right of the

screen shows the current elapsed time in the mission and will include an

additional message, "(x10)" or "(x60)", when fast time is in effect.

The combination of the FAST and PAUSE modes will let you "look

ahead" and then freeze the display at a time of interest -- such as when

the satellite enters your circle of visibility. You can then use the "+"

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 4

and "-" keys to move the satellite position back and forward in time (using

Function Key F6 to select the time step interval, 1, 10 or 60 seconds) to

determine the times and positions throughout a pass.

USERS WITHOUT A MATH COPROCESSOR CHIP

-------------------------------------

STSPLUS makes very complex calculations to determine the position of

the Earth, Satellite, and Sun. In addition, graphics operations required to

display the maps and the satellite track and position also require many

calculations. Slow computers or computers not equipped with a math

coprocessor chip (8087, 80287, 80387, etc.) may not be able to perform all

of the required calculations within the desired one second.

Note that the Sun terminator feature is NOT automatically disabled;

however, slower computers without a math coprocessor chip may be unable to

keep up with real time if this feature is enabled. From the Main Menu, use

F10+F3+F8 to disable this feature if necessary.

PROGRAM REGISTRATION

--------------------

See the file README for information on registering STSPLUS. While I

do not require registration of STSPLUS, I've put a great deal of time and

effort into the program for more than two years. Registering your copy and

sending along the very modest donation of $10.00 will encourage me to

continue maintaining and improving the program.

David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, CA 90274 USA

#########################################################################################################################