This is a document listing VT102/ANSI codes as observed by experiments

with Telemate 3.01. Most codes are also supported in Telix and FD's terminal.

The "#"'s in code represents numbers in ASCII digits, such as "12".

All the codes below are preceded by an Esc (ASCII code 27).

[#;#H        Position cursor on screen

[#A          Cursor Up

[#B          Cursor Down

[#C          Cursor Forward

[#D          Cursor Backward

[#E          Repeat a CRLF

[#F          Repeat a reverse CRLF (upwards)

[s           Save Cursor Position

[u           Restore Cursor Position

[0K or [K    Erase Line

[1K          Deletes current cursor position to beginning of line

[2K          Deletes current line containing cursor

[0J or [J    Erase from cursor to end of screen

[1J          Erase from beginning of screen to cursor

[2J          Erase Display

[6n          Device Status Report

[#;#R        Cursor Position Report

[#@          Insert spaces and pushes text at right towards right

[#P          Delete characters and pull text at right towards left

[#L          Insert lines and pushes text below down

[#M          Deletes lines and pulls text below up (or ANSI music prefix code)

[4h          Insert mode on

[4l          Insert mode off

E            Unconditial equivalent of CR and LF (Regardless of CR-LF setting)

D            Linefeed (Screen scroll if at bottom)

M            Reverse Linefeed (Screen scroll if at top)

[#;#;  ...etc... ;#m

All the "#"'s are the numbers below. For example,

"[0m" turns all attributes off, and "[0;5;31m" resets

the attributes, turns on bold, and sets the foreground

color to red.

0    All Attributes off, set color to non-bold white

1    Bold on

5    Blink on

7    Reverse video on

8    Concealed on

30   Black foreground

31   Red foreground

32   Green foreground

33   Yellow foreground

34   Blue foreground

35   Magenta foreground

36   Cyan foreground

37   White foreground

40   Black background

41   Red background

42   Green background

43   Yellow background

44   Blue background

45   Magenta background

46   Cyan background

47   White background