AI-LANBIOS Low-Level Driver For XIRCOM Pocket Ethernet Adapter III

(PE3AI50.DOC)

-----------------------------------------------------

(C) Copyright 1991 ARTISOFT, Inc.

(Revised 6/26/93)

Portions (C) Copyright 1993 Xircom, Inc.

INTRODUCTION

============

The PE3AI50.COM low-level AI-LANBIOS driver allows you to use your Xircom

Pocket Ethernet Adapter III in a LANtastic 5.x network.  The driver interfaces

with the following models:

PE3-10B2   Thin Ethernet adapter

PE3-10BT   Twisted Pair Ethernet adapter

In order for the Xircom adapter to operate with the LANtastic Network

Operating System (NOS), you must first install the adapter as instructed in

the Xircom installation guide, then run the supporting software (PE3AI50.COM

and AILANBIO.EXE).  PE3AI50.COM communicates with the Xircom adapter, while

AILANBIO.EXE provides high-level communications across the local area

network.  Because of this, you cannot run any network software until after

you load PE3AI50.COM and AILANBIO.EXE.  (Note: For information on running

AILANBIO.EXE, refer to your LANtastic manuals.)

There are command line switches that you can use when you run PE3AI50.COM and

AILANBIO.EXE.  These switches allow you greater flexibility in using these

programs.  For information on the AILANBIO.EXE command line options refer

to SWITCHES.DOC in your LANtastic installation directory.  For information

on PE3AI50.COM command line switches, refer to "PE3AI50.COM COMMAND LINE

OPTIONS" in this file.

RUNNING PE3AI50.COM

===================

The syntax for running PE3AI50.COM is

PE3AI50[switches . . . ] [ ; comment]

where "switches" denotes any optional command line switches.  If you are

using more than one switch in a command line, use a space or a forward

slash (/) to separate the arguments.  If a switch takes a value (such as

when you designate the DOS multiplex number with the MPX= switch) the

switch must be followed by an equal sign (=) or a colon (:).  All switch

values are checked to make sure they are valid.  Illegal values produce

error messages.

These are valid switch formats:

SWITCH

/SWITCH

SWITCH:VALUE

/SWITCH=VALUE

You would use one of the first two formats for a switch that does not take

a value.  For example:

PE3AI50/HELP

You would use the third or fourth formats for a switch that takes a value.

Such as:

PE3AI50/MPX=D7

PE3AI50.COM COMMAND LINE OPTIONS

================================

The valid PE3AI50.COM switches appear below.  The letters "ddd" denote that

you must provide a decimal number.  The letters "hh" denote a required

hexadecimal value.  Values enclosed in brackets "[]" indicate default

values if the switch is omitted.  Values after the brackets denote the

valid range of variables for the switch.

----------

@switch-file

This option specifies that further switches are to be taken from a

switch file.  Any switches after the @switch-file name on the command line

will be used, and those switch settings take precedence over any settings

in the switch file.  You may invoke switch files from within switch files

as often as you wish.  The file should contain valid switches and may contain

comment characters (;) at the beginning of each line or after switches.

For example,

PE3AI50 @setup

The file SETUP contains

; PE3AI50.COM AI-LANBIOS driver setup file

;

MPX=D7            ; Use Multiplex number D7

IRQ=5             ; Use interrupt request 5

VERBOSE           ; Display verbose information

----------

HELP or ?

This switch instructs PE3AI50.COM to display information about the valid

command line switches, but does not install the PE3AI50.COM driver.  For

example,

PE3AI50/help

PE3AI50 ?

See the messages section for a detailed description of the HELP display

output.

---------

IEEE

This switch causes all transmitted packets to be in the IEEE 802.3 standard

format instead of the default XEROX format.  When using this switch

PE3AI50.COM will receive both XEROX and IEEE 802.3 packets.

----------

IRQ=d [auto] Range 5 to 7

This switch specifies the interrupt request (IRQ) level used by the parallel

port.  Normally this switch is not required, since by default the driver

automatically determines the IRQ level of the port.  However, in certain

cases, such as having devices connected to more than one parallel port, a

unique IRQ must be specified.  The IRQ switch value must match the IRQ of

the parallel port used by the adapter.

----------

LINKDISABLE

This switch, only applicable to the PE3-10BT model, tells the adapter to not

check for link integrity upon reception or transmission of a packet.  It

should only be used if a PE3-10BT adapter is connected to older 10-BaseT

hardware that does not support link integrity.

----------

LPT=d [auto] Range 1 to 3

This switch allows you to specify the parallel port where the Pocket Ethernet

Adapter III is installed.  Normally this switch is not required, since by

default the driver automatically determines the LPT number of the port in

use.  However, if the automatic process adversely affects a device on another

parallel port or fails to detect the adapter, you will need to use this switch

to specify a port.

----------

MPX=hh [C7] C0-FF

This switch specifies the DOS multiplex (MPX) number to use for

communication with AILANBIO.EXE.  You will need to change this value only

if the default MPX number is in use by another application or NETBIOS.

Multiplex numbers 00 through BF are reserved for DOS use; therefore, you

must use a multiplex number between C0 and FF.

----------

NON

This switch forces the driver to operate in non-bidirectional parallel port

mode.  Normally this switch should not be used, since the driver automatically

tests the parallel port and determines the optimum mode for that port.

However, you may wish to try the NON switch if improper operation is

encountered with the automatically selected mode.

----------

PACKET_SIZE=dddd [1500] 600-1500

This switch specifies the maximum size of transmitted IEEE 802.3 or

XEROX-type packets.  You may need to change this value when using certain

bridges that do not support the full 1500-byte packet size.  Even when you

reduce the packet size, the low-level driver can still receive larger packets.

----------

REMOVE

This switch removes PE3AI50.COM from memory.  You must remove TSR programs in

the opposite order they were loaded.  For example, if you loaded PE3AI50.COM

and then AILANBIO, you must remove AILANBIO before PE3AI50.COM.

----------

VERBOSE

This switch causes the PE3AI50.COM driver to be installed and detailed

information about the configuration of the PE3AI50.COM driver to be displayed.

See the messages section for a detailed description of the VERBOSE output.

---------

XEROX

This switch causes all transmitted packets to be in the XEROX format.  This

switch isn't necessary since by default the XEROX format is used.  When using

the XEROX format, PE3AI50.COM will receive both XEROX and IEEE 802.3 packets.

PE3AI50.COM MESSAGES

====================

The PE3AI50.COM driver produces two types of messages: Informative and Error.

All error messages are preceded by the text "ERROR:" and in those cases

the PE3AI50.COM driver is not installed.

--------

MESSAGE:

Xircom Pocket Ethernet Adapter III

PE3AI50 AI-LANBIOS(R) V4.05 Compatible Driver (Vx.xx)

(C) Copyright 1993 by ARTISOFT Inc.   Portions (C) Copyright 1993 Xircom, Inc.

This message appears each time the low-level driver is invoked.

--------

MESSAGE:

Hardware Configuration     LPT1:, IRQ7, Bidirectional

---- PE3AI50 AI-LANBIOS(R)-Compatible Driver Installed ----

This or a similar message appears after the low-level driver is

successfully installed.

--------

MESSAGE:

---- PE3AI50 AI-LANBIOS(R)-Compatible Driver NOT Installed ----

This message appears if the low-level driver is not installed.  The driver

will not be installed if HELP information is displayed or an error occurred.

--------

MESSAGE:

Command line              /verbose

IEEE 802.3 node address   0080C7OC69AE          MPX interface number   C7

Network packet size       1500                  Bytes of memory used   9968

Packet type               XEROX

Hardware Configuration    LPT1:, IRQ7, Bidirectional

This message appears if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.

--------

MESSAGE:

Valid command line switches:

@<filename>   Read command line switches from file

HELP or ?     Display this help information

REMOVE        Unload driver from memory

VERBOSE       Display detailed configuration information

MPX           MPX interface number in hex  (C0 - FF,  default C7)

PACKET_SIZE   Maximum packet size in decimal  (600 - 1500,  default 1500)

XEROX         Set the packet type to XEROX  (Default)

IEEE          Set the packet type to IEEE 802.3

IRQ           Interrupt used by the adapter

LPT           LPT port number  (1, 2, or 3)

NON           Force non-bidirectional parallel port mode

LINKDISABLE   Disable link integrity

Note:  The parallel port number, interrupt, and optimum mode are determined

automatically unless overridden with the LPT, IRQ, and NON switches.

This message is displayed when the HELP or ? switch is used.  Each valid

switch is listed with the acceptable range of values (if any) and the base

(hex or decimal) that the numbers must be entered in.  The low-level driver

is not installed when this switch is used.

-------

MESSAGE:

---- PE3AI50 AI-LANBIOS(R)-Compatible Driver REMOVED ----

The PE3AI50.COM driver has been removed from memory.

--------

ERROR:   Can't REMOVE -- PE3AI50 AI-LANBIOS(R)-Compatible Driver is not loaded

Meaning: You have issued the PE3AI50/REMOVE command and do not have

PE3AI50.COM loaded into memory.

Remedy:  Only issue this command after the program is loaded.

--------

ERROR:   Can't REMOVE -- AI-LANBIOS(R) still installed

Meaning: You have issued the PE3AI50/REMOVE command while AILANBIO.EXE

is still in memory.

Remedy:  Remove AILANBIO.EXE before removing PE3AI50.COM.

-------

ERROR:   Can't REMOVE -- interrupts re-hooked

Meaning: You have attempted to remove PE3AI50.COM and the interrupts are in

use by another TSR, or PE3AI50.COM was not the last TSR loaded.

Remedy:  Remove any TSRs loaded after PE3AI50.COM.  For example, if you load

PE3AI50.COM then AILANBIO.EXE, you must first remove AILANBIO.EXE,

then you can remove PE3AI50.COM.

------

ERROR:   Cannot open switch file - @file

Meaning: The file "file" cannot be opened as an indirect switch file.

Remedy:  The file must exist before it can be used as an indirect file.

-------

ERROR:   Installed PE3AI50 is different version

Meaning: You have loaded a version of the PE3AI50.COM program and attempted to

remove it from memory with the /REMOVE switch.  The version with

which you are attempting to remove PE3AI50.COM is not the same

version that was loaded.

Remedy:  Remove PE3AI50.COM with the same version of the program that was

loaded.

------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

------

ERROR:   Illegal character after switch name - ??????xnnnn

Meaning: The switch ?????? is followed by a character other than "=" or ":",

or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) occurrred in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value "ssss"

represents the lowest acceptable value and the value "eeee"

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

------

ERROR:   MPX number hh is already in use - Try another number

Meaning: The multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

------

ERROR:   A Pocket LAN Adapter could not be found

Meaning: PE3AI50.COM was unable to communicate with the Pocket Ethernet

Adapter III.

Remedy:  Make sure the adapter is connected to your computer and powered up.

Also be sure any command line switches match the hardware

configuration.  If the error persists, contact Xircom Technical

Support.

------

ERROR:   A Pocket LAN Adapter driver is already loaded

Meaning: You have attempted to load a second copy of PE3AI50.COM into the same

adapter.

Remedy:  Load only one copy of PE3AI50.COM per adapter.

------

ERROR:   Pocket Ethernet Adapter III Address EEPROM unreadable.

Meaning: PE3AI50.COM was unable to read the node address in the adapter EEPROM

chip.

Remedy:  Contact Xircom Technical Support.

------

ERROR:   Pocket Ethernet Adapter III failed initialization.

Meaning: PE3AI50.COM was unable to reset the adapter's NIC chip.

Remedy:  Try loading the program with the NON switch.  If that doesn't work,

contact Xircom Technical Support.

------

ERROR:   Pocket Ethernet Adapter III failed memory test.

Meaning: PE3AI50.COM was unable to read or write properly to the adapter's

on-board RAM.

Remedy:  Try loading the program with the NON switch.  If that doesn't work,

contact Xircom Technical Support.

------

ERROR:   The selected hardware configuration is not supported

Meaning: PE3AI50.COM was unable to operate properly using the selected

LPT, IRQ, and parallel port mode.

Remedy:  Be sure any values you specify for LPT and IRQ match those used by

the parallel port.  If this message appears when using the default

auto-configuration, try specifying the NON switch.

------

ERROR:   This driver does not support the currently connected adapter

Meaning: The installed adapter is not a Xircom Pocket Ethernet Adapter III.

Remedy:  This driver only supports the Xircom Pocket Ethernet Adapter III.

If your Xircom adapter is a different model, contact Xircom Technical

Support for a driver that supports that model.

TESTING PE3AI50.COM RETURN CODES (ERROR LEVEL)

==============================================

You may test the PE3AI50.COM return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No errors.  PE3AI50.COM driver installed or removed successfully,

or Help requested.

1     MPX number already in use

2     Switch error

3     Adapter not found or hardware error

4     Cannot remove - PE3AI50.COM not loaded

5     Cannot remove - Different PE3AI50.COM version loaded

6     Cannot remove - Interrupts rehooked

7     Cannot remove - AILANBIO.EXE still installed

------------------