IC v2.1

Copyright (C) 1993, Geoff Friesen B.Sc.

All rights reserved.

#-1TABLE OF CONTENTS#-0

INTRODUCTION.........................................................3

IC.COM...............................................................4

EXISTING INTERNAL COMMANDS...........................................5

SND API.............................................................10

CREATING YOUR OWN INTERNAL COMMANDS.................................18

CONCLUSION..........................................................23

COMMAND.COM contains the user interface to the MS-DOS/PC-DOS  operating

system. This  program displays  a prompt  and solicits  input from  the

user  in an  endless  loop.  The  input  consists  of  a  command  name

followed  by an  optional  arguments  list referred  to as  the command

tail. The  command name  and command  tail taken  together are referred

to  as the  command  line.  Commands are  divided into  two categories:

internal  and external.  Internal  commands  are  commands  that remain

resident in  memory. External  commands are  resident on  disk and must

be loaded  into memory  before they  can be  run. Internal commands are

stored in memory  for performance reasons.  Examples of these  commands

include CLS,  ERASE, MD,  and TYPE.  COMMAND.COM contains  a variety of

internal commands that  perform useful tasks  but there is  always room

for  improvement.  Beginning  with  version  3.30,  the  DOS  operating

system provided support  for extending this  set of internal  commands.

My program,  Internal Commands  (IC) takes  advantage of  this support.

This documentation  is divided  into four  sections. The  first section

looks at the IC.COM program.  The second section details each  internal

command found in  IC.COM. The third  section talks about  sound and the

SND API.  The final  section provides  an indepth  look at writing your

own  internal commands  and  grafting  them into  IC. I  had originally

planned to include an  index but this manual  is alot smaller and  less

complex  than I  envisioned.  I  do  not  think you  will have  trouble

finding relevant material.

- 3 -

#-1IC.COM#-0

IC.COM is  an easy-to-install  memory resident  program. Type  "IC" and

press the RETURN/ENTER key and  IC.COM will be installed provided  that

IC is not  already resident. IC  can be removed  by typing "IC  /U" and

pressing  RETURN/ENTER  provided  that  IC.COM  is  the  most  recently

installed program  and that  no external  installable command  has been

grafted (more on grafting later)  into IC. IC is capable  of displaying

the following messages at the command line.

� this version of DOS is not supported

IC   works   with   DOS   versions   3.30   through   5.0    inclusive.

� bad command line

IC  takes an  optional  /U  argument to  uninstall itself  from memory.

Nothing else is permitted on the command line.

� not resident, cannot uninstall

You cannot  remove IC  from memory  if it  was never  installed in  the

first place.

� unable to uninstall, reboot if desired

IC is not the most  recently installed memory resident program.  Remove

resident programs  that were  installed after  IC before  attempting to

remove  IC  (or  reboot  your  computer  -  you  may  need  to   change

AUTOEXEC.BAT  if it  contains  commands  that  reinstall  all of  these

programs by placing the  line that installs IC.COM  at the end of  this

batch file).

� successfully uninstalled

IC was able to uninstall itself from memory.

� already resident, not reinstalled

IC  is already  resident  in  memory  and  has  not  been  reinstalled.

� successfully installed

IC was able to install itself into memory.

� ungraft installed command before removing

An external installable command has been grafted into IC via the  GRAFT

command.  This command  must  be  ungrafted before  IC can  be removed.

Some commands take over  interrupt vectors and these  interrupt vectors

must be  restored before  IC is  removed otherwise  a system crash will

occur. A command is ungrafted  by typing "GRAFT" without any  argument.

- 4 -

#-1EXISTING INTERNAL COMMANDS#-0

IC.COM contains  the following  internal commands:  CLOCK, CLS, CSHAPE,

GRAFT, HIDE, ICH, LOCATE, ND,  NMDIR, NLOCK, PLAY, PPORT, PSEND,  SHOW,

SLOCK, SOUND,  TALLY, and  WAIT. This  section documents  each command.

� CLOCK: toggle CAPS LOCK state

Syntax: CLOCK [state]

The CLOCK command can  be followed by an  optional state. A zero  turns

off  CAPS LOCK  while  a  one turns  CAPS LOCK  on. If  no argument  is

passed  then the  current  state  is  displayed.  For example,  CLOCK 0

turns  off   CAPS  LOCK   and  CLOCK   1  turns   CAPS  LOCK  back  on.

� CLS: clear screen

Syntax: CLS [attr] [>filespec]

The  CLS command  can  be  followed  by  an  optional  video  attribute

(represented  by attr)  to  clear  the  screen  to  a  particular color

combination. Two hexadecimal digits  must be specified if  an attribute

will be  used. Either  a digit  0-9 or  alphabetic letter  A-F (or a-f)

can be  used for  specifying a  hexadecimal digit.  The left-most digit

controls the  background attribute.  The right-most  digit controls the

foreground.  For  example,  CLS  70  clears  the  screen  to  BLACK  on

LIGHTGRAY.  The  >filespec  is  optional  and  not  commonly  used.  If

specified then  the ANSI  CLS sequence  ESC[2J is  written to filespec.

The attribute  defined by  attr can  be specified  as xx  where each  x

specifies  a  4-bit  hexadecimal  digit  and  is  decoded  as  follows.

xx: Bbbbffff

The most significant bit of  the left-most digit (B) controls  blinking

and is set  to one if  blinking is desired.  Three b-bits follow.  They

define the background color as follows.

000 - BLACK

001 - BLUE

010 - GREEN

011 - CYAN

100 - RED

101 - MAGENTA

110 - BROWN

111 - LIGHT GRAY

The  second  hex  digit  consists  of  four  f-bits.  They  define  the

foreground color.

- 5 -

0000 - BLACK

0001 - BLUE

0010 - GREEN

0011 - CYAN

0100 - RED

0101 - MAGENTA

0110 - BROWN

0111 - LIGHT GRAY

1000 - DARK GRAY

1001 - LIGHT BLUE

1010 - LIGHT GREEN

1011 - LIGHT CYAN

1100 - LIGHT RED

1101 - LIGHT MAGENTA

1110 - YELLOW

1111 - WHITE

The 70 that  was mentioned in  the previous example  (CLS 70) would  be

decoded as follows.

70 = 01110000

The blink bit  is 0 so  there is no  blinking. The three  ones match up

to  a LIGHTGRAY  background.  The  four  zeros  match  up  to  a  BLACK

foreground.

� CSHAPE: change cursor shape

Syntax: CSHAPE [shape]

The CSHAPE command is  used to change the  cursor shape. The new  shape

is  passed via  an optional  shape argument.  If no  shape argument  is

given,  CSHAPE  displays  the  current  cursor  shape  in   hexadecimal

notation.  The shape  argument must  be specified  as four  hexadecimal

digits.   As    an   example,    CSHAPE   2000    hides   the   cursor.

� GRAFT: graft .com file into IC

Syntax: GRAFT [filespec]

The GRAFT  command takes  a specially  written .com  file and merges it

into IC making  this .com file  into a new  internal command. The  .com

file  is specified  by  filespec  (and  may  contain a  drive and  path

designation).  The  creation  of  these  .com  files  is  an   advanced

technique  described  in  section  four.  GRAFT  specified  by   itself

removes any  currently grafted  command. For  example, GRAFT  ASCII.COM

grafts  the  ASCII  .com  file   into  the  memory  image  of   IC.COM.

� HIDE: hide file

Syntax: HIDE filespec

The HIDE  command is  used to  make files  or directories invisible (to

minimize  snooping). The  filespec  argument  can  contain  an optional

drive  and  path  specification.  For  example,  HIDE   C:\AUTOEXEC.BAT

causes  AUTOEXEC.BAT to  be  hidden  from  DIR  listings. Make  sure to

unhide AUTOEXEC.BAT or DOS cannot find  and execute it the next time  a

reset occurs.

- 6 -

� ICH: get help on IC commands

Syntax: ICH [command]

The  ICH command  provides quick  help on  IC commands.  If no  command

argument is provided, ICH lists available commands.

� LOCATE: locate cursor on screen

Syntax: LOCATE row, column

The  LOCATE command  positions  the  cursor  to  the specified  row and

column  on the  current  page.  LOCATE  does  not check  to see  if the

row/column  values are  valid.  Row  and  column  are given  as decimal

numbers relative to one.

� ND/NMDIR: rename directory

Syntax: ND oldname newname

NMDIR oldname newname

The ND  or NMDIR  command renames  an existing  directory whose name is

oldname to newname.  Because this command  uses the File  Control Block

or FCB method of file  access, only a drive and  filename specification

is allowed  (paths cannot  be specified).  For example,  ND wordpro  wp

renames the wordpro directory to wp.

� NLOCK: toggle NUM LOCK state

Syntax: NLOCK [state]

The NLOCK command can  be followed by an  optional state. A zero  turns

off NUM LOCK while  a one turns NUM  LOCK on. If no  argument is passed

then the  current state  is displayed.  For example,  NLOCK 0 turns off

NUM    LOCK    and    NLOCK    1    turns    NUM    LOCK    back    on.

� PLAY: play music in the background

Syntax: PLAY [filespec]

The  PLAY command  plays music  in the  background. A  music string  is

read  into a  buffer  from  filespec  and  passed onto  the SND  API. A

maximum of 512 bytes  will be read. If  no filespec appears after  PLAY

then  any  current  background  music  is  terminated.  Section   three

documents the  SND API  and the  PLAY language.  PLAY displays messages

to indicate file I/O trouble, SND API in use, or a syntax error in  the

music  string. For  example, PLAY  C:\MUSIC\SONAT will  play the  music

string  found in  file  SONAT  of directory  MUSIC on  drive C:.  Three

sample music files have been included: BIRTHDAY, HORNPIPE, and  SAINTS.

� PPORT: view or swap printer port addresses

Syntax: PPORT [S]

The PPORT command displays the  addresses of the LPT1 and  LPT2 printer

ports if  S is  not specified.  The S  argument stands  for swap. S may

be entered in uppercase or  lowercase. PPORT S swaps the  addresses for

LPT1    and   LPT2    and    then    displays    the    new   settings.

- 7 -

� PSEND: send character(s) to the printer

Syntax: PSEND portnum "string"

The PSEND command sends the string of characters specified between  the

" characters to  a printer connected  to the printer  port specified by

portnum. A value of 0 specifies  LPT1 and a value of 1  specifies LPT2.

No other value  is accepted. Provision  has been made  to enter control

codes  and quote  characters into  the string  via the  ^ character.  A

decimal value  from 0  to 255  inclusive must  be specified immediately

after  the ^.  As an  example: psend  0 "^34String  Doubled ^34"  sends

"String Doubled" to LPT1.

� SHOW: show hidden file

Syntax: SHOW filespec

The  SHOW command  unhides  a  hidden file.  The filespec  argument may

contain  optional drive  and path  information. See  HIDE for  example.

� SLOCK: toggle SCROLL LOCK state

Syntax: SLOCK [state]

The SLOCK command can  be followed by an  optional state. A zero  turns

off SCROLL LOCK  while a one  turns SCROLL LOCK  on. If no  argument is

passed  then the  current  state  is  displayed.  For example,  SLOCK 0

turns  off  SCROLL  LOCK  and  SLOCK  1  turns  SCROLL  LOCK  back  on.

� SOUND: generate sound

Syntax: SOUND frequency, duration

The SOUND command  adds sound capability  to batch files.  The tone has

the  specified  frequency  and  persists  for  the  specified  duration

measured  in eighteenths  of  a  second.  At  least one  digit must  be

specified  for both  arguments.  Once  sound is  started, it  cannot be

interrupted  without  resetting  the  computer.  Therefore,  use  small

values when specifying  the duration. SOUND  expects it's values  to be

entered  as decimal  numbers.  Sound  is not  generated if  a frequency

less than  20 HZ  is specified.  SOUND uses  the internal  SND API.  If

the SND  API is  in use  (background music)  and SOUND  requests access

then SOUND displays  a message indicating  this fact. More  information

on  the SND  API  can  be  found  in  section  three.  SOUND  1000,  36

generates  a 1000  HZ  tone  for  about  2  seconds.  The lowest  valid

frequency is 20 HZ.

� TALLY: tally file sizes

Syntax: TALLY filespec

The  TALLY  command  accumulates  the  sizes  of  all  files   matching

filespec.  Files must  have normal,  system, or  hidden attributes  (or

any combination) to be considered.  An example: TALLY *.doc tallys  the

sizes  of all  files with  a doc  extension in  the current  directory.

- 8 -

� WAIT: suspend execution

Syntax: WAIT hh:mm

The WAIT  command suspends  execution until  a specified  time. A  time

value is  specified as  two hour  digits followed  by a  colon and  two

minute digits.  The time  is entered  in 24-hour  format (WAIT  13:00 -

wait until one PM). A leading zero  must be given if hour is less  than

10   (WAIT   09:00).   Wait    aborts   if   any   key    is   pressed.

- 9 -

#-1SND API#-0

This section  documents the  SND API  along with  the PLAY language and

ends  with a  complete  discussion  of  how  sound is  generated on  PC

computers compatible with the IBM PC standard.

Sound support has  been added to  the BIOS by  extending interrupt 1ah.

The  SND API  is  accessed  by  loading  the AH  register with  ffh and

loading a  function code  in register  AL (0-5  are valid codes). Other

registers, as defined by that  particular function, are loaded and  int

1ah is  issued. The  SND API  places a  status code  in AH  that can be

read  by a  program  when  int  1ah  returns.  A  status  code  of  255

indicates    that   an    invalid    code    was    passed    in    AL.

The SND  API was  designed to  work with  both resident and nonresident

programs.  A  resident  program  might  activate  while  a  nonresident

program is running and this  can cause problems. Suppose a  nonresident

has enabled sound  via the SND  API and is  delaying (also via  the SND

API) and  a resident  activates. Suppose  that this  resident wants  to

use the SND API as well and no safeguards have been built into the  SND

API.  The resident  would  enable  sound to  a frequency  that probably

differs from  the nonresident's  choice. If  the original  frequency is

not restored before  exit, the nonresident  will be reactivated  with a

totally different sound.  It is not  possible to restore  the frequency

by examining the  TIMER ports. Therefore,  the SND API  prevents access

while sound  is in  effect. A  status code  of 1  is returned  in AH if

this is the case.

The following functions are defined.

� Function 0: Get version number

mov ax, ff00h

int 1ah

Exit: AL = 2 (version number)

AH = 0 (no function currently in progress)

AH = 1 (another function in progress)

Function 0  makes it  possible to  test the  SND API  to see if another

function  is in  progress.  A  resident  can  use  this  information to

decide  whether or  not it  should pop  up. A  resident program  should

execute a  code sequence  like the  following before  accessing the SND

API.

cli

mov ax, ff00h

int 1ah

cmp ah, 1

jz SND_IN_USE

mov ax, ff02h

mov dx, 1000

int 1ah

SND_IN_USE:

sti                                    - 10 -

In the above example, a resident program wants to enable sound at  1000

HZ. Before it can do so, the  resident must make sure that the SND  API

is not  currently being  used (perhaps  a foreground  program is  using

background music). The first thing  that the resident must do  is clear

interrupts.  Why? Suppose  that  a  second resident  tied to  the timer

tick  interrupt  and  activating   18.2  times  per  second   has  been

installed. Suppose this  other resident wants  to use background  music

and that background  music is not  currently active. If  interrupts are

not cleared then the following can happen.

1.  Our resident  calls  int  1ah and  gets a  return code  of 0  in AH

signalling that the SND API is not active.

2. The  second resident  activates and  gets the  all clear  signal and

then initiates background music.

3. Our resident gets to the  cmp instruction and sees a return  code of

0 so it starts the enable sound function.

4. Our resident receives an error  code indicating that the SND API  is

in use even though it was not in step 1.

The  SND API  disables  interrupts  until it  has set  an active  flag.

Therefore, once int 1ah has  been called, it is guaranteed  to succeed.

However, if interrupts are enabled in the current resident, then  there

is no guarantee that another resident will not activate and  monopolize

the SND API. Interrupts need only  be disabled for a short duration  as

seen in the example.

� Function 1: Generate a delay in timer ticks

mov ax, ff01h

mov dx, ticks

int 1ah

Exit: AH = 0 (success)

AH = 1 (another function in progress)

� Function 2: Enable sound

mov ax, ff02h

mov dx, frequency

int 1ah

Exit: AH = 0 (success)

AH = 1 (another function in progress)

AH = 2 (frequency less than 20 HZ)

� Function 3: Disable sound

mov ax, ff03h

int 1ah

Exit: AH = 0 (success)

AH = 1 (another function in progress)

- 11 -

� Function 4: Start background music

mov ax, ff04h

lds dx, music_string

int 1ah

Exit: AH = 0 (success)

AH = 1 (another function in progress)

AH = 2 (syntax error in music string)

Version 2 does  not return an  error code if  a syntax error  occurs in

the music  string. Instead,  music is  stopped. However,  your programs

should test for an error code of  2 because I may support this code  in

a future version of IC.

� Function 5: Stop background music

mov ax, ff05h

int 1ah

Exit: AH = 0 (success)

This  function can  be called  whether or  not another  function is  in

progress.  Since sound  is  disabled,  this  function  should  only  be

called when background music is in effect.

-----------------

The PLAY language

-----------------

The SND API contains an  internal language modelled after the  language

found in the BASIC PLAY command.  I refer to this language as  the PLAY

language. In addition to the  notes A, A#, B, C,  C#, D, D#, E, F,  F#,

G, and G#; the following commands are defined.

� O (set octave)

Syntax: O#

Set the current octave  to the octave specified  by # (# can  take on a

value from  1 to  5 inclusive).  The default  octave is  1 which is two

octaves   below  the   octave   containing   middle   C   (octave   3).

� L (set note length)

Syntax: L#

Set the  length of  all succeeding  notes to  that defined  by # (# can

take on a value of 1 for  whole notes, 2 for half notes, 4  for quarter

notes,  8 for  eighth notes,  or 6  for sixteenth  notes). The  default

length is 1.

� R (rest)

Syntax: R

Generate a rest equal to the  length of a note using the  most recently

specified note length.

- 12 -

� X (cycle)

Syntax: X

Reset    music   to    start    of    music    string    and    repeat.

� S (switch frequency table)

Syntax: S

Toggle back and forth between frequency tables.

� 0 (select normal frequency table)

Syntax: 0

This   table    defines   music    the   way    it   normally   sounds.

� 1 (select alternate frequency table)

Syntax: 1

This table  defines music  using an  inverse frequency  table providing

interesting effects.

------------------------------

PC Timing and Sound Generation

------------------------------

The  heart  of  the  PC  timing  mechanism  is  a  crystal  oscillator.

Circuitry is used to condition  and divide the oscillator output  until

it  reaches the  manufacturer's  rated  CPU  speed  (4.77  MHZ  on  the

original IBM PC).

The output frequency is further  divided until it reaches 1.19318  MHZ.

This frequency is available to programmers.

The 1.19318 MHZ frequency  is input to an  Intel 8235 timer chip  (8254

on an AT  or an ASIC  on some compatibles).  This timer chip  has three

different channels and each channel receives the same input  frequency.

These channels are labeled  A, B, and C.  Each channel has a  gate that

can allow the input  frequency into the channel  or keep it out  of the

channel. A pair of data latches,  a counter, and an output line  finish

off  the channel.  All  three  channels  are  controlled  by  a  single

control register. The gates for  channels A and B are  permanently open

to allow the frequency into these  channels. The channel C gate can  be

programmed to be  open or shut.  The following is  a detailed schematic

of a channel on the 8253/8254 timer chip.

- 13 -

�������������Ŀ ����������Ŀ

���Ĵ Clock input � � Computer �

�   � 1.19318 MHZ � � I/O  BUS �

�   ��������������� ��������^���

�          ������������ �����

�  ����������������������������ͻ

�  �  �����v���Ŀ  ���������Ŀ  �

�  �  � Input   �  � Output  �  �

�  �  � Data    �  � Data    �  �

�  �  � Latch   �  � Latch   �  �

�  �  �����������  �����^�����  �

���������Ŀ���������Ŀ  ��Ŀ    �

�  ����v����Ŀ  ��v������Ŀ  �

�������> Gate    ���> Counter ������Ŀ

�   �  �����������  �����������  �   �

�   �  8253 - One of 3 Channels  �   �

�   ����������������������������ͼ   v

Control                               Output

In most  of it's  operating modes,  a channel  loads a  value from it's

input data latch into the counter,  counts down toward 0 on each  input

pulse,  and outputs  a  pulse  when  the  counter  reaches  0. It  then

reloads the value  into the counter  and repeats as  long as the  input

gate is  open. This  allows the  input frequency  to be  divided into a

more convenient value.

On  the original  IBM  PC,  channel  A  controls  the  real-time clock,

channel  B controls  the  DRAM  refresh  circuitry,  and channel  C the

speaker.  Channel  A  is  also   used  by  a  variety  of   disk  drive

controllers.

Each channel may operate in any one of six different modes (0-5)  which

are   summarized  shortly.   Modes   2   and  3   are  normally   used.

The  timer control  register  is  located  at  port 43h.  This port  is

write-only.  The read/write  data  latches  are visible  at ports  40h,

41h,   and   42h   for    channels   A,   B,   and    C   respectively.

To use  the timer  chip, the  following sequence  of operations must be

performed.

1. Issue a command through the control port.

2. Read or write values through the data latches.

The command to be  executed is defined by  bits 5 and 4  of the control

register.  The last  three  commands  allow  the  counter  to  be  read

directly without first  having it's value  latched via the  output data

latch register.  This approach  causes a  problem for  the last command

which  allows both  bytes  of  the  16-bit  counter to  be read:  least

significant byte followed by most significant.

- 14 -

Because the ports (41h-42h) are only 8-bits wide, two successive  reads

are required.  After the  least significant  byte has  been read, it is

possible for the counter to decrement by one or more pulses before  the

most significant byte is read. As  a result, the true counter value  is

distorted since  the most  significant byte  may be  decremented during

the count.  For example,  suppose the  counter value  is 400  (decimal)

when the  least significant  byte is  read and  then decrements  to 399

before the most significant byte  is read. The resulting value  is 300,

not the  correct 400  or even  399. This  is the  reason for  the first

command which copies the counter  to the output data latch.  Both reads

will  return the  latch  value  rather  than  the  counter  value which

results in accuracy.

Control Register: Port 43h

���������������Ŀ

�7�6�5�4�3�2�1�0�

�����������������

Bits 7 and  6 select the  counter to be  used. Bits 5  and 4 select the

read/load command. Bits  3 and 2  and 1 select  the counting mode.  Bit

0    selects    the    type    of    counting    (binary    or    BCD).

7 6

- -

0 0 - select channel A counter (port 40h)

0 1 - select channel B counter (port 41h)

1 0 - select channel C counter (port 42h)

1 1 - illegal value

5 4

- -

0 0 - copy counter register to output data latch

0 1 - read/load most significant byte only

1 0 - read/load least significant byte only

1 1 - read/load least significant byte followed by most significant

byte

3 2 1 (all modes count from loaded value down to zero)

- - -

0 0 0 - generate an interrupt when counter reaches 0

0 0 1 - programmable one-shot

0 1 0 - rate generator

0 1 1 - square wave generator: used by channel A and C (speaker)

1 0 0 - software triggered strobe

1 0 1 - hardware triggered strobe

Modes 2 and 3 are normally used  on the PC. Mode 2 counts down  to zero

and triggers a  1-clock low pulse.  Mode 3 counts  down by 2's  setting

the  output low  for  an  entire  cycle  and  high  for the  next cycle

resulting in a square wave.

If  bit 0  is  0  then  a  16-bit binary  counter is  used otherwise  a

four-decade   binary   coded    decimal   (BCD)   counter    is   used.

Channel B has it's input gate  wired open so it is always  active. This

channel  is responsible  for  maintaining  DRAM  refresh.  It is  of no

interest in this discussion.

- 15 -

During power-up, the BIOS loads the channel A input latch with a  value

of 0  which is  interpreted as  65536. The  input frequency  of 1.19318

MHZ divided by  65536 results in  the 18.20648193359 HZ  signal that is

associated  with  interrupt  8h  (commonly  known  as  the  timer  tick

interrupt).  About 18.2  times  each  second,  the  channel  A  counter

generates  an IRQ0  hardware interrupt.  This interrupt  is fielded  by

the 8259 priority interrupt controller resulting in interrupt 8h  being

activated. Interrupt 8h has  the responsibility for updating  the timer

tick  counter found  at BIOS  addresses 0000:046Ch  to 0000:046Fh.  The

timer tick  frequency can  be changed  if used  with care  by loading a

different value into the channel A input latch.

There are two ways to generate  sound from the speaker. Toggling bit  1

of port B  under program control  is one way.  When bit 1  is set to  1

then current flows  through the speaker.  When bit 1  is set to  0 then

current is blocked. A program could set  bit 1 and sit in an idle  loop

for a  certain number  of cycles.  Then bit  1 would  be set  to 0. The

program  would sit  in another  idle loop.  The length  of these  loops

would determine  the frequency  of the  tone produced  by the  speaker.

Unfortunately, the tone frequency is dependent upon the loop  counters.

A loop running on an XT is slower than a loop running on a 386  because

of the higher  clock speed on  the 386. Therefore,  a pleasant-sounding

tone  on the  XT  would  sound like  a high-pitched  whistle on  a 386.

Also, these  loops would  be interrupted  approximately 18.2  times per

second by interrupt  8h resulting in  distortion. A third  disadvantage

is that the computer is forced to spend all of its time handling  these

loops.

The    second    and    better    way    of    handling    sound    is:

1.  Turn off  bits  0  and  1  of port  B. This  blocks current  to the

speaker  and  prevents  channel  C  from  sending  pulses  to  port  C.

2.  Load the  channel  C  input data  latch with  the frequency  value.

3. Turn  on bit  5 of  port C  (it's on  by default) if necessary. This

allows channel C  pulses to reach  a tristate buffer  just in front  of

the    speaker   and    controlled    by    bit    1    of   port    B.

4. Turn on bits 0 and 1 of port B.

5. Go  and do  something else  until the  tone has  lasted long enough.

The correct frequency value can  be determined by dividing the  1.19318

MHZ frequency by the tone frequency  desired. An example for a 1000  HZ

tone is:  1,193,180/1000 =  1,193. Place  1,193 in  the channel C input

data latch for this tone.

Turning off the speaker  is as simple as  turning off bits 0  and 1 and

the   following   schematic   illustrates   the   speaker   connection.

- 16 -

�������������Ŀ ����������Ŀ

���Ĵ Clock input � � Computer �

�   � 1.19318 MHZ � � I/O  BUS �

�   ��������������� ��������^���

�          ������������ �����

�  ����������������������������ͻ

�  �  �����v���Ŀ  ���������Ŀ  �

�  �  � Input   �  � Output  �  �

�  �  � Data    �  � Data    �  �

�  �  � Latch   �  � Latch   �  �

�  �  �����������  �����^�����  �

���������Ŀ���������Ŀ  ��Ŀ    �

�  ����v����Ŀ  ��v������Ŀ  �

��������> Gate    ���> Counter ������Ŀ

�    �  �����������  �����������  �   �

�    �   Channel C - 8253 Timer   �   �

�    ����������������������������ͼ   �

���������������������������ͻ                                 �

�                     ��������������Ŀ                        �

� Port B ����������������  �        �                        �

�  61h   �7�6�5�4�3�2�1�0�  �        �                        �

�        �����������������  �   �����v���Ŀ                   �

�             ������������Ŀ�   � SPEAKER �                   �

� Port C ���������������Ŀ ��   �����^�����                   �

�  62h   �7�6�5�4�3�2�1�0� �����������                        �

�        �����^�����������  �                                 �

�             �������������������������������������������������

� 8255 Peripheral Interface �

���������������������������ͼ

Summary:

1. Port B  (61h) bit 0  controls channel C.  If set then  the channel C

counter will send it's pulses to port  C (62h) bit 5. On some PCs,  bit

5 of  port C  is permanently  set to  1 so  that whenever  channel C is

active, it's pulses are  going to the speaker  as long as port  B bit 1

is set.

2. Port B (61h) bit 1 controls  access to the speaker. If set then  the

speaker will  receive current  and can  generate sound.  This sound  is

constant  and the  frequency is  determined by  software timing  loops.

3. Port C (62h) bit 5 controls  access to the speaker by channel C.  If

set   then   the   channel   C   pulses   will   reach   the   speaker.

- 17 -

#-1CREATING YOUR OWN INTERNAL COMMANDS#-0

IC makes it possible for you  to write your own internal commands.  You

will notice  several .asm  files accompanying  this documentation file.

These .asm files contain the source code to the ASCII, DRIVERS,  EQUIP,

LCASE, MOVE,  TYPE, UCASE,  and VECTORS  internal commands.  I refer to

these commands  as External  Installable Commands  (EICs) because  they

exist in separate .com  files and must be  grafted into IC before  they

can be used (see the GRAFT command in section two for more  information

on grafting).

An  EIC begins  as  a  specially written  .com file.  This file  is not

designed  to be  run from  the command  line. The  message "NONCOM"  is

displayed  to users  who  attempt  this. Only  after the  EIC has  been

grafted into IC  will anything happen  when it's name  is typed on  the

command line.

The GRAFT command loads the .com file into a buffer within the IC  code

space located at offset 110h  relative to the code segment.  The buffer

size is set to 5K but the  last 256 bytes are reserved for the  startup

stack  and GRAFT  reserves  an  additional  16  bytes.  Therefore,  the

maximum    size   of    an   EIC    .com   file    is   4,848    bytes.

Execution begins  at offset  address 110h  relative to  the resident IC

code  segment. Therefore,  the  EIC  .com  file  should have  it's code

begin  at  offset  110h.  The   EIC  has  limited  access  to   the  IC

environment. Offset 10ah within IC  contains the offset address of  the

EIC command tail. Offset 10ch  contains the offset address of  the hexw

procedure  that is  useful  for  debugging.  Offset  10eh contains  the

offset  address of  the  stdinit  procedure  that  is  used  to perform

standard initialization.

When an EIC begins execution,  the following conditions have been  met.

� the direction flag is clear

� the DS and  ES segment registers contain  the segment address of  the

command.com transient area. This  is needed for internal  commands that

use file control blocks.

�  a data  structure  called  cline contains  the command  tail of  the

internal command. This  tail may contain  leading spaces and  tabs. The

first byte contains the length (not including the terminating  carriage

return). The maximum length is  128 characters (one for the  count byte

and 127 for  the tail). The  command tail is  terminated by a  carriage

return.

� the stack  has been switched  to the top  of the buffer  reserved for

the  grafted  internal  command.  This  allows  256  bytes  for   stack

storage.  The  SP  register  will  need  to  be  further  directed   if

additional  stack  space  is  required.  This  stack  belongs  to   IC.

� internal commands cannot use  the int 21h memory functions  48h, 49h,

and   4ah    because   there    is   not    enough   available   memory

� commands should  exit via a  RET instruction. Return  codes passed by

int   21h  function   4ch   cannot   be   picked   up  by   ERRORLEVEL.

- 18 -

The following skeleton shows the format of a typical .com file  written

to conform to EIC specifications.

_TEXT     SEGMENT   BYTE PUBLIC 'CODE'

ASSUME    cs:_TEXT, ds:_TEXT, es:_TEXT, ss:_TEXT

ORG       100H

CR        EQU       13

LF        EQU       10

CLINE     EQU       10aH

HEXW      EQU       10cH

STDINIT   EQU       10eH

start:

lea       dx, msg

mov       ah, 9

int       21h

int       20h

msg       DB        "NONCOM$"

code_entry     DW   _code

help_entry     DW   _help

byline_entry   DW   _byline

setup_entry    DW   _setup

cleanup_entry  DW   _cleanup

; -----------------------------------------------

; The EIC begins execution in the code procedure.

; -----------------------------------------------

_code     PROC      NEAR

call      cs:[STDINIT]   ; For standard initialization only.

mov       bx, ds:[CLINE]

mov       al, [bx]

xor       ah, ah

call      ds:[HEXW]

...

ret

_code     ENDP

_help     DB        "This is command help"

DB        CR, LF

DB        '$'

_byline   DB        "command - this is a sample command"

DB        CR, LF

DB        '$'

_setup    PROC      NEAR

ret

_setup    ENDP

_cleanup  PROC      NEAR

ret

_cleanup  ENDP

- 19 -

_TEXT     ENDS

END       start

Standard initialization performs the following tasks.

� Set DS segment register to CS segment

� Set SI to the address of the first nonspace character

� Set AL to the first nonspace character

This makes it easy to access  arguments on the command line. Note  that

the  command line  terminates with  a CR  (carriage return)  character.

In   the  above   skeleton,   I've   included   the   following  lines:

mov bx, ds:[CLINE]

mov al, [bx]

xor ah, ah

call ds:[HEXW]

The  first line  gets  the  address  of  the  command  line  to  the BX

register. Note  that DS  is pointing  to the  same segment  as CS  as a

result of the STDINIT  call. The length of  the command line is  stored

in the first byte  of the command line  and this value is  fetched into

the  AL  register.  The  length  does  not  take  the  terminating   CR

character into  effect. Finally,  the length  is converted  to a 16-bit

value by zeroing  the AH register  and displayed in  hexadecimal format

via the HEXW call.

If you've played with ASCII, you know that it's possible to graft  EICs

that act  like memory  resident programs.  When an  EIC is installed by

GRAFT,  GRAFT  calls  the  setup  routine  allowing  the  EIC  to  hook

interrupt vectors. When  GRAFT is called  without arguments or  another

EIC is  to be  grafted then  the cleanup  routine is  called to  unhook

these  vectors. The  DS  and  ES segment  registers point  to the  same

segment as CS.

This is pretty  well all you  need to know  in order to  write your own

EICs. If you study  the source code for  the included EICs, you'll  get

a pretty good idea of how they are written.

ASCII is  a hotkey  activated memory  resident utility.  When the ALT-A

hotkey is  pressed, a  listbox appears  displaying ASCII  codes in  the

decimal and hexadecimal  number systems along  with the character.  The

UP,  DOWN, HOME,  END,  PGUP,  and  PGDN  keys are  used to  select the

character of interest and ESC  exits ASCII. The hotkey and  location of

this  listbox can  be  changed.  The syntax  of the  ASCII command  is:

ASCII [/kx] [/x##] [/y#]

The  /k parameter  is  used  to define  a new  hotkey. The  x value  is

A-Z/0-9. The  /x parameter  defines the  left column  (0-59) and the /y

parameter   defines   the   upper    row   (0-6)   of   the    listbox.

DRIVERS displays a  list of installed  device drivers. The  driver name

is displayed  if the  driver drives  a character  device. Eight  dashes

are    displayed   if    the    driver    drives   a    block   device.

- 20 -

EQUIP displays  some of  the internal  equipment configuration settings

such as video adaptor type,  number of floppy disk drives,  and whether

or not a math coprocessor is present.

LCASE converts  standard input  to lowercase  and sends  the results to

standard output. UCASE converts  standard input to uppercase  and sends

the results to standard output.

DOS  has never  included a  command for  moving files.  However, it  is

possible to create a batch file containing the COPY and ERASE  commands

to simulate  a move.  Although this  works, copying  files is  slow and

this batch file would not  work for disks almost (or  completely) full.

The DOS RENAME function (function 56h of interrupt 21h) can be used  to

move a file  from one area  to another. It  does so not  by copying the

file and then erasing the  original, but by moving the  directory entry

from  one location  to  another.  MOVE  makes  use  of  this  approach.

Unfortunately, function 56h cannot  make use of wildcard  parameters so

only a single file  can be moved at  a time. However, DOS  comes to the

rescue  with an  undocumented  technique  for allowing  wildcards. This

technique is stable through DOS versions 3.10 to 5.00 inclusive and  IC

only works with DOS 3.30 to 5.00 inclusive.

The MOVE command  has a few  quirks and may  take a little  time to get

used to because MOVE relies on  the RENAME function. I will attempt  to

explain this through the use of examples.

Assume that drive C: and  directory \DOC is current. Assume  that there

is a  directory called  \BACKUP on  the same  drive and  a file  called

X.DOC in \DOC.

� MOVE X.DOC \

X.DOC now appears  in the root  directory. It is  no longer located  in

\DOC. This is the  only case that I  have found where only  a directory

name appears in the target filespec.

� MOVE X.DOC \BACKUP\X.DOC

X.DOC now  appears in  \BACKUP. It  is no  longer located  in \DOC.  If

instead of  typing the  preceeding command  line, "MOVE  X.DOC \BACKUP"

had  been typed,  an  error  message  would  be  displayed.  You cannot

specify only a directory.

� MOVE X.DOC \BACKUP\XYZ.DOC

X.DOC now appears  in \BACKUP as  XYZ.DOC. It is  no longer located  in

\DOC.

� MOVE *.DOC \BACKUP\*.DOC

All  files havin  a .DOC  extension are  moved from  \DOC into  \BACKUP

(they are no longer located in \DOC).

� MOVE *.DOC PRN

You  cannot move  filespecs  to  a  device  name. An  error message  is

displayed.

- 21 -

TYPE is an  alternate form of  the standard internal  TYPE command. The

new TYPE  command has  the same  syntax as  the standard  TYPE command.

VECTORS displays the  current addresses assigned  to all 256  interrupt

vectors.  A  brief  description  is  included  beside  some  of   these

vectors.

- 22 -

The  designers of  DOS  saw  fit  to  add an  extension to  COMMAND.COM

beginning with version 3.30 to allow for additional internal  commands.

This feature has been used to accomodate APPEND and the DOS 4.0  SHELL.

I believe  that IC  is the  first program  to really  take advantage of

this  feature. Although  Windows  and  other  shell  programs hide  the

command line interface from users, I think there will always be a  need

to access  DOS at  the command  line level.  And Internal Commands will

be there  to make  using the  command line  interface more pleasurable.

- 23 -