(C) Micro Staff 1988-1993 Copyrighted programs.

MODR-ASPI ( Micro Staff ASPI 3.5" MO Disk Drive Device Driver module and

Utilities)

MODR-ASPI is a package of ASPI 3.5" MO disk drive device driver module

and utility software for the IBM PC/AT and compatible computers.

MODRVR.SYS, MO disk drive driver, works with ASPI manager for

Adaptec 152x, 154x, Trantor Mini SCSI plus and BusLogic BT542 SCSI

adapters under MS-DOS 3.3x, 4.0x and 5.0x.  MOFORM.EXE, MO disk formatter,

formats 3.5" MO media to IBM 3.5" MO compatible format.

MODR-ASPI supports the following 3.5" MO disk drives:

SONY RMO-S350, RMO-S360

RICOH RS-3100E

PANASONIC LF-3000

OLYMPUS MOS300S

FUJITSU M2511A

TOSHIBA OD-D300B

This program is a Shareware ($15).  If you find it convenient, register via

CompuServe Shareware Registration database.

Registration ID:  935

Micro Staff Co., Ltd.

1-46-9 Matsubara, Setagaya-ku, Tokyo, Japan 156

Phone 03-3325-8128

Fax   03-3327-7037

CompuServe ID 100157,1053

1. Installation

Type MOINST and press Enter key, and follow the instructions displayed on

the screen.  This will copy the driver and utility programs into SCSI

sub directory (default) and add MO disk drive device driver, MODRVR.SYS,

in your CONFIG.SYS file.

2. Formatting MO disk

MOFORM.EXE, MO disk formatting program, performs physical and logical

format. Type MOFORM and press Enter key.  The program will check for MO

drive(s) connected to the SCSI adapter and display the following messages:

ISO-type 3.5"/5" MO disk format utility ver x.x

Copyright (c) 1989-1993 Micro Staff

checking unit: 0

If the program detects MO drives with SCSI ID 3 and 4 (for example), it

will display the following message:

ISO-type 3.5"/5" MO disk format utility ver x.x

Copyright (c) 1989-1993 Micro Staff

Enter unit no. (3-4,  'q' for exit) ->

(If the program does not detect any drive, no unit number is displayed.

Press q to exit and make sure that the cable and the terminator are

properly connected  and also MODRVR.SYS is added to the CONFIG.SYS file.)

To format a MO disk in the drive with SCSI ID 3, press 3 and enter key.

The following messages will be displayed:

Unit 3: Drive name  RICOH RO-3010E

are you sure? (y/n)

Enter y to proceed.  The following messages will be displayed:

do physical format? (will take 20 min.)  (y/n)

Enter y to perform both physical and logical format or n to perform only

logical format.  After formatting is completed, you will return to DOS

prompt.

3. Other utilities

a. BFC.EXE

Compares binary files.

Use to verify the writing after copying data including sub directory on a

hard disk to a MO disk, using a command such as XCOPY.EXT.

<syntax>

bfc [option switches] file_name_1 file_name_2

bfc [option switches] path_1 path_2 ..... directory

bfc -r [option switches] directory_1 directory_2

option switches:

-r: compares sub directories too.

-s: compares even if sizes are different.

-m<number>: displays number of differences up to the number

specified (default: 0).

<example>

bfc -r a: c:

b. MOMAP.EXE

Displays the SCSI ID number(s) and drive letter(s) assigned by DOS.

c. MOLOCK.EXE

Disables the eject switch on the MO drive.

<syntax>

MOLOCK <drive number>

<example>

molock d:

d. MOUNLOCK.EXE

Enables the eject switch on the MO drive.

<syntax>

MOUNLOCK <drive number>

<example>

mounlock d:

e. MOEJECT.EXE

Ejects the  MO disk from the drive.

<syntax>

MOEJECT <drive number>

<example>

MOEJECT d:

3. Disk Driver

MODRVR.SYS, driver module for MO disk drive, needs to be added to

CONFIG.SYS file to use MO disk drives.  With the driver the following

disk medium can be used:

Under MS-DOS 3.x:  disks formatted by MOFORM.EXE

Under MS-DOS 5.x:  IBM formatted and MOFORM formatted disks

Option switch settings:

-h

Enables to control hard disks as well as MO disk drives.

-u <SCSI_ID.....>

Sets SCSI ID(s) for 3.5" MO disk drive(s).

To connect the drives with the ID numbers 3, 4 and 5 (for example),

set -u345.

Default setting:  -u0123456.

-v<SCSI_ID.....>

Sets SCSI IDs for 3.5" MO disk drives that use Write & Verify when

writing.  This will increase data integrity as the data is verified

when written.  However, write speed will decrease.

We recommend setting this switch as MO disks are structurally subject

to dirt due to its removability.

***************** WARRANTY DISCLAIMER ****************

This programs  is provided on an "as is" basis. MICRO STAFF CO., LTD.

makes no warranty of any kind.

In no  event will  MICRO STAFF CO., LTD. be liable  for any  errors,

damages including  but  not  limited  to  special,  incidental,

consequential or other damages.

#