One percent resistor calculator program

by Andrew Dart

Box 380508

Duncanville, TX   75138

C version, revison 1.1

6/10/93

This program finds the two best values of standard 1% resistors

to use in a simple two-resistor voltage divider like this:

V1  ----------/\/\/\/\/------*--------/\/\/\/\/-----------  V2

|

R1          |            R2

|

V3

You specify the voltage at V1 and V2, and the voltage you

want at V3.  Usually V1 is a positive supply and V2 is

ground, or zero volts, but V1 need not be greater than V2.

If V3 is midway between V1 and V2, then you could use any

two identical resistors, and you don't need this program,

but you may still find it useful even then, because it will

adjust the values according to the power rating of the

resistors (see below).

One percent resistor values are 96 evenly spaced powers

of the 96th root of 10.  That is the key to the algorithm

used here.  Refer to EIA standard RS-385.   While I've

never seen RS-385 myself, I understand it has something

to do with this topic.

Updated features since the first BASIC version:

*   The result is printed immediately.  There is no delay.

Updated features since the last BASIC version:

*   The previous version would select the correct values to

achieve the desired output voltage, but typically each

resistor was only a few ohms.  You probably are not going

to use 100-watt resistors, so this program asks you what

size resistors you're using (any reasonable size; for example,

quarter-watt resistors) and multiplies the result by some power

of ten, to bring the current down to a level that the selected

resistors can handle.

*   This program draws the diagram above and shows the voltages

and resistor values, and the power dissipated by each

resistor, instead of simply listing the voltages and

resistor values.  This makes it a little easier to use.

Future modifications:

*   With a little extra programming, I could draw the resistors

in color and let you see what the stripes should look like.

*   After selecting the ideal resistor pair, you could increment

or decrement each resistor value (by one place in the table

of standard values) and come out with almost exactly the same

voltage at V3, but a little higher or lower total resistance.

So after the program computes the result, you should be able

to hit the up or down arrow keys and have this adjustment made

for you, and the values instantly recomputed.  Well, I've tried

that, and in many cases it works, but to cover every situation,

like one value rolling over to a new order of magnitude (from

9.76 to 10.00 and beyond), there are a number of flags and

special conditions to keep track of, and it hasn't yet been

demonstrated to me that it would be worthwhile.  Keep this in

mind, however, if you are using your two-resistor divider to

replace a trim-pot and you need the total resistance (R1+R2)

to equal the value of the trim-pot (e.g., 1000 ohms).  Adjust

the two resistor values up or down (by the same number of steps)

in the table, and output voltage will be almost the same as

with the ideal combination of values.  Another application for

this is in the event you have run out of 715 ohm resistors but

still have plenty of 732's.  Still another would be the case

where you know R1 must be a particular value, because you have

a thousand of them, or because you're modifying an existing design

and only want to change one value.  You could then increment or

decrement the value of R1 until it was the value you wanted.

There's nothing to prevent you from doing this manually, or

modifying the program to include this feature, or motivating

me to append to the program.

This program was written with Microsoft QuickC / Quick Assembler

and probably will run through other compilers as well with little

or no modification.  The most likely exception would be cursor-

positioning routines, clearscreen, header files, etc.

Naturally, the author assumes no responsibility for the use of

this software, and while every effort has been made to keep it

free of bugs and test it thoroughly, you may use it at your own

risk.  Also, I'll take no credit or blame for any circuit or

system designed (partially) with the aid of this program.

These remarks are included in the C source code in case you would

like to edit them out of the .C file or discard this .DOC file.

Even though the software is provided to you as is, at no charge,

if you find it to be of value, or you find the source code to be

educational, or the comments to be entertaining, or you would

simply like to support my efforts, please feel free to send a

generous token of your appreciation, in umarked U.S. currency,

to the author at the address above.  Job offers will be accepted

in lieu of donations.