The G8BPQ AX25 Networking Package.

1. Introduction.

This software allows an IBM PC, or similar machine, equiped with suitable

Communications hardware, to act as a Node in a NET/ROM compatible AX25

network, and/or to support a multiuser Mailbox, or other similar applications.

The switch section of the code supports up to 16 AX25 ports, and the

application interface supports up to 64 connections. The basic applications

interface is called 'BPQ Host Mode', and is defined in file BPQHOST.DOC.

Additional drivers are provided to convert this to other common Amateur

interface standards (eg TNC2 via COMBIOS, and WA8DED Host Mode).

Note the software is NOT Public Domain. It may be used only by licenced

Radio Amateurs in Amateur Packet Radio systems.

2 Hardware Requirements.

The system is written in 8086 assembler, and is currently only available for

the MSDOS/PCDOS environment, although once loaded it makes no use of operating

system facilities. It has been tested on IBM PC and XT machines, and a number

of similar machines. (AST Premium, Toshiba T1100+, OPUS). The software needs

about 48 - 80k RAM, varying with number of buffers, comms links and COMBIOS

ports configured.

The software supports the following comms hardware:

PAC-COM  PC120 card

DRSI PCPA card

Software Forge HDLC card for the Toshiba T1100 (with external modem).

KISS mode TNC via a standard PC ASYNC card or QUADRAM Quadport .

Link to NET/ROM or TheNet Async port.

RLC100 4 port HDLC card.

Baycom USCC 4 port HDCL card.

A version is also available for the Kantronics Data Engine.

3. Networking Facilities.

The system is designed to be operationally compatible with existing NET/ROM

or TheNet nodes, but there are a few minor differences. As it can support

an integrated BBS, it has 2 Callsign/Alias pairs, one for accessing the node,

and the other the BBS. Note, however, that a multiport node does not need a

different callsign for each port. Because of the multiport support, the

'Downlink' connect command needs a port number as the first parameter (eg

C 2 G8BPQ). A connect to another Node doesnt need this - the system selects

the 'best' port as found from the NODEs and ROUTEs lists.

Although multiple applications are allowed, only one (BBS) has its own

Callsign/Alias pair. Others are accessed by connecting to the switch, then

entering the service name.

There are  extra commands for normal use (BBS, or other application name, to

connect to the applications, and PORTS to display available port descriptions),

and 2 commands intended primarily for the system operator (STATS and LINKS).

Two extra commands PACLEN and L4T1 are available to set Paclen for NODE

generated messages (nodes lists, etc), and the Transport timeout. At the

moment these are primarily for me to experiment with, but feel free to fiddle.

The system is also able to link to the 'back end' async port on a normal

NET/ROM or TheNet node.

4 Getting Started.

A. Decide what you want.

The software can be configured as a mailbox access system, a packet switching

node, or a combination of both. I would expect most intallations to use either

one or the other, but if you run a well sited BBS (particularly if multiband),

and there is a shortage of nodes in your area, you may wish to run both.

B� Choos� you� hardwar� an� software.

You need an IBM PC/XT/AT or compatible. A switch needs very little RAM (

about 128k), but a BBS system will probably need a minimum of 640K. You can

connect your radios via an internal packet adaptor (PC120 from PACCOM,

PCPA from DRSI, etc), or via normal TNC's running in KISS mode (or both). For

a new installation I recommend the internal card, as it will normally be

cheaper (especially for multiport nodes), and faster, but if you already have

a TNC which has (or can be fitted with) a KISS option, then by all means use

it.

The software has been tested with the BBS systems written by W0RLI, F6FBB,

G1NNA, and G4YFB, and with AK1A's PacketCluster. It is likely to work with

other system which use a similar interface - if in doublt, consult the

author of the program concerned.

C. Edit the configuration file (BPQCFG.TXT)

I hope the comments in the file make it largely self-documenting. You need

one Callsign/Alias pair for BBS access, and one for the Node. If you are

running both, they must be different, but if you are running only one or

the other, you must still put BOTH entries in the file, but they MUST be the

SAME. The software doesn't currently verify this, so be careful!

Most of the configuration params can be left alone - the ones to look at

are:

IDMSG and INFOMSG

BBS, NODE	to select BBS support and NODE support as required

ROUTES		to 'Lock In' the routes you want to use

Note that the supplied Networking parameter conform (more or less) to the

recommendations made at SYSOPS 5.

There are sample 'PORTS' configuration files for the various link types

supported in file PORTS.DOC - copy and customise the ones you want to use.

D. Run BPQCFG.

This will convert the configuration file to a format suitable for use by the

main software. Validation isn't wonderful, but it may produce error messages.

E. Load it.

I suggest you test the system first using a simple terminal program, before

trying it with the BBS (if required).  I have included on the disk a suitable

terminal program (PAC4), which may be used with the system.

When running with a multitasker (eq DesqView), you must load my software

before the multitasker. You should run BPQCODE, then any required interface

drivers. The available drivers are described in DRIVERS.DOC, but for initial

testing, just run BPQHTNC2. A copy of my AUTOEXEC.BAT file follows (I use

W0RLI BBS). Once the switch is running, load Desqview (if required), then

your BBS or other application programs.

My AUTOEXEC.BAT:

cd \bbs

LM

SHARE

cd \net

bpqcode

bpqhtnc2

cd \bbs

mbinit

cd \dv

dv

The BBS software is loaded by a Desqview script file. The following saves

the ROUTES and NODES to disk before shutting down.

cd \net

bpqnodes

Appendix 1 - Fowarding files

Both MBL and RLI BBS software requires that the reply to the first connect in

a forward file connect sequence is of the form *** CONNECTED to xxxx. This is

not normally provided by the switch code, as the first connect is effectively

at the Net/Rom level. To get round this, I have provided a dummy command,

C SWITCH, which returns the string '*** CONNECTED to SWITCH'. This should be

included as the first entry in each set of forwarding commands. See below

for sample forwarding files for MBL and RLI.

MBL:

FA GB7NTS

00-23

@SWITCH

.C LRG7

?u

+to

.c GB7NTS-7

+to

?u

G4MTP

GB7NTS

-----

RLI:

CC SWITCH

NC GB7NTS

HB0023GB7NTS

GB7NTS

_271

_1111

*** EOF

John Wiseman, G8BPQ

76,Haywood Road,

Mapperley,

Nottingham.

NG3 6AE.

Phone 0115 9522669    (Please phone only between 20:00 - 22:00 UTC)

1 October August 1994.

#####################