This document will provide a list of the default settings for the

supported Braille Displays when using SLIMWARE WINDOW BRIDGE.

If your Braille Display has Status Cells that have Cursor Routing keys

above each Cell, these keys have been assigned to Functions as well.

By default, each Status Cell Cursor Routing key will identify the

WINDOW BRIDGE information that is being currently shown in this Status

Cell.  This feature is described in detail in the WINDOW BRIDGE Manual.

If you have a Cursor Routing key in the Blank position between the

Status Cells and the main Display, this Cursor Routing key has been

defined to "Refresh" the Status Cells.  "Status Cell Refresh" is

described in the Manual in the Chapter concerning the KEYMAP Program.

**ALVA KEYBOARD SUMMARY

The ALVA Braille Displays have 7 Thumb Keys.  For identification

purposes, these keys are labelled #1 to #7 from

left to right.  A sample definition for these Thumb keys has been

included with your SYNTHA-VOICE Product.  The default

orientation for these keys is as follows:

KEY           FUNCTION

1             CONTROL PANEL

2             Scroll Top

3             READ CURSOR LINE

4             Scroll Up

5             Scroll Left

6             Scroll Right

7             Scroll Down

12            Braille Refresh

13            WINDOWS Scan Line

14            WINDOWS Scan Up

15            WINDOWS Resolution, Decrease

16            WINDOWS Resolution, Increase

17            WINDOWS Scan Down

25            PREVIOUS ICON

26            NEXT ICON

27            SCROLL BOTTOM

In the previous list, whenever a reference is made to the use of 2

Thumb keys, hold the first key, and then press

the second Thumb key.  Release both keys to execute the function.

If your ALVA Display has 3 Status Cells, the default assignments for

these Cells in WINDOWS is as follows:

Cell #1 and #2 show the Mouse Pointer location.

Cell #3 shows the Insertion Pointer Column

If your ALVA Display has 5 Status Cells, the default assignments for

these Cells in WINDOWS is as follows:

Cell #1 and #2 show the Mouse Pointer location.

Cell #3 and #4 show the Insertion Pointer location.

Cell #5 shows the Feature Status for some WINDOW BRIDGE features.

**HANDY TECH KEYBOARD SUMMARY

There are 10 Thumb Keys along the front Panel of the HANDY TECH

keyboard.  These keys are labelled from left to right.  Thumb Keys

Number 1 to 4 are on the Left side.  Keys 5 and 6 are larger keys.

Keys 7 to 10 are on the Right side.  They are organized as follows:

KEY            FUNCTION

1              PREVIOUS ICON

2              WINDOWS SCAN UP

3              SCROLL TOP

4              SCROLL LEFT

5              SCROLL UP

6              SCROLL DOWN

7              SCROLL RIGHT

8              SCROLL BOTTOM

9              WIDNWS SCAN DOWN

10             NEXT ICON

The default settings for the Status Cells in WINDOWS is as follows:

Status Cell #1 and #2 show the Mouse Pointer location.

Status Cell #3 and #4 show the Insertion Pointer location.

The Numeric Pad keys have not been assigned by default.  You can

assign these keys by using the KEYMAP Utility program discussed in

the manual.

**KTS KEYBOARD SUMMARY

There are 3 BRAILLOTERM Displays supported by your SYNTHA-VOICE

Product.  These are the BRAILLOTERM 20, 40, and

80.  These systems have a SCROLL LEFT and SCROLL RIGHT key on the

extreme Left and Right side of the unit.  To the

right of the Left Scroll key is a key labelled as "0"  A similar

key is positioned to the left of the Right

Scroll key.  This key is labelled as "#".  To the left of the "#"

key is the "M" key.  The remaining 3 groups of

horizontal keys are labelled as the Numbers 1 to 9 from left to

right.  The following SYNTHA-VOICE Commands have

been assigned to these keys:

KEY                    FUNCTION

Scroll Left            Scroll Left

Scroll Right           Scroll Right

0+Scroll Left or Right Scroll Up

1+Scroll Left or Right Scroll Down

#                      Cursor Locator

M                      Control Panel

0                      Help Mode

1                      Left Mouse Button

2                      Mouse Locator

3                      Right Mouse Button

4                      Previous Icon

5                      List Icons

6                      Next Icon

7                      Previous Item

8                      Current Item

9                      Next Item

**TSC NAVIGATOR KEYBOARD SUMMARY

There are 2 groups of 5 Command keys on the Braille Display.  These

keys are organized as 5 keys on the Left, and 5

keys on the Right.  Four of the five keys are oriented in the shape

of a Diamond, with the fifth key representing

a Command Function.  The position of the first 4 keys in each group

represent an Arrow pointing Up, Down, Left, and

Right.  These keys have been assigned to logical Commands in the

following list.

KEY                            FUNCTION

Left Group Up                  Cursor Up

Left Group Down                Cursor Ddown

Left Group Left                Cursor Left

Left Group Right               Cursor Right

Left Group Command key         Enter

Left Group Command key + Up    Cursor Locator

Left Group Command key + Down  Control Panel

Left Group Command key + Left  Left Mouse Button

Left Group Command key + Right Escape

Left + Right Command keys      Help Mode

Right Group Up                 Scroll Up

Right Group Down               Scroll Down

Right Group Left               Scroll Left

Right Group Right              Scroll Right

Right Group Command key        Refresh Display

Right Group Command key + Up   Scroll Top

Right Group Command key + Dn   Scroll Bottom

The reference to "+" in the Command List means that you should hold

the first key and then press the second key to access

the designated function.  The function will be executed when all of

the keys have been released.  The NAVIGATOR

requires these key combinations to be pressed by pressing the

second key fairly quickly after pressing the first

key.  If you find that the key combination is not working, you

might try pressing the combination a little

faster.

**TSC POWERBRAILLE KEYBOARD SUMMARY

The T.S.I POWERBRAILLE Display has a key on the Left

and Right side of the Braille Display.  These have been assigned to

Scroll the Display Left and Right.

There are a group of Thumb Keys along the front panel of the

POWERBRAILLE.  These keys have been assigned as follows:

Key                      Function

Left Rocker Up           Scroll Top

Left Rocker Down         Scroll Bottom

Right Rocker Up          Scroll Up

Right Rocker Down        Scroll Down

Left Small Key           Cursor Locator

Right Small Key          Control Panel

Small key + Rocker Up    Previous Icon

Small key + Rocker Down  Next Icon

Both Small keys together Braille Refresh

**TIEMAN MULTIBRAILLE KEYBOARD SUMMARY

There are 10 Thumb Keys along the front Panel of the MULTIBRAILLE

keyboard.  There are 6 more keys along the top panel of the

system.  There is an optional group of Numeric keys available.  The

Thumb Keys will begin with the Letter "T" in the following list,

while the upper group of  Keys will begin with the Letter "U".  Both groups

of keys are labelled from left to right.

**TIEMAN 85 SUMMARY

KEY            FUNCTION

T1      SCROLL TOP

T2      SCROLL LEFT

T3      WINDOW SCAN; UP

T4      READ LINE

T5      SCAN MODE

T6      WINDOW SCAN; DOWN

T7      SCROLL RIGHT

T8      SCROLL BOTTOM

T9      WINDOWS BRAILLE RESOLUTION; INCREASE

T10     WINDOWS BRAILLE RESOLUTION; DECREASE

U1      TOGGEL PAD (mouse/pixel/window)

U2      IBM PAD

U3      LEFT MOUSE BUTTON

U4      RIGHT MOUSE BUTTON

U5      BRAILLE CURSOR ON / OFF

U6      DISPLAY; REFRESH

U123    SCROLL TOP

U456    SCROLL BOTTOM

U13     9

U46     3

U23     4

U56     6

U12     8

U45     2

U1234   7

U1456   1

U1235   WINDOW BRAILLE RESOLUTION; INCREASE

U2456   WINDOW BRAILLE RESOLUTION; DECREASE

U12346  Scan Mode

U13456  Editing Mode

U123456 MENU

OPTIONAL KEYPAD * LEFT MOUSE BUTTON

OPTIONAL KEYPAD / RIGHT MOUSE BUTTON

OPTIONAL KEYPAD 0 READ LINE

The default settings for the Status Cells in WINDOWS is as follows:

Status Cell #1 and #2 show the Mouse Pointer location.

Status Cell #3 and #4 show the Insertion Pointer location.

Status Cell #5 shows the Feature Status for some WINDOW BRIDGE features.

**TIEMAN 45 SUMMARY

T1      SCROLL LEFT

T2      WINDOW SCAN; UP

T3      SCAN MODE

T4      WINDOW SCAN; DOWN

T5      SCROLL RIGHT

U1      TOGGLE PAD (mouse/pixel/window)

U2      IBM PAD

U3      LEFT MOUSE BUTTON

U4      RIGHT MOUSE BUTTON

U5      BRAILLE CURSOR ON / OFF

U6      DISPLAY; REFRESH

U123    SCROLL TOP

U456    SCROLL BOTTOM

U13     9

U46     3

U2      4

U56     6

U12     8

U45     2

U1234   7

U1456   1

U1235   WINDOW BRAILLE RESOLUTION; INCREASE

U2456   WINDOW BRAILLE RESOLUTION; DECREASE

U12346  Scan Mode

U13456  Editing Mode

U123456 CONTROL PANEL

The default settings for the Status Cells in WINDOWS is as follows:

Status Cell #1 and #2 show the Mouse Pointer location.

Status Cell #3 and #4 show the Insertion Pointer location.

Status Cell #5 shows the Feature Status for some WINDOW BRIDGE features.

**TIEMAN COMBIBRAILLE KEYBOARD SUMMARY

The THUMB KEYS (from left to right T1-T5)

T1                  SCROLL LEFT

T2                  SCROLL UP (SCAN UP)

T3                  SCAN LINE

T4                  SCROLL DOWN (SCAN DOWN)

T5                  SCROLL RIGHT

The BRAILLE KEYS (from left to right B3 B2 B1  B4 B5 B6)

B1                  SCROLL LEFT

B2                  SCAN UP

B3                  SCROLL TOP

B4                  SCROLL RIGHT

B5                  SCAN DOWN

B6                  SCROLL BOTTOM

B1+B2+B3+B4+B5+B6   MENU/WINDOW BRIDGE CONTROL PANEL

B1+B4+B5+B6         HELP

B1+B2+B3            SCANWIDTH DECREASE

B4+B5+B6            SCANWIDTH INCREASE

B2+B5                    SCAN MODE ON

CURSOR ROUTING KEYS IN THE STATUS CELLS:

C1                  VOLUME, DECREASE

C2                  VOLUME, INCREASE

C3                  RATE, DECREASE

C4                  RATE, INCREASE

C5                  VOICE, ON/OFF

C6                  MENU / WINDOW BRIDGE CONTROL PANEL

THE STATUS CELLS (from left to right S1-S5)

S1 - S4                  MOUSE/INSERTION POINTER (4 CELL)

S5                  ENHANCEMENT (1 CELL)

COMBINATIONS  T1 + BRAILLE KEYS ( windows specific commands)

B1                  ITEM LEFT

B2                  ITEM UP

B3                  ITEM, READ/SPELL

B1+B2                    PREVIOUS ITEM

B2+B3                    PREVIOUS ICON

B1+B3                    DEFINE ICON

B1+B2+B3            FIRST ITEM

B4                  ITEM RIGHT

B5                  ITEM DOWN

B6                  ITEM PROTECTION SENSOR ON/OFF

B4+B5                    NEXT ITEM

B5+B6                    NEXT ICON

B4+B6                    LIST ICONS

B4+B5+B6            LAST ITEM

COMBINATIONS  T2 + BRAILLE KEYS (mouse and pad commands)

B1                  MOUSE RIGHT

B2                  JOIN MOUSE TO CURSOR

B3                  MOUSE LEFT

B1+B2                    MOUSE AUTOTRACK ON/OFF

B2+B3                    MOUSE LOCATOR

B1+B3                    MOUSE SEARCH

B1+B2+B3            MOUSE/PIXEL/WINDOW PAD

B4                  MOUSE SCROLL BAR HORIZONTAL LOCK

B5                  MOUSE SCROLL BAR VERTICAL LOCK

B6                  IBM / SCAN PAD TOGGLE

B4+B5                    COLUMN PAD INTERMEDIATE

B5+B6                    COLUMN PAD NUMERIC/CURSOR

B4+B6                    DOCUMENT PAD INTERMEDIATE

B4+B5+B6            DOCUMENT PAD NUMERIC/CURSOR

COMBINATIONS  T3 + BRAILLE KEYS ( area / configuration commands)

B1                  READ AREA ENABLE/DISABLE

B2                  SCROLL AREA ENABLE/DISABLE

B3                  MONITOR AREA ENABLE/DISABLE

B1+B2                    IGNORE AREA ENABLE/DISABLE

B2+B3                    READ ALL ON/OFF

B1+B3                    {NOT DEFINED}

B1+B2+B3            CONFIGURATION LOAD

B4                  READ CURRENT READ AREA

B5                  READ CURRENT SCROLL AREA

B6                  MONITOR AREA DEFINE

B4+B5                    IGNORE AREA DEFINE

B5+B6                    READ ALL TERMINATOR DEFINE

B4+B6                    MACRO DEFINE/DELETE

B4+B5+B6            CONFIGURATION SAVE

COMBINATIONS  T4+ BRAILLE KEYS ( keyboard commands)

B1                  ARROW LEFT

B2                  ARROW UP

B3                  PAGE UP

B1+B2                    HOME

B2+B3                    NUM LOCK

B1+B3                    CAPS LOCK

B1+B2+B3            SCROLL LOCK

B4                  ARROW RIGHT

B5                  ARROW DOWN

B6                  PAGE DOWN

B4+B5                    END

B5+B6                    DELETE

B4+B6                    INSERT

B4+B5+B6            BACKSPACE

B1+B5                    ESCAPE

B2+B4                    TAB

COMBINATIONS  T5 + BRAILLE KEYS ( braille/speech commands)

B1                  LANGUAGE NEXT

B2                  LANGUAGE PREVIOUS

B3                  VOICE NEXT

B1+B2                    VOICE PREVIOUS

B2+B3                    VOICE INFLECTION ON/OFF

B1+B3                    DICTIONARY ADD

B1+B2+B3            DICTIONARY DELETE

B4                  BRAILLE CURSOR TYPE

B5                  BRAILLE CURSOR ON/OFF

B6                  BRAILLE 6/8 DOTS

B4+B5                    BRAILLE TRACKING ON/OFF

B5+B6                    BRAILLE WORDWRAP ON/OFF

B4+B6                    BRAILLE TREMBLE

B4+B5+B6            ROUTING MACRO ON/OFF