SYNTHA-VOICE COMPUTERS INC.

SPEECH SYNTHESIZER NOTES

TABLE OF CONTENTS

**GENERAL NOTES

**SUPPORTED SYNTHESIZERS

**ACCENT PREPARATION

**APOLLO AND JUNO PREPARATION

**ARTIC TRANSPORT PREPARATION

**BRAILLE 'N SPEAK PREPARATION

**DECTALK VERSION 2.0 PREPARATION

**DECTALK EXPRESS PREPARATION

**DECTALK PC PREPARATION

**INFOVOX PREPARATION

**KEYNOTE PREPARATION

**KURZWEIL PERSONAL READER AND READING EDGE PREPARATION

**MULTIVOICE PREPARATION

**SMARTALK PREPARATION

**SOUNDING BOARD AND SPEAK-OUT PREPARATION

**TELEVOX PSOLA PREPARATION

**TEXT ASSIST PREPARATION

**GENERAL NOTES

If your SYNTHA-VOICE Product supports the use of a Speech

Synthesizer, this document will provide you with specific

information on many of the currently supported synthesizers.  If

you are experiencing problems installing your SYNTHA-VOICE Product,

verify that you have configured your synthesizer and Interface

Cable as specified in this file.

Once this is complete, refer to the file called INSTALL.DOC for the

correct Install Command.

SUPPORTED SYNTHESIZERS

ACCENT PC, SA, and MESSENGER, APOLLO 1 Serial, APOLLO 2 Serial and

Parallel, ARTIC TRANSPORT, AUDAPTER, BRAILLE 'N SPEAK Classic and

640, DECTALK 2.0, PC, and EXPRESS, DOLPHIN JUNO, DOUBLETALK PC and

LT, ECHO SERIAL and THE NEW ECHO PC, EUROVOCS, INFOVOX 100, 200,

500, and 700, KEYNOTE GOLD PC, SA, SE, and VC, KURZWEIL PERSONAL

READER and READING EDGE, MULTIVOICE, SMARTALK, SPEAK-OUT, SOUNDING

BOARD, TELEVOX PSOLA, and TEXT ASSIST with a CREATIVE LABS SOUND BLASTER.

The original ECHO Serial, and BRAILLE 'N SPEAK are supported as

Generic Synthesizers.  Refer to the INSTALL COMMAND section

in INSTALL.DOC for the correct Install Command.

**ACCENT PREPARATION

Your SYNTHA-VOICE Product supports the ACCENT PC, SA, and

MESSENGER.  The MESSENGER requires the Speech Drivers included with

this device.  The PC Model can be used with or without these

Drivers.  The SA Model does not use these Drivers.

When using the ACCENT Drivers, be sure to load these Drivers before

running the Install Program.  These Drivers must also be loaded

before loading your SYNTHA-VOICE Product each day.  The ACCENT

Drivers must be set to communicate with LPT3.

The ACCENT PC can also be used without the ACCENT Drivers.  To use

the ACCENT PC without these Drivers, a couple of changes must be

made to the ACCENT PC prior to installation.

There are a group of 3 Pins in the lower, left corner on the

synthesizer, close to the group of 8 Dip Switches.  A wire

connector, called a Jumper normally connects Pins 1 and 2.  This

Jumper should be moved to connect Pins 2 and 3.  Connect a standard

Serial cable from the Serial connector on your computer's Serial

Port to the Serial Connector on the ACCENT PC.  It will be

important to be aware of the Serial Port Number used for this

connection.  This was explained in Chapter 1.  If your computer has

only 1 Serial Port; it is normally assigned as COMM 1.

When using the ACCENT SA, or the ACCENT PC without the included

Drivers, it must be set to 9600 Baud, 8 Data Bits, 1 Stop Bit, and

No Parity.  The ACCENT is shipped with the DATA BITS set to 7.

Change Switch #8 so that Switch #4 is the only Switch in a

different position to set the DATA BITS to 8.

**APOLLO AND JUNO PREPARATION

The APOLLO synthesizers require different Pins to be used in the

Serial Cable to establish the Communications Rate (BAUD RATE)

required for your software.  Refer to the APOLLO Manual to verify

that your cable is designed for 9600 BAUD.

The DOLPHIN JUNO uses the same communications as the APOLLO 2.

Therefore, the Installation Screen will show that you are

installing for the APOLLO 2.

The APOLLO II is also supported as a Parallel device.  However, it must

be connected to LPT1.

The APOLLO does not process Numbers in terms of hundreds and

thousands.  As a result; the NUMBERS; ON/OFF command discussed in

Chapter 8 is not applicable.  The APOLLO will read all numeric text

a digit at a time.

Your SYNTHA-VOICE Product uses INDEX MARKERS to identify words in

Block Upper Case if this Sensor is enabled, as discussed earlier in

this Manual.  The Marker Processing on the APOLLO 1 only allows one

tone to be heard per line when using the BLOCK CASE SENSOR with the

Line-reading commands.  If a tone is heard when this feature is

active, it is informing you that this line contains a Block Upper

Case word.

Index Marking is not supported on the APOLLO II as a Parallel device.

The Volume Control for the APOLLO is located on the unit itself.

Although this is also the case on the APOLLO 2; this newer version

allows the Volume to be set and controlled from the keyboard as

well.

Index Markers do not work especially well on the DOLPHIN JUNO.  As

a result the READ ALL and UPPER CASE SENSOR Commands should not be

used with the JUNO.

**ARTIC TRANSPORT PREPARATION

This Synthesizer is capable of emulating the ACCENT SA Synthesizer.

Make sure that you are using TRANSPORT Firmware Revision 1.5 or later.

When the Install Menu appears, the recognized Synthesizer will be the

ACCENT SA.  This is fine.

**BRAILLE 'N SPEAK PREPARATION

Your SYNTHA-VOICE Product is compatible with the BRAILLE 'N SPEAK

and BRAILLE 'N SPEAK 640.  The 640 Version supports Index Markers.

If you are not sure if your BRAILLE 'N SPEAK supports Index

Markers, contact BLAZIE ENGINEERING, or your BRAILLE 'N SPEAK

Dealer.

Both models of BRAILLE 'N SPEAK are supported as Generic

Synthesizers.  There are several steps that are required by this

device in order to use it as a Synthesizer.

Connect the small end of the cable supplied with your device into

the left side of the BRAILLE 'N SPEAK.  You may need to connect the

Gender Connector accessory to make the cable compatible with your

computer.  Connect this cable into Serial Port COM1 or COM2.

Reset your BRAILLE 'N SPEAK by pressing all of the seven keys on

the device at the same time.  When you are prompted to confirm this

command, press all of the keys again.

You are going to use the default Communication settings for the

BRAILLE 'N SPEAK.  To set the Handshaking to Hardware, begin by

executing the "P" Chord.  Type the letter "H" for Handshaking, and

then "H" again to set the Handshaking to Hardware.

Press Key Number 3, 4, 5, and the SPACEBAR  together to enter the

Speech Settings Mode.  Press all 6 Braille Keys together to enter

Speech Box Mode.  Now type an "E" Chord to Exit from the Setup.

You are now ready to Install your SYNTHA-VOICE Product.  Refer to

the Chapter at the beginning of this manual for instructions on the

"INSTALL G" (Generic) Installation Command Option.  As the

Installation Menu appears, it will be spoken through your

synthesizer.  Use the ARROW DOWN to move to the correct model of

BRAILLE N' SPEAK, and press ENTER.

Your SYNTHA-VOICE Product can be used with a DOS based

Telecommunications program without any additional steps or commands

being set.  However, if you plan to transfer files from DOS; there

is one step to be executed before beginning a file transfer.  This

command is used to temporarily disable all of the SYNTHA-VOICE

Product "monitoring" capabilities which may interfere with file

transfers with the BRAILLE N' SPEAK.

Execute the SYNTHA-VOICE command used to ENABLE A MONITOR AREA; as

discussed earlier in this manual.  At the prompt; type "999", and

press ENTER.  Since this is an invalid MONITOR AREA Reference

Number; you will be prompted:

"DISABLE ALL MONITORING TEMPORARILY? NO"

Press "Y" to temporarily disable all monitoring of the screen.

Now, you can commence a file transfer between your computer and the

BRAILLE N' SPEAK without any side effects.

To re-enable the monitoring feature, you can repeat the above

process; answering the question by typing "N".  In addition;

loading any SYNTHA-VOICE Product Configuration Files; either

manually, or as the result of AUTO-LOAD will internally re-activate

all of the normal monitoring activities.

**DECTALK VERSION 2.0 PREPARATION

If the original DECTALK Version 2.0 External synthesizer is used;

the following process is required in order to set the

Communications Rate (BAUD RATE to 9600).

1.        Connect the Serial Interface Cable to the connector

closest to the center of the rear panel on the

synthesizer.

2.        Insert the SYNTHA-VOICE MASTER disk into the Floppy

Drive.

3.        Connect your DECTALK synthesizer to Serial Communications

Port 1.

4.        From the MASTER Disk; type DEC 9600; and press ENTER.

5.        Change the cable on the rear panel of the synthesizer to

the port closest to the edge of the synthesizer's rear

panel.

**DECTALK EXPRESS PREPARATION

Although the DECTALK EXPRESS includes Driver files to allow your

program to communicate with the synthesizer, these Drivers are not

required with your SYNTHA-VOICE Product.  Therefore, do not load

these Drivers into memory before Installing or loading your SYNTHA-

VOICE Product.  Communicating directly with the synthesizer will

require less memory, and will not have any limitations.

**DECTALK PC PREPARATION

Your DECTALK PC includes a disk containing the Drivers for the

synthesizer.  The revision Number for these Drivers must not be

older than Revision 4.0E.  These Drivers must be installed and

loaded into memory before installing your SYNTHA-VOICE Product.

These Drivers must also be loaded each day before loading your

SYNTHA-VOICE Product.

Be sure that your DECTALK PC Drivers are not loaded from within

your AUTOEXEC.BAT file through the use of the "CALL DT" statement.

Loading memory resident programs through the use of the "CALL"

statement will make your system unstable.  If you have a Serial

Device connected to COMM2, you should edit the DECTALK PC Driver

statements to remove "-C" from the "DT_DRIV statement once you have

installed your SYNTHA-VOICE Product.  This Command Option is

required during the Installation process to provide proper Speech

Output, but can cause Interrupt conflicts with another Serial

Device connected to Comm 2 during normal operation.

**INFOVOX PREPARATION

INFOVOX ROM Revision 3.4, 3.5, 3.6, 3.61, 3.70, and 3.71 have been

tested for compatibility with your SYNTHA-VOICE Product.  It is

compatible with Driver Revisions 1.00, 1.01, and 1.20.  Be sure to

load the INFOVOX Drivers into memory using the "/I", "/C2" and any

other necessary INFOVOX Options as described in your INFOVOX Manual

prior to Installing or loading your SYNTHA-VOICE Product.

Be sure to use the Serial Cable included with your External

synthesizer.  If a cable was not included, you should use a cable

that is designed to meet the specifications provided in your

INFOVOX Manual.

The normal BAUD RATE for the INFOVOX is 4800 BAUD.  Your SYNTHA-

VOICE Product requires this to be set to 9600 BAUD.

INFOVOX documents the steps required for setting the BAUD RATE.  In

the following steps, replace "#" with "1" or "2" to represent the

Serial Port used by your synthesizer.  The reference to "^Z" refers

to holding the CONTROL key, and typing the Letter "Z".  Also notice

that the final command requires the letters "iv" to be typed in

Lower Case.

Connect the correct Serial Cable to Serial Port 1 or 2.  From the

DOS Prompt, type the following commands:

MODE COM#,96,N,8,1 (ENTER)

COPY CON COM# (ENTER)

iv^Z

Before pressing ENTER for this final Command, hold the button on

the rear panel of the INFOVOX.  Turn your synthesizer; ON.  Press

ENTER.  Keep holding the button on the rear panel of the INFOVOX

until you see the DOS Prompt on your screen.

The INFOVOX will normally memorize your new BAUD RATE.  However, if

your synthesizer is not connected to a Power Supply for an extended

period, these changes will be lost.  If your INFOVOX is not

responding properly, it is possible that the BAUD RATE has been

changed back to the original setting.

**KEYNOTE PREPARATION

Be sure to connect the AC Adaptor to the KEYNOTE SA before

beginning the SYNTHA-VOICE Installation program.  This step should

be completed even if your synthesizer battery is fully charged.

Otherwise, it is possible that the synthesizer will not be

recognized properly.

When using the KEYNOTE SE synthesizer on a Toshiba Laptop, this

synthesizer is recognized as COM1.  Use the TEST3 Utility provided

with your Toshiba to verify that the first Line is set to COM2 and

the second Line is set to COM1.

If you connect a device to the Serial Port on the Toshiba, it will

be recognized as COM1, even though the KEYNOTE is connected to this

Port as well.  If your device requires an Address (such as a

modem), it must be assigned to 2F8.

When using the KEYNOTE VC synthesizer, be sure to load the KEYNOTE

Drivers prior to installing your SYNTHA-VOICE Product.  These

Drivers must also be loaded each day prior to loading your SYNTHA-

VOICE Program.

**KURZWEIL PERSONAL READER AND READING EDGE PREPARATION

To communicate with the KURZWEIL Personal Reader; the

Communications for the voice must be set properly on this device.

The following settings must be selected on the KPR:

9600 BAUD

8 DATA BITS

1 STOP BIT

NO PARITY

XON/XOFF HANDSHAKING

DECTALK or VOICE INPUT MODE

COMMUNICATIONS PORT 1 or 2

Be sure to use the Serial Cable supplied with your KPR system.  If

a straight through 25 Pin cable is not recognizing your synthesizer

as a DECTALK during the Installation process, try the following

Serial Cable configuration:

Pin 1 to Pin 1.

Pin 2 and 3 crossed.

Pin 6 and 20 crossed.

Pin 7 to Pin 7.

Your SYNTHA-VOICE Product also supports the KURZWEIL READING EDGE.

Be sure to connect the READING EDGE to Serial Communications Port

1 or 2 by using the Serial Interface Cable supplied with your

READING EDGE.  Use the "SELECT INPUT" button on the READING EDGE to

set the system to "RECEIVE FROM SCREEN READER".  The default

Communications settings on the READING EDGE are already set to

communicate properly with your SYNTHA-VOICE Product.  The Volume

and Rate of Speech are controlled by the knobs on the READING EDGE,

while the Pitch and Voice Selections are controlled by your SYNTHA-

VOICE Product.

**MULTIVOICE PREPARATION

Be sure that your MULTIVOICE is configured for 9600 BAUD, 8 DATA

BITS, 1 STOP BIT, NO PARITY, XON/XOFF Handshaking.

Your SYNTHA-VOICE Product can be used with the 9 to 9 Pin Serial

Cable supplied with the MULTIVOICE.  If your computer has a 9 Pin

Serial Connector, simply connect the 9 to 9 Pin Cable supplied with

the MULTIVOICE to your computer.  If your computer only has a 25

Pin Serial Connector, you must use a standard 9 to 25 Pin Adaptor

Cable.

**SMARTALK PREPARATION

The SMARTALK Program must be loaded before installing or running

your SYNTHA-VOICE Program.  The correct Command is:

S_TALK F3

**SOUNDING BOARD AND SPEAK-OUT PREPARATION

Your SYNTHA-VOICE Product is compatible with Version 2.4C or later

of the SOUNDING BOARD Drivers.  These Drivers must be installed,

and present in memory as described in your SOUNDING BOARD Manual

prior to installing your SYNTHA-VOICE Product.

The SPEAK-OUT Synthesizer is only supported as a Serial Interface

device.  Do not use the Drivers included with this device when

using your SYNTHA-VOICE Product.

**TELEVOX PSOLA PREPARATION

Your SYNTHA-VOICE Product has been tested with Version 2.4 of the

TELEVOX PSOLA Drivers.  Load this file followed by the TELEVOX

Dictionary File before installing, or running your SYNTHA-VOICE

Product.  Otherwise, the program will not be able to establish

proper communications with this Serial Synthesizer.  For example:

PSOLA A=348 V=65 (ENTER)  PARLEKDG (ENTER)

GENERIC SYNTHESIZERS

Your SYNTHA-VOICE Product supports several Generic Synthesizers.

A Generic Synthesizer is one that does not support User Definable

Index Markers.

Files or "Tables" containing the correct Communications Codes for

some Generic Synthesizers are contained on your MASTER DISK.  They

have the File Extension ".TBL".  They include:

BNS.TBL             BRAILLE 'N SPEAK.

BNS640.TBL          BRAILLE 'N SPEAK 640.

ECHO.TBL       The original ECHO Serial Synthesizer.

Since a Generic Synthesizer cannot support multiple User Definable

Index Markers, they will have a few limitations.  These include:

1. No Cursor synchronization when using the READ ALL feature in

DOS.

2. READ ALL is not available in WINDOWS.

3. BLOCK UPPER CASE SENSOR is not available.

4. DUAL VOICE MODE is not available.

5. Executing a Command in the CONTROL PANEL will always exit the

MENU.

6. The WAIT FOR VOICE Option in Macro's is always 1 second.

7. Do not press a key for 2 seconds after Loading a Configuration.

**TEXT ASSIST PREPARATION

WINDOW BRIDGE is compatible with the "Text Assist" Product from

CREATIVE LABS.  This Product does not run on all SOUND BLASTER

Cards.  The AWE32 is the most common Card for use with Text Assist.

Test your SOUND BLASTER with TEXT ASSIST to verify that this Card

works with TEXT ASSIST before attempting to use WINDOW BRIDGE.

If TEXT ASSIST does not run with your SOUND BLASTER without WINDOW

BRIDGE, adding WINDOW BRIDGE is not going to change this.

TEXT ASSIST is a WINDOWS Application.  The WINDOW BRIDGE Install is a

Program that must be run from DOS, not WINDOWS.  Therefore, it is not

possible to inter-actively install WINDOW BRIDGE for TEXT ASSIST with full

Speech output.  However, it is possible to perform an automatic

Installation without the need to respond to Prompts when installing

for TEXT ASSIST.

If you are using a Demo Version of WINDOW BRIDGE, the correct Install

Command is:

INSTALL 0 TA DRIVE:

If this is a Licensed Version of WINDOW BRIDGE, the Install Command

is as follows:

INSTALL 0 TA DRIVE: U:User Name

"0" in the previous Commands specifies that you are using an Internal

Synthesizer.

"TA" in the previous Commands specifies the Synthesizer as TEXT ASSIST.

"DRIVE:" should be replaced with C:, etc..  Since you cannot install

WINDOW BRIDGE to a floppy Drive, A: and B: are not supported.

Licensed Users must follow the DRIVE: with a SPACE.

Now, type U: followed by the Licensed User Mame that will appear as

the Registered User.

Do not use an alias as a User Name, as this will affect your eligibility

to Customer Support and Upgrades.

The use of U:User Name should not be used on the Demo Version of

WINDOW BRIDGE.

Here is an example of how to install the Demo to Drive C:

INSTALL 0 TA C:

Here is how the Licensed Version would be installed to Drive C:

INSTALL 0 TA C: U:Syntha-voice Computers Inc.

Of course, you would replace "Syntha-voice Computers Inc." with your Name.

When you hear a series of tones that raise in pitch followed by a pause,

then a double tone, this indicates that you should insert the second

disk.  Once inserted, press the SPACEBAR.

When you hear a series of tones that raise in pitch followed by a pause,

then a single tone, this indicates that you should insert the first

disk.  Once inserted, press the SPACEBAR.

At the completion of the Install Program, you will hear a series of

tones that decrease in sound.  Once this is heard, wait for about

10 seconds.  Remove the Disk from the Drive, and Reset your PC.

The previous Install Commands for WINDOW BRIDGE with TEXT ASSIST will

create a BRIDGE Directory or BRDEMO Directory on the specified Drive.

All Sample Configuration Files and an ASCII Manual will be installed

as well.

No modifications will be made to AUTOEXEC.BAT.

Rather, a file called SWB.BAT will be created to load WINDOW BRIDGE.

This should be executed before loading WINDOWS if you would like to have

WINDOW BRIDGE communicate with TEXT ASSIST.

Once WINDOW BRIDGE has been installed, make sure that Text Assist

has been installed as well.  WINDOW BRIDGE will automatically

detect the presence of Text Assist.  Your SOUND BLASTER will speak

as you load WINDOWS.

WINDOW BRIDGE can be used with the Text Assist Program in WIN 3.1,

3.11, and WINDOWS 95.  You can also access the DOS Prompt, and run

DOS Applications from within WINDOWS with WINDOW BRIDGE and Text

Assist.

The general responsiveness of Text Assist in a DOS environment is

not as high as WINDOWS itself.  This is especially noticeable

during typing.  It is recommended that WORD MODE (SHIFT-CONTROL-W)

is used with Text Assist within DOS to improve responsiveness.  A

second consideration in DOS is that Text Assist may stop speaking

for a moment.  However, it will resume.

WINDOW BRIDGE does not use any additional third party Communications

Drivers when communicating with TEXT ASSIST.  For example, the

SSIL Drivers from ARKENSTONE.

TEXT ASSIST wants to spell words ending in "...".  There is a

Dictionary File in your CONFIG Sub-Directory called TA.DIC.

Add the contents of this file to your other .DIC files in the CONFIG

Sub-Directory to avoid this spelling in Program Manager and the

WINDOW BRIDGE CONTROL PANEL.

If you encounter Menus in WINDOWS that spell the Option, follow the

format shown in the TA.DIC file to add additional Words to the desired

User Dictionary (.DIC) files.