DSKWATCH.COM - A program that reports on all disk i/o errors.

Notes by Bernie Eisenberg, 213/391-0487. 9/15/85.

This program was published in PC Magazine.  The assembly language

version was used, but it did not assemble correctly with the Microsoft

MASM (presumably, it did assemble with IBM's macro assembler).  So, I

made the corrections and this is the version that you have now.

To use it, just put the following line in your autoexec.bat file:

dskwatch

If you have set your path correctly, or if the .com file resides in the

root directory, the program will load and become memory resident.

The program will report each and every i/o error that the bios

encounters, while not interfering with normal DOS error reporting.  It

will report on errors in the upper right corner of your screen.  The

following errors are reported:

Message	      What the Message Means

-------	      ----------------------

1. No response	      drive probably empty

2. Failed seek

3. NEC error

4. Bad CRC seen       intermittant error; during FORMAT, the sector will

be marked 'bad' by DOS

5. DMA overrun	      NEC has to wait until the PC bus is available

6. Impos sector       NEC received request to i/o a non-existant sector

7. No addr mark       usually when NEC tries to read unformatted disk, or

when a foreign disk's tracks do not entirely coincide

8. W. protected       write protected disk

9. Err unknown	      bad error, since NEC doesn't even know what happened

NEC is the i/o controller chip used in the PC. Good Luck.