MSCAN 2.02 MANUAL SUPPLEMENT

============================

PAGE IV

Agent for North America: HAMVision NA, P.O. Box 5073, Salisbury, NC 28147-

7806, Tel/Fax: (704) 636 3308.

Agent for Hungary & neighbouring countries: KATE BT., 3529 Miskolc, Perczel M.

u. 45/A, Hungary, Tel: +36 46 36 56 43, Fax: +36 46 36 00 09.

PAGE 1

From now on MSCAN will always have the TX option installed. All parts in the

manual marked [TX] are now relevant, even if you used to have the RX version.

PAGE 2

A number of chipsets and the VESA/fast option have been added to the

configuration menu. Some video cards have a very slow VESA driver that slow

down MSCAN, or even prevent it from operating. This problem will be solved for

some video cards by choosing the VESA/fast option.

PAGE 3

If you have the Microscan version of MSCAN, you will be prompted for the IRQ

number of your com port. This enables MSCAN to operate on PCs with non-

standard IRQ numbers.

The 'Initial program mode' and 'Default path to picture directory' entries

are no longer available. If you quit the program, MSCAN now stores all

selected modes, the current program mode and the selected picture path.

PAGE 8

The calibrationfactor can now also be set in the new SSTV modes, in the same

way as in the FAX modes.

PAGE 10

In previous versions of MSCAN pictures that were loaded will be automatically

scaled so that they fit in the screen or box selected. Now the pictures will

also be enlarged in vertical direction so that it will fill the entire screen.

This auto fill will only be done when the picture will not be enlarged for

more than 25%, and only when the target is a screen. For boxes and in FAX

modes the auto fill will be supressed.

A visual directory has been added to the path command. After clicking on the

'path' button, click again (or press <Enter>) to enable this option. You can

locate the desired directory by clicking on the presented buttons. Each time

you click a directory the new subdirectories (if any) will be shown. Clicking

on a '..' button will place you one directory back. If you have found the

desired directory press the 'ok' indicator to accept, or press your right

mousebutton to abort.

PAGE 11+12

Loading several images in alphabetical order is now much easier. First load a

picture like you are used to. Now click on the 'next' button and MSCAN will

load the next picture in the same screen or box you just selected. For easy

deleting you can alternate the next and delete commands. The next command is

available in both the diskload and autoload menus.

PAGE 12

The 'transbox' button has been added to the autoload and read menus. This

command will load a picture in a box, but will not place the black pixels.

This simple command offers many possibilities. You can now for example use

some external program to create a nice text or drawing, import it in MSCAN and

use the transbox command to place it on any picture. Or you can fill the black

characters of MSCAN with parts of a picture. The possibilities are endless and

can only be discovered by experimenting. Pixels that are not to be placed have

to be completely black (0 red, 0 green and 0 blue). Because JPEG can change

the intensity of colours you can not use this format to import pictures for

'transboxing' into MSCAN, instead use GIF. Some examples are supplied with the

registered version of MSCAN.

PAGE 13

MSCAN now supports both the old model (2 inputs) and new model (3 inputs) Iris

videodigitizer, and the original Video Blaster (full size card). For correct

operation of the Video Blaster, the following commands must have been executed

before starting MSCAN, you might want to place these commands in your

AUTOEXEC.BAT file:

vblstdrv                    (load resident driver)

vbset source 0              (select the source you want to use)

vbset range 640 400         (set resolution)

vbset window 0 0 640 400    (set window)

Since the hardware-overlay option of the Video Blaster will not operate in

truecolour modes, you do not have to install the flatcable between the Video

Blaster and the graphic adapter. MSCAN will auto-detect which videodigitizer

is installed, if both the Iris and Video Blaster are present the Iris will be

supported. Due to the design of the Video Blaster, the digitizing process will

distort the timing of MSCAN. It is therefore recommended to digitize pictures

only when not transmitting or receiving images.

MSCAN now also supports the ComputerEyes/RT model videodigitizer.

PAGE 14

By clicking on the 'display' button in the memwrite menu, the complete image

on your monitor will be saved to memory.

PAGE 15

The C indicator now also selects the colour for the 'draw' function in the

toolkit menu.

The scottie DX mode has been added to the new SSTV modes.

MSCAN now features auto mode setting for M1, M2, S1, S2 and DX modes by

detecting the transmitted VIS codes. MSCAN reports a auto mode setting in the

message indicator with the text 'VIS -> M1' etc. This message will be erased

when the MODE or OK indicator is clicked on.

Previous versions of MSCAN and the program JV-FAX both have faulty scottie

modes due to misinterpretation of its specifications. MSCAN now auto detects

which implementation it receives: the correct or faulty one. The faulty

scottie mode is indicated by a ' sign in the mode indicator. MSCAN now

transmits in the correct scottie modes.

If the display sequence is changed from e.g. 1X to X2, MSCAN will continue to

receive on screen #1 until the start of a next picture is detected. If you

want MSCAN to immediately switch to the other screen, and there wait for a new

picture, the first select XX and then the new setting.

PAGE 16

The VIS synchronization mode has been added. When this mode is selected, MSCAN

will only start receiving a picture if a valid VIS code is detected. The

setting of the synchronization mode is stored.

PAGE 22

The spectrum menu is now available for all MSCAN version.

PAGE 24

The 'esc' indicator has been replaced with 'ok', this command is used for the

new path option. Press this indicator twice (or press <alt-q> or <alt-x>) to

quit the program.

PAGE 26

If loading a picture from disk causes the receiving (or transmitted) image to

shift to the left, and no cache program is installed, your harddisk/BIOS

combination is the cause of the problem. This cannot be helped.

Loading a picture from disk can cause little noise dots to appear in the

received image, this is a result from using the Microscan interface. Using the

Multiscan interface will avoid this problem, and will give additional

improvements like sharper and cleaner images.