The purpose of this program is to allow the addition of an extra communications

port to IBM-family machines.  This is a programmable port addressed as com3: andlocated at address 3e8h.  It may be configured

using the TERM.EXE program.   Type TERM HELP for details.

The existing IBM-supplied single communications port may be used in this application if installed multi-function boards are

presently supplying COM1: and COM2:

MODS to ASYNC BOARD to SUPPORT COM3:

1.	Break trace A27 to U2-5

2.	Connect A27 to U6-12

3.	Connect U6-11 to U2-5

4.	Connect U6-13 to +5Vdc. (U6-14)

5.	Break trace from U15-6 to B25

6.	Connect U15-6 to B4.

Refer to IBM Technical Reference Personal Computer XT for above designations.

These wiring changes should only be undertaken by personnel experienced and

equipped with PC board tools and the techniques involved.

Install async3.sys in the config.sys file as DEVICE=ASYNC3.SYS

The new serial port will not appear in the port table located at Segment40:0

This modification may be used with any software that allows you to specify either COM3: or port 3E8h.

Direct inquiries to:

Art Shulman,

Box 850

Waterford, Ontario, Canada.

N0E 1Y0

Telephone (519) 443-4321 after 6:00 Pm. EST.

Files included: 	This file you are reading.

ASYNC3.SYS

TERM.EXE

EOF